3GPP TSG-RAN WG2#132	R2-2508948

Dallas, USA, Nov 17th – 21st 2025

Source: 	Fujitsu

Agenda item:	10.3.3.3

Title:	Discussion on Access Stratum Security

Document for:	Discussion and decision

# Introduction

The SID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

In this contribution, we would like to provide our views on access stratum security requirements and general security principles.

# Discussion

## Background

SA3 WG is the main group responsible for the security requirements and overall security framework for 3GPP specified systems. Nevertheless, access stratum security (i.e. control plane signalling using RRC messages and user plane data), the requirements and solutions are discussed within RAN2 and in consultation with SA3. Historically, this has been the working model for 3G, 4G LTE and 5G NR access stratum security. Overall security framework and requirements for 4G LTE system are captured in SA3 owned specifications TS 33.401 and for 5G NR system it is specified in TS 33.501. Further, in RAN2 specifications the access stratum security aspects are described in TS 36.300 for 4G LTE and UE security requirements in TS 36.331. Similarly, 5G NR access stratum security aspects are covered in TS 38.300 and UE security requirements in TS 38.331.

The security procedures include integrity protection and encryption. Integrity protection ensures the mutual authentication (i.e. verification of the authenticity) between the UE and gNodeB. Encryption ensures the communication between the UE and gNodeB is ciphered providing confidentiality between the communicating entities. The security procedures are performed independently at the Non-Access Stratum (NAS) and Access Stratum (AS) layer respectively. This means the AS security keys are cryptographically separated from the NAS security keys. Further, separate AS and NAS level Security Mode Command (SMC) procedures are applied. Access stratum refers to the communication between the UE and the gNodeB over the air interface (i.e. using the radio protocols). The Non-Access Stratum refers to the communication between the UE and the Core Network (CN) over the air interface but transparent to the gNodeB (i.e. using the CN protocols). We think this framework is also applied to 6G.

Proposal#1: Like LTE/NR, we assume the following security aspects for the 6GR system

- The AS security keys shall be cryptographically separated from the NAS security keys.

- Separate AS level Security Mode Command (SMC) procedures and NAS level SMC procedures shall be applicable between the gNodeB/UE and CN/UE respectively.

- Above security requirements can be confirmed with SA3

### Access Stratum Security Principles

As per [2], at RAN#109 the “Requirements for architecture and migration” were agreed to be captured in the TR 38.914 section 5.2. One of the requirements states “The 6G RAN architecture shall allow for control plane and user plane separation”. 5G NR supports the separation of control plane and user plane not only from protocol perspective but also from RAN architecture perspective. This means the control plane signalling (i.e. RRC and NAS messages) is mapped to SRB and the user data is mapped to DRB. Further, discussions on split RAN architecture i.e. CU-CP and CU-UP functions separation and CU-CP and DU split discussions are ongoing in RAN3.

Observation#1: The requirement “The 6G RAN architecture shall allow for control plane and user plane separation” is already captured in TR 38.914.

As mentioned above overall security requirements will be captured in SA3 specification. From RAN2 perspective we need to focus on access stratum security requirements. Like LTE/NR access stratum security requirements can be expected to be same for 6G system.

Therefore, we propose the following:

Proposal#2: The following security requirements related to user plane data for the 6G RAN

- The gNodeB and UE shall support encryption (ciphering) of user data (i.e. confidentiality protection) between the UE and the 6G gNodeB.

- The encryption (ciphering) of the user plane data (i.e. confidentiality protection) between the UE and the 6G gNodeB may be optional to use depending on compliance to local regulation.

- The gNodeB and UE shall support integrity protection of user data (i.e. authenticity verification) between the UE and the 6G gNodeB.

- The integrity protection of user data (i.e. authenticity verification) may be optional depending on the data rate supported by the UE.

Proposal#3: The following security requirements related to control plane signalling data for the 6G RAN

- The gNodeB and UE shall support encryption (ciphering) of Radio Resource Control (RRC) signalling (i.e. confidentiality protection) carried by the signalling radio bearers (SRBs).

- The encryption (i.e. ciphering or confidentiality protection) of the RRC-signalling and NAS-signalling may be optional to use depending on compliance to local regulation.

- The gNodeB and UE shall support integrity protection of the RRC-signalling (i.e. authenticity verification) carried by the SRBs.

- The integrity protection of the RRC-signalling (i.e. authenticity verification) is mandatory to use and there may be some exceptions for RRC-signalling during emergency calls

As per the RAN2#132 meeting agenda contribution on MAC CE/L2 security is not expected in this meeting. However, MAC CE/L2 security related aspects were discussed at RAN2#131bis meeting and subsequently an LS was sent to SA3 for clarifications [3]. Even though RAN2 wait for reply from SA3 on the security requirements for MAC CE, one can assume if SA3 respond with need of security of MAC CE security protection, these requirements would be like RRC signalling. This means integrity protection of the MAC CE is mandatory while the encryption (i.e. confidentially protection) is optional.

Observation#2: If SA3 responds with need of security protection for MAC CE then one can assume integrity protection of the MAC CE is mandatory while the encryption (i.e. confidentially protection) is optional.

In our understanding if any information in the RAN (whether user data or signalling data) needs to be security protected, then to avoid security vulnerability, the information should be protected either by the node which generates the information or by the node which is ingress point for the information in the RAN. We therefore propose to the following principle to be taken into consideration in RAN2 during the access stratum security studies.

Proposal#4: RAN2 to agree that “If any information in the RAN (whether user data or signalling data) needs to be security protected, then to avoid security vulnerability, the information should be security protected either by the node which generates the information or by the node which is ingress point for the information in the RAN.

### AS Security Key Derivation

We assume SA3 will study the KgNB key derivation function and the related input parameters. Further, SA3 will also specify the requirements on forward security and backward security applicable to KgNB key derivation like those defined for 5G and specified in TS 33.501. RAN2 need not spend time in these security requirements of KgNB derivation and follow the SA3 agreements in due course. However, RAN2 needs to discuss the access stratum security key derivation hierarchy (i.e. in what scenarios) the AS security keys (i.e. KgNB, Krrc_Int, Krrc_Enc, Kup_Int and Kup_Enc) are refreshed and how they are refreshed.

In LTE, due to connected mode mobility resulting in handover the security key is always refreshed by default. This results in reestablishment of PDCP, RLC and MAC reset. However, in NR, due to the distributed split architecture, connected mode mobility resulting in a handover did not warrant a security key refresh by default. Security key refresh was needed only if the PDCP termination point (i.e. the host of security function) changed, otherwise, it was not needed. As the NR user plane is modular and distributed, handover did not result into PDCP/RLC re-establishment by default but only when the PDCP termination point (i.e. security function host) changes. Further, when the PDCP COUNT is about to wrap around, since it is used as input to the encryption and integrity function, if key is not refreshed, it would result in keystream reuse issue and violate replay protection respectively. In other scenarios, if the Kamf is refreshed then AS security keys and NAS security keys need to be refreshed. AS security keys further changes during the RRC Reestablishment and RRC Resume procedures respectively.

Observation#3: AS security keys need to be refreshed if the PDCP termination point (i.e. security function host) changes due to connected mode mobility.

Observation#4: AS security keys need to be refreshed if the PDCP COUNT which is used as an input parameter in the encryption and the integrity function is about to wrap around.

Observation#5: If the root key (e.g. Kamf) is refreshed, then the AS and NAS security keys which are derived from Kamf need to be refreshed.

Observation#6: AS security keys changes during the RRC Reestablishment and RRC Resume procedures respectively.

Proposal#5: AS security keys shall be refreshed when the security function host in the radio protocol changes due to connected mode mobility or other reasons (e.g. RRC procedures like RRE, Resume).

Proposal#6: When the input parameters used in the encryption and the integrity function are expected to repeat then the AS security keys shall be refreshed.

### Security keys for split RRC design

We proposed the modular RRC design based on the split RRC architecture as described in our companion contribution [4] as shown in Figure 1. The benefits of the split RRC design are well articulated in [4]. This section focuses on the security aspects for the split RRC design.

Figure 1 - 6GR RRC Architecture

As mentioned in earlier sections the integrity protection of the RRC signalling is mandatory, so if the Auxiliary RRC or Assisting RRC in the DU generates the RRC Reconfiguration message then such messages shall be security protected. One simple way is to provision a PDCP-C instance in the DU which is a duplicate of the PDCP-C instance in CU-CP. Such modelling then follows that the Krrc_Int and Krrc_Enc derived from the KgNB by the CU-CP can be shared securely via the F1-C interface with the DU and there is no need to derive separate Krrc_Int and Krrc_Enc at the DU. After the F1 setup, the security algorithms and Krrc_Int/Krrc_Enc can be included in the UE Context Setup Request message to configure the PDCP-C in the DU. Such an approach is simple and secure without any security vulnerability. Another advantage of using the same Krrc_Int/Krrc_Enc for the security function hosted by the CU-CP PDCP-C instance and the security function hosted by the DU duplicate PDCP-C instance is that the intra-CU mobility (i.e. intra-DU and inter-DU in same CU) does not result into security key refresh as the security host function does not change during such mobility. Therefore, there is no data interruption to the L2 stack re-establishment. Apart from above mentioned advantages, the main advantage of the duplicate PDCP-C instances in CU-CP and DU and the usage of same Krrc_Int/Krrc_Enc is that there is no impact at the UE side for processing the RRC Reconfiguration messages generated by the CU-CP or DU. In other words, the DU is transparent to whether the RRC Reconfiguration message is generated by CU-CP or DU.

Observation#7: Split RRC design does not impose complex security requirements and the same Krrc_Int/Krrc_Enc can be configured securely for the duplicate PDCP-C in the DU. In our view there are no security vulnerabilities due to such split RRC design.

Observation#8: There is no impact at the UE side due to the split RRC design.

Proposal#7: In Split RRC design, the security function in the duplicate PDCP-C hosted in DU can be configured securely with the same RRC Krrc_Int and Krr_Enc.

Since the DU hosts the duplicate PDCP-C instance for security protection of the RRC Reconfiguration message generated by DU, one can argue how the PDCP COUNT is handled. The RRC Reconfiguration message generated by CU-CP is security protected, wherein the PDCP SN in the PDCP PDU of the message along with HFN which constitutes the PDCP COUNT is used as input parameter in the security function of the PDCP-C hosted in the CU-CP. The RRC Reconfiguration message is included in the RRC Container which is encapsulated in the F1 message i.e. DL RRC TRANSFER MESSAGE. The PDCP SN value or directly the 32-bit PDCP COUNT value can be included in the F1 message towards the DU. The DU updates the duplicate PDCP-C with the received PDCP SN value and PDCP COUNT value. If the DU generates the next RRC Reconfiguration message, then the PDCP SN is simply incremented by the DU and the next PDCP COUNT is used as input parameter by the security function of the PDCP-C instance hosted by the DU. After the DU sends the RRC Reconfiguration message to the UE, it sends a F1 message towards the CU-CP including the PDCP SN value or PDCP COUNT value. Subsequently, the CU-CP updates the PDCP-C instance with the received PDCP SN value and PDCP COUNT value. Since RRC Reconfiguration procedure is sequential, if consecutive RRC Reconfiguration messages are generated by CU-CP or DU, the respective node includes the PDCP SN or PDCP COUNT in the F1 message to update the duplicate PDCP-C.

Observation#9: PDCP SN and PDCP COUNT can be handled in a very simple manner in the Split RRC design by exchanging the used PDCP SN and/or PDCP COUNT by the CU-CP and DU respectively on the F1.

### Security requirement for System Information

In 5G NR, the system information (i.e. MSI and OSI) which is broadcasted is not security protected. There are concerns raised on the unprotected system information resulting in fake gNodeB wherein the UEs camping on cells with such fake gNodeB can result in denial-of-service attack.

In our companion contribution [5] on the system information provisioning, we have proposed the OSI is provided by default in connected mode in on-demand unicast manner. Once the UE is in connected mode the access stratum security is applied and if any OSI is provided to the UE in dedicated manner it is by default security protected. In this way, no additional security mechanism is required to protect the OSI.

Observation#10: If OSI is provided by default in connected mode in on-demand unicast manner, then security protection for the OSI comes for free.

Consequently, we need to study the security solution for the essential SI (i.e. MIB and SIB1) which will be broadcasted either periodically or in on-demand manner. RAN2 should liaison with SA3 as soon as possible to consult on the security requirements for the essential system information which is expected to be broadcasted.

We therefore propose the following

Proposal#8: RAN2 send LS to SA3 on the security requirement for essential system information which is expected to be broadcasted.

# Conclusion

In this paper we presented some views on access stratum security aspects and have come up with the following observation and proposals to be discussed and agreed:

Proposal#1: Like LTE/NR, we assume the following security aspects for the 6GR system

- The AS security keys shall be cryptographically separated from the NAS security keys.

- Separate AS level Security Mode Command (SMC) procedures and NAS level SMC procedures shall be applicable between the gNodeB/UE and CN/UE respectively.

- Above security requirements can be confirmed with SA3

Observation#1: The requirement “The 6G RAN architecture shall allow for control plane and user plane separation” is already captured in TR 38.914.

Proposal#2: The following security requirements related to user plane data for the 6G RAN

- The gNodeB and UE shall support encryption (ciphering) of user data (i.e. confidentiality protection) between the UE and the 6G gNodeB.

- The encryption (ciphering) of the user plane data (i.e. confidentiality protection) between the UE and the 6G gNodeB may be optional to use depending on compliance to local regulation.

- The gNodeB and UE shall support integrity protection of user data (i.e. authenticity verification) between the UE and the 6G gNodeB.

- The integrity protection of user data (i.e. authenticity verification) may be optional depending on the data rate supported by the UE.

Proposal#3: The following security requirements related to control plane signalling data for the 6G RAN

- The gNodeB and UE shall support encryption (ciphering) of Radio Resource Control (RRC) signalling (i.e. confidentiality protection) carried by the signalling radio bearers (SRBs).

- The encryption (i.e. ciphering or confidentiality protection) of the RRC-signalling and NAS-signalling may be optional to use depending on compliance to local regulation.

- The gNodeB and UE shall support integrity protection of the RRC-signalling (i.e. authenticity verification) carried by the SRBs.

- The integrity protection of the RRC-signalling (i.e. authenticity verification) is mandatory to use and there may be some exceptions for RRC-signalling during emergency calls

Observation#2: If SA3 responds with need of security protection for MAC CE then one can assume integrity protection of the MAC CE is mandatory to use while the encryption (i.e. confidentially protection) is optional to use.

Proposal#4: RAN2 to agree that “If any information in the RAN (whether user data or signalling data) needs to be security protected, then to avoid security vulnerability, the information should be security protected either by the node which generates the information or by the node which is ingress point for the information in the RAN.

Observation#3: AS security keys need to be refreshed if the PDCP termination point (i.e. security function host) changes due to connected mode mobility.

Observation#4: AS security keys need to be refreshed if the PDCP COUNT which is used as an input parameter in the encryption and the integrity function is about to wrap around.

Observation#5: If the root key (e.g. Kamf) is refreshed, then the AS and NAS security keys which are derived from Kamf need to be refreshed.

Observation#6: AS security keys changes during the RRC Reestablishment and RRC Resume procedures respectively.

Proposal#5: AS security keys shall be refreshed when the security function host in the radio protocol changes due to connected mode mobility or other reasons (e.g. RRC procedures like RRE, Resume).

Proposal#6: When the input parameters used in the encryption and the integrity function are expected to repeat then the AS security keys shall be refreshed.

Observation#7: Split RRC design does not impose complex security requirements and the same Krrc_Int/Krrc_Enc can be configured securely for the duplicate PDCP-C in the DU. In our view there are no security vulnerabilities due to such split RRC design.

Observation#8: There is no impact at the UE side due to the split RRC design.

Proposal#7: In Split RRC design, the security function in the duplicate PDCP-C hosted in DU can be configured securely with the same RRC Krrc_Int and Krr_Enc.

Observation#9: PDCP SN and PDCP COUNT can be handled in a very simple manner in the Split RRC design by exchanging the used PDCP SN and/or PDCP COUNT by the CU-CP and DU respectively on the F1.

Observation#10: If OSI is provided by default in connected mode in on-demand unicast manner, then security protection for the OSI comes for free.

Proposal#8: RAN2 send LS to SA3 on the security requirement for essential system information which is expected to be broadcasted

# References

- RP-252912, Revised SID: Study on 6G Radio.

- RP-252870, pCR for 38.914 on requirements for architecture and migration for 6G.

- R2-2507933, LS on Early Alignment on Access Stratum security aspects, Vodafone.

- R2-2508946, Discussion on RRC Structure and (re)configuration in 6G Fujitsu.

- R2-2508947, Discussion on Initial and System Access in 6G Fujitsu