3GPP TSG-RAN WG2 Meeting #132		       R2-2508958

Dallas, USA, Nov. 17th-21st

Agenda Item:	9.3.2

Source:	ASUSTeK

Title:	Discussion on RRM prediction

Document for:	Discussion and Decision

- Introduction

In WI for AI mobility, one objective is to specify the applicability report and inference configuration for RRM prediction. In this contribution, we’d like to discuss this procedure.

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. … Note 5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible.

- Discussion

In RAN2 #129bis, RAN2 made the following agreements, confirming that the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework:

Agreements - The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework - For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message. - FFS whether association ID is required and should provide motivation on per use case bases - As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework. - Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline. Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20. - Model transfer will not be discussed in this study item 7 For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring. FFS metrics per use case

In RAN2 #130, the following parameters are identified for the inference of different use cases:

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

In the RRM measurement framework, MeasId, MeasObject, and ReportConfig are used to configure the UE to perform RRM measurement. The UE is configured to perform measurement and reporting for a MeasObject when the MeasObject is included in a MeasId or is a ServingCellMO and there is at least one ReportConfig associated with a MeasId. The MeasObject indicates the target to measure (e.g., SSB, cell, frequency), and the ReportConfig indicates the condition and content to report (e.g., report quantity, trigger quantity).

In our opinion, the inference configuration for RRM prediction can be based on the MeasId, MeasObject, and/or ReportConfig. Therefore, some of the candidates for reporting applicability for RRM prediction could be MeasId, MeasObject, and/or ReportConfig.

However, we think MeasId and ReportConfig may not be suitable for reporting applicability for RRM prediction because these configurations may be associated with more than one functionalities. For example, in a measurement report based on a MeasId, measurement results for one or more frequencies and cells may be included, wherein the applicability for each frequency and cell may be different. Therefore, reporting applicability for RRM prediction based on MeasId may not be sultiable, as the applicability of a MeasId may need to consider both serving cell and neighbouring applicability, a serving ccell being inapplicable may cause all MeasId to be inapplicable. Additionally, the ServingCellMO may not be associated with a MeasId. For another example, a ReportConfig may be associated with one or more MeasObject of different frequencies, cells, and applicabilities. This leaves us with MeasObject as the only candidate left.

Proposal 1: The inference configuration for RRM prediction can be identified by a MeasObject.

The steps for applicability reporting used in AI/ML PHY may be adopted as follows, serving as a baseline for discussion:

Step 1 and 2: Capability exchange.

Step 3a (for option A): The UE receives one or more MeasObject for inference. FFS on the parameters and container/where to put the parameters (e.g., report/trigger quantity, PW, skipping pattern, frequency).

Step 3b (for option B): The UE receives one or more sets of inference related parameters. FFS on the parameters and container (e.g., applicability set, MeasObject).

Step 4: The UE reports applicability based on the received one or more MeasObject for inference and sets of inference related parameters. FFS on how to activate/deactivate the inference configuration.

Step 5: The NW may provide further reconfiguration based on the received applicability report.

Proposal 2: Use the following steps for applicability reporting for RRM prediction as a starting point:

Step 1 and 2: Capability exchange.

Step 3a (for option A): The UE receives one or more MeasObject for inference. FFS on the parameters and container/where to put the parameters (e.g., report/trigger quantity, PW, skipping pattern, frequency).

Step 3b (for option B): The UE receives one or more sets of inference related parameters. FFS on the parameters and container (e.g., applicability set, MeasObject).

Step 4: The UE reports applicability based on the received one or more MeasObject for inference and sets of inference related parameters. FFS on how to activate/deactivate the inference configuration.

Step 5: The NW may provide further reconfiguration based on the received applicability report.

After the inference configuration is activated (i.e., the UE is configured to perform inference), one issue is regarding which measurement results should the UE report. For example, when performing temporal prediction case A or inter-frequency prediction, the UE may have actually measured results and predicted results available. But when performing temporal prediction case B, the actually measured results for a time instance may be aging and may not be useful for the NW. In our opinion, since different use cases may have different characteristics, and whether the information is useful depends on the NW implementation, one way forward is for the NW to configure the UE which type or types of measurement result to report/trigger an event.

Proposal 3: The NW may configure the UE which type or types of measurement result (e.g., actual and/or predicted rsrp) to report measurement results and/or trigger an event.

- Conclusion

Proposal 1: The inference configuration for RRM prediction may becan be identified by a MeasIdMeasObject.

Proposal 2: Use the following steps for applicability reporting for RRM prediction as a starting point:

Step 1 and 2: Capability exchange.

Step 3a (for option A): The UE receives one or more MeasObject for inference. FFS on the parameters and container/where to put the parameters (e.g., report/trigger quantity, PW, skipping pattern, frequency).

Step 3b (for option B): The UE receives one or more sets of inference related parameters. FFS on the parameters and container (e.g., applicability set, MeasObject).

Step 4: The UE reports applicability based on the received one or more MeasObject for inference and sets of inference related parameters. FFS on how to activate/deactivate the inference configuration.

Step 5: The NW may provide further reconfiguration based on the received applicability report.

Proposal 3: The NW may configure the UE which type or types of measurement result (e.g., actual and/or predicted rsrp) to report measurement results and/or trigger an event.

References

[1] RP-252899

[2] Meeting Report of RAN2 #129bis

[3] Meeting Report of RAN2 #130

Annex