3GPP TSG-RAN WG2 Meeting #132	R2-2509008

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.3.2.3

Source:	Panasonic

Title: 	Considerations for 6GR system access and carrier aggregation

Document for:	Discussion

# Introduction

RAN2 has started studying 6GR protocol based on the RAN plenary SID[1] and control plane architecture is one of the aspects to be studied.

Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

This document discusses some key aspects to be considered in 6GR studies for control plane, one specific to system information security and identification of fake base stations, and another related to spectrum aggregation and multi carrier deployment scenarios, using existing NR control plane design as the baseline.

# Discussion

## Securing system information

Authentication procedure for mutual verification between UE and gNB is one of the essential steps to ensure that the connection establishment is secured and not spoofed from some fake base stations. Despite having secured connection or verification procedure, the vulnerabilities to a security breach can be majorly caused by replay attacks. Where the authentication or verification messages are recorded to be replayed and spoofed at a later point in time, while the lack of freshness to generate newer authentication or verification parameters is exploited by the attacker to breach security.

In 5G NR, UE while receiving system information broadcasted from gNBs, initiates authentication procedure with the network and only then the user data or essential information is transmitted over the air interface. Also, the encryption method to convert plain text into ciphered text provides full security of the transmitted data. This ensures mutual authentication from the perspective of both UE side verification of the gNB (or the network) or the network side verification of the UE. This applied to both connected mode and inactive mode, whereas in inactive mode the AS and NAS security context established during connected mode will be retained for integrity and security.

Considering the existing methods in 5G NR for mutual authentication and verification, the next question would be the need for verification of the broadcast information transmitted by the base station, which is not protected or cannot be verified until the legacy authentication steps are completed (i.e. not until the secured bearers for communication is established). The following aspects should be considered in RAN2 to further study the requirements for security of system information, or to identify fake base stations earlier while receiving broadcasted information.

- How to protect sensitive information in broadcast messages?

System information broadcasted to UEs to perform initial access or to convey useful information to the UEs can be easily replayed by fake base stations to spook UEs and enforce executing malicious operations. Furthermore, even if the broadcasted information is encrypted (or security protected) to conceal such sensitive information, it may not be very difficult for the attacker to record and replay the original ciphered text even though the actual information may not be revealed. Hence directly concealing the sensitive information present in system information may not completely resolve security vulnerabilities.

And such vulnerabilities may be crucial in case of emergency calls or PWS/ETWS services provided to the UE, where the UE or the network (i.e AMF) bypasses authentication procedure to establish quick connection without delays caused due to verification steps.

Observation 1: Sensitive information in broadcasted messages can be vulnerable to security threats, where the information/ pattern in such messages can be replayed/ repeated to spoof UEs. Causing vulnerabilities to services where authentication and verification steps are especially bypassed (like emergency calls, PWS or ETWS).

Such sensitive information, example network related configurations or the early radio configurations sent in SIBs etc, can be shared to UE only after authentication and verification step are completed. For example, using on demand SIBs shared when the connection with the UE is established. Here the assumption is the messages can be encrypted/ ciphered and possibly integrity protected using the established AS security context.

Proposal 1: RAN2 to study if the broadcast information (e.g. SIBs, PWS/ETWS messages) includes any sensitive information to cause security vulnerabilities where it needs further protection.

Proposal 2: To study methods to protect the broadcast information (including e.g. SIBs, PWS/ ETWS messages) using one of the following options.

Option 1: Using on-demand like SI messages to send sensitive information after authentication/ verification is complete. FFS to identify all the sensitive information in the NR SI messages and the impact of delayed transmission of such information util the verification is completed.

Option 2: Conceal or encrypt SI messages before broadcasting onto the air interface. FFS the feasibility of encrypting the first message received by UE, and to involve SA3 to investigate the feasibility.

- How to verify broadcast messages?

While receiving broadcast information, in the current NR design UE first initiates authentication procedure with the NAS layer and eventually verification of the AS layer is also performed. This verification structure could be sufficient to transmit regular traffic in UL or DL. But some of the SI messages are specifically meant to bypass the authentication procedure to achieve faster connection, e.g. during emergencies.

SA3 has worked on the aspect of protecting the SI information and the identified security concerns related to broadcasted messages, one such issue and its requirements have been reported in TR 33.809. In 5G NR the solutions were not adopted due to complexity of modifying the signalling aspects of the SI transmission and system access procedures. But in 6GR we can study again and see if this issue needs to be addressed.

Observation 2: Concern for security of SI has been studied before but may have to consider for further study about the additional verification steps executed while receiving SI, which could introduce delays before the required service could start.

Proposal 3: RAN2 to study whether the threat from false base station needs be address by including authentication/ verification solutions to SI information. FFS the feasibility of authentication procedure to the first message received by UE and impact to the delays cause due to additional verification steps (e.g. in case of emergency services).

## Spectrum aggregation

Carrier Aggregation (CA) has been used in 5G NR to combine multiple frequency carriers into a single logical channel and deliver higher bandwidth and improved performance in terms of throughput and coverage. It was introduced in 5G NR to address several challenges like fragmented spectrum allocations due to non-availability of contiguous spectrum or the requirement to combine spectrum across multiple bands.

In RAN2#131bis meeting in Prague, following agreements were made to study the CA requirements for 6GR.

Agreements on Spectrum aggregation

- 1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

- 2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

One of the principles of CA in 5G NR, the configuration for multiple carrier components (CC) is performed using RRC signalling to configure one Primary Cell (PCell) and additional Secondary Cells (SCell), based on the capability information of supported multiple carrier bands at UE. And further additions or modification to CC could be handled using RRC signalling or by lower layers to activate/deactivate SCells using MAC CE commands, giving more flexibility to adapt the number of active CC based on the dynamic load or the performance requirements at each carrier.

Although such configuration of PCell and SCell would still be required in 6GR, multiple frequencies within the same cell needs to be further explored. Each frequency can be deployed by different TRP or DU but to be served by the same base station or the same CU. Compared with the configurations of the present CA design, to setup PCell and SCells using RRC signalling, the dynamic activation of each frequency can be further explored to enable faster activation (e.g. at UE side) tailing to the new requirements for improved throughput performance or coverage in 6GR. Such quicker adaptation is important also from network energy saving perspective. The handling of each frequency is the object within a cell, which could be a BWP.

Proposal 4: RAN2 to study single cell multi carrier aggregation to enable simplified configuration and dynamic and faster activation/ deactivation in 6GR, based on the co-located or non-co-located deployment scenarios of the cells.

# Conclusions

Securing system access

Observation 1: Sensitive information in broadcasted messages can be vulnerable to security threats, where the information/ pattern in such messages can be replayed/ repeated to spoof UEs. Causing vulnerabilities to services where authentication and verification steps are especially bypassed (like emergency calls, PWS or ETWS).

Proposal 1: RAN2 to study if the broadcast information (e.g. SIBs, PWS/ETWS messages) includes any sensitive information to cause security vulnerabilities where it needs further protection.

Proposal 2: To study methods to protect the broadcast information (including e.g. SIBs, PWS/ ETWS messages) using one of the following options.

Option 1: Using on-demand like SI messages to send sensitive information after authentication/ verification is complete. FFS to identify all the sensitive information in the NR SI messages and the impact of delayed transmission of such information util the verification is completed.

Option 2: Conceal or encrypt SI messages before broadcasting onto the air interface. FFS the feasibility of encrypting the first message received by UE, and to involve SA3 to investigate the feasibility.

Observation 2: Concern for security of SI has been studied before but may have to consider for further study about the additional verification steps executed while receiving SI, which could introduce delays before the required service could start.

Proposal 3: RAN2 to study whether the threat from false base station needs be address by including authentication/ verification solutions to SI information. FFS the feasibility of authentication procedure to the first message received by UE and impact to the delays cause due to additional verification steps (e.g. in case of emergency services).

Spectrum aggregation

Proposal 4: RAN2 to study single cell multi carrier aggregation to enable simplified configuration and dynamic and faster activation/ deactivation in 6GR, based on the co-located or non-co-located deployment scenarios of the cells.

# References

[1]		RP-251881; New SID: Study on 6G Radio.