3GPP TSG-RAN WG2 Meeting #132	R2-2509011

Dallas, USA, Nov. 17th – 21st, 2025

Agenda item:	8.1.3

Source:	Samsung

Title:	Discussion on [S042], [S046], [S052], [S053], [S054]

Document for:	Discussion & Decision

Introduction

This document discusses on remaining issues for NW-side data collection (i.e., NSDC) for Rel-19 AI/ML air interface WI.

Discussion

S042

According to current text, UE stores NSDC (NW-side data collection) configuration and corresponding UAI configuration in UE Inactive AS Context, “before” they are released. So, they would be restored during RRCResume procedure, which is not the intention of RAN2 agreement.

#### 5.3.8.3 Reception of the RRCRelease by the UE 2> if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1: <…> 2> else: 3> store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the ncr-FwdConfig (if configured), the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for: - parameters within ReconfigurationWithSync of the PCell; - parameters within ReconfigurationWithSync of the NR PSCell, if configured; - parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured; - servingCellConfigCommonSIB; - sl-L2RelayUE-Config, if configured; - sl-L2RemoteUE-Config, if configured; - aerial-Config, if configured; - cellDTX-DRX-Config, if configured; 2> release CSI-LoggedMeasurementConfig, if configured; 2> release loggedDataCollectionAssistanceConfig, if configured; 2> discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

There are two options to address this issue.

- Option 1. The same proposed text is captured before UE stores them in the UE Inactive AS Context

#### 5.3.8.3 Reception of the RRCRelease by the UE 2> if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1: <…> 2> else: 3> release CSI-LoggedMeasurementConfig, if configured; 3> release loggedDataCollectionAssistanceConfig, if configured; 3> discard the logged measurement entries included in VarCSI-LogMeasReport, if any; 3> store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the ncr-FwdConfig (if configured), the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for: - parameters within ReconfigurationWithSync of the PCell; - parameters within ReconfigurationWithSync of the NR PSCell, if configured; - parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured; - servingCellConfigCommonSIB; - sl-L2RelayUE-Config, if configured; - sl-L2RemoteUE-Config, if configured; - aerial-Config, if configured; - cellDTX-DRX-Config, if configured; 2> release CSI-LoggedMeasurementConfig, if configured; 2> release loggedDataCollectionAssistanceConfig, if configured; 2> discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

- Option 2. UE stores both configurations in UE Inactive AS Context, and releases them upon RRC resume initiation and discards logged data together.

| #### 5.3.8.3 Reception of the RRCRelease by the UE 2> if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1: 2> else: 3> store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the ncr-FwdConfig (if configured), the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for: - parameters within ReconfigurationWithSync of the PCell; - parameters within ReconfigurationWithSync of the NR PSCell, if configured; - parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured; - servingCellConfigCommonSIB; - sl-L2RelayUE-Config, if configured; - sl-L2RemoteUE-Config, if configured; - aerial-Config, if configured; - cellDTX-DRX-Config, if configured; 2> release CSI-LoggedMeasurementConfig, if configured; 2> release loggedDataCollectionAssistanceConfig, if configured; 2> discard the logged measurement entries included in VarCSI-LogMeasReport, if any; ### 5.3.13 RRC connection resume #### 5.3.13.2 Initiation Upon initiation of the procedure, the UE shall: 1> if the UE is in NE-DC or NR-DC: 2> if the UE does not support maintaining SCG configuration upon connection resumption: 3> release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored; 1> if the UE does not support maintaining the MCG SCell configurations upon connection resumption: 2> release the MCG SCell(s) from the UE Inactive AS context, if stored; <…> 1> release delayBudgetReportingConfig from the UE Inactive AS context, if stored; - stop timer T342, if running; <…> 1> release CSI-LoggedMeasurementConfig, from the UE Inactive AS context, if stored; - release loggedDataCollectionAssistanceConfig, from the UE Inactive AS context, if stored; - discard the logged measurement entries included in VarCSI-LogMeasReport, if any; |  |
| --- | --- |

Although both options can work, we suggest to follow the legacy principle (i.e., Option 2) as the configurations stored in UE Inactive AS context could be re-used in future releases (like SCell and SCG configuration based on restoreMCG-SCells or restoreSCG)

Proposal 1 [S042]. Adopt S042 (i.e., UE stores both main configuration and UAI configurations for NW-side data collection in UE Inactive AS Context, and releases them upon RRC resume initiation and discards logged data together.)

S046

According to current ASN.1 structure, there are two issues on redundancy of logged data.

- Issue 1. Whenever UE includes CSI-RS or SSB resource measurement results (i.e., csi-RS-MeasResultList-r19 and ssb-MeasResultList-r19), UE has to include refCSI-LoggedMeasurementConfigId-r19 redundantly.

- Issue 2. Whenever UE includes L1-RSRP, UE has to include resourceId-r19 (or ssb-Id-r19) redundantly.

CSI-LogMeasInfoCellList-r19 ::= SEQUENCE (SIZE (1..maxNrofServingCells)) OF CSI-LogMeasInfoCell-r19 CSI-LogMeasInfoCell-r19 ::= SEQUENCE { cellId-r19 CHOICE { cellGlobalId-r19 CGI-Info-Logging-r16, pci-arfcn-r19 PCI-ARFCN-NR-r16 }, csi-LogMeasInfoList-r19 SEQUENCE (SIZE (1..maxLogCSI-MeasReport-r19)) OF CSI-LogMeasInfo-r19, ... } CSI-LogMeasInfo-r19::= SEQUENCE { refCSI-LoggedMeasurementConfigId-r19 CSI-LoggedMeasurementConfigId-r19, csi-RS-MeasResultList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF CSI-RS-MeasResult-r19 OPTIONAL, ssb-MeasResultList-r19 SEQUENCE (SIZE (1..maxNrofSSBs-r16)) OF SSB-MeasResult-r19 OPTIONAL, timeGap-r19 ENUMERATED {true} OPTIONAL, ... } CSI-RS-MeasResult-r19 ::= SEQUENCE { resourceId-r19 NZP-CSI-RS-ResourceId, l1-RSRP-r19 RSRP-Range } SSB-MeasResult-r19 ::= SEQUENCE { ssb-Id-r19 SSB-Index, l1-RSRP-r19 RSRP-Range

If the logged data is included per refCSI-LoggedMeasurementConfigId-r19, the redundancy in issue 1 could be addressed. Similarly, if the logged data is included per resourceId-r19 (or ssb-Id-r19), the redundancy in issue 2 could be addressed. As the issue 1 has been addressed in Z008, we propose further update on Z008’s proposed text to resolve issue 2.

CSI-LogMeasInfoCell-r19 ::= SEQUENCE { cellId-r19 CHOICE { cellGlobalId-r19 CGI-Info-Logging-r16, pci-arfcn-r19 PCI-ARFCN-NR-r16 }, csi-LogMeasInfoMeasConfigList-r19 SEQUENCE (SIZE (1..maxNrofLoggedMeasurementConfigurations-r19)) OF CSI-LogMeasInfoMeasConfig-r19, ... } CSI-LogMeasInfoMeasConfig-r19 ::= SEQUENCE { refCSI-LoggedMeasurementConfigId-r19 CSI-LoggedMeasurementConfigId-r19, csi-RS-MeasResultList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF CSI-RS-MeasResult-r19 OPTIONAL, ssb-MeasResultList-r19 SEQUENCE (SIZE (1..maxNrofSSBs-r16)) OF SSB-MeasResult-r19 OPTIONAL, ... } CSI-RS-MeasResult-r19 ::= SEQUENCE { resourceId-r19 NZP-CSI-RS-ResourceId, rsrp-Results-r19 SEQUENCE (SIZE (1..maxLogCSI-MeasReport-r19)) OF RSRP-Result-r19 } SSB-MeasResult-r19 ::= SEQUENCE { ssb-Id-r19 SSB-Index, rsrp-Results-r19 SEQUENCE (SIZE (1..maxLogCSI-MeasReport-r19)) OF RSRP-Result-r19 } RSRP-Result-r19 ::= SEQUENCE { l1-RSRP-r19 RSRP-Range timeGap-r19 ENUMERATED {true} OPTIONAL, ... }

After [POST131bis][011][AI PHY] RRC CR (Ericsson), the endorsed CR (i.e., R2-2507934) address the 1st issue, but still has the 2nd issue. So, we still propose to adopt the ASN.1 structure in S046.

Proposal 2 [S046]. Adopt S046 to remove redundancy of logged data.

S052

Event triggered CSI measurement logging can be performed based on L3 events. It is agreed to use CSI framework and not measurement framework and the event is captured as below in RRC CR.

CSI-LoggedMeasurementEventTriggerConfig-r19 ::=          SEQUENCE {

threshold-r19                     CHOICE {

aboveThreshold-r19               MeasTriggerQuantity,

belowThreshold-r19               MeasTriggerQuantity

},

hysteresis                        Hysteresis,

timeToTrigger                     TimeToTrigger,

...

}

Now it is not clear how does the UE get the quantity configuration for the L3 filtering for deriving L3 measurements for the event evaluation for logging CSI measurements.

There can be two options for the quantity configuration for deriving L3 measurements for event evaluation for logging CSI measurements.

- Quantity configuration is provided in CSI measurement framework.

- UE uses the Quantity configuration from the measurement configuration, for e.g. the quantity configuration associated to measurement object through ServingCellMO. This means UE should be provided with the measurement configuration when CSI-LoggedMeasurementEventTriggerConfig is received.

RAN2 can discuss these options and select one of these options for the quantity configuration to be used for event evaluation of CSI measurements. The TPs with required changes for both options are included in the section 5- Annexure: TPs for S052.

Proposal 3 [S052]. RAN2 to discuss the below options for quantity configuration for deriving L3 measurements for event evaluation for logging CSI measurements and adopt the corresponding TP.

- Quantity configuration is provided in CSI measurement framework.

- UE uses the Quantity configuration from the measurement configuration, for e.g. the quantity configuration associated to measurement object through ServingCellMO.

S053

RAN2 needs to discuss if AI/ML for Phy is supported for NTN. If it is supported, there is a change required for SRBx, i.e. allowedHARQ-mode should be applicable for SRBx. i.e. below change is needed.

LogicalChannelConfig field descriptions

allowedCG-List This restriction applies only when the UL grant is a configured grant. If present, UL MAC SDUs from this logical channel can only be mapped to the indicated configured grant configuration. If the size of the sequence is zero, then UL MAC SDUs from this logical channel cannot be mapped to any configured grant configurations. If the field is not present, UL MAC SDUs from this logical channel can be mapped to any configured grant configurations. If the field configuredGrantType1Allowed is present, only those configured grant type 1 configuration indicated in this sequence are allowed for use by this logical channel; otherwise, this sequence shall not include any configured grant type 1 configuration. Corresponds to "allowedCG-List" as specified in TS 38.321 [3]. This field is ignored when SDT procedure is ongoing.

allowedHARQ-mode Indicates the allowed HARQ mode of a HARQ process mapped to this logical channel. If the parameter is absent, there is no restriction for HARQ mode for the mapping. This field applies to SRB1, SRB2, SRB4, SRBx and DRBs.

Proposal 4 [S053]. Discuss whether AI/ML for Phy is supported for NTN. If supported, allowedHARQ-mode should be applicable for SRBx.

S054

RAN2 needs to discuss if AI/ML for Phy is applicable for NR-U. If it is not explicitly disallowed, Channel Access Priority Class (CAPC) needs to be configured for SRBx, as this has impact on other technologies like WiFi which use the same spectrum

4.2.2 Signalling radio bearers <change 1> For operation with shared spectrum channel access in FR1, SRB0, SRB1 and SRB3 are assigned with the highest priority Channel Access Priority Class (CAPC), (i.e. CAPC = 1) while CAPC for SRB2,SRBx is configurable. <change 2> LogicalChannelConfig field descriptions. channelAccessPriority Indicates the Channel Access Priority Class (CAPC), as specified in TS 38.300 [2], to be used on uplink transmissions for operation with shared spectrum channel access in FR1. The network configures this field only for SRB2, SRBx and DRBs..

Proposal 5 [S054]: Discuss whether AI/ML for Phy is supported for NR-U. If it is not explicitly disallowed, Channel Access Priority Class (CAPC) should be applicable for SRBx.

Conclusion

RAN2 is requested to discuss and agree the following proposals:

Proposal 1 [S042]. Adopt S042 (i.e., UE stores both main configuration and UAI configurations for NW-side data collection in UE Inactive AS Context, and releases them upon RRC resume initiation and discards logged data together.)

Proposal 2 [S046]. Adopt S046 to remove redundancy of logged data.

Proposal 3 [S052]. RAN2 to discuss the below options for quantity configuration for deriving L3 measurements for event evaluation for logging CSI measurements and adopt the corresponding TP.

- Quantity configuration is provided in CSI measurement framework.

- UE uses the Quantity configuration from the measurement configuration, for e.g. the quantity configuration associated to measurement object through ServingCellMO.

Proposal 4 [S053]. Discuss whether AI/ML for Phy is supported for NTN. If supported, allowedHARQ-mode should be applicable for SRBx.

Proposal 5 [S054]: Discuss whether AI/ML for Phy is supported for NR-U. If it is not explicitly disallowed, Channel Access Priority Class (CAPC) should be applicable for SRBx.

References

Annexure: TPs for S052

TP for Option A

<Change 1>

–	CSI-MeasConfig

The IE CSI-MeasConfig is used to configure CSI-RS (reference signals) belonging to the serving cell in which CSI-MeasConfig is included, channel state information reports to be transmitted on PUCCH on the serving cell in which CSI-MeasConfig is included and channel state information reports on PUSCH triggered by DCI received on the serving cell in which CSI-MeasConfig is included. See also TS 38.214 [19], clause 5.2.

CSI-MeasConfig information element

-- ASN1START

-- TAG-CSI-MEASCONFIG-START

CSI-MeasConfig ::=                  SEQUENCE {

nzp-CSI-RS-ResourceToAddModList     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource   OPTIONAL, -- Need N

nzp-CSI-RS-ResourceToReleaseList    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToAddModList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToReleaseList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need N

csi-IM-ResourceToAddModList         SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource           OPTIONAL, -- Need N

csi-IM-ResourceToReleaseList        SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId         OPTIONAL, -- Need N

csi-IM-ResourceSetToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet     OPTIONAL, -- Need N

csi-IM-ResourceSetToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId   OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddModList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet   OPTIONAL, -- Need N

csi-SSB-ResourceSetToReleaseList    SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSetId OPTIONAL, -- Need N

csi-ResourceConfigToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig

OPTIONAL, -- Need N

csi-ResourceConfigToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfigId

OPTIONAL, -- Need N

csi-ReportConfigToAddModList        SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig  OPTIONAL, -- Need N

csi-ReportConfigToReleaseList       SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId

OPTIONAL, -- Need N

reportTriggerSize                   INTEGER (0..6)                                                            OPTIONAL, -- Need M

aperiodicTriggerStateList           SetupRelease { CSI-AperiodicTriggerStateList }                            OPTIONAL, -- Need M

semiPersistentOnPUSCH-TriggerStateList    SetupRelease { CSI-SemiPersistentOnPUSCH-TriggerStateList }         OPTIONAL, -- Need M

...,

[[

reportTriggerSizeDCI-0-2-r16        INTEGER (0..6)                                                            OPTIONAL -- Need R

]],

[[

sCellActivationRS-ConfigToAddModList-r17  SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-Config-r17   OPTIONAL, -- Need N

sCellActivationRS-ConfigToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-ConfigId-r17 OPTIONAL  -- Need N

]],

[[

ltm-CSI-ReportConfigToAddModList-r18   SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfig-r18

OPTIONAL, -- Need N

ltm-CSI-ReportConfigToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfigId-r18

OPTIONAL  -- Need N

]],

[[

csi-LoggedMeasurementConfigToAddModList-r19   SEQUENCE (SIZE (1..maxNrofLoggedMeasurementConfigurations-r19)) OF CSI-LoggedMeasurementConfig-r19

OPTIONAL, -- Need N

csi-LoggedMeasurementConfigToReleaseList-r19  SEQUENCE (SIZE (1..maxNrofLoggedMeasurementConfigurations-r19)) OF CSI-LoggedMeasurementConfigId-r19                                                                                                               OPTIONAL,  -- Need N

csi-LoggedMeasurementConfig-quantityConfig    QuantityConfigRS 														  OPTIONAL,  -- Need N

]]

}

-- TAG-CSI-MEASCONFIG-STOP

-- ASN1STOP

CSI-MeasConfig field descriptions

aperiodicTriggerStateList Contains trigger states for dynamically selecting one or more aperiodic and semi-persistent reporting configurations and/or triggering one or more aperiodic CSI-RS resource sets for channel and/or interference measurement (see TS 38.214 [19], clause 5.2.1).

csi-IM-ResourceSetToAddModList Pool of CSI-IM-ResourceSet which can be referred to from CSI-ResourceConfig or from MAC CEs.

csi-IM-ResourceToAddModList Pool of CSI-IM-Resource which can be referred to from CSI-IM-ResourceSet.

csi-LoggedMeasurementConfigToAddModList Configured CSI logged measurements for network-side data collection.

csi-ReportConfigToAddModList Configured CSI report settings as specified in TS 38.214 [19] clause 5.2.1.1.

csi-ResourceConfigToAddModList Configured CSI resource settings as specified in TS 38.214 [19] clause 5.2.1.2.

csi-SSB-ResourceSetToAddModList Pool of CSI-SSB-ResourceSet which can be referred to from CSI-ResourceConfig.

ltm-CSI-ReportConfigToAddModList Configured CSI report settings for LTM as specified in TS 38.214 [19].

nzp-CSI-RS-ResourceSetToAddModList Pool of NZP-CSI-RS-ResourceSet which can be referred to from CSI-ResourceConfig or from MAC CEs.

nzp-CSI-RS-ResourceToAddModList Pool of NZP-CSI-RS-Resource which can be referred to from NZP-CSI-RS-ResourceSet.

reportTriggerSize, reportTriggerSizeDCI-0-2 Size of CSI request field in DCI (bits) (see TS 38.214 [19], clause 5.2.1.5.1). The field reportTriggerSize applies to DCI format 0_1 and the field reportTriggerSizeDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 5.2.1.5.1).

scellActivationRS-ConfigToAddModList Configured RS for fast SCell activation as specified in TS 38.214 [19] clause 5.2.1.5.3.

csi-LoggedMeasurementConfig-quantityConfig Specifies L3 filter configurations for cell measurement results for the configurable RS Types (e.g. SS/PBCH block and CSI-RS) for theevent evaluation for network side data collection.

<Change 2>

- 5.5x.1.3	Reception of CSI-LoggedMeasurementConfig by the UE

Upon receiving csi-LoggedMeasurementConfigToAddModList in the csi-MeasConfig of a serving cell, the UE shall:

1>	for each CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList:

2>	if the current UE configuation for the serving cell includes the CSI logged measurement configuration associated with the given csi-LoggedMeasurementConfigId:

3>	modify the CSI logged measurement configuration according to the configuration received in csi-LoggedMeasurementConfigToAddModList;

3> reset the associated information (e..g. timeToTrigger) of the CSI logged measurement configuration;

2>	else:

3>	add the received CSI logged measurement configuration to the UE configuration;

<Change 3>

- 5.5.3.1	General

An RRC_CONNECTED UE shall derive cell measurement results by measuring one or multiple beams associated per cell as configured by the network, as described in 5.5.3.3. For all cell measurement results, except for RSSI, and CLI measurement results in RRC_CONNECTED, the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria, measurement reporting or the criteria to trigger conditional reconfiguration execution or event-triggered measurement logging for network-side data collection. For cell measurements, the network can configure RSRP, RSRQ, SINR, RSCP or EcN0 as trigger quantity. For CLI measurements, the network can configure SRS-RSRP or CLI-RSSI as trigger quantity. For cell and beam measurements, reporting quantities can be any combination of quantities (i.e. only RSRP; only RSRQ; only SINR; RSRP and RSRQ; RSRP and SINR; RSRQ and SINR; RSRP, RSRQ and SINR; only RSCP; only EcN0; RSCP and EcN0), irrespective of the trigger quantity, and for CLI measurements, reporting quantities can be either SRS-RSRP or CLI-RSSI. For conditional reconfiguration execution, the network can configure up to 2 quantities, both using same RS type. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the CBR measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the Rx-Tx time difference measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the altitude measurements.

<Change 4>

- 5.5.3.2	Layer 3 filtering

The UE shall:

1>	for each cell measurement quantity, each beam measurement quantity, each sidelink measurement quantity as needed in clause 5.8.10, for each CLI measurement quantity that the UE performs measurements according to 5.5.3.1, for each L2 U2N Relay UE measurement quantity according to 5.5.3.4, for evaluating the selected or detected NR sidelink U2N Relay UEs according to 5.8.15.3, for evaluating the SyncRef UE according to 5.8.5 and 5.8.6, for evaluating the NR sidelink U2U Relay/Remote UE threshold conditions according to 5.8.16.2 and 5.8.17.2, for evaluating the conditions for selection and reselection of NR sidelink U2U Relay UE according to 5.8.17.3, and for evaluating the detected NR sidelink U2U Relay UEs according to 5.8.17.4:

2>	filter the measured result, before using for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation, for evaluating the SyncRef UE, or for evaluation for the event triggered CSI measurement logging by the following formula:

Fn = (1 – a)*Fn-1 + a*Mn

where

Mn is the latest received measurement result from the physical layer;

Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE;

Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and for MeasObjectNR, a = 1/2(ki/4), where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR or for event-triggered measurement logging,the filterCoefficient of the RS from CSI measurement configuration; for other measurements, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig; for UTRA-FDD, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigUTRA-FDD in the QuantityConfig;

2>	adapt the filter such that the time characteristics of the filter are preserved at different input rates, observing that the filterCoefficient k assumes a sample rate equal to X ms; The value of X is equivalent to one intra-frequency L1 measurement period as defined in TS 38.133 [14] assuming non-DRX operation, and depends on frequency range.

NOTE 1:	If k is set to 0, no layer 3 filtering is applicable.

NOTE 2:	The filtering is performed in the same domain as used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE, i.e., logarithmic filtering for logarithmic measurements.

NOTE 3:	The filter input rate is implementation dependent, to fulfil the performance requirements set in TS 38.133 [14]. For further details about the physical layer measurements, see TS 38.133 [14].

NOTE 4:	For CLI-RSSI measurement, it is up to UE implementation whether to reset filtering upon BWP switch.

NOTE 5:	For SSB measurements when multiple altitude range-based ssb-ToMeasure are configured, it is up to UE implementation whether to reset filtering upon entering a different altitude range.

NOTE 6:	Upon satellite switch with resynchronization, it is up to UE implementation to reset filtering for the serving cell.

TP for Option b

<Change 1>

- 5.5.2.8	Quantity configuration

The UE shall:

1>	for each RAT for which the received quantityConfig includes parameter(s):

2>	set the corresponding parameter(s) in quantityConfig within VarMeasConfig to the value of the received quantityConfig parameter(s);

1>	for each measId included in the measIdList within VarMeasConfig:

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId.

- reset the associated information (e..g. timeToTrigger) of the EventTriggeredConfig for event-triggered measurement logging for network-side data collection.

<Change 2>

- 5.5.3.1	General

An RRC_CONNECTED UE shall derive cell measurement results by measuring one or multiple beams associated per cell as configured by the network, as described in 5.5.3.3. For all cell measurement results, except for RSSI, and CLI measurement results in RRC_CONNECTED, the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria, measurement reporting or the criteria to trigger conditional reconfiguration execution or event-triggered measurement logging for network-side data collection. For cell measurements, the network can configure RSRP, RSRQ, SINR, RSCP or EcN0 as trigger quantity. For CLI measurements, the network can configure SRS-RSRP or CLI-RSSI as trigger quantity. For cell and beam measurements, reporting quantities can be any combination of quantities (i.e. only RSRP; only RSRQ; only SINR; RSRP and RSRQ; RSRP and SINR; RSRQ and SINR; RSRP, RSRQ and SINR; only RSCP; only EcN0; RSCP and EcN0), irrespective of the trigger quantity, and for CLI measurements, reporting quantities can be either SRS-RSRP or CLI-RSSI. For conditional reconfiguration execution, the network can configure up to 2 quantities, both using same RS type. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the CBR measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the Rx-Tx time difference measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the altitude measurements.

<Change 3>

- 5.5.3.2	Layer 3 filtering

The UE shall:

1>	for each cell measurement quantity, each beam measurement quantity, each sidelink measurement quantity as needed in clause 5.8.10, for each CLI measurement quantity that the UE performs measurements according to 5.5.3.1, for each L2 U2N Relay UE measurement quantity according to 5.5.3.4, for evaluating the selected or detected NR sidelink U2N Relay UEs according to 5.8.15.3, for evaluating the SyncRef UE according to 5.8.5 and 5.8.6, for evaluating the NR sidelink U2U Relay/Remote UE threshold conditions according to 5.8.16.2 and 5.8.17.2, for evaluating the conditions for selection and reselection of NR sidelink U2U Relay UE according to 5.8.17.3, and for evaluating the detected NR sidelink U2U Relay UEs according to 5.8.17.4:

2>	filter the measured result, before using for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation, for evaluating the SyncRef UE or for evaluation for the event triggered CSI measurement logging by the following formula:

Fn = (1 – a)*Fn-1 + a*Mn

where

Mn is the latest received measurement result from the physical layer;

Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE;

Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and for MeasObjectNR, a = 1/2(ki/4), where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR; for other measurements, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig; for UTRA-FDD, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigUTRA-FDD in the QuantityConfig;

2>	adapt the filter such that the time characteristics of the filter are preserved at different input rates, observing that the filterCoefficient k assumes a sample rate equal to X ms; The value of X is equivalent to one intra-frequency L1 measurement period as defined in TS 38.133 [14] assuming non-DRX operation, and depends on frequency range.

NOTE 1:	If k is set to 0, no layer 3 filtering is applicable.

NOTE 2:	The filtering is performed in the same domain as used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE, i.e., logarithmic filtering for logarithmic measurements.

NOTE 3:	The filter input rate is implementation dependent, to fulfil the performance requirements set in TS 38.133 [14]. For further details about the physical layer measurements, see TS 38.133 [14].

NOTE 4:	For CLI-RSSI measurement, it is up to UE implementation whether to reset filtering upon BWP switch.

NOTE 5:	For SSB measurements when multiple altitude range-based ssb-ToMeasure are configured, it is up to UE implementation whether to reset filtering upon entering a different altitude range.

NOTE 6:	Upon satellite switch with resynchronization, it is up to UE implementation to reset filtering for the serving cell.

<Change 4>

- –	CSI-LoggedMeasurementConfig

The IE CSI-LoggedMeasurementConfig is used to configure a CSI logged measurement configuration. It defines a group of one or more CSI resources for which the UE logs the associated L1 radio measurements.

CSI-LoggedMeasurementConfig information element

-- ASN1START

-- TAG-CSI-LOGGEDMEASUREMENTCONFIG-START

CSI-LoggedMeasurementConfig-r19 ::=          SEQUENCE {

csi-LoggedMeasurementConfigId-r19         CSI-LoggedMeasurementConfigId-r19,

csi-LoggedResourceConfig-r19              CSI-ResourceConfigId,

loggingPeriodicity-r19                    ENUMERATED {n2, n3, n4, n5, spare4, spare3, spare2, spare1}            OPTIONAL,  -- Need M

csi-LoggedMeasurementEventTriggerConfig-r19         CSI-LoggedMeasurementEventTriggerConfig-r19                OPTIONAL,  -- Need R

...

}

CSI-LoggedMeasurementEventTriggerConfig-r19 ::=          SEQUENCE {

threshold-r19                     CHOICE {

aboveThreshold-r19               MeasTriggerQuantity,

belowThreshold-r19               MeasTriggerQuantity

},

hysteresis                        Hysteresis,

timeToTrigger                     TimeToTrigger,

...

}

-- TAG-CSI-LOGGEDMEASUREMENTCONFIG-STOP

-- ASN1STOP

CSI-LoggedMeasurementConfig field descriptions

csi-LoggedMeasurementConfigId This field indicates the instance of CSI-LoggedMeasurementConfig.

csi-LoggedResourceConfig Resources in which the UE performs channel measurement whose associated measurement results are logged by the UE. The csi-LoggedResourceConfig indicated here contains only NZP-CSI-RS resources and/or SSB resources.

csi-LoggedMeasurementEventTriggerConfigThis field is used to configure the UE with event-triggered measurement logging. If this field is included and threshold is set to aboveThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.2 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.2 is met. If this field is included and threshold is set to belowThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.3 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.3 is met. If this field is not included, the UE starts the measurement logging according to csi-LoggedResourceConfig upon reception. This field is configured if the serving cell is configured with servingCellMO and a measurement identity associated to the measurement object indicated by servingCellMO..

loggingPeriodicity The periodicity that the UE shall use for the logging of the CSI measurements. The loggingPeriodicity is given as a multiple of the periodicity of the resources indicated by csi-LoggedResourceConfig. If loggingPeriodicity is included and set to 'n2', the UE performs the logging of CSI measurements for every 2nd occasion of the resources, if it is set to 'n3', the UE performs logging of CSI measurements for every 3rd occasion of the resources, and so on. If loggingPeriodicity is not included, the UE performs the logging of CSI measurements according to the periodicity of the resources indicated by csi-LoggedResourceConfig, i.e. for every occasion of the resources.