3GPP TSG-RAN WG3 Meeting #130	R3-258767

Dallas, US, 17 – 21 November 2025

Agenda item:	10.3.2

Source:	Nokia

Title:	6G RAN-CN Cloud-friendly P2P Solution

WID/SID:	FS_6G_Radio

Document for:	Discussion and Decision

# 1	Introduction

At RANP#108 the new SID on 6G RAN was agreed in [1] and was further revised at RAN#109 in [2].

The main architectural impacts for RAN3 are listed below:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

As part of point b/, RA3 was tasked with a first milestone to achieve:

#### 4.1.1	Interim Milestone

Interim results shall be delivered as per the milestones below, in coordination with the RAN Plenary 6G Study [RP-250810].

TSG#112 (June/2026):

RAN3 to provide interim study results to allow TSGs to make a decision on:

- RAN-CN interface: P2P vs SBI

- RAN internal interfaces: CU-DU split, CP-UP split.

Tdoc [3] has derived the key requirements and associated criteria to select the RAN-CN CP interface solution.

This paper proposes our solution for the section 6.1.3 of the TR with:

- a first part concerning the Control Plane of the RAN-CN interface for which the interim milestone applies, and

- a second part related to the User Plane of the RAN-CN interface.

# pCR for TR 38.760-3

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 6.1.x	RAN-CN User Plane

Editor’s note 1: The aim of this section is to describe the user plane protocol stack for the RAN-CN interface.

Editor’s note 2: On hold waiting for CT4 input.

The user plane interface between the 6G RAN and CN for 6G is based on a point to point tunnel.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>