3GPP TSG-RAN WG3 Meeting #130	R3-258792

Dallas, Texas, USA, 17-21 November 2025

Agenda Item:		12.2.1

Source:		NEC (moderator)

Title:			Summary of Offline Discussion for CB: # AIMLMultiHop

Document for:		Discussion and decision

# 1	Introduction

CB: # 17_AIMLMultiHop

-  For the subsequent hop, configuration of measurement feedback

-  The receiving node behavior for subsequent handovers

-  TP for TR

(NEC - moderator)

Summary of offline discussion in R3-258792

# 2	For the Chair Notes

Editor’s Note: For Rel-20 study/work items, please consider that when agreements/FFSes are captured in a TP, additional inclusion in the Chair Notes may be unnecessary (particularly for stage 3 details).

Propose the following:

R3-258791 rev in R3-258247 – agreed

Propose to capture the following:

- For the subsequent hop, configuration of measured UE trajectory:

Data Collection Reporting Initiation procedure is in place between the initiating gNB and a subsequent gNB to request the reporting of the measured UE trajectory.

- For the subsequent handovers, multi-hop predicted UE trajectory includes a list of cells where the UE is expected to connect to, in chronological order.

Open issue:

How to configure the reporting of the measured UE trajectory in each subsequent hop?

- Hop by hop via Data Collection Reporting Initiation procedure

- Hop by hop via Handover request

# 3	Discussion

## 3.1 Configuration of measurement feedback

For the subsequent hop, configuration of measurement feedback:

Option 1: From initiating node to each hop via the Data Collection Reporting Initiation procedure

Option 2: Hop to hop

Option 2a: via Data Collection Reporting Initiation procedure

Option 2b: via Handover request

#### 9.2.3.185	UE Trajectory Collection Configuration

This IE contains configurations for UE trajectory collection after successful handover.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Trajectory | M |  | INTEGER (1..4096, …) | Time duration starting at successful handover within which UE Trajectory Information is collected. Unit: second |
| Number of Visited Cells | O |  | INTEGER (1..16, …) | Maximum number of intra-node visited cells. |

For the subsequent hop, configuration of measured UE trajectory:

Data Collection Reporting Initiation procedure is in place between the initiating gNB and a subsequent gNB to request the reporting of the measured UE trajectory.

How to configure the reporting of the measured UE trajectory in each subsequent hop?

- Hop by hop via Data Collection Reporting Initiation procedure

- Hop by hop via Handover request

## 3.2 Receiving node behavior for subsequent handovers

#### 9.2.3.180	Cell Based UE Trajectory Prediction

The Cell Based UE Trajectory Prediction IE contains information related to NG-RAN cells where the UE is predicted to connect.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Cell Based UE Trajectory Prediction |  | 1 |  | List of cells where the UE is predicted to connect, in chronological order. The first predicted cell added to the top of this list, is the cell in the target node where the UE will move to after the serving cell at the source node. |
| >Predicted UE Trajectory Item |  | 1..<maxnoofCellsTrajectoryPredict> |  |  |
| >>Predicted Trajectory Cell Information | M |  | 9.2.3.181 |  |

Follow the Semantics Description of Cell Based UE Trajectory Prediction IE in TS 38.423 section 9.2.3.180.

## 3.3 TP

Agree the TP reflecting the following agreements:

- Reuse the Cell Based UE Trajectory Prediction IE in HANDOVER REQUEST message for multi-hop UE Trajectory Prediction transfer.

- How to transfer measurement feedback:

Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1)

- For the subsequent hop, configuration of measured UE trajectory:

Data Collection Reporting Initiation procedure is in place between the initiating gNB and a subsequent gNB to request the reporting of the measured UE trajectory.

- For the subsequent handovers, multi-hop predicted UE trajectory includes a list of cells where the UE is expected to connect to, in chronological order.