3GPP TSG-RAN WG4 Meeting #117                        R4-2520048                                                                                                       Dallas, Texas, USA, 17 – 21 November 2025

Source:	China Telecom

Title:	On maintenance for BS demodulation performance requirements for MMSE-IRC

Agenda Item:	4.14.1

Document for:	Discussion

Introduction

According to the last meeting WF [1], we will make decision on the final SNR requirement for BS MMSE-IRC in this meeting:

Alignment on simulation results for BS MMSE-IRC requirements - Way forward - Keep the way forward from RAN4#116 and continue the discussions in RAN4#117 - Make decision on final SNR requirements in RAN4#117

In this contribution, our views are given.

Discussion

Early in the RAN4#115, we have agreed the SNR requirement value deriving rule as below.

SNR requirement value deriving rule - Online agreement: - Reuse the SNR requirement value deriving rule for Rel-15 BS demodulation SNR requirement derivation procedure as agreed in R4-1904713 Procedure to derive the performance requirements: – Only inputs that consist of a pair of ideal and impaired results can be taken into account. – If the ideal span <= [2]dB: • The AVERAGE impairment results can be used for the performance requirement with [] in the draftCRs/CRs; – Else if the ideal span is larger than [2]dB: • The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available: – The ultimate AVERAGE impairment results with corresponding ideal span <=2dB can be used for performance requirement with [] in the draftCRs/CRs. • Otherwise put TBD for the related performance requirements. – If the span of the impairment results after removal the outliers (if any) are larger than 4dB, then the procedure cannot be applied, related performance requirement remain TBD..

The SNR values in the current spec is derived based on the above requirement deriving rules to remove the outliers to make the SPAN for each test cases within 2dB.

Some companies think that the mis-alignment between different companies’ results is due to different MMSE-IRC implementation for ICI power estimation:

MMSE-IRC receiver assumption - estimation of interference covariance matrix Samsung: Estimation method of interference covariance matrix based on DMRS symbol per RB and per TTI for MMSE-IRC is up to the gNB implementation. Nokia: There was agreement in previous meeting was to consider the DMRS RE’s, not the DMRS symbols which also include data. Huawei: Agree with Samsung Ericson: We agree with Nokia.

However, based on our simulation results provided in RAN4#115, for 2Rx and 4Rx, there should be no performance difference if companies use symbol-based or data-RE-based MMSE-IRC implementation.

For 8Rx cases, as we checked the latest simulation result summary in [2], we do not observe better performance (lower SNR values) from the companies who propose better ICI power estimation.

Observation 1: For 2Rx and 4Rx, there is no performance difference if companies use symbol-based or data-RE-based MMSE-IRC implementation.

Observation 2: Even for 8Rx cases, we do not observe better performance (lower SNR values) from the companies who proposed better ICI power estimation (DMRS symbol based power estimation).

As a summary, we do not think that different MMSE-IRC implementation could be the reason of the mis-alignment of simulation result. And we think it is natural to have some outliers for requirement definition considering we have 11 companies who provided simulation results.

Considering all the above, we propose to simply follow the agreed SNR deriving rules in RAN4#115 and no additional margin or further simulation result alignment is needed.

Proposal 1: Follow the agreed SNR deriving rules in RAN4#115 and no additional margin or further simulation result alignment is needed.

Conclusion

Observation 1: For 2Rx and 4Rx, there is no performance difference if companies use symbol-based or data-RE-based MMSE-IRC implementation.

Observation 2: Even for 8Rx cases, we do not observe better performance (lower SNR values) from the companies who proposed better ICI power estimation (DMRS symbol based power estimation).

Proposal 1: Follow the agreed SNR deriving rules in RAN4#115 and no additional margin or further simulation result alignment is needed.

Reference

R4-2514823, WF on NR_demod_Ph6_Ph5, China Telecom, RAN4#116bis

R4-2509405, Simulation results summary for BS requirement with MMSE-IRC receiver, Samsung, RAN4#116