3GPP TSG-RAN WG4 Meeting #117                        R4-2520052                                                                                                       Dallas, Texas, USA, 17 – 21 November 2025

Source:	China Telecom

Title:	Discussion on UE requirements for FR1 SU-MIMO with spatial channel model

Agenda Item:	7.5.3

Document for:	Discussion

Introduction

In RAN #109 meeting, the new WID on NR demodulation performance: Phase6 was approved [1], with the following objectives for the normative work for spatial channel model (SCM):

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1 - Using rCDL-C1 channel model detailed in TR 38.753 as starting point - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case. - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding. - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

The RAN4#116bis is the first meeting companies discussed the test scopes, parameters and simulation assumptions and the latest status are captured in [2].

In this contribution, our views on UE requirements for FR1 SU-MIMO with spatial channel model is provided.

Discussion

According to the WID, the normative work includes mainly 3 parts: update of rCDL channel model considering the outcome in SI FS_NR_7_24GHz_Chmod (if necessary), test scope and test parameter selection, test applicability.

Possible update to rCDL channel model

Frequency generalization for rCDL channel modeling

- Agreement: - Define the requirements based on the same channel model for all different FR1 frequencies for this WI - The maximum Doppler fD should be fixed as X Hz - As baseline for Rel-20 demod requirement, X =10 - FFS if other fD can be considered - On the note to Maximum Doppler: - The ratio between the UE speed v, i.e., magnitude of the velocity vector, and the wavelength is normalized to be X Hz (maximum Doppler).

In addition to the maximum Doppler 10Hz as will be used for this WI, we are supportive to consider other Doppler values to cover more practical scenarios as we usually use TDLA30-10, TDLB100-400 and TDLC300-100. But considering the progress, we recommend to deprioritize this part when the SCM requirements with maximum Doppler 10Hz are stable.

Proposal 1: Study other Doppler values to cover more practical scenarios when the SCM requirements with maximum Doppler 10Hz are stable.

Test scope and test parameters

Duplex mode and SCS

- Cover TDD 30kHz SCS with 40MHz CHBW and FFS whether to also cover FDD 15kHz SCS with 10MHz CHBW

It has been agreed in the last meeting that for all FR1 frequencies, the same channel model will be used.

We expect limited workload for covering FDD 15kHz SCS with 10MHz CHBW. Considering we are defining requirement for basic 5G UE features, we propose to cover both 30kHz SCS with 40MHz CHBW and FDD 15kHz SCS with 10MHz CHBW.

Proposal 2: Cover both 30kHz SCS with 40MHz CHBW and FDD 15kHz SCS with 10MHz CHBW.

Precoding, MCS and test metric for PDSCH requirements

- Agreement and candidate options: - For 4T4R 4L test, it is agreed to use the test metric: SNR @ 70% max TP. - For 8T8R 8L test, it is agreed to use test metric: Define single SNR and further discuss the SNR is based on: - Option a: Based on per CW thput with Different MCS per CW - Option b: Based on total thput with different MCS per CW - Option c: Based on total thput with same MCS per CW - Further discuss: How to define the SNR for option a. How to select the MCS for different CW.

For 8T8R rank8, from Table 7.1-2 of TR38.753, we observe huge performance difference between the 2 codewords. (SNR difference of 5.9~11.8dB). Since different performance for different codewords is one of the main motivations to study spatial channel model, if companies are not interested in defining SNR requirement values for each codeword separately, we support to at least use different MCS for each CW.

Observation 1: For 8T8R rank8, the performance difference between the 2 codewords is at the range of 5.9~11.8dB as captured in TR38.753.

Proposal 3: For 8T8R rank8, if companies are not interested in defining SNR requirement values for each codeword separately, at least use different MCS for each CW.

PMI bias

- Candidate options: - Option 1: RAN4 shall exploit approaches to address the PMI bias issue - Option 1a: Apply the existing simple beam steering approach specified in TS 38.101-4 B.2.3.2.3 - Option 1b: time variant mean angles (AOA/AOD/ZOA/ZOD) - Other options are not precluded - Option 2: No need to consider time-wise evolution for PMI reporting tests - Option 3: Defer a consideration of a comprehensive long-term solutions to the 6G Radio Study Item

We believe the existing beam steering approach is a important method to ensure the reported PMI can dynamically catch up with the latest channel condition. In addition, it can verify the UE can report the PMIs covering all directions.

Therefore, we support the idea to apply the existing simple beam steering approach specified in TS 38.101-4 B.2.3.2.3 as a start point. Companies can do some additional result alignment first.

Proposal 4: Apply the existing simple beam steering approach specified in TS 38.101-4 B.2.3.2.3 as a start point. Companies can do some additional result alignment first.

Whether to define eType II PMI reporting requirements based on SCM

According to the WID, it has been approved that RAN4 will define requirements for PDSCH demodulation performance requirements for the 8T8R rank8 and 4T4R rank4 case as studied in TR38.753. Same time, it remains undecided whether to define eType II PMI test cases, given that the the simulation results for eType II PMI is not well aligned in terms of the SNR @70% and @90% max TP, as captured in Table 7.1-3 in TR38.753.

Table 7.1-3 Simulation result summary for FR1 SU-MIMO PMI 8Tx4Rx with 4 layers, follow PMI SNR at Norm. Throughput [dB]; Source #1; Source #2; Source #3; Source #4; Source #5; Source #6; Source #7; Source #8; Source #9; Average; Span. Type-I; 70%; 12.4; 11.0; 9.7; 10.3; 10.2; 9.7; 9.5; 10.2; 9.8; 10.4; 2.9. 90%; 14.4; 13.1; 12.5; 12.8; 12.6; 12.6; 12.0; 12.5; 12.3; 12.7; 2.4. eType-II; 70%; 14.0; 14.3; 8.4; 11.6; 10.1; 10.8; 8.2; 13.8; 10.8; 11.5; 6.1. 90%; 15.9; 17.1; 12.3; 14.0; 12.6; 13.9; 10.6; 16.8; 12.9; 14.0; 6.5.

We also observe that companies’ results for Type I PMI as well as SU-MIMO PDSCH are well-aligned, which means that the rCDL-C1 channel modeling has been aligned between companies.

eType II PMI enables much more accurate CSI feedback compared with Type I PMI. Thus it is naturally difficult for companies to align their simulation results especially under SCM condition, considering different vendors will implement different channel estimation accuracy and different CSI calculation algorithms.

Since Rel-16 Type II PMI, for original TDL model, it was hard for companies to align simulation results. RAN4 defines enhanced PMI requirements by selecting the lowest value among the ones companies provided.

Observation 2: According to TR38.753, companies’ results for Type I PMI as well as SU-MIMO PDSCH are well-aligned.

Observation 3: Since Rel-16 Type II PMI, for original TDL model, it was hard for companies to align simulation results. RAN4 defines enhanced PMI requirements by selecting the lowest value among the ones companies provided.

We believe PMI reporting is one of the crucial aspects for UE performance verification under SCM, thus we propose to specify requirements for PMI performance with 8T4R eType II precoding.

For 4T4R eType II as captured in the WID, we notice that it was not studied in the Rel-19 SI, which means additional simulation work will be required.

Proposal 5: Specify requirements for PMI performance with 8T4R eType II precoding by selecting the lowest value among the ones companies provided.

Test metric for eType II PMI reporting

As for the test metric for eType II PMI reporting, we prefer to follow the same rule for the existing eType II requirement based on TDL, i.e., following PMI (eType II)/Random PMI (Type I codebook).

Proposal 6: Reuse the same test metric for the existing eType II requirement based on TDL, i.e., Thput of following PMI (eType II)/ Thput of Random PMI (Type I codebook) with SNR @ 90 % of the maximum throughput for following PMI.

Test parameters for eType II PMI reporting and PDSCH demodulation

As for the test parameters, we propose to reuse the simulation assumptions for the Rel-19 SI as much as possible to avoid duplicate simulation workload, i.e., Table 6.1-1 and Table 6.1-2 for PDSCH requirements and Table 6.2-1 and Table 6.2-2 for PMI reporting as captured in TR38.753.

Proposal 7: Reuse the simulation assumptions/test parameters for the Rel-19 SI, i.e., Table 6.1-1 and Table 6.1-2 for PDSCH requirements and Table 6.2-1 and Table 6.2-2 for PMI reporting as captured in TR38.753.

Test applicability

It is clearly captured in the WID that all SCM new requirements are optional. Therefore, RAN4 needs to introduce new declaration for the SCM testing and UE can choose whether to pass the tests based on declaration.

It remains unclear that if RAN4 will define one single declaration on SCM requirements, or define multiple declarations for each individual test cases.

In addition, RAN4 is now defining requirements for the existing UE features, e.g., 4T4R rank4 requirement applies to Rel-15 UEs and 8T8R rank8 requirement applies to Rel-17 8Rx UEs supporting max 8 layers. Therefore, RAN4 needs to discuss whether to define release independent rules for the new SCM requirements.

Proposal 8: Introduce new declaration for the SCM testing and UE can choose whether to pass the tests based on declaration. RAN4 needs to discuss whether to define one single declaration on SCM requirements, or define multiple declarations for each individual SCM test cases.

Proposal 9: RAN4 needs to discuss whether to define release independent rules for the new SCM requirements.

Conclusion

Proposal 1: Study other Doppler values to cover more practical scenarios when the SCM requirements with maximum Doppler 10Hz are stable.

Proposal 2: Cover both 30kHz SCS with 40MHz CHBW and FDD 15kHz SCS with 10MHz CHBW.

Observation 1: For 8T8R rank8, the performance difference between the 2 codewords is at the range of 5.9~11.8dB as captured in TR38.753.

Proposal 3: For 8T8R rank8, if companies are not interested in defining SNR requirement values for each codeword separately, at least use different MCS for each CW.

Proposal 4: Apply the existing simple beam steering approach specified in TS 38.101-4 B.2.3.2.3 as a start point. Companies can do some additional result alignment first.

Observation 2: According to TR38.753, companies’ results for Type I PMI as well as SU-MIMO PDSCH are well-aligned.

Observation 3: Since Rel-16 Type II PMI, for original TDL model, it was hard for companies to align simulation results. RAN4 defines enhanced PMI requirements by selecting the lowest value among the ones companies provided.

Proposal 5: Specify requirements for PMI performance with 8T4R eType II precoding by selecting the lowest value among the ones companies provided.

Proposal 6: Reuse the same test metric for the existing eType II requirement based on TDL, i.e., Thput of following PMI (eType II)/ Thput of Random PMI (Type I codebook) with SNR @ 90 % of the maximum throughput for following PMI.

Proposal 7: Reuse the simulation assumptions/test parameters for the Rel-19 SI, i.e., Table 6.1-1 and Table 6.1-2 for PDSCH requirements and Table 6.2-1 and Table 6.2-2 for PMI reporting as captured in TR38.753.

Proposal 8: Introduce new declaration for the SCM testing and UE can choose whether to pass the tests based on declaration. RAN4 needs to discuss whether to define one single declaration on SCM requirements, or define multiple declarations for each individual SCM test cases.

Proposal 9: RAN4 needs to discuss whether to define release independent rules for the new SCM requirements.

Reference

RP-252957, WID: NR demodulation performance: Phase 6, Apple, RAN#109, Sep 2025

R4-2514823, WF on NR_demod_Ph6_Ph5, China Telecom, RAN4#116bis, Oct 2025