3GPP TSG-RAN WG4 Meeting #117                                                                                                      R4-2520068

Dallas, USA, 17th –21st November, 2025

Source: 	CATT

Title: 	Further discussion on 6G demod

Agenda item:	8.7

Document for:	Approval

- Introduction

In RAN4#116bis meeting, a WF on [116bis][106] 6G Demod was agreed [1], and the remaining issues on 6G demod in WF require further discussion.

In this contribution, we further provide our views on 6G demod.

- Discussion

2.1 General aspects

RAN4 demod study timeline

As per the WF [1], the open issue concerning RAN4 demod study timeline is shown as below:

RAN4 demod study timeline - Proposals - Option 1: For RAN4 6G Demodulation, RAN4 establish a more realistic and structured timeline to ensure adequate depth in technical deliberation and a well-paced progression toward completion. - Option 2: RAN4 closely follow RAN1's discussion progress on other parameters related to 6G physical layer structure. Once relevant conclusions are available, RAN4 could consider adopting them for RAN4 6G Demodulation study. - Option 3: Postpone RAN4 discussion on demodulation requirements related to physical layer channel and procedure design until sufficient progress reached in RAN1 i.e., no early than Q2’26. - Option 4: 5GA Topics that were studied and proven useful should be introduced from day 1 in 6G, and RAN4 needs start discussions on corresponding test specifications to enable early adoption.

Generally, the demod includes the following parts:

- System requiring demodulation, such as TN, NTN, ISAC, etc, which require independent study by RAN4.

- Test parameter for PUSCH, PUCCH, etc, RAN4 need to follow RAN1 progress.

- Test methodologies, including test procedure, measurement setup, etc, which require independent study by RAN4.

- Channel model for system requiring demodulation, which require RAN4 to study the channel model applicable for TN, NTN, and ISAC etc.

So, for Test parameter, it is related to RAN1, but RAN4 need to understand and follow RAN1 progress. For system requiring demodulation, test methodologies, channel model for system requiring demodulation etc. need RAN4 independent study.

From our understanding, Option 1 includes the principle of Option 4, and Option 1 is the most proactive, structured, and balanced approach. It establishes a clear and "realistic" framework for the work, which is crucial for a complex, long-term 6G standardization project. Option 2 is reactive to RAN1 and Option 3 is a significant delay. So Option 1 is preferable.

Proposal 1: For RAN4 demod study timeline, Option 1 is preferable.

  - Option 1: For RAN4 6G Demodulation, RAN4 establish a more realistic and structured timeline to ensure adequate depth in technical deliberation and a well-paced progression toward completion.

Waveform and modulation study

As per the WF [1], the open issue concerning waveform and modulation study is shown as below:

Waveform and modulation study - Proposals - Option 1: RAN4 6G Demodulation could start with CP-OFDM and DFT-s-OFDM waveforms for 6G uplink demodulation study, and CP-OFDM waveform for 6G downlink demodulation study. - Option 2: RAN4 6G Demodulation study should cover following modulation schemes at least - For downlink, QPSK, 16QAM, 64QAM, 256QAM and 1024QAM - For uplink with CP-OFDM waveform, QPSK, 16QAM, 64QAM, 256QAM - For uplink with DFT-s-OFDM waveform, pi/2 BPSK, QPSK, 16QAM, 64QAM, 256QAM - Option 3: Develop testable specifications for waveform, frame structure, channel coding, and modulation as defined by RAN1, ensuring that these can be practically implemented and validated in real products.

The waveforms in Option 1 were supported by NR, and modulation schemes in Option 2 were supported by NR. From perspective of RAN4, it is reasonable for 6G demodulation to cover the waveforms and modulation schemes supported by NR. So option 1 and option 2 are preferable. RAN4 also need to focus on the new channel coding and modulation technology defined by RAN1, such as probabilistic shaping QAM, geometric shaping QAM, etc. The Option 3 indicates RAN4 should focus on waveform, frame structure etc. defined in RAN1 and implementation and testable specifications are considered, so it is also preferable.

Proposal 2: For waveform and modulation study, Option 1, Option 2 and Option 3 are preferable.

  - Option 1: RAN4 6G Demodulation could start with CP-OFDM and DFT-s-OFDM waveforms for 6G uplink demodulation study, and CP-OFDM waveform for 6G downlink demodulation study.

  - Option 2: RAN4 6G Demodulation study should cover following modulation schemes at least

    - For downlink, QPSK, 16QAM, 64QAM, 256QAM and 1024QAM

    - For uplink with CP-OFDM waveform, QPSK, 16QAM, 64QAM, 256QAM

    - For uplink with DFT-s-OFDM waveform, pi/2 BPSK, QPSK, 16QAM, 64QAM, 256QAM

  - Option 3: Develop testable specifications for waveform, frame structure, channel coding, and modulation as defined by RAN1, ensuring that these can be practically implemented and validated in real products.

SCS

As per the WF [1], the open issue concerning waveform and modulation study is shown as below:

SCS - Proposals - Option 1: RAN4 6G Demodulation study could start with following SCS options. - For sub 6GHz, 15kHz SCS for FDD, 30KHz SCS for TDD - For around 7GHz, 30kHz, 60kHz - For around 15GHz, 60kHz - For between 24.25GHz - 52.6GHz, 120kHz

For sub 6GHz, wider bandwidth for re-farming band may be need to be considered, and wider bandwidth FDD need 30kHz SCS. The SCSs in Option 1 were supported by 5G NR, for 6G, single numerology principle is needed, so the choice of SCS for demod is not issue. So Option 1 can be as starting point.

Proposal 3: For SCS for 6G demod, Option 1 can be as starting point.

  - Option 1: RAN4 6G Demodulation study could start with following SCS options.

    - For sub 6GHz, 15kHz SCS for FDD, 30KHz SCS for TDD

    - For around 7GHz, 30kHz, 60kHz

    - For around 15GHz, 60kHz

    - For between 24.25GHz - 52.6GHz, 120kHz

Demodulation specification principles

As per the WF [1], the open issue concerning waveform and modulation study is shown as below:

Demodulation specification principles - Proposals - Option 1: For 6G Demodulation specification structures, take TS38.101-4 as a starting point. - Option 2: For 6G Demodulation specification drafting principles, the descriptions of test parameters should be aligned with RAN1/RAN2 descriptions as much as possible, in order to avoid ambiguous understanding. - Option 3: For FRCs in 6G Demodulation specification, prefer to use a formula-based or pseudo-code-based definition for FRCs instead of table-based approach listing every parameter combination. - Option 3A: RAN4 needs to discuss how to specify FRC table in the specification for both BS and UE demodulation performance, considering the discussion in SI modernization of specification format and procedures for 6G.

For 6G demodulation specification structures, we can take TS 38.101-4 for UE demod as starting point, and take TS 38.104 for BS demod as starting point. It is reasonable that the descriptions of test parameters should be aligned with RAN1/RAN2 descriptions as much as possible to avoid ambiguous understanding. Option 1 and Option 2 is perferable. Option 3 prefers to use a formula-based or pseudo-code-based definition for FRCs instead of table-based approach listing every parameter combination. From our understanding, the formula method is simple, while table method is clear. Option 3 and Option 3A needs more discussion, from perspective of standard simplification, Option 3 and Option 3A is preferable.

Proposal 4: For demodulation specification principle, from perspective of standard simplification, Option 3 and Option 3A is preferable.

  - Option 3: For FRCs in 6G Demodulation specification, prefer to use a formula-based or pseudo-code-based definition for FRCs instead of table-based approach listing every parameter combination.

    - Option 3A: RAN4 needs to discuss how to specify FRC table in the specification for both BS and UE demodulation performance, considering the discussion in SI modernization of specification format and procedures for 6G.

ISAC study

As per the WF [1], the open issue concerning ISAC study is shown as below:

ISAC study - Proposals - Option 1: Study the demodulation for ISAC for 6G.

Generally, the demod for ISAC/sensing includes test parameter, channel model and test methodologies, including test procedure, measurement setup. The test parameter for ISAC/sensing is related to RAN1 progress, unlike channel model and test methodology for TN and NTN demodulation for 5G NR have been defined, channel model and test methodology for ISAC/sensing demodulation has not yet been defined. So RAN4 can study channel model and test methodology for ISAC/sensing demodulation for 6G. The channel model in 38.901 for sensing can be as starting point.

Proposal 5: Study channel model and test methodology for ISAC/sensing demodulation, channel model in 38.901 for sensing can be as starting point.

Conducted and radiated testing

As per the WF [1], the open issue concerning conducted and radiated testing is shown as below:

Conducted and radiated testing - Proposals - Option 1: Conducted test method can be considered for FR1 frequency range, and OTA test method can be considered for FR2 frequency range for both UE and BS demodulation requirements. - Option 2: Define test methodologies for FR3, including whether measurements are conducted or OTA.

From our understanding, conducted test method can be used for conducted requirement, and OTA test method can be used for radiated requirement. The conducted requirement and radiated requirement are dependent on BS and UE type. For NR FR1, the BS type 1-C, 1-H and 1-O were defined, so both conducted requirement and radiated requirement are defined, and both conducted and OTA test method are needed. For FR2, only radiated requirement is defined, so it is sufficient that OTA test method can be considered for FR2 frequency range for both UE and BS demodulation requirements. FR3 in Option 3 is not defined, so to discuss Option 2 is too earlier. Generally, conducted and OTA test methodologies for FR1 and FR2 can be applicable for FR3((7-24GHz). So the “OTA test method can be considered for FR2 frequency range for both UE and BS demodulation requirements” in Option 1 is preferable.

Proposal 6: For conducted and radiated testing, “OTA test method can be considered for FR2 frequency range for both UE and BS demodulation requirements” in Option 1 is preferable.

  - Option 1: Conducted test method can be considered for FR1 frequency range, and OTA test method can be considered for FR2 frequency range for both UE and BS demodulation requirements.

2.2 Channel models

Channel type

As per the WF [1], the open issue concerning channel type is shown as below:

Channel type - Proposals - Option 1: Clarify use of TDL and SCM models for 6G - Option 1A: Use rCDL baseline for MIMO. - Option 1B: Use CDL/rCDL as baseline for 6G. - Option 1C: Maintain TDL and CDL. - Option 1D: Continue TDL for simplicity. - Option 1E: Include xTDL. - Option 1F: Define default CDL propagation models. - Option 1G: Adoption of SCM should be justified for each test purpose. - Option 1H: Study practical MIMO correlation matrices for TDL. - Option 1I: Use rCDL baseline for MIMO and a limited number of TDL requirements for MIMO features with single layer transmission. - Option 1J: select one channel model (either TDL or CDL) for one specific feature. - Option 1Ja: The criteria of selection should be clarified and applied for all features.

In 5G NR, the TDL channel was defined for BS demod requirement and UE demod requirement, The SCM models (rCDL and xTDL )in TR 38.753 will be defined for UE demod requirements. From perspective of simplicity, it is reasonable to continue TDL for simplicity. For UE demodulation requirement, since The rCDL and xTDL in TR 38.753 can be more applicable for multi-port beamforming transmission of BS, so for channel type for UE  demod for 6G,  it is sufficient to select one channel model from TDL CDL, rCDL, xTDL for one specific feature, and criteria of selection should be clarified and applied for all features, For channel type for UE demod, Option 1J and 1Ja is reasonable. For channel type for BS demod, only 1Tx , 2Tx and 3Tx port for UE is defined, and multi-port beamforming transmission for UE is not necessary, so TDL channel can be as starting point.

So, we propose,

Proposal 7: For channel type for UE demod for 6G, Opion 1J and 1Ja is preferable.

    - Option 1J: select one channel model (either TDL or CDL) for one specific feature.

      - Option 1Ja: The criteria of selection should be clarified and applied for all features.

Proposal 8: For channel type for BS demod for 6G, Option 1D is preferable.

    - Option 1D: Continue TDL for simplicity.

Specialized propagation channels

As per the WF [1], the open issue concerning specialized propagation channels is shown as below:

Specialized propagation channels - Proposals - Option 1: Evaluate candidate channel model for DL and UL considering new use cases including AI, ISAC, NTN, HST. - Option 2: Initiate a similar study for NTN scenarios, focusing on the CDL-D variant to reflect the LOS-dominant nature of NTN links and enable more accurate performance evaluations for NTN systems under practical deployment scenarios.

For some special scenario case, such as HST, ISAC, AI, the normal TDL channel is not applicable, RAN4 need to study and evaluate candidate channel model for those, so option 1 is reasonable. For NTN scenario case, CDL and TDL model were defined in TR 38.811, for LOS-dominant nature of NTN links, the TDL-D channel is sufficient, since SAN does not have multi-port transmission beamforming like BS.

Proposal 9: For specialized propagation channels, Option 1 is preferable.

  - Option 1: Evaluate candidate channel model for DL and UL considering new use cases including AI, ISAC, NTN, HST.

Uplink aspects of channel model

As per the WF [1], the open issue concerning uplink sapects of channel model is shown as below:

Uplink aspects of channel model - Proposals - Option 1: Study UL CDL for BS variants. - Option 1a: Study UL CDL for BS variants if key issues in DL CDL study are settled.

From our understanding, the UE antenna number is significantly fewer than BS antenna number, for UE 1Tx and 2Tx port, whether to study UL CDL for 1Tx and 2Tx port requires more discussion, since UE does not have multi-port transmission beamforming like BS. In current stage, We can focus on DL link CDL study.

Observation 1: Whether to study UL CDL for BS variants needs more discussion since UE does not have multi-port transmission beamforming like BS and focus on DL link CDL study in current study.

2.3 Receiver assumptions

Receiver assumption for UE

As per the WF [1], the open issue concerning receiver assumption for UE is shown as below:

Receiver assumption for UE - Proposals - Option 1: MMSE-IRC as a baseline receiver. - Option 2: MMSE-IRC and R-ML as baseline receivers. - CMCC: with the prerequisite that the receiver is transparent to the network and does not require any PHY layer modification and additional assistance information. - Option 3: Cover advanced receivers (R-ML, soft-IC) - CT: Study the required information for advanced Rec for MU-MIMO. - Option 4: Study baseline and simplified structures. - Option 5: Study widely linear MMSE-IRC.

For 5G NR, the MMSE, MMSE-IRC and R-MLwere defined for UE demodulation requirement, for 6G UE demodulation, MMSE-IRC and R-MLcan be considered as baseline receivers. RAN4 can further consider new receiver for UE,  The major benefit for the usage of widely linear MMSE-IRC(WLMMSE-IRC) is to increase the detector’s degrees of freedom from NR to 2NR. Particularly, this holds strictly if both desired and interfering signals employ a real-valued modulation scheme. As a consequence, the interference suppression capability of linear interference-aware detector can be significantly enhanced, from NR -1 to 2NR -1, leading to an improved performance in interference limited scenarios. So for new receiver, option 5 is preferable.

Proposal 10: For receiver assumption for UE, Option 2 and Option 5 are preferable.

  - Option 2: MMSE-IRC and R-ML as baseline receivers.

    - CMCC: with the prerequisite that the receiver is transparent to the network and does not require any PHY layer modification and additional assistance information.

  - Option 5: Study widely linear MMSE-IRC.

Receiver assumption for BS

As per the WF [1], the open issue concerning receiver assumption for BS is shown as below:

Receiver assumption for BS - Proposals - Option 1: MMSE-IRC as a baseline receiver. - Option 2: Study feasibility of considering higher than 8Rx scenarios.

For 5G NR, MMSE is assumed as baseline receiver, and MMSE-IR is also defined for BS demodulation. For 6G BS demodulation, the MMSE-IRC can be considered as baseline receiver. As frequency increases, antenna number of BS increases. So to consider higher than 8Rx scenarios for BS demodulation is reasonable, so it is necessary to study feasibility of considering higher than 8Rx scenarios.

Proposal 11: For receiver assumption for BS, Option 1 and Option 2 are preferable.

  - Option 1: MMSE-IRC as a baseline receiver.

  - Option 2: Study feasibility of considering higher than 8Rx scenarios.

2.4 Interference modelling aspects

Interference profile

As per the WF [1], the open issue concerning interference profile is shown as below:

Interference profile - Proposals - Option 1: Study the interference profile for 6G DL/UL intra-cell and inter-cell interference scenarios. - Option 1A: RAN4 further evaluate interference profiles for intra-cell/interference cell scenarios: gNB and UE configuration e.g., power class, antenna configuration Homogenous and heterogenous scenarios Asynchronization TDD or dynamic TDD scenario Semi-static/Dynamic SBFD operation in gNB. - Option 1B: For 6G Demodulation with interference modelling, further discussion, and analysis on the modelling of directions, INRs, modulation orders of interference(s), number of layers from interference(s) are needed. - Option 2: Study possible interference caused by MRSS, by interference cancellation or mitigation.

For MMSE-IRC receiver, the interfence model is more curical. INR model were used for BS MMSE-IRC demodulation requirement and UE MMSE-IRC demodulation requirement. So for 6G demodulation, interference profiles for intra-cell/interference cell scenarios need to be evaluated, interference modelling needs discussion. So Option 1 is preferable. The interference cancellation or mitigation in NR DSS was studied, for 6G MRSS, it is needed to study possible interference caused by MRSS, and Option 2 is also preferable.

Proposal 12: For interference profile, the Option 1 and Option 2 are preferable.

  - Option 1: Study the interference profile for 6G DL/UL intra-cell and inter-cell interference scenarios.

    - Option 1A: RAN4 further evaluate interference profiles for intra-cell/interference cell scenarios: gNB and UE configuration e.g., power class, antenna configuration Homogenous and heterogenous scenarios Asynchronization TDD or dynamic TDD scenario Semi-static/Dynamic SBFD operation in gNB.

    - Option 1B: For 6G Demodulation with interference modelling, further discussion, and analysis on the modelling of directions, INRs, modulation orders of interference(s), number of layers from interference(s) are needed.

  - Option 2: Study possible interference caused by MRSS, by interference cancellation or mitigation.

2.5 Uplink demod

UE RF Impairment Modelling and Compensation

As per the WF [1], the open issue concerning UE RF impairement modelling and compensation is shown as below:

UE RF Impairment Modelling and Compensation - Proposals - Option 1: Beyond just the PA model, the entire UE RF front-end needs to be studied by RAN4, with particular attention to the potential variation in impairments across different UEs. - Option 2: Study feasibility of UE non-linearity estimation methods with reference signals or actual data and assess their suitability for supporting post-distortion and compensation techniques. - Option 3: Evaluate RF front-end variation from multiple UEs in the market into account and define suitable baseline/reference models for UL post-distortion studies. - Option 4: Evaluate UE PA non-linearity and related impairments across both FR1 and FR2 for UL-Post distortion compensation at BS receiver. Further, this would impact both BS and UE model.

From BS demodulation, the demodulation requirement is defined based on impairment result from companies, from our understanding, the impairment includes impairment of UE and BS, generally, the impairment margin is about 2.5dB. The UL-Post distortion compensation at BS receiver can significantly improve system performance, if RAN4 want to evaluate demodulation requirement for UL-Post distortion compensation at BS receiver, the UE PA non-linearity etc. need to be considered. All Options are related to PA model, so they are preferable.

Proposal 13: for UE RF impairment modelling and compensation, the all Options 1, 2, 3 and 4 are preferable.

  - Option 1: Beyond just the PA model, the entire UE RF front-end needs to be studied by RAN4, with particular attention to the potential variation in impairments across different UEs.

  - Option 2: Study feasibility of UE non-linearity estimation methods with reference signals or actual data and assess their suitability for supporting post-distortion and compensation techniques.

  - Option 3: Evaluate RF front-end variation from multiple UEs in the market into account and define suitable baseline/reference models for UL post-distortion studies.

  - Option 4: Evaluate UE PA non-linearity and related impairments across both FR1 and FR2 for UL-Post distortion compensation at BS receiver. Further, this would impact both BS and UE model.

Evaluation Methods and Simulation Models

As per the WF [1], the open issue concerning Evaluation Methods and Simulation Models for DPoD is shown as below:

Evaluation Methods and Simulation Models - Proposals - Option 1: Study CP-OFDM and DFT-s-OFDM for UL evaluation of this feature. - Option 2: Evaluate channel models for link-level simulation with DPoD feature, considering their impact on test metrics under higher UE transmit power.

The CP-OFDM and DFT-s-OFDM were defined for Uplink in 5G NR, if both are defined for 6G, the Option 1 is reasonable. For link-level simulation with DPoD feature, the channel model, test metric under higher UE transmit power are need to be considered. So generally, the Option1 and Option 2 are OK.

Proposal 14: For evaluation Methods and Simulation Models for DPoD, Option 1 and Option 2 are preferable.

  - Option 1: Study CP-OFDM and DFT-s-OFDM for UL evaluation of this feature.

  - Option 2: Evaluate channel models for link-level simulation with DPoD feature, considering their impact on test metrics under higher UE transmit power.

- Conclusion

This contribution provides analysis on 6G demod, and the following proposal is concluded as follows:

Proposal 1: For RAN4 demod study timeline, Option 1 is preferable.

  - Option 1: For RAN4 6G Demodulation, RAN4 establish a more realistic and structured timeline to ensure adequate depth in technical deliberation and a well-paced progression toward completion.

Proposal 2: For waveform and modulation study, Option 1, Option 2 and Option 3 are preferable.

  - Option 1: RAN4 6G Demodulation could start with CP-OFDM and DFT-s-OFDM waveforms for 6G uplink demodulation study, and CP-OFDM waveform for 6G downlink demodulation study.

  - Option 2: RAN4 6G Demodulation study should cover following modulation schemes at least

    - For downlink, QPSK, 16QAM, 64QAM, 256QAM and 1024QAM

    - For uplink with CP-OFDM waveform, QPSK, 16QAM, 64QAM, 256QAM

    - For uplink with DFT-s-OFDM waveform, pi/2 BPSK, QPSK, 16QAM, 64QAM, 256QAM

  - Option 3: Develop testable specifications for waveform, frame structure, channel coding, and modulation as defined by RAN1, ensuring that these can be practically implemented and validated in real products.

Proposal 3: For SCS for 6G demod, Option 1 can be as starting point.

  - Option 1: RAN4 6G Demodulation study could start with following SCS options.

    - For sub 6GHz, 15kHz SCS for FDD, 30KHz SCS for TDD

    - For around 7GHz, 30kHz, 60kHz

    - For around 15GHz, 60kHz

    - For between 24.25GHz - 52.6GHz, 120kHz

Proposal 4: For demodulation specification principle, from perspective of standard simplification, Option 3 and Option 3A is preferable.

  - Option 3: For FRCs in 6G Demodulation specification, prefer to use a formula-based or pseudo-code-based definition for FRCs instead of table-based approach listing every parameter combination.

    - Option 3A: RAN4 needs to discuss how to specify FRC table in the specification for both BS and UE demodulation performance, considering the discussion in SI modernization of specification format and procedures for 6G.

Proposal 5: Study channel model and test methodology for ISAC/sensing demodulation, channel model in 38.901 for sensing can be as starting point.

Proposal 6: For conducted and radiated testing, “OTA test method can be considered for FR2 frequency range for both UE and BS demodulation requirements” in Option 1 is preferable.

  - Option 1: Conducted test method can be considered for FR1 frequency range, and OTA test method can be considered for FR2 frequency range for both UE and BS demodulation requirements.

Proposal 7: For channel type for UE demod for 6G, Opion 1J and 1Ja is preferable.

    - Option 1J: select one channel model (either TDL or CDL) for one specific feature.

      - Option 1Ja: The criteria of selection should be clarified and applied for all features.

Proposal 8: For channel type for BS demod for 6G, Option 1D is preferable.

    - Option 1D: Continue TDL for simplicity.

Proposal 9: For specialized propagation channels, Option 1 is preferable.

  - Option 1: Evaluate candidate channel model for DL and UL considering new use cases including AI, ISAC, NTN, HST.

Observation 1: Whether to study UL CDL for BS variants needs more discussion since UE does not have multi-port transmission beamforming like BS and focus on DL link CDL study in current study.

Proposal 10: For receiver assumption for UE, Option 2 and Option 5 are preferable.

  - Option 2: MMSE-IRC and R-ML as baseline receivers.

    - CMCC: with the prerequisite that the receiver is transparent to the network and does not require any PHY layer modification and additional assistance information.

  - Option 5: Study widely linear MMSE-IRC.

Proposal 11: For receiver assumption for BS, Option 1 and Option 2 are preferable.

  - Option 1: MMSE-IRC as a baseline receiver.

  - Option 2: Study feasibility of considering higher than 8Rx scenarios.

Proposal 12: For interference profile, the Option 1 and Option 2 are preferable.

  - Option 1: Study the interference profile for 6G DL/UL intra-cell and inter-cell interference scenarios.

    - Option 1A: RAN4 further evaluate interference profiles for intra-cell/interference cell scenarios: gNB and UE configuration e.g., power class, antenna configuration Homogenous and heterogenous scenarios Asynchronization TDD or dynamic TDD scenario Semi-static/Dynamic SBFD operation in gNB.

    - Option 1B: For 6G Demodulation with interference modelling, further discussion, and analysis on the modelling of directions, INRs, modulation orders of interference(s), number of layers from interference(s) are needed.

  - Option 2: Study possible interference caused by MRSS, by interference cancellation or mitigation.

Proposal 13: for UE RF impairment modelling and compensation, the all Options 1, 2, 3 and 4 are preferable.

  - Option 1: Beyond just the PA model, the entire UE RF front-end needs to be studied by RAN4, with particular attention to the potential variation in impairments across different UEs.

  - Option 2: Study feasibility of UE non-linearity estimation methods with reference signals or actual data and assess their suitability for supporting post-distortion and compensation techniques.

  - Option 3: Evaluate RF front-end variation from multiple UEs in the market into account and define suitable baseline/reference models for UL post-distortion studies.

  - Option 4: Evaluate UE PA non-linearity and related impairments across both FR1 and FR2 for UL-Post distortion compensation at BS receiver. Further, this would impact both BS and UE model.

Proposal 14: For evaluation Methods and Simulation Models for DPoD, Option 1 and Option 2 are preferable.

  - Option 1: Study CP-OFDM and DFT-s-OFDM for UL evaluation of this feature.

  - Option 2: Evaluate channel models for link-level simulation with DPoD feature, considering their impact on test metrics under higher UE transmit power.

- References

- R4-2514645, WF on [116bis][106] 6G Demod, RAN4#116bis, Feature lead (MediaTek inc.)