3GPP TSG-RAN WG4 Meeting #117	                                                                                            R4-2520166

Dallas, US, 13th –17th Oct, 2025

Source: 	CATT

Title: 	Discussion on remaining issue for A-IoT BS

Agenda item:	4.20.1.2

Document for:	Approval

- Introduction

In RAN4 #116-bis meeting, a maintenance CR [1] for ambient IoT BS and CW spec TS 38.194 has been approved. However, there remain some issues that require further clean up. This paper shares some views on maintenance for ambient IoT BS spec.

- Discussion

Typically, for multiple in-band or in-channel Rx requirements (such as ACS/blocking), an interfering signal should be placed at adjacent channel during the testing. The types of interfering signals will be specified in the table of requirements, whilst the characteristics of interfering signals are typically captured in the annex. Taking ACS requirements as an example, the relevant elements related to interfering signals are highlighted in yellow shades as follows:

### 7.3.1 Adjacent Channel Selectivity #### 7.3.1.1 General Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency at the antenna connector for BS type 1-C in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system. #### 7.3.1.2 Minimum requirement for BS type 1-C The MDR performance shall be [1%] of the reference measurement channel. The wanted and the interfering signal coupled to the BS type 1-C antenna connector are specified in table 7.4.1.2-1 and the frequency offset between the wanted and interfering signal in table 7.4.1.2-2 for ACS. The reference measurement channel for the wanted signal is identified in table 7.2.2-1, 7.2.2-2 and 7.2.2-3 for each BS D2R channel bandwidth in any operating band and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D. The ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges. Minimum conducted requirement is defined at the antenna connector for BS type 1-C. Table 7.3.1.2-1: Base station ACS requirement A-IoT channel bandwidth of the lowest/highest carrier received [kHz]; Wanted signal mean power [dBm]; Interfering signal mean power [dBm]; Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge [kHz]; Type of interfering signal. 200; PREFSENS + 6dB (Note); -53; ±100; 3 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal. 3520; PREFSENS + 6dB (Note); -53; ±100; 3 MHz DFT-s-OFDM NR signal. Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1. Table 7.3.1.2-2: Base Station ACS interferer frequency offset values BS channel bandwidth of the lowest/highest carrier received (kHz); Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz); Type of interfering signal. 200; ±100; 3 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal. 3520; ±100; 3 MHz DFT-s-OFDM NR signal.

However, TS 38.194 does not include relevant annex D to specify the characteristics of the interfering signal and should be supplemented accordingly.

Observation 1: The A-IoT BS spec TS 38.194 lacks an annex which specifies the characteristics of the interfering signal.

Meanwhile, for type of interfering signal, the signal type/SCS/number of RBs should be specified. Regarding the A-IoT BS ACS requirements, the interfering signal types for ACS requirements at 3520 kHz CBW lack SCS and number of RB, which should be supplemented accordingly.

Observation 2: The A-IoT BS spec TS 38.194 lacks the SCS and number of RB for the type of the interfering signal of the ACS requirements.

In addition, regarding the Rx requirements for A-IoT BS, there are some inconsistencies in the description of the BS channel bandwidth under different Rx requirements. For instance, the BS CBW description for A-IoT BS REFSENS are shown as below:

Table 7.2.2-1: A-IoT Medium range BS reference sensitivity levels BS D2R channel bandwidth (KHz); DSB (kHz); Reference measurement channel; Reference sensitivity power level, PREFSENS (dBm). 200; 15; A-FR1-A1-1; -95.2. A-FR1-A1-2; -92.2. 3520; 2880; A-FR1-A1-3; -72.4. A-FR1-A1-4; -69.4. NOTE: Reference sensitivity power level is defined based on the CW power at the BS antenna connector as -38dBm without the cancellation of CW phase noise considered..

However, for ACS/IBB/intermodulation requirements, the descriptions of BS CBW are not aligned:

Table 7.6.2-1: Narrowband intermodulation performance requirement for A-IoT Medium Range BS Channel bandwidth of the lowest/highest carrier received [kHz]; Wanted signal mean power [dBm]; Interfering signal mean power [dBm]; Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz]; Type of interfering signal. 200; PREFSENS + 6dB*; -53; ±340; CW. -53; ±880; 5MHz E-UTRA signal, 1 RB**. 3520; PREFSENS + 6dB*; -53; ±270; CW. -53; ±780; 3.0 MHz E-UTRA signal, 1 RB**. Note*: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1. Note**: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge..

To ensure the above description better aligns with 3GPP conventions and keeping good alignment, we believed that the approach in TS 38.104 could be considered as a template. For REFSEN, the description of the BS D2R channel bandwidth may be adopted. For other Rx requirements associated with REFSENS, the description of the BS D2R channel bandwidth for the lowest/highest received carrier may be adopted, in accordance with the TS 38.104 approach.

Observation 3: For A-IoT BS spec TS 38.194, the descriptions of the BS D2R channel bandwidth for some Rx requirements are not aligned.

- Conclusion

This contribution provides our views on maintenance for ambient IoT BS spec. The following proposals are concluded as follows:

Observation 1: The A-IoT BS spec TS 38.194 lacks an annex which specifies the characteristics of the interfering signal.

Observation 2: The A-IoT BS spec TS 38.194 lacks the SCS and number of RB for the type of the interfering signal of the ACS requirements.

Observation 3: For A-IoT BS spec TS 38.194, the descriptions of the BS D2R channel bandwidth for some Rx requirements are not aligned.

- References

- R4-2515143, CR for TS 38194 on A-IoT BS RF requirements, RAN4 #116-bis