3GPP TSG-RAN WG4 Meeting #117                                 R4-2520300

Dallas, USA, Nov. 17 – 21, 2025

Agenda Item:	4.2.8

Source: 	Murata Manufacturing Co Ltd.

Title: 	MSD of CA_n29A-n71A with n71 UL 25 MHz

Document for:	Approval

Introduction

In the RAN4#116-bis meeting, one company proposed that since BCS4/5 has been introduced in CA_n29A-n71A, as shown in the following table, n71 UL 25 MHz should be applied to this band combination. In this paper, we provide a brief analysis of the Tx noise of n71 UL 25 MHz and the PC3 MSD for n29 DL.

Table 5.5A.3.1-1h: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n29A-n71A17 | n718 | n29 | 5, 10 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | - | n29 | n29 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |

Discussion

Tx noise

The frequency separation between n71 UL and n29 DL is narrow ---- only 19 MHz, as shown in Figure 1. In the case of n71 UL at 25 MHz, the IM3 product of the Tx signal and IQ image falls into n29 DL, and as a result, the Tx noise at n29 DL increases significantly. As our evaluation, the Tx noise of UL 25 MHz is 18.4 dB higher compared to UL 20 MHz.

Figure 1. CA_n29A-n71A frequency relationship

Figure 2. n71 f=685.5MHz/BW=25MHz@RB(20, 113), n29 f=719.5MHz/5MHz

Observation 1: The Tx noise of n71 UL 25 MHz is 18.4 dB higher than that of UL 20 MHz due to the IM3 product of the Tx signal and IQ image falls into n29 DL.

MSD proposal

Using the delta in Tx noise between UL 25 MHz and UL 20 MHz, we estimated the PC3 MSD for UL 25 MHz based on the existing MSD for UL 20 MHz. The result is highlighted in Table 1.

For the PC2 MSD, it can be determined via the LUT of HPUE [1] once the PC3 MSD has been determined.

Table 1. PC3 MSD for CA_n29A-n71A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n29 | 688 | 20 | 15 | 20 (RBstart=86) | 719.5 | 5 | 17.5 | ACLR2 |
| n71 | n29 | 685.5 | 25 | 15 | 20 (RBstart=113) | 719.5 | 5 | 35.8 | ACLR1 |

Proposal 1: RAN4 to consider the MSD value highlighted in Table 1 for PC3 CA_n29A-n71A.

Proposal 2: RAN4 to determine the PC2 MSD via the LUT of HPUE [1] once the PC3 MSD has been determined.

Conclusion

In this paper, we provide an analysis of the Tx noise of n71 UL 25 MHz and the PC3 MSD for n29 DL.

Observation 1: The Tx noise of n71 UL 25 MHz is 18.4 dB higher than that of UL 20 MHz due to the IM3 product of the Tx signal and IQ image falls into n29 DL.

Proposal 1: RAN4 to consider the MSD value highlighted in Table 1 for PC3 CA_n29A n71A.

Proposal 2: RAN4 to determine the PC2 MSD via the LUT of HPUE [1] once the PC3 MSD has been determined.

Table 1. PC3 MSD for CA_n29A-n71A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n29 | 688 | 20 | 15 | 20 (RBstart=86) | 719.5 | 5 | 17.5 | ACLR2 |
| n71 | n29 | 685.5 | 25 | 15 | 20 (RBstart=113) | 719.5 | 5 | 35.8 | ACLR1 |

References

[1] R4-2505143, “WF on HPUE MSD simplification”, Skyworks/Qualcomm/Murata/…, 3GPP TSG-RAN WG4 Meeting #114bis, Apr 7th – 11th, 2025