3GPP TSG-RAN4 Meeting #117	R4-2520314

# Dallas, US, Nov. 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6049 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Ku-band NTN RRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR is a resubmission of the previously endorsed contribution R4-2515167 from the last meeting. Clarify terminology and complete missing definitions for FR2 satellite switching requirements, and align exception handling consistently between hard and soft switch. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The requirements for FR2 satellite switching were updated for consistency and completion. - replace “cell” with “satellite” in 6.1C.3.3 - include ssb-TimeOffset alongside t-ServiceStart-18 in the SI condition for the exception, and change “is acquired by” to “are available for” in 6.1C.3.3.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The core requirements for FR2 satellite switching will remain incomplete and ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of change>>

#### 6.1C.3.3	NR SAN FR2 – NR SAN FR2 Satellite switching with re-synchronization

The requirements in this clause are applicable to both hard and soft switch over quasi-earth fixed scenario from FR2-NTN satellite to FR2-NTN satellite. The requirements in this clause apply provided that UE has the valid and applicable parameters of ephemeris information, common TA, DL and UL Polarization information, Koffset, and Kmac for target NR SAN satellite during Dswitch_unchangedPCI, otherwise interruption time may be longer than the requirements in clause 6.1C.3.3.2 for hard satellite switch and satellite switch delay may be longer than the requirements in clause 6.1C.3.3.3 for soft satellite switch.

Requirements for soft satellite switching are applicable for UEs that support softSatelliteSwitchResyncNTN-r18 [14] when network configures soft satellite switching with resynchronization [2].

Requirements for hard satellite switching are applicable for UEs that support hardSatelliteSwitchResyncNTN-r18 or softSatelliteSwitchResyncNTN-r18 [14] when the network configures hard satellite switching with resynchronization [2]; or for UEs that support hardSatelliteSwitchResyncNTN-r18 but do not support softSatelliteSwitchResyncNTN-r18 when the network configures soft satellite switching with resynchronization.

Note: UE indicating ‘mechanical’ via UE capability ntn-VSAT-AntennaType-r19 is not expected to indicate support of softSatelliteSwitchResyncNTN-r18 [14].

##### 6.1C.3.3.1	Satellite switching delay

The requirements in clause 6.1C.3.2.1 shall apply except that

-	clause 6.1C.3.2.2 is replaced with clause 6.1C.3.3.2, and

-	clause 6.1C.3.2.3 is replaced with 6.1C.3.3.3.

##### 6.1C.3.3.2	Interruption time for hard satellite switch with re-sync

The requirements in clause 6.1C.3.2.2 shall apply except that Tinterrupt is replaced with

Tinterrupt = Tsat_beam +Tsearch + Tprocessing + T∆ + Tmargin ms

where:

-	Tsearch, Tprocessing, T∆ and Tmargin are as defined in clause 6.1C.3.2.2,

-	Tsat_beam is additional time for UE to steer the downlink spatial domain reception filter to the target satellite.

-	For UE indicating ‘electronic’ via UE capability ntn-VSAT-AntennaType-r19, Tsat_beam is 3*TSSB, where TSSB is the periodicity of the SSB of the source satellite.

-	For UE indicating ‘mechanical’ via UE capability ntn-VSAT-AntennaType-r19, Tsat_beam is Oangle / 22.5 s, where Oangle is the angle offset observed from UE in degree between the satellite for the serving cell and the satellite for the target cell.

##### 6.1C.3.3.3	Satellite switch delay for soft satellite switch with re-sync

The requirements in clause 6.1C.3.2.3 shall apply except that

-	clause 9.2C.5.3 is replaced with clause 9.2C.7.3, and

-	Tsoft_switch is replaced with

Tsoft_switch = max(t-service-t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing  ms

where

-	Tsearch, Tprocessing, T∆ and Tmargin are as defined in clause 6.1C.3.2.3,

-	Tsat_beam is additional time for UE to steer the downlink spatial domain reception filter to the target satellite.

-	For UE indicating ‘electronic’ via UE capability ntn-VSAT-AntennaType-r19, Tsat_beam is 3*TSSB, where TSSB is the periodicity of the SSB of the source satellite.

In addition, following scheduling restriction applies during the time period from t-serviceStart to t-service,

-	If deriveSSB-IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and KRX_switch data symbol before and after each consecutive SSB symbols to be measured within SMTC window duration.

-	If deriveSSB-IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration, and KRX_switch data symbol before and after SMTC window duration.

- 	where KRX_switch is for the switch period of UE to steer the downlink spatial domain reception filter toward a target cell. KRX_switch = 1.

The requirements in this clause do not apply if the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-ServiceStart-r18 and ssb-TimeOffset are available for the UE for the first time to the first slot corresponding to t-ServiceStart-r18 is less than 1s.

<<End of change>>