3GPP TSG-RAN WG4 Meeting #117                                                               R4-2520373

Dallas, USA, 17th – 21st November, 2025

Source:	Samsung

Title:	Discussion on AI framework and use cases for 6GR

Agenda item:			8.8.2

Document for:	Approval

# 1. Introduction

In RAN#108, RAN plenary approved a new RAN working group level SID on 6G Radio [RP-251881], which has been further revised into [1, RP-252912] in RAN#109. Specifically, AI/ML is the indispensable enabler for 6G RAN, for which at least AI/ML framework and new AI/ML use cases are highlighted as key aspects to be studied, with objectives provided as follows [1]:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

Based on RAN4#116bis discussion, the WF is approved on 6G AI/ML, in which the principle of RAN4-driven use cases selection and the aspects of RAN4-driven use cases to be studied are agreed [4]. Additionally, the aspects of RAN4-driven use case to be studied

In this contribution, we would like to provide our viewpoints on RAN4 driven use cases.

# 2. RAN4-driven AI/ML use cases for RF/Demod

## 2.1 Use Case-1: AI/ML-enabled non-linearity compensation

2.1.1 Motivation and use case description

In general, the ever-growing data rate demands in mobile communications motivate the exploitation of higher order modulations (HOM) such as 256QAM and 1024QAM. At a given transmitter-receiver distance, the HOM requires larger transmission power than QPSK, 16QAM, and 64QAM due to its reduced distance between constellation points, which results in smaller coverage for HOM. A natural way to improve the HOM coverage would be to increase the transmission power level. However, the possible increase of transmission power level is limited by the maximum power that can be provided by the power amplifier (PA). To maximize the HOM coverage, it is expected that the PA output power should be as close to its saturation level as possible. This would, however, result in severe performance loss due to the nonlinear distortion caused by typical characteristics of radio frequency (RF) components such as PA. It is clear that such non-linear distortion needs to be compensated at either transmit side (TX) with pre-distortion mechanism or receive side (RX) with equalization to ensure block error rate (BLER) or coverage requirements for HOM.

To leverage HOM, it is essential to develop a more effective scheme to compensate the nonlinear distortion. One alternative is to embrace artificial intelligence (AI), which has been recognized as a promising tool to deal with complicated issues in many fields.

AI-enabled non-linearity compensation (AI-NC) at RX side is proposed, which can support fast online training without additional overhead.  The receiver block diagram of AI-NC is shown in Figure 1.

Rx block diagram of AI/ML-enabled non-linearity compensation

Figure 1is a schematic of AI-NC receiver structure with NRx-antenna. The received signal of each RX antenna is transformed into frequency domain by using FFT. Then conventional CE is performed based on local frequency domain demodulation reference symbol (DMRS) to estimate channel coefficients per RX antenna. By using frequency domain RX data and estimated channel coefficients of each RX antenna, conventional MIMO equalization is performed to distinguish TX data streams as well as remove the impact caused by multipath channel.

After conventional frequency domain MIMO equalization, neural network is applied to compensate the non-linear distortion. As non-linear distortion of PA is generated and introduced in time domain, using time-domain neural NW is a better way compared with frequency domain processing. Meanwhile, our investigation and analysis show that if frequency domain processing is applied, training overhead and computational complexity will be much larger compared with a time-domain neural NW with similar performance. As a result, after conventional frequency domain CE and MIMO equalization, IFFT is applied per data stream for time domain processing of neural NW. In addition, one high order term construction is applied to derive the 3rd term (e.g., the so-called high order term) of the time domain signal samples. Per our analysis, such term can further improve the performance of using ESN.

Then neural network is utilized to do the online training based on time domain DMRS samples and inference based on RX data samples after training to compensate non-linear distortion. After that, the resulted signal is transformed into frequency domain again and subsequent procedures, including de-modulation and decoding, are performed to retrieve the transmitted bits as convention. To summarize the model structure and training related information in Table 1.

Model structure and training related information for AI/ML-enabled non-linearity compensation

| Model location | Network side |
| --- | --- |
| Model input | Training: estimated RS signal time domain sample with 1st, 3rd terms Inference: estimated data signal time domain sample with 1st, 3rd terms |
| Model output | Training Label: original/known RS signal Inference Output: data signal with non-linear distortion compensated |
| Training types | Online training |
| Collaboration between UE and NW | no collaboration/interaction |

Note: the details model input format related with time domain sample with 1st, 3rd terms can be referred in equation (3) and (4) as the following details of neural network design of section 2.1.1.1.

2.1.1.1 Details of neural network design

In our proposed AI/ML-enabled non-linearity solution, the ESN solution with some enhancements was applied for model training. The ESN can be regarded as a simplified recursive neural network (RNN) and consists of input layer, middle layers and output layer, the coefficients for input layer and middle layer are generated before training phase based on predefined rule and only coefficients for output layer are updated in training phase. Such property significantly reduces the training complexity and overhead, making fast online training possible. To make ESN better fitting for the desired functionally in realistic wireless communication system. The enhancements include additional buffer, noise variance estimation, as well as the high-order construction with following considerations:

- Input buffer is introduced even though conventional channel equalization is performed before ESN. The benefit of function of input buffer, one is to deal with non-flat equivalent channel due to the fading channel and non-linear distortion. Another one is to deal with merry effect considering the memory effort may cause correlation between adjacent samples in the real PA

- Noise variance is estimated during training phase. For coded system, noise variance is important since its accuracy has impact on LLR calculation in the following soft decoding. Using training phase to estimate noise variance can provide much better estimation accuracy and can reduce implementation complexity by removing conventional noise variance estimation module

- High-order construction additionally used as input, which provides information of high order non-linear characteristic. It can consider as priori information for training, can further improve the performance

The details can be expressed as follows in [7]. Assuming Nn as the number of nodes in middle layer, the Lin denotes the input buffer length. For time index   t, the state vector is m[t]∈CNn×1, then the state update function is expressed as follows

mt+1=1-αmt+αtanh⁡(Amt+Winut+ε)                                                (1)

Where A∈CNn×Nn denotes the weight matrix for middle layers. Win∈CNn× Lin is the is the weight matrix between input layer and middle layers. Both A  and Winare randomly generated and fixed during receiving process. α∈[0,1] denotes weighting factor and ε denotes the offset vector. The state update is applied for both training phase and inference phase. For training phase, the time-domain samples of DMRS after channel equalization are used. After the processing of state update function with all the training samples state matrix for training phase can be obtained with the following optimization issue

min|Y-WoutMRS|                                                                   (2)

Where A ∈C1×Ntraining denotes the vector of local time-domain DMRS samples, which is used as label in training process. Wout∈C1×Nn denotes the weighted matrix between middle layers and output layer, which is regarded as training target. Ntraining  is the number of samples used for training. And ut is the input with including additional buffer and high-order terms in time domain as following

ut=[u1t,⋯,uTxt  ]T                                                            (3)

Where

uit=[uit-(Lin-1),⋯,uit, ui3t-(Lin-1),⋯, ui3t]T                 (4)

In our proposed solution and evaluation, the 3rd high-order is considered. For the inference phase, the samples of received time-domain data symbols after channel equalization are input to the trained network right after fast online training to get state matrix MDATA∈CNn×Nsymbol for inference, where Nsymbol denotes the number of data samples. Then the estimated data is calculated as follows

Rest=WoutMDATA                                                                            (5)

2.1.2 Preliminary evaluation results

2.1.2.1 Comparison with legacy receiver without compensation

Link level evaluations have been conducted to verify the ability of proposed AI-NC scheme. The performance is evaluated by BLER with different number of layers/users, different numbers of RX antennas, and multiple EVM values. For current results, the benchmark is the normal reception schemes without distortion compensation.

The evaluation assumptions are listed in Table 2. Note that each TX layer corresponds to 1 TX antenna. Both one TX layer (1T) and two TX layers (2T) are evaluated. For 2T case, frequency domain multiplexing (FDM) is considered in the evaluation, which means that 2 layers occupy two sets of adjacent frequency resources and out-of-band non-linear interference is concerned and handled by AI-NC. It should be noted that one layer can be regarded as one user for evaluating multi-user case equivalently. NR frame structure with 10 data symbols and 2 DMRS symbols per slot is applied in LLS evaluation. For conventional scheme, 2 DMRS symbols are used for linear CE and equalization. For proposed AI-NC scheme, after linear CE and EQ, the first DMRS symbol is reused for neuron network training.

Configuration of link level simulation

| Parameters | Value |
| --- | --- |
| Carrier frequency | 3.5 GHz |
| # of TX layers/users | 1/2 |
| # of RX antennas | 4/8/64 |
| Waveform | NR based |
| # of PRB | 50 |
| # of symbol per slot | 12 |
| # of DMRS per slot | 2 |
| # of DMRS re-used for training | 1 |
| DMRS subcarrier position | Odd index (0 for even index) |
| FFT size | 2048 |
| CP length | 144 |
| Modulation | 256 / 1024/ 4096 QAM |
| Coding rate | LDPC 0.9395 |
| Transport block size | 45120 for 1Tx/256 QAM 56392 for 1Tx/1024 QAM 67680 for Tx/4096 QAM 90200 for 2Tx/256 QAM |
| Channel model | TDL-A 30ns |
| Subcarrier spacing | 15 kHz |
| Evaluation metric | BLER |
| Benchmark | Conventional receiver (without non-linearity compensation) GMP-NC(with non-linearity compensation ) |

The evaluation assumptions are listed above. Note that each TX layer corresponds to 1 TX antenna. Both one TX layer (1T) and two TX layers (2T) are evaluated. For 2T case, frequency domain multiplexing (FDM) is considered in the evaluation, which means that 2 layers occupy two sets of adjacent frequency resources and out-of-band non-linear interference is concerned and handled by AI-NC. It should be noted that one layer can be regarded as one user for evaluating multi-user case equivalently. NR frame structure with 10 data symbols and 2 DMRS symbols per slot is applied in LLS evaluation. For conventional scheme, 2 DMRS symbols are used for linear CE and equalization. For proposed AI-NC scheme, after linear CE and EQ, the first DMRS symbol is reused for neuron network training.

Figure 2(a) shows the results of 256QAM with 4 and 64 RX antennas. Several EVM values are considered to observe the performance of AI-NC compared with conventional linear scheme. It should be noted that 3.5% is the minimum EVM requirement for 256QAM and 2.5% is for 1024QAM specified by RAN4 in 5G NR. We relax those two values to verify the capability of AI-NC to compensate non-linearity with larger EVM values. For 1T case, the performance is good enough even without high-order term construction. As a result, high-order term construction is turned off. From those results, it can be seen that with 4 RX antennas, the performance gain of proposed AI-NC over conventional scheme gets larger with the increase of EVM, from less than 1dB with 2% EVM to around 2dB with 3.5% EVM at BLER of 10%. When EVM increases to be 6% and 8%, proposed scheme can still achieve BLER performance of 10% at SNR of 24.4 dB and 27.2 dB, respectively. However, conventional scheme fails to deal with such serious non-linear distortion with EVM of 6% and 8%, so the corresponding curves are not shown in the figure. If the number of RX antennas increases to 64, more obvious gain can be observed. Even with small EVM value of 2%, the performance gap between the proposed and conventional scheme at BLER of 10% can reach 3.6 dB.

Figure 2(b) shows evaluation results of 1024QAM with 4 and 64 RX antennas. For 2.5% EVM with 4 RX antennas, conventional scheme fails to decode within the evaluated SNR range, while the performance of proposed scheme can meet the typical 10% BLER requirement within a reasonable SNR range. Even with 1.5% EVM with 4 RX antennas, more than 1dB gain can be observed compared with convention scheme. If the number of RX antennas increases to 64, conventional scheme fails to decode 1024QAM within the evaluated low SNR range but the proposed scheme can support 1024QAM reaching 10% BLER performance.

Figure 2(c) shows evaluation results of 4096QAM with 4 RX antennas. Note that current wireless communication systems do not support such high modulation order. The results show that with proposed AI-NC, the performance gain over conventional scheme is around 1.8 dB with 1% EVM. Meanwhile, if EVM increases to 1.5%, conventional scheme failed to decode within evaluated SNR range while AI-NC still shows acceptable performance.

- AI-NC provides 1-2dB improvement at 10% BLER when using 256QAM, 1024QAM and 4096QAM for the given EVM values.

Evaluation results for (a)256QAM, (b)1204 QAM, and (c)4096QAM

2.1.2.2 Comparison with GMP-NC receiver

In the existing study of non-linearity handing, there is some non-AI method to compensate the non-linearity. For example, the GMP-NC [8] is trying to modeling the PA nonlinearity. However, based on our study, such kind method highly relies on the purity of the non-linearity source, e.g., only coming from single source that can be modelled.  But in the reality, the situation is not that ideal, many aspects will contribute the overall non-linearity experienced by the signal. Thus, in this evaluation, I/Q imbalance is added on top of typical PA non-linearity [6]. The modelling of I/Q imbalance is listed in the table, which are basically two dimensions of uniformly distributed random value.

BLER performance comparison with non-AI NC (i.e., GMP-NC) method

Evaluation step for comparison with AI based non-linearity and non-AI based non-linearity compensation

| Parameters | Value |
| --- | --- |
| Carrier frequency | 3 GHz |
| SCS | 15 kHz |
| BW | 11.88 MHz |
| Code rate | 0.92 |
| Modulation | 256 QAM |
| Antenna Config. | 1T1R |
| Channel | TDL-D |
| Input buffer size (AINC & MPNC) | 2 |
| # of reservoirs | 30 |
| Order (AINC & MPNC) | 5 |
| Non-linearity factor | PA non-linearity Rx EVM=4.5%, corresponds to additional I/Q imbalance (Amp imbalance ~ [0,0.1dB], phase imbalance ~ [-5, 5 degree]) Rx EVM=6%, corresponds to additional I/Q imbalance (Amp imbalance ~ [0,0.5dB], phase imbalance ~ [-5, 5 degree]) |

Based on BLER performance comparison, it can be observed that

- When nonlinearity is dominated by PA component, e.g., I/Q imbalance is slight (Rx EVM=4.5% case). For a memoryless PA, GMP-NC can already fit PA model so that AI-NC improves only a few over GMP-NC

- When nonlinearity is contributed by both of I/Q imbalance and nonlinear PA, e.g., Rx EVM=6% case. In such case, GMP-NC exhibits obvious under-fitting while the nonlinearity can still be well-canceled by AI-NC.

2.1.3 Study focus in RAN4

The following Note, including a check point, was captured in Rel-20 WID discussion (New WID: UE RF enhancements for NR FR1, Phase 5) [2] during RANP#109 as the guidance for RAN4 work. Motivated by the potential commonality of non-linearity modeling of transmission signals and feasibility study steps, which are likely to be agnostic of 5G-A and 6G solutions, the group decided to combine the 5G and 6G discussion into one track to avoid the duplicated efforts and inconsistence. Specifically, it was emphasized that the 5G-A solution should be non-AI based and should not be any impact to RAN1. In other words, for 6G solution, new waveforms and/or AI-based solution can be considered.

In our view, the spirit of this study is to increase the possibility of low-end PA transmitting higher power for higher modulation schemes, with advanced compensation solutions implemented at BS side. Therefore, we believe the non-linearity model(s) selected to evaluate BS compensation capabilities and UE MPR reduction limit should also account other RFIC impairments including at least I/Q imbalance, other factors such as fractional timing error are not precluded. In addition, to simplify the discussion considering the time constrains, it is advisable to prioritize FR1 single CC operation, for PC3 with 1Tx and PC2 with both 1Tx and 2Tx. Further, selecting CP-OFDM as the waveform is recommended which is suitable for both Layer-1 and Layer-2.

- The goal of the work in 6G is to enable low-end PA and extreme high modulation to transmit higher power, leveraging the capabilities of advanced BS receiver.

2.1.3.1 General assumption for both AI and Non-AI solutions

In accordance with the chair's guidance, discussions under the UE RF agenda should focus solely on non-AI-related UE RF topics, while AI-related matters are to be addressed in the AI agenda. However, it is our understanding that certain aspects, such as the non-linearity model(s) selected for evaluation purpose and steps to confirm feasibility, can be determined in UE RF session. These are agnostic to AI/Non-AI solutions and can help best organize the study. Additionally, by assuming the same waveform and non-linearity model(s), results for both AI and non-AI solutions can be effectively compared. This approach ensures a more organized and efficient study process.

At the initial stage of 6G discussion, while the discussion on the new waveform and new PA modeling for 6G are going, it is advisable to prioritize the study of AI and non-AI solutions based on 5G waveforms and 5G PA models. Once 6G waveforms and 6G PA models are agreed, they can be incorporated into the study.

- Non-linearity model(s) of transmission signals (selected for the evaluation of BS compensation capabilities and UE MPR reduction limits) can be determined in 6G UE RF session, which should be common for BS compensation with AI and Non-AI solutions.

To evaluate the effectiveness of proposed techniques related to nonlinearity, PA models in TR 38.803 can be considered for evaluation purpose, e.g., Rapp model and generalized memory-based polynomial (GMP) model, in conjunction with modelling of other non-ideal factors including I/Q imbalance, fractional timing error and so on. Our detailed proposals are provided in our accompanying Tdoc [6].

2.1.3.2 Working steps for feasibility study for both AI and Non-AI solutions

To efficient the complex discussion, it is essential to establish clear criteria for confirming feasibility. We propose to agree on the following steps as below. As detailed below, the feasibility study involves extensive demodulation work.

- Adopt the following working steps for feasibility study for both AI and Non-AI solutions.

  - Study item phase (demod centric)

    - Step-1: Selection of Non-linearity model(s) of transmission signals

- UE RF session firstly to determine the non-linearity model(s)

    - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

      - Note: For feasibility study, demodulation relevant evaluation is performed in UE RF session for non-AI based solution, and demodulation relevant evaluation is performed in AI session for AI based solution

- Including the non-linearity model(s) into the RAN4 PUSCH demodulation test case(s) for both 64QAM and 256QAM

- For a given Tx EVM value derived based on the non-linearity models, evaluate the performance with following assumptions on selected RAN4 test cases to find the target SNR to meet RAN4 requirement or test metric, e.g., 70% of maximum throughput or 10% BLER

  - Performance evaluation

    - Case 1: Conventional receiver including non-linearity TX model: SNR-a

    - Case 2: Non-AI based non-linearity receiver including non-linearity TX model: SNR-b

    - Case 3: Conventional receiver without any non-linearity TX model: SNR-c

    - Case 4: AI based non-linearity (AI-NC) receiver with non-linearity TX model: SNR-d

    - Note: Case 3 is considered as upper bound performance, Case 1 is expected to be low bound performance

  - Performance comparison

    - Compare SNR-d and SNR-c, the results should be comparable

      - FFS the criteria to be claimed as “comparable” (e.g., within certain x dB), and the target SNR-c should be feasible for network deployment

    - Compare SNR-d and SNR-b, for AI-NC advantage over non-AI receiver

- Perform evaluation with different Tx EVM values, find/determine the maximum Tx EVM with the target SNR-d derived based on Case 4 to meet RAN4 requirement or test metric and comparable with the target SNR-c derived based on Case 3.

  - Perform different PA models, if needed

  - WI phase (UE RF centric)

    - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation and elevate the Net Gain :

  - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

  - For instance for 256QAM Tx EVM, 3.5% -> 6%,

    - For selected PA model(s), evaluate the allowed TX power boosting/gain (i.e., MPR reduction) for different power class

    - Net Gain [dB] = (Tx power boosting/gain relative to reference) − (SNR degradation relative to reference at 10% BLER)

    - FFS the necessity for new EVM model for RAN4 demodulation test cases.

Note: Above approaches are applicable for both 5G waveforms and 6G waveforms

2.1.3.3 Summary

In order to facilitate the RAN4 study on RAN4-driven use cases, it is benefit to provide the performance benefit, specification impact and evaluation assumption. The following aspect related with AI/ML-enabled non-linearity compensation was proposed

| Use Case 1# |  | AI/ML-enabled non-linearity compensation |
| --- | --- | --- |
| AI model input | Input in training | The estimated RS signal time domain sample with 1st, 3rd terms |
|  | Input in inference | The estimated data signal time domain sample with 1st, 3rd terms |
| AI model output | Label in training (if applicable) | original/known RS signal |
|  | Output in inference | data signal with non-linear distortion compensated |
| Assumption on training | Training type | Online training/finetune |
|  | Label construction (if applicable) | DMRS with 1st, 3rd terms |
| model location for inference |  | NW-sided model |
| Collaboration/interaction between UE and NW |  | Similar to NW-sided model as NR |
| Evaluation methodology |  | Link level simulation |
| Evaluation assumption |  | Parameters; Value. Carrier frequency; 3.5 GHz. # of TX layers/users; 1/2. # of RX antennas; 4/8/64. Waveform; NR based. # of PRB; 50. # of symbol per slot; 12. # of DMRS per slot; 2. # of DMRS re-used for training; 1. DMRS subcarrier position; Odd index (0 for even index). FFT size; 2048. CP length; 144. Modulation; 256 / 1024/ 4096 QAM. Coding rate; LDPC 0.9395. Transport Block size; 45120 for 1Tx/256 QAM 56392 for 1Tx/1024 QAM 67680 for Tx/4096 QAM 90200 for 2Tx/256 QAM. Channel model; TDL-A 30ns. Subcarrier spacing; 15 kHz. Evaluation metric; BLER. Benchmark; Conventional receiver (without non-linearity compensation) GMP-NC (with non-linearity compensation). |
| Evaluation KPI |  | BLER, MPR, EVM, throughput |
| Evaluation benchmark |  | Legacy receiver without DPoD Receiver with non-AI based DPoD, GMP-NC |
| High level potential specification impact |  | RAN4 requirements, e.g, EVM, MPR reporting Signalling/procedure related to LCM for NW-sided model |
| Feasibility, complexity/cost |  | Performance benefit: AI-NC provides 1-2dB improvement at 10% BLER when using 256QAM, 1024QAM and 4096QAM for the given EVM values. |

## 2.2 Use Case-2: AI/ML-SRS-assisted channel reconstruction

2.2.1 Motivation and use case description

In the last RAN4#116 bis meeting, as one of AI/ML use cases, AI/ML-SRS-assisted channel reconstruction was identified. In this use case, the main purpose is for DL CSI acquisition, considering that when the channel dimensions are large, e.g., large number of ports for X-MIMO, mTRP, large CSI bandwidth, prediction/measurement instances, DL CSI acquisition at the network faces several challenges.

Challenges of DL CSI acquisition based on SRS measurements: The overhead of SRS based DL channel measurement is agnostic to the number of NW’s antenna ports. This might make SRS an attractive alternative for DL CSI acquisition for a large number antennas in X-MIMO. However, SRS-based DL CSI acquisition is constrained by various factors, including limited coverage (especially when bandwidth increases), power imbalance and SRS overhead when the number of served UEs increases. One approach to address SRS coverage could be frequency-domain partial sounding, i.e., to boost SRS power. However, this incurs large latency to cover large bandwidth and results in channel ageing. Error! Reference source not found. shows the SGCS degradation (as compared to the reference, i.e., unit SGCS) as a result of channel ageing and power imbalance.

Impact of channel ageing and SRS power imbalance in the DL channel acquisition

- SRS based DL CSI acquisition for large channel dimensions is challenged by factors including SRS coverage or channel ageing and SRS power imbalance.

As one of the limitation factors, SRS IL power imbalance issue, RAN4 has identified this issue and some candidate solutions were proposed. However, there is no consensus achieved whether and how to support SRS IL imbalance reporting in RAN4.  Without the extract IL reporting by UE if UE cannot compensate, Network side may not handle the SRS power imbalance issue when deriving the DL CSI information. Even that UE can report the extract IL, it may result in some performance degradation especially in low SNR region, considering it may potentially amplify the noise power level with using the report IL for compensation. To address this issue, AI based solution has potential to recover CSI even under low SNR, by modeling the relationship between CSI without and with IL imbalance, based on SRS measurement. However, besides the SRS IL power imbalance issue, SRS coverage or channel ageing are also important factors which will also limit on the accuracy of DL CSI acquisition as analyzed above considering limited coverage when number of UE increases.

To address the aforementioned issues, we discuss approaches that combine the benefits of CSI-RS based CSI feedback and SRS-based measurement. As an example, the UE may measure spatially sub-sampled CSI-RS and feedback explicit CSI (channel matrix) for sub-sampled channel without prediction to larger dimension (up sampling). The sub-sampling could also be in the frequency and temporal domains. The network may combine SRS measurements with the CSI feedback on the explicit (channel matrix) as shown in Figure 4. The CSI feedback corresponding to spatially sub-sampled measurements, e.g., 64 ports out of 256 ports, carries the channel information for the entire CSI BW. On the other hand, one-shot SRS transmission covers only the fraction of CSI BW, e.g., 3 RBs, to achieve comparable coverage with CSI-RS transmission. In contrast, the SRS measurement provides full spatial domain information, e.g., 256 ports, as it is assumed to be received by all the receive ports. Here, the SRS may also be transmitted from the subset of the received ports as opposed to the entirety of them. In particular, it may be sufficient if the UE transmits SRS on the antenna ports with higher power. Additional advantage of such NW-side up sampling or reconstruction is that the reconstruction models could easily be site-specific, trained for the specificity of deployment scenario and network-side additional conditions, e.g., antenna layout, subsampling patter, TxRU mapping, etc.

The benefits of this scheme include

- Addressing the limitations of DL CSI acquisition via SRS

  - SRS IL power imbalance, by using the fused IL-free CSI report as reference for CSI reconstruction.

  - SRS coverage especially when CSI bandwidth is large, i.e., SRS is sounded for small portion of the CSI bandwidth wherein the additional frequency-domain information is completed by the CSI report

  - SRS overhead, especially in the presence of a large number of UEs, i.e., SRS is sounded for the small portion of the CSI bandwidth and/or subset of SRS ports and complemented by the CSI port

As illustrated in Figure 5, for the above purpose, two cases can be considered.

- Case 1: NW-side model-based approaches. Here, the network-side model may combine CSI feedback and SRS measurement. The CSI feedback may not necessarily rely on AI-based feedback as long as it is for explicit channel matrices, e.g., we may consider CB-based feedback or linear projection-based approach, as outlined in the previous section.

- Case 2: Two-sided model-based approach. Here, the CSI encoding and the corresponding reconstruction must be trained to be compatible, e.g., based on the training collaboration approaches outlined in the NR.

Two cases for SRS-assisted CSI reconstruction from spatially sub-sampled CSI-RS measurement

2.2.2 Preliminary evaluation results

To evaluate performance of SRS-assisted explicit channel reconstruction, the parameters in Table 4 are considered. The two-sided model-based approach as illustrated in Figure 6 is considered. It is shown that the two-sided model-based approach achieves a significant gain, e.g., close to 100% SGCS gain for layer 1, as compared to the impaired SRS measurement. The SRS measurement is impaired by channel aging as a result of 30ms interval to cover the CSI bandwidth as well as power imbalance. It is to be noted these results are for non-fixed power imbalance ratios. The SRS power imbalance coefficients are varied across their mean value with uniform distribution in the range [-1, 1] dB. Therefore, the results can be considered reasonably generalizable across diverse UE-side implementations and variations.

Simulation parameters for results for joint SRS and CSI feedback

| Parameter | Value | Parameter | Value |
| --- | --- | --- | --- |
| M (measured CSI-RS ports) | 64 | Channel | UMa Dense-urban |
| N (NW antenna ports) | 256 | SRS measurement interval | 30ms |
| Nr (number of UE receive ports) | 4 | Mean power imbalance (4 antennas) | [0 -3 -6 -9]dB |
| Carrier frequency/BW | 7 GHz/ 20MHz 13 SBs | Power imbalance distribution | Uniform [-1,1] dB centered on the mean. |

SRS-assisted explicit CSI reconstruction: SGCS gain as compared to impaired SRS-based DL CSI acquisition

- Two-sided model for SRS-assisted explicit channel reconstruction from spatially sub-sampled CSI-RS measurement achieves significant SGCS gain over SRS-based DL-CSI acquisition for large channel dimensions.

2.2.3 Study focus in RAN4

The SRS-assisted explicit channel reconstruction can work together with two-sided model, and also NW-sided model, as long as the explicit channel can be obtained for data collection for training. Since SRS-assisted DL CSI acquisition with two-sided model was under discussed in RAN1-led CSI compression use cases, we can prioritize the NW-sided model. After RAN1 identified it as one case with two-sided model for CSI compression, RAN4 can proceed the RAN4 analysis and evaluation relevant to implementation, performance as well as the interoperability similar as CSI compression in NR.

- Study SRS-assisted explicit channel reconstruction from spatially sub-sampled CSI-RS measurement at least for NW-sided model.

2.2.4 Summary

The following table provides the performance benefit, specification impact and evaluation assumption related with AI/ML-SRS-assisted channel reconstruction.

| Use Case 2# |  | AI-based SRS assisted channel reconstruction Network-side model |
| --- | --- | --- |
| AI model input | Input in training | CSI measurement and channel matrix feedback for subset of CSI-RS ports Estimated channel based on SRS |
|  | Input in inference | CSI measurement and channel matrix feedback for subset of CSI Estimated channel based on SRS |
| AI model output | Label in training (if applicable) | Ideal CSI measurement with full channel dimension |
|  | Output in inference | The reconstructed CSI with full channel dimension |
| Assumption on training | Training type | Offline |
|  | Label construction (if applicable) | Ideal CSI measurement with full channel dimension |
| model location for inference |  | Network-sided |
| Collaboration/interaction between UE and NW |  | Similar to NW-sided model in NR |
| Evaluation methodology |  | Link level simulation |
| Evaluation assumption |  | Parameter; Value; Parameter; Value. M (measured CSI-RS ports); 64; Channel; UMa Dense-urban. N (NW antenna ports); 256; SRS measurement interval; 30ms. Nr (number of UE receive ports); 4; Mean power imbalance (4 antennas); [0 -3 -6 -9]dB. Carrier frequency/BW; 7 GHz/ 20MHz 13 SBs; Power imbalance distribution; Uniform [-1,1] dB centered on the mean.. |
| Evaluation KPI |  | SGCS, Tput |
| Evaluation benchmark |  | SRS without IL imbalance SRS only based NW-sided model |
| High level potential specification impact |  | - Signalling/procedure to support lower overhead or simpler explicit CSI feedback - Signalling/ procedure related to LCM for NW-sided model - Potential UE RF related testing |

## 2.3 Use Case-3: AI/ML-enabled PRACH receiver

2.3.1 Motivation and use case description

In the last RAN4#116 bis meeting, as one of AI/ML use cases, AI/ML-enabled PRACH was identified for further discussion.

In mobile communication system, the random access (RA) procedure serves as a critical mechanism for establishing uplink synchronization by resolving two key parameters: the preamble index (RAPID) and timing advance (TA) between UE and BS. For contention-based random access (CBRA), UEs autonomously select preambles from a limited pool of preamble sequence, creating inherent collision risks when multiple UEs select the identical preamble. Such collisions force contending UEs to initiate power-ramped preamble retransmissions, resulting in higher power consumption and latency on the UE side, especially for power-constrained or latency-sensitive devices.

The NR PRACH resource partitioning exacerbate this collision challenge during each Random Occasion (RO). While Rel-15 5G NR systems allocated 64 dedicated preambles per RO, emerging services in 5G evolution accelerates preamble resource fragmentation, thus reducing the available contention-based preamble subset for random access and thereby amplifying the challenge of multi-user collision.

Conventional RAPID detection relies on threshold-based peak identification utilizing the power delay profile (PDP), derived by correlating the PRACH preamble with a locally generated ZC sequence. A preamble is declared detection if the maximum peak of PDP in the search window (SW) exceeds the predetermined threshold, simultaneously resolving the RAPID and TA. This approach exhibits two fundamental limitations: (a) Static-setting threshold through empirical methods: Predefined threshold could fail to adapt to dynamic channel condition, particularly in low SNR scenarios. (b) Near-far power disparity: In multi-user environments, strong interferers mask weaker signal, thus complicate the threshold setting for multi-user detection. The analysis underscores the need to develop an effective preamble detection algorithm designed to operate efficiently in scenarios involving multiple users.

Meanwhile, from random access procedure perspective, BS can only feedback one set of attributes related to detected preamble in RAR. Assuming two UEs initiate a random access using the same preamble ID at the same time, then both UEs will receive the same RAR and decode it to obtain the corresponding parameters for the following MSG3 transmission. Subsequently, both UEs will use the same uplink parameters for MSG3 transmission, which will result in mutual interference of MSG3 between these two UEs on the BS side. Then, it will result in unnecessary long access delay and high-power consumption. Therefore, from random access procedure perspective, it will also need to design the random-access procedure for more than one UEs with selected the same PRACH preamble.

- From PRACH preamble detection perspective, it is necessary to investigate effective preamble detection algorithm to operate well in the multiple users

- From random access procedure perspective, it is necessary to investigate the early contention resolution

Machine learning (ML)-based PRACH detection has merged as a transformative approach and drawn attention in both academic and industrial societies. With AI-based PRACH receiver, early contention resolution can be feasible in which the BS employs the AI-based receiver to detect a preamble and estimate the number of UEs that selected and transmitted the same preamble. Based on the estimated multiplicity for a given preamble, the BS can transmit a random-access response (RAR) associated with the RA-RNTI corresponding to the preamble. The RAR may include multiple PUSCH grants, where the number of grants corresponds to the BS’s estimation of how many UEs transmitted the same preamble in the same PRACH resource. Based on the estimated multiplicity for a given preamble, the BS can transmit a random-access response (RAR) associated with the RA-RNTI corresponding to the preamble. The RAR may include multiple PUSCH grants. The contented UE can choose the corresponding TA and UL grand for msg3 transmission to solve the preamble collision in early stage. By enabling this kind of contention resolution at the Msg1/Msg2 stage for a substantial portion of collisions, it reduces access delay, improves first-attempt success probability, and enhances uplink resource efficiency, making it particularly beneficial for 6G networks with massive and bursty access demands.

To enable the early contention resolution with AI/ML-enabled PRACH receiver, from RAN1 and RAN2 prospective, random-access procedure to solve multi-user collision is needed, especially for msg2 to support grants for multi-user corresponding to one preamble, and msg3 transmission in UE side.

In general, assisting by AI/ML technologies, we expect that the following benefits can be achieved

- Detection performance improvement including both missing detection and false alarm detection compared with legacy receiver in both single-user and multi-user collision scenario

- Access delay and power consumption reduction for UE random access

Based on random access procedure designed by RAN1 and RAN2, RAN4 should specify the corresponding PRACH detection requirement and RRM requirement. For demodulation requirement, only a single preamble detection performance without considering the multi-user collision was specified in current NR system, it may be difficult to reflect the multi-user collision performance considering 6G requirement on massive connectivity put challenges on random access. Meanwhile, current RAN4 performance requirement for PRACH detection is 99% detection probability and 0.1% false alarm probability, it is also difficult to meet the requirement and real field in cases the multi-user in collision scenario.

For RRM requirement, RAN4 defined the corresponding random-access requirement with following the random-access procedure specified in RAN1 and RAN2 for both contention and non-contention based random access. For example, the correct behavior for UE when receiving random access response is that UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random-Access Response contains a Random-Access Preamble identifier corresponding to the transmitted Random-Access Preamble. In case RAR including multiple grands and TA corresponding to one preamble ID, the UE correct behavior may need to be studied.

- For RAN4 requirement, both demodulation requirement and RRM requirement for random access are specified.

- For PRACH requirement, only single preamble detection performance without considering the multi-user collision was specified.

- For RRM requirement, RAN4 defined the corresponding random-access requirement with following the random-access procedure specified in RAN1 and RAN2 for both contention and non-contention based random access.

2.3.2 Details of Neural Network Design

(a) overall procedure of PRACH receiver                        (b) Proposed AI-based PRACH receiver

Figure 7(a) provides the overall procedure of PRACH receiver. In our proposed solution as indicated Figure 7(b), AI receive is mainly to replace the traditional threshold-based blocks after obtained PDP of PRACH signal, highlighted in the green box of Figure 7(a). Specifically, the proposed solution includes the data processing module to facilitate the identification of candidate clusters. Then, considering various PRACH configuration and channel conditions, a unified feature selection was design. After that, a simple AI network was applied for classification to obtain the predicted PRACH preamble ID and the corresponding TA. In our solution, the Convolutional Neural Network (CNN) architecture is specifically designed for RAPID detection.

2.3.3 Preliminary evaluation results

The following shows a preliminary result of detection performance at BS AI/ML-based PRACH receiver for both single-user and multi-user collision scenario. For simulation, we utilize the typical assumption to assess the performance of the proposed method in relation to both detection probability and false detection probability, referred to as Pd and Pf, respectively. According to the targeting requirements outlined in 3GPP, Pd must be equal to or greater than 99%, while Pf should be less than 0.1%. The conditions for the simulation are delineated in Table 5.

Simulation conditions for AI/ML-enabled PRACH receiver

| Parameter | Values |
| --- | --- |
| Channel type | AWGN/TDLC300-100 |
| PRACH format | C0 |
| Ncs | 0/23 |
| Logical index | 0/1 |
| Preamble ID | 0/5 |
| Number of Preamble | 64 |
| Number of UE | 1/2 |
| Doppler Shift | 2334Hz for restrict set B |
| Number of Tx/Rx | 1/2 |
| Timing offset between 2UE | 20samples |

Multi-user collision detection performance in both AWGN and fading scenario

The Figure 8 illustrates the detection performance Pd of two UEs in AWGN and TDLC300-100, respectively. -8dB and -2dB correspond to the minimum SNR required by 3GPP specifications to meet the performance for a single user under AWGN and TDLC300-100, respectively. When the two UE have equal power, the corresponding SNR in AWGN is -8dB and in TDLC300-100 it is -2 dB, with the probability of correct multi-user detection being 99.96% and 97.06%, respectively. When the power difference between the two user devices is 3 dB, the probability of multi-user detection reaches 95.6%.

Meanwhile, for single user scenario, the proposed AI based solution can also achieve the outstanding performance for single UE detection under non-HST scenario, almost 2dB gain compared with conventional threshold-based approaches, as illustrated in Figure 9. Also, the outstanding performance can be achieved with around 2dB gain under high-speed scenario in the low SNR region as illustrated in Figure 10.

Single-user detection performance in AWGN scenario, (Ncs, RAPID, logical root sequence index) as (0,0,0)

Single-user detection performance in HST scenario, (Ncs, RAPID, logical root sequence index) as (23,5,1) for HST restriction setB

- For PRACH detection performance perspective, it is feasible to improve the detection performance for single UE and multiple-UE collision scenario with AI/ML-enabled advanced receiver

2.3.4 Study focus in RAN4

With supporting of AI/ML-enabled advanced receiver for PRACH detection, from RAN4 perspective, the PRACH performance should be investigated pending on Network AI/ML capability. The input and output of AI/ML model should be studied, as well as the corresponding LCM procedure for NW-sided model. Although the preamble sequence and format for 6G is designed by RAN1, from feasibility study perspective, the existing preamble sequences and formats can be considered for study of AI/ML-enabled PRACH receiver.

Meanwhile, pending on the study of random-access procedure to enable the early contention resolution with AI/ML-enabled PRACH receiver, the RRM requirement for random access also need to be specified.

- RAN4 study AI/ML-enable PRACH receiver for 6GR, including both single-user and multi-user collision scenario based on existing PRACH preamble formats as starting point, including LCM procedure, data collection, performance monitoring, and model interference pending on the assumption of AI/ML receiver.

- Pending on the study of random-access procedure to enable the early contention resolution with AI/ML-enabled PRACH receiver, RRM requirement for random access also need to be studied, including the UE correct behavior when receiving random access response.

2.3.5 Summary

In order to facilitate the RAN4 study on RAN4-driven use cases, it is benefit to provide the performance benefit, specification impact and evaluation assumption. The following aspect related with AI/ML-enabled PRACH receiver was proposed for RAN4 study

| Use Case 3# |  | AI-PRACH receiver |
| --- | --- | --- |
| AI model input | Input in training | Detected PRACH signal with PDP or truncated PDP in time domain for each preamble detection window |
|  | Input in inference | Detected PRACH signal with PDP or truncated PDP in time domain for each preamble detection window |
| AI model output | Label in training (if applicable) | Ground-truth number of UEs that transmitted the same preamble |
|  | Output in inference | Predicted number of UEs that transmitted the same preamble for given PRACH resources |
| Assumption on training | Training type | Offline training |
|  | Label construction (if applicable) | Ground-truth number of UEs that transmitted the same preamble |
| model location for inference |  | NW-sided model |
| Collaboration/interaction between UE and NW |  | Similar to NW-sided model in NR |
| Evaluation methodology |  | Link level simulation |
| Evaluation assumption |  | Parameter; Values. Channel type; AWGN/TDLC300-100. PRACH format; C0. Ncs; 0/23. Logical index; 0/1. Preamble ID; 0/5. Number of Preamble; 64. Number of UE; 1/2. Doppler Shift; 2334Hz for restrict set B. Number of Tx/Rx; 1/2. Timing offset between 2UE; 20samples. |
| Evaluation KPI |  | Miss detection rate and false detection rate |
| Evaluation benchmark |  | Legacy receiver with threshold-based method |
| preliminary evaluation results |  | For multi-UE scenario, when the two UE have equal power, the corresponding SNR in AWGN is -8dB and in TDLC300-100 it is -2 dB, with the probability of correct multi-user detection being 99.96% and 97.06%, respectively. When the power difference between the two user devices is 3 dB, the probability of multi-user detection reaches 95.6%. For single UE scenario, the proposed AI based solution can also achieve the outstanding performance for single UE detection under non-HST scenario, almost 2dB gain compared with conventional threshold-based approaches And also, for HST scenario, the outstanding performance can be achieved with around 2dB gain under high-speed scenario in the low SNR region |
| High level potential specification impact |  | 1. RAN 4 requirements including both demodulation requirement and RRM requirement for random access 2. Signaling/procedure related to random access for more than one UEs with selected the same PRACH preamble 3. Signaling/procedure related to LCM for NW-sided model |
| Feasibility, complexity/cost |  | CNN-based AI model |

# 3. RAN4-driven AI/ML use cases for RRM

In last meeting, 14 candidate use cases were agreed in the WF [4]. In this contribution, we would like to give some analyses use case-by-use case from dependence with other WG-led use cases (in 5GA or 6G), expected benefits and which specific aspect RAN4 should study perspective.

From our understanding, the 14 use cases can be classified into two aspects: AI-RRM prediction models and application scenarios for AI-RRM prediction models. For AI-RRM prediction model, the use case 1) ~ 3) can belongs to this basket.  For application scenarios for AI-RRM prediction models, the use case 4) ~ 14) can belongs to this basket.

- The 14 candidate use cases for 6G AI RRM can be classified into two aspects: AI-RRM prediction models and application scenarios for AI-RRM prediction models.

## 3.1 AI-RRM prediction models

3.1.1 AI-RRM prediction model-1: L1 beam measurement

- AI-RRM prediction model-1 (L1 beam measurement): (a) Time domain (intra-cell); (b) Time and spatial domain (intra-cell); (c) Freq. and spatial domain (inter-cell); (d) Spatial domain (inter-cell); (e) Time and spatial domain (inter-cell).

During Rel-18 study on AI/ML for air interface, RAN1 studied and evaluated the spatial and temporal beam prediction based on smaller set of L1 beam measurements or historic measurements. For AI-based BM, from RAN1 perspective, BM-Case1 and BM-Case 2 are considered for normative work:

- BM-Case1: Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams;

- BM-Case2: Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams. However, all the predictions are applied for FR2-to-FR2 and serving cell-to-serving cell.

- The investigation scope of the AI/ML for air interface (in 5G/5GA) prediction use cases has been focused on FR2-to-FR2 intra-cell (i.e., serving cell).

Based on the potential RAN4-driven RRM L1 beam measurement use cases discussed in last meeting, it can be observed that there are two main methodologies that RAN4 can study for L1 beam RRM measurement prediction: intra-cell and inter-cell. However, from our understanding, no matter intra-cell prediction or inter-cell prediction, it corresponds to a large extent to what RAN1 has already studied or will be evaluated in 5G/5GA/6G, what RAN4 in 6G highly possible can do on top of that is to check how such predicted measurements impact the RRM requirement in certain FR1/FR2 evaluation use case.

- For L1 beam measurement, candidate AI-RRM prediction model use cases (a)~(e) discussed in RAN4#116bis have dependence with RAN1-led use cases (in 5G/5GA/6G) except for different FR consideration.

Specifically, the AI-RRM prediction models analysis including scenario, methodology and the dependence with other WG-led use cases (in 5GA or 6G) from our side for each sub use-case are summed up in Table 6.

Summarization on methodology, scenario and dependence with other WG-led use cases (in 5G, 5GA or 6G) for L1 beam measurement use cases in 6G

| Methodology | Scenario | Dependence with other WG-led use cases (in 5GA or 6G) |
| --- | --- | --- |
| Intra-cell | (a) Time domain | Similar to RAN1-led L1 beam use cases (in 5G/5GA) except for different FR consideration |
|  | (b) Time and spatial domain | Similar to RAN1-led L1 beam use cases (in 5G/5GA) except for different FR consideration |
| Inter-cell | (c) Frequency and/or spatial domain | Potential RAN1-led L1 beam use cases or the extension (in 6G) except for different FR consideration |
|  | (d) Spatial domain | Merged into (c) |
|  | (e) Time and/or spatial domain | Potential RAN1-led L1 beam use cases or the extension (in 6G) except for different FR consideration |

- For L1 beam measurement, the AI-RRM prediction models analysis including scenario, methodology and the dependence with other WG-led use cases (in 5G, 5GA or 6G) for each sub-case are summed up in Table 6.

- To avoid overlapping scope with other WGs, suggest RAN4 to deprioritize the study of feasibility and applicability of L1 beam measurement use cases in 6G.

3.1.2 AI-RRM prediction model-2: L3 beam measurement

- AI-RRM prediction model-2 (L3 beam measurement): (a) Time domain (intra-cell); (b) Time and spatial domain (intra-cell); (c) Freq. and spatial domain (inter-cell); (d) Time and freq. domain (inter-cell); (e) Freq.  domain prediction for non-collocated cells (inter-cell); (f) Freq.  domain prediction for N carriers single cell

In RAN2#131 meeting, it was agreed that for RRM measurement prediction, L3 beam-level measurement prediction is feasible for network sided mode, and agreed that L3 beam-level prediction is feasible for UE sided model but there are concerns on complexity and other WGs. And later on, in Rel-20 AI MOB WID [5], there is a note indicating that for UE sided model, L3 beam level prediction is not supported. In this sense, UE sided model based L3 beam level measurement prediction has no dependence with RAN2-led AI MOB use cases (in 5GA) but the feasibility was already considered in 5G.

- For L3 beam measurement use cases:

- In Rel-19 RAN2 AI MOB, it was agreed that L3 beam-level prediction is feasible for UE sided model but with concerns on complexity and other WGs;

- For UE sided model, L3 beam level prediction is not supported in RAN2 Rel-20 AI MOB WI.

From our understanding, although L3 beam-level prediction for UE-sided model was excluded from Rel-20 AI/ML MOB scope, the feasibility of such AI/ML model was already studied in RAN2 Rel-19 MOB. In this sense, some duplicated work should be avoided in RAN4 6G SI.

- For L3 beam level intra-cell use case, it can be understood as beam-level measurement for L3 mobility, and we think the principle/ways can be inherited from L1-beam level prediction but with different AI/ML model input data, and what RAN4 can do is to study how such predicted measurements impact the L3 mobility performance.

- For L3 beam level inter-cell use case, it is more challenging and meaningful to perform inter-cell prediction since spatial consistency issue between the two cells is more severe. From this, the feasibility of achieving gain by AI/ML-assisted algorithm should to be verified by RAN4 through simulations if use cases (c) ~ (d) we discussed in last meeting will be agreed in RAN4 6G SI. However, since LTM (L3 beam-level) handover will be highly possible as a benchmark use case in 6G RAN2, AI/ML-assisted inter-cell L3 beam prediction can be studied to prove beneficial for L3 beam prediction with serving cell and target cells from temporal and/or spatial domain naturally. Furthermore, it is very likely that frequency domain prediction will be considered as one of candidate directions in RAN2 6G also due to the feasibility verification in Rel-19 AI MOB SI. At the same time, since non-collocated deployment was not considered in any of the frequency domain prediction use cases, RAN4 can to consider the study of feasibility and applicability of AI/ML model use cases (e) with non-collocated deployments in 6G.

Besides, in 5G/5GA we mainly studied single-domain prediction models and limited application scenarios, however, to widen the scope of application scenarios, it is justified to study the feasibility and applicability of multi-domain prediction AI-RRM prediction models in RAN4. Exemplified, joint frequency and Tx/Rx beam pair prediction, as well as joint frequency, cell, and Tx/Rx beam pair prediction can be as candidates of the RAN4-driven use cases in 6G. After feasibility verification, the joint frequency and Tx/Rx beam pair prediction AI/ML model or joint time, frequency and spatial domain can be considered at least in spatial domain Rx beam sweeping reduction (L1/L3) application scenario.

Regarding the use case (f): Frequency domain prediction for N-carriers single cell, it refers to a new scenario will be studied in 6G: single cell multi-carrier, as shown in Figure 11.

new scenario: Single cell multi-carrier

While, as one of 6G new frameworks, N-carrier single cell is on-going discussed in RAN1. From this, the study of the feasibility of inter-carrier intra-cell time and/or carrier domain prediction can postpone in RAN4 at current stage.

- For L3 beam measurement use cases, since frequency domain single cell, N-carriers prediction is on-going discussed in RAN1 6G as new scenario, RAN4 to postpone the discussions in 6G AI-RRM at current stage.

According to the above discussions, for the potential RAN4-driven RRM L3 beam measurement use cases discussed in last meeting, the AI/ML models including the methodology, scenario and dependence with other WG-led use cases (in 5GA or 6G) for each sub use-case are summed up in Table 7.

Summarization on methodology, scenario and dependence with other WG-led use cases (in 5G, 5GA or 6G) for L3 beam measurement use cases in 6G

| Methodology | Scenario | Dependence with other WG-led use cases (in 5GA or 6G) |
| --- | --- | --- |
| Intra-cell | (a) Time domain | - The feasibility of UE sided model L3 beam level prediction was considered in RAN2-led AI MOB use cases (in 5G) - Similar as RAN1-led L1 beam use cases (in 5G/5GA/6G) except for different FR consideration |
|  | (b) Time and spatial domain | - No dependence with RAN2-led AI MOB use cases (in 5G/5GA) - Similar as RAN1-led L1 beam use cases (in 5G/5GA/6G) except for different FR consideration |
| Inter-cell (Non-collocated and collocated deployments) | (c) Frequency and spatial domain | - No dependence with RAN2-led AI MOB use cases (in 5G/5GA) - RAN4-driven use case in 6G |
|  | (d) Time and frequency domain | - No dependence with RAN2-led AI MOB use cases (in 5G/5GA) - RAN4-driven use case in 6G |
|  | (e) Time, frequency and spatial domain | - No dependence with RAN2-led AI MOB use cases (in 5G/5GA) - RAN4-driven use case in 6G |
|  | (f) Frequency domain | The feasibility of UE sided model L3 beam level prediction (collocated) was considered in RAN2-led AI MOB use cases (in 5G) |
|  | (g)Time and/or spatial domain | - Potential RAN2-led L3 beam use cases, LTM handover (in 6G) |
| Single cell, N carriers | (h) Frequency domain | - No dependence with RAN2-led AI MOB use cases (in 5G/5GA) - New scenario, on-going discussion in RAN 1 6G |

- For L3 beam measurement, the AI-RRM prediction models analysis including scenario, methodology and the dependence with other WG-led use cases (in 5GA or 6G) for each sub use-case are summed up in Table 7.

- For L3 beam level measurement prediction, in addition to single-domain AI-RRM prediction models, RAN4 can to study the feasibility and applicability of the following multi-domain AI-RRM prediction models in 6G accommodating to different application scenarios with non-collocated and collocated inter-cell deployments through simulations:

- Joint frequency and/or spatial domain

- Joint time and frequency domain

- Joint time, frequency and spatial domain

3.1.3 AI-RRM prediction model-3: L3 cell measurement

- AI-RRM prediction model-3 (L3 cell measurement): (a) Time domain (intra-cell); (b) Freq. domain (inter-cell); (c) Freq.  domain prediction for non-collocated cells (inter-cell)

After checking RAN2-led AI/ML MOB discussions, some of the L3-cell level RRM measurement prediction AI/ML model, i.e., (a) and (b) were involved in 5G/5G-A AI MOB scope. According to the AI/ML MOB use cases for RRM measurement prediction defined in TR 38.744, the agreed sub-use cases in 5G/5G-A AI MOB can be categorized in diverse domains: FR1-to-FR1, FR2-to-FR2; Space domain, time domain and frequency domain; Intra-frequency and inter-frequency; Inter-cell and intra-cell. By means of down selection, the prediction scenario combinations and methodologies shown in Table 3 were considered as higher priority and to investigate the potential RRM requirement impact with the target study goal in Rel-20 WI.

Summarization on scenario, methodology and target study goal for 5G/5G-A AI MOB

| Scenario | Methodology | Target study goal |
| --- | --- | --- |
| FR1 to FR1 intra-frequency temporal domain case A | Intra-cell | 2nd goal |
| FR1 to FR1 intra-frequency temporal domain case B | Intra-cell | 1st goal |
| FR2 to FR2 intra-frequency temporal domain case A | Intra-cell | 2nd goal |
| FR2 to FR2 intra-frequency temporal domain case B | Intra-cell | 1st goal |
| FR1 to FR1 inter-frequency frequency domain | Inter-cell, co-located | 1st goal |
| FFS: FR2 to FR2 inter-frequency frequency domain | Inter-cell, co-located | 1st goal |

Based on above table, we have the following observations for L3 cell measurement:

- For L3 cell measurement use cases:

- L3 cell level time domain (intra-cell) prediction (i.e., use case (a)) no matter for FR1-FR1 or FR2-FR2 has dependence with RAN2-led AI MOB use cases (in 5G/5GA)

- L3 cell level frequency domain (inter-cell, collocated deployment) prediction has dependence with RAN2-led AI MOB use cases (in 5G/5GA)

- L3 cell level frequency domain (inter-cell, non-collocated deployment) prediction has no dependence with RAN2-led AI MOB use cases (in 5G/5GA)

Most of companies have negative attitude on non-collocated scenario prediction in Rel-19 SI due to the low spatial correlation between the two frequencies, so inter-frequency prediction starts evaluation from co-located scenario. From our understanding, the application scope of inter-frequency prediction can be extended to non-collocated scenario as long as the frequencies of two CCs is not very far away. And based on the observation in 5G SI that, the closer the input and output frequencies, the higher the correlation between input and output, we think from the strong learning capability of AI/ML model, the key features between two non-collocated frequencies in the same band can be learnt especially based on frequency-cluster based consideration. However, the impact on timing issue and how to define generalization requirement and the corresponding testing need to be studied.

On top of single-domain prediction, multi-domain prediction can also be considered as one candidate of the AI-RRM prediction models in 6G L3 cell measurement use case. Exemplified, joint time, frequency and/or spatial domain under non-collocated inter-cell methodology, and the corresponding multi-domain AI-RRM prediction models can be used for at least measurement-gap reduction, the details can be found in Section 3.2.1.

According to the above discussions, for the potential RAN4-driven RRM L3 cell measurement use cases discussed in last meeting, the AI/ML models including the methodology, scenario and dependence with other WG-led use cases (in 5GA or 6G) for each sub use-case are summed up in Table 9.

Summarization on methodology, scenario and dependence with other WG-led use cases (in 5G, 5GA or 6G) for L3 cell measurement use cases in 6G

| Methodology | Scenario | Dependence with other WG-led use cases (in 5GA or 6G) |
| --- | --- | --- |
| Intra-cell | (a) Time domain | Same to RAN2-led L3 cell use cases (in 5G/5GA) |
| Inter-cell | (b) Frequency domain (Collocated deployment) | Same to RAN2-led L3 cell use cases (in 5G/5GA) |
|  | (c) Frequency domain (Non-collocated deployment) | FFS RAN4-driven use case in 6G |
|  | (d) Time, frequency domain (Non-collocated deployment) | FFS RAN4-driven use case in 6G |

- For L3 beam measurement, the AI-RRM prediction models analysis including scenario, methodology and the dependence with other WG-led use cases (in 5GA or 6G) for each sub use-case are summed up in Table 9.

Take all the candidate use cases defined in Table 1, Table 2 and Table 4 into consideration, for such AI-RRM prediction models related use cases which have/will have dependence with other WG-led use cases (in 5G/5GA/6G), from our understanding, it does not mean they are not essential and should to preclude in RAN4 6G, instead, RAN4 can to study how such predicted measurements impact the RRM requirement in certain application scenarios in 6G based on other WG’s feasibility verification outcome.

- For AI-RRM prediction models related use cases, which have or will have dependence with other WG-led use cases (in 5G/5GA/6G), RAN4 can to study how such predicted measurements impact the RRM requirement in certain application scenarios in 6G based on other WG’s feasibility verification outcome

## 3.2 Application scenarios for AI-RRM prediction models

Based on our understanding, from RAN4/UE behaviour impact perspective, at least the following four scenarios can be deemed as proactive RAN4-driven application scenarios for AI-RRM prediction models: Measurement gap reduction, dynamic adaptation of measurement procedure, RX beam sweeping reduction (L1/L3) and AI/ML based RLM/CBD, LTM by measurement prediction

- From RAN4 impact perspective, at least the following four scenarios can be as proactive RAN4-driven application scenarios in 6G AI RRM:

- Measurement gap reduction

- Dynamic adaptation of measurement procedure

- RX beam sweeping reduction (L1/L3)

- AI/ML based RLM/CBD, LTM by measurement prediction

3.2.1 Multi-Domain L3 level Prediction for Measurement-Gap Reduction

RRM measurements on serving and neighbor cells are performed by the UE both in idle/inactive and connected modes in order to ensure that the UE is at all times served by a cell, and measurements are almost involved in all RRM procedures. Based on the current measurement mechanism, when UE to perform inter-frequency measurement, measurement gap (MG) shall be configured. However, regarding MG, two main UE capabilities are considered: per-UE gap and per-FR gap, for the gap types, we have: Type 1 gap (Gap(s) configured via GapConfig without suffix), Type 2 gap (Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17), NCSG, MUSIM gap, Positioning gap, Rel-18 concurrent gap, PreConfigured gap, and in 6G, maybe more gap types will be introduced/defined. And based on the gap pattern configuration defined in TS 38.133 9.1.2, we have #0~#25 gap patterns. Take FR1 as example, the MGs in FR1 are used for inter-frequency measurements, inter-RAT measurements with NR target RAT, intra-frequency measurements on PCC and SCCs when the SSB is not within the active DL BWP. Such cumbersome gap design make UE implementation becomes complicated. The obstacles can be summarized as below

- Throughput degradation. Scheduling restriction and interruption are unavoidable. When UE use MG to perform measurement, UE cannot receive/or transmit on downlink/uplink shared channels of the serving cell during such MG

- UE power consumption is relatively high.

- Trade-off between measurement overhead and mobility performance. Based on the current requirement definition, the measurement period for inter-frequency measurements with gaps is related to MGRP configuration, when MGRP is large, the measurement overhead is shortened, while the measurement period becomes long, the subsequent mobility action will be delayed, but short MGRP will lead to significant measurement overhead.

- The motivation of reducing MG in 6G AI RRM contain: Throughput improvement, UE power consumption reduction, measurement overhead and mobility performance balance.

Whilst, MG reduction is in parallel discussed in 6G non-AI. In such discussions, the mainstream includes MG pattern reduction from 5G, gap-less measurement, unified MG, etc. While, to show the performance gain from AI-assisted approaches, the comparable non-AI baseline performance under certain simulation environment should be provided. In this sense, to understand how the performance of AI/ML predictors compares to the baseline clearly, and to verify AI/ML-assisted approach is significantly beneficial for facilitating NW resource scheduling/configuration and improving the performance, RAN4 should to figure out first the MG design in which G (5G or 6G) can be as the comparable non-AI baseline.

- For measurement-gap reduction application scenarios, to verify AI/ML-assisted approach is beneficial for facilitating NW resource scheduling and obtaining performance gain, RAN4 should to figure out the MG design in which G (5G or 6G) can be as the comparable non-AI baseline.

From our understanding, MG reduction can be interpretated from two aspects, MG occasion skipping per a MG periodicity for a certain frequency layer or MG reduction of a certain frequency for all configured frequency layers, as shown in Figure 12.

Measurement prediction example on MG reduction in 6G

To reduce the MG in 6G AI, multi-domain L3 level AI-RRM prediction model can be as the backbone. As an example, joint frequency, time and/or spatial domain for non-collocated inter-cell prediction.

- UE measures a multi-dimensional minimal selected Set B of its serving frequency layer and a limited subset of neighbour frequency layers and cells, together with a short time sequence of historical measurement results

- Alternatively, multi-dimensional minimal selected Set B can be constructed by combining frequency layers and cells, Tx/Rx beam pairs, and a short time sequence of historical measurement results

An AI/ML model reconstructs the multi-dimensional complete Set A of all L3 RSRP values across the candidate frequency layers and cells for future time instances

- Alternatively, multi-dimensional complete Set A of all RSRP values can be reconstructed across the candidate frequency layers and cells, candidate Rx-Tx beams for future time instances

In this sense, the free frequency-time resources can facilitate enhanced performance including throughput improvement while maintaining the accurate mobility performance, and as a promising benefit, the AI/ML model can forecast the strongest target frequency/cell with predicted ranked Top-K (L3/L1) RSRP, exhaustive measurement on neighbour frequencies/cells can be avoided and timely handover decisions can be supported with the accurate time-aware knowledge. The details of MG reduction use case from our side is shown in Table 5.

- RAN4 to study and evaluate the feasibility of multi-domain L3 level prediction for measurement-Gap reduction in 6G AI RRM

- Multi-domain combining time and frequency domain L3 level AI-RRM prediction model can be evaluated as baseline

- Both co-located and non-collocated inter-cell prediction are considered

- For measurement-gap reduction application scenarios, the details of MG-reduction use case are shown in Table 10, wherein multi-domain combining time and frequency domain L3 cell level AI-RRM prediction model is as example.

The details of measurement-gap reduction use case (for information)

| Sub-use case: MG reduction |  |  |
| --- | --- | --- |
| AI model input | Input in training (Multi-domain: Joint time and frequency) | Measured RSRPs for all candidate frequency layers/cells for time series until t |
|  | Input in inference | Measured L3 RSRPs corresponding to multi-dimensional minimal selected Set B |
| AI model output | Label in training (if applicable) | Legacy measured L3 RSRPs |
|  | Output in inference | Predicted L3 RSRPs corresponding to multi-dimensional complete Set A |
| Assumption on training | Training type | Offline training |
|  | Label construction (if applicable) | Through legacy measurement |
| model location for inference |  | UE-sided model |
| Collaboration/interaction between UE and NW |  | TBA |
| Evaluation methodology |  | System-level simulation |
| Evaluation assumption |  | TBA |
| Evaluation KPI |  | Intermediate KPIs：RSRP difference as baseline Performance KPI: e.g., QoS related KPI and/or HO performance KPI |
| Evaluation benchmark |  | Time domain: Sample and hold Frequency domain: Predict based on co-located/ non-collocated cell measurement with pathloss modification |
| Preliminary evaluation results |  | TBA |
| High level potential specification impact |  | TBA |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Measurement delay requirements and scheduling requirements |
| Expected benefits |  | MG reduction Measurement delay reduction HO performance enhancement |

3.2.2 Dynamic Adaptation of Measurement Procedure

From our understanding, dynamic adaptation of meaurement precedure is, to some extent, refers to UE measurement adaptation. And the dynamic UE measurement adaptation can be NW agnostic or need NW-UE interaction to assist NW measurement scheduling.

- For NW-UE interaction method. As one of the high-level implementation methods, shown in Fig. 3

Fig. 3 Dynamic adaptation of measurement procedure with NW-UE interaction

Originally, UE to perform measurement based on NW measurement configurations always. Currently, UE can 1) to perform inference based on the inference related configuration with associated ID containing gNB-side additional condition and the trained AI/ML model to predict quality of some cells/frequencies and 2) to perform adaptive measurement based on its local information/ measurement preference. After that UE can to report object measurement preference with/without identified ID to assist updating NW measurement scheduling/configuration. From this, UE can smartly spend its more efforts/resources on the important measurement objects (e.g., frequency layers), and the measurement period can be also adjusted flexibly for different measurement task.

- For NW agnostic method. We think it can up to UE implementation, and it is not clear how to define requirement impact on RAN4 specification, since non-AI delay requirement can always be met and it is worst case.

- RAN4 to study and evaluate the feasibility of dynamic adaptation of measurement procedure in 6G AI RRM

- Which L3 level single-domain AI-RRM prediction model (s) can be evaluated as baseline

- Potential specification impact should be clarified no matter AI/ML-assisted UE dynamic measurement is or is not transparent to the network

- Both co-located and non-collocated inter-cell prediction are considered.

3.2.3 Spatial domain RX beam sweeping reduction (L1/L3)

At current spec., the main contributor of measurement period no matter for L1 measurement or L3 measurement, is Rx beam sweeping factor. In this sense, from our understanding, spatial domain RX beam sweeping reduction (L1/L3) is a typical RAN4-driven use case no matter for AI or non-AI perspective for FR2. As an AI-RRM prediction model example in application scenarios, single spatial-domain DL Rx beam prediction for Set A of beams can be performed based on measurement results of Set B of beams of RS, that means the UE does not need to measure all RSs with traversing through all Rx beams, and the measurement to select the best RX beam or the optimal Rx-Tx beam pair can be reduced, in other words, the RX beam sweeping factor can be reduced. To evaluate the application scenarios, L1 beam measurement and/or L3 beam measurement AI-RRM prediction models with/without multi-domain consideration can be involved.

- RAN4 to study and evaluate the feasibility and testability of FR2 RX beam sweeping reduction (L1/L3) in 6G AI RRM

- Which L1/L3 beam measurement AI-RRM prediction models with/without multi-domain consideration can be evaluated as baseline to evaluate beam sweeping factor reduction

- Both co-located and non-collocated deployment can be considered if inter-cell prediction is involved.

3.2.4 AI/ML based RLM/CBD, LTM by measurement prediction

From our understanding, AI/ML based RLM/CBD, LTM by measurement prediction can be as candidate application scenarios naturally as long as the feasibility of L1/L3 beam measurement AI-RRM prediction models are verified in RAN4 6G SI or other WG. And one of the motivations of studying RLM prediction is to use predictions for recovery resource (SSB or CSI-RS beams) selection instead of performing actual measurement. Besides, indirect and direct methods can be both considered for AI/ML based RLM in 6G, wherein indirect methods refer to UE use AI/ML model to predict radio link quality in across-domain, and then follow the existing RLM/RLF declaration criteria/modelling, and direct method is that to treat RLM as an event, and the output of the model can be the probability of the event in a future time instance/window.

- RAN4 to study and evaluate the feasibility and applicability of AI/ML based RLM/BFD/CBD, LTM by measurement prediction based on L1/L3 beam measurement multi-domain AI-RRM prediction models in 6G AI RRM

- Whether to/How to consider indirect and direct methods in RLM measurement prediction application scenario

# 4. Conclusion

In this contribution, we have provided our viewpoints on RAN4 driven use cases, with following proposals and observations obtained:

Use Case-1: AI/ML-enabled non-linearity compensation

- AI-NC provides 1-2dB improvement at 10% BLER when using 256QAM, 1024QAM and 4096QAM for the given EVM values.

- The goal of the work in 6G is to enable low-end PA and extreme high modulation to transmit higher power, leveraging the capabilities of advanced BS receiver.

- Non-linearity model(s) of transmission signals (selected for the evaluation of BS compensation capabilities and UE MPR reduction limits) can be determined in 6G UE RF session, which should be common for BS compensation with AI and Non-AI solutions.

- Adopt the following working steps for feasibility study for both AI and Non-AI solutions.

  - Study item phase (demod centric)

    - Step-1: Selection of Non-linearity model(s) of transmission signals

- UE RF session firstly to determine the non-linearity model(s)

    - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

      - Note: For feasibility study, demodulation relevant evaluation is performed in UE RF session for non-AI based solution, and demodulation relevant evaluation is performed in AI session for AI based solution

- Including the non-linearity model(s) into the RAN4 PUSCH demodulation test case(s) for both 64QAM and 256QAM

- For a given Tx EVM value derived based on the non-linearity models, evaluate the performance with following assumptions on selected RAN4 test cases to find the target SNR to meet RAN4 requirement or test metric, e.g., 70% of maximum throughput or 10% BLER

  - Performance evaluation

    - Case 1: Conventional receiver including non-linearity TX model: SNR-a

    - Case 2: Non-AI based non-linearity receiver including non-linearity TX model: SNR-b

    - Case 3: Conventional receiver without any non-linearity TX model: SNR-c

    - Case 4: AI based non-linearity (AI-NC) receiver with non-linearity TX model: SNR-d

    - Note: Case 3 is considered as upper bound performance, Case 1 is expected to be low bound performance

  - Performance comparison

    - Compare SNR-d and SNR-c, the results should be comparable

      - FFS the criteria to be claimed as “comparable” (e.g., within certain x dB), and the target SNR-c should be feasible for network deployment

    - Compare SNR-d and SNR-b, for AI-NC advantage over non-AI receiver

- Perform evaluation with different Tx EVM values, find/determine the maximum Tx EVM with the target SNR-d derived based on Case 4 to meet RAN4 requirement or test metric and comparable with the target SNR-c derived based on Case 3.

  - Perform different PA models, if needed

  - WI phase (UE RF centric)

    - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation and elevate the Net Gain :

  - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

  - For instance for 256QAM Tx EVM, 3.5% -> 6%,

    - For selected PA model(s), evaluate the allowed TX power boosting/gain (i.e., MPR reduction) for different power class

    - Net Gain [dB] = (Tx power boosting/gain relative to reference) − (SNR degradation relative to reference at 10% BLER)

    - FFS the necessity for new EVM model for RAN4 demodulation test cases.

Note: Above approaches are applicable for both 5G waveforms and 6G waveforms

Use Case-2: AI/ML-SRS-assisted channel reconstruction

- SRS based DL CSI acquisition for large channel dimensions is challenged by factors including SRS coverage or channel ageing and SRS power imbalance.

- Two-sided model for SRS-assisted explicit channel reconstruction from spatially sub-sampled CSI-RS measurement achieves significant SGCS gain over SRS-based DL-CSI acquisition for large channel dimensions.

- Study SRS-assisted explicit channel reconstruction from spatially sub-sampled CSI-RS measurement at least for NW-sided model.

Use Case-3: AI/ML-enabled PRACH receiver

- From PRACH preamble detection perspective, it is necessary to investigate effective preamble detection algorithm to operate well in the multiple users

- From random access procedure perspective, it is necessary to investigate the early contention resolution

- For RAN4 requirement, both demodulation requirement and RRM requirement for random access are specified.

- For PRACH requirement, only single preamble detection performance without considering the multi-user collision was specified.

- For RRM requirement, RAN4 defined the corresponding random-access requirement with following the random-access procedure specified in RAN1 and RAN2 for both contention and non-contention based random access.

- For PRACH detection performance perspective, it is feasible to improve the detection performance for single UE and multiple-UE collision scenario with AI/ML-enabled advanced receiver

- RAN4 study AI/ML-enable PRACH receiver for 6GR, including both single-user and multi-user collision scenario based on existing PRACH preamble formats as starting point, including LCM procedure, data collection, performance monitoring, and model interference pending on the assumption of AI/ML receiver.

- Pending on the study of random-access procedure to enable the early contention resolution with AI/ML-enabled PRACH receiver, RRM requirement for random access also need to be studied, including the UE correct behavior when receiving random access response.

RAN4-driven AI/ML use cases for RRM

- The 14 candidate use cases for 6G AI RRM can be classified into two aspects: AI-RRM prediction models and application scenarios for AI-RRM prediction models.

- The investigation scope of the AI/ML for air interface (in 5G/5GA) prediction use cases has been focused on FR2-to-FR2 intra-cell (i.e., serving cell).

- For L1 beam measurement, candidate AI-RRM prediction model use cases (a)~(e) discussed in RAN4#116bis have dependence with RAN1-led use cases (in 5G/5GA/6G) except for different FR consideration.

- For L1 beam measurement, the AI-RRM prediction models analysis including scenario, methodology and the dependence with other WG-led use cases (in 5G, 5GA or 6G) for each sub-case are summed up in Table 6.

- To avoid overlapping scope with other WGs, suggest RAN4 to deprioritize the study of feasibility and applicability of L1 beam measurement use cases in 6G.

- For L3 beam measurement use cases:

- In Rel-19 RAN2 AI MOB, it was agreed that L3 beam-level prediction is feasible for UE sided model but with concerns on complexity and other WGs;

- For UE sided model, L3 beam level prediction is not supported in RAN2 Rel-20 AI MOB WI.

- For L3 beam measurement use cases, since frequency domain single cell, N-carriers prediction is on-going discussed in RAN1 6G as new scenario, RAN4 to postpone the discussions in 6G AI-RRM at current stage.

- For L3 beam measurement, the AI-RRM prediction models analysis including scenario, methodology and the dependence with other WG-led use cases (in 5GA or 6G) for each sub use-case are summed up in Table 7.

- For L3 beam level measurement prediction, in addition to single-domain AI-RRM prediction models, RAN4 can to study the feasibility and applicability of the following multi-domain AI-RRM prediction models in 6G accommodating to different application scenarios with non-collocated and collocated inter-cell deployments through simulations:

- Joint frequency and/or spatial domain

- Joint time and frequency domain

- Joint time, frequency and spatial domain

- For L3 cell measurement use cases:

- L3 cell level time domain (intra-cell) prediction (i.e., use case (a)) no matter for FR1-FR1 or FR2-FR2 has dependence with RAN2-led AI MOB use cases (in 5G/5GA)

- L3 cell level frequency domain (inter-cell, collocated deployment) prediction has dependence with RAN2-led AI MOB use cases (in 5G/5GA)

- L3 cell level frequency domain (inter-cell, non-collocated deployment) prediction has no dependence with RAN2-led AI MOB use cases (in 5G/5GA)

- For L3 beam measurement, the AI-RRM prediction models analysis including scenario, methodology and the dependence with other WG-led use cases (in 5GA or 6G) for each sub use-case are summed up in Table 9.

- For AI-RRM prediction models related use cases, which have or will have dependence with other WG-led use cases (in 5G/5GA/6G), RAN4 can to study how such predicted measurements impact the RRM requirement in certain application scenarios in 6G based on other WG’s feasibility verification outcome

- From RAN4 impact perspective, at least the following four scenarios can be as proactive RAN4-driven application scenarios in 6G AI RRM:

- Measurement gap reduction

- Dynamic adaptation of measurement procedure

- RX beam sweeping reduction (L1/L3)

- AI/ML based RLM/CBD, LTM by measurement prediction

- The motivation of reducing MG in 6G AI RRM contain: Throughput improvement, UE power consumption reduction, measurement overhead and mobility performance balance.

- For measurement-gap reduction application scenarios, to verify AI/ML-assisted approach is beneficial for facilitating NW resource scheduling and obtaining performance gain, RAN4 should to figure out the MG design in which G (5G or 6G) can be as the comparable non-AI baseline.

- RAN4 to study and evaluate the feasibility of multi-domain L3 level prediction for measurement-Gap reduction in 6G AI RRM

- Multi-domain combining time and frequency domain L3 level AI-RRM prediction model can be evaluated as baseline

- Both co-located and non-collocated inter-cell prediction are considered

- For measurement-gap reduction application scenarios, the details of MG-reduction use case are shown in Table 10, wherein multi-domain combining time and frequency domain L3 cell level AI-RRM prediction model is as example.

- RAN4 to study and evaluate the feasibility of dynamic adaptation of measurement procedure in 6G AI RRM

- Which L3 level single-domain AI-RRM prediction model (s) can be evaluated as baseline

- Potential specification impact should be clarified no matter AI/ML-assisted UE dynamic measurement is or is not transparent to the network

- Both co-located and non-collocated inter-cell prediction are considered

- RAN4 to study and evaluate the feasibility and testability of FR2 RX beam sweeping reduction (L1/L3) in 6G AI RRM

- Which L1/L3 beam measurement AI-RRM prediction models with/without multi-domain consideration can be evaluated as baseline to evaluate beam sweeping factor reduction

- Both co-located and non-collocated deployment can be considered if inter-cell prediction is involved.

- RAN4 to study and evaluate the feasibility and applicability of AI/ML based RLM/BFD/CBD, LTM by measurement prediction based on L1/L3 beam measurement multi-domain AI-RRM prediction models in 6G AI RRM

- Whether to/How to consider indirect and direct methods in RLM measurement prediction application scenario

# 5. Reference

[1] RP-252912 (revision from RP-251881), “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN#109.

[2] R4-2511652, “RAN4 6G Plan and Meeting Arrangement”, RAN4 chair, RAN4#116.

[3] R4-2514636,	“Topic Summary for [116bis][107] 6G AI”, Samsung, RAN4#116bis.

[4] R4-2514637,	“WF on 6G AI”, Samsung, RAN4#116bis.

[5] RP-242393, “Revised SID on AIML for mobility in NR”, OPPO, RAN#105.

[6] R4-2520358, “Discussion on general RF and UE RF for 6GR”, Samsung, RAN4#117.

[7] Bin Yu, Chen Qian, et.al, Light-Weight AI Enabled Non-Linearity Compensation Leveraging High Order Modulations. IEEE Trans. Commun. 72(1): 539-552 (2024)

[8] TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”, 3GPP.