3GPP TSG-RAN WG4 Meeting #117	R4-2520394

Dallas, USA, Nov. 17-21, 2025

Source: 	vivo

Title: 	On remaining issues for LP-WUR maintenance

Agenda Item:	4.22.3

Document for:	Discussion

Introduction

In RAN Plenary #102 meeting, WI for “Low-power wake-up signal and receiver for NR” was agreed [1]. The WI was further updated at RAN Plenary #103 meeting as [2] and RAN #105 [3].

The objectives of WI are duplicated as below:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

  - Specify necessary RRM requirements

The WI was officially closed at RAN4 116 meeting. The topic summary and WF at previous meeting in the maintenance phase can be found at [4] and [5].

Discussion

Issue 1-1-1-1: LP-WUR operation with Rel-18 EMR

- Proposals

  - P1: RAN4 confirms that no spec change is needed for R18 EMR. (Huawei)

  - P2: For Rel-18 eEMR if the UE has been woken up by LP-WUS from fully offloading scenario, UE may skip the validityCheck for the configured carriers (measIdleCarrierListNR or measReselectionCarrierListNR). (Nokia)

  - P3: For a UE which supports Rel-18 EMR, UE shall perform the configured Rel-18 EMR carriers’ measurement by following the higher priority frequency measurement requirements in MR offloading mode. (Ericsson)

Background:

Agreement (from RAN4 116):

When both Rel-16 EMR and Rel-19 LP-WUR are configured:

When T331 is running

  - When Case#1 (RRM offloading) conditions are met:

    - The UE shall search for NR inter-frequency layers configured for idle mode CA/DC measurements by following the higher priority frequency measurement requirements (i.e., based on 60s).

    - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements (i.e., based on 60s).

  - When Case#3 (RRM relaxation) conditions are met:

    - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ

      - The UE shall search for NR inter-frequency layers configured for idle mode CA/DC measurements by following the higher priority frequency measurement requirements (i.e., based on 60s).

      - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements (i.e., based on 60s).

    - if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ

      - the UE shall search for NR inter-frequency layers configured and not configured for idle mode CA/DC measurements by following the legacy measurement requirements (no relaxation)

when T331 is not running,

  - Serving cell, intra-frequency and NR inter-frequency layers not configured for idle mode CA/DC measurements and NR inter-frequency layers configured for idle mode CA/DC measurements and mobility measurement follow corresponding agreed requirements defined in case 1 and case 3 when they are satisfied.

Regarding the interworking between Rel-18 EMR and LP-WUR, the following specs for Rel-18 EMR is copied:

4.7.1	Introduction

The requirements of measurement report for fast CA/DC setup apply in this clause for:

-	UE supporting measValidationReportEMR-r18 and configured with measIdleCarrierListNR-r16 and/or measIdleCarrierListEUTRA-r16 by higher layers. (refer as A in this contribution)

-	UE supporting measValidationReportReselectionMeasurements-r18 and configured with measReselectionCarrierListNR-r18 by higher layers. (refer as B in this contribution)

A UE supporting measValidationReportEMR-r18  and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall report the results of the idle mode measurement which is specified in clause 4.4, and according to the measurement reporting requirements specified in clause 4.7.3.

A UE supporting measValidationReportReselectionMeasurements-r18 shall report the results of the idle mode measurement specified in the clause 4.2.2, and according to the measurement reporting requirements specified in the clause 4.7.3.

For case A, measValidationReportEMR-r18 is configured with measIdleCarrierListNR-r16 and/or measIdleCarrierListEUTRA-r16 hence it was covered by existing agreements already.

For base B, when measValidationReportReselectionMeasurements is configured the measurement requirements will be based on the idle mode measurement specified in 4.2.2 hence there is no further specs update needed for Rel-18 EMR.

Proposal 1: For the interworking between Rel-18 EMR and LP-WUR, besides existing agreement at RAN4 116, there is no further specs impact.

Issue 1-1-4: UE behavior when both Rel-16 relaxation and Rel-19 LP-WUR offloading/relaxation are satisfied; or when both Rel-17 relaxation and Rel-19 LP-WUR offloading/relaxation are satisfied

- Proposals

  - P1: UE is allowed to follow the most relaxation requirements when multiple relaxation criteria are configured and satisfied. (Xiaomi Huawei)

  - P2: In case #1 and case #3 for LP-WUR based RRM, if R16/17 low mobility or stationary condition/criteria is configured (including with/without not-at-cell-edge(NCE) criteria), UE shall follow the principle of RRM relaxation in R16/17 power saving requirements for IDLE/Inactive mode. (Apple)

Background:

Agreement at RAN4 116:

UE is allowed to follow the most relaxation requirements when only Rel-16 not at cell edge is configured, and Rel-16 not at cell edge and Rel-19 LP-WUR offloading/relaxation are both satisfied and Rel-16 EMR is not configured or Rel-16 EMR is configured and T331 is not running.

The scenario to be further discussed are:

1. The NW configures both Rel-16/17 UE power saving feature and Rel-19 LP-WUR, and low-mobility criteria is configured in Rel-16/17 UE power saving.

2. The UE behavior for a UE supporting both Rel-16 UE power saving and Rel-19 LP-WUR, and the Rel-19 LP-WUR related criteria are satisfied however Rel-16 low mobility criteria are not satisfied.

Under this scenario it is still preferred to allow UE to follow the most relaxed requirements due to the following reasons: 1. Comparing with a Rel-19 UE, a UE supporting both Rel-19 and Rel-16 power saving feature should have consistent power saving gain; 2. From NW point of view, when a NW supports Rel-19 LP-WUR, the corresponding Rel-19 UE power saving gain should not be weaken simply if this NW also provides power saving opportunity for Rel-16/17 UEs, when comparing a NW only implements Rel-19 LP-WUR. 3. Logically, a requirement shall apply when its corresponding conditions are met, it will cause extra ambiguity to determine a requirement does not apply when its corresponding conditions are met however other conditions for other features are not met.

Proposal 2:

UE is allowed to follow the most relaxation requirements when both Rel-16 criteria are configured, and both Rel-16 and Rel-19 LP-WUR offloading/relaxation are both satisfied;

UE is allowed to follow Rel-19 relaxation requirements when both Rel-16 criteria and Rel-19 criteria are configured, and Rel-19 LP-WUR offloading/relaxation criteria are satisfied however Rel-16 low mobility criteria is not satisfied.

Issue 1-1-6: On whether to consider MR wake up delay after MR exits from case 3 due to relaxation conditions cannot be met

- Proposals:

  - P1: Do not apply the wake-up delay in the scenario when exiting case 3. (oppo xiaomi)

  - P2: When exiting from case 3 due to exiting conditions of RRM relaxation is met, MR reuse the existing wake up delay (vivo Huawei Nokia)

  - P2-1: Select a single value for wake-up time from 10ms, 100ms or 800ms. During the wake-up time UE uses SSB periodicity for the measurements. (Nokia)

At RAN4 116 meeting, the following agreements are available:

Issue 1-5-3: MR wake up delay after exiting case 1 due to offloading conditions cannot be met

Agreement: When exiting case 1 due to exiting conditions of offloading is met, MR reuse the existing wake up delay.

FFS on the measurement periodicity after wake up.

FFS on whether the wake up delay apply in the scenario when exiting from case 3.

The exiting from case 3 to legacy state can be triggered by either when MR condition is not satisfied or when LR condition is not satisfied, providing LR condition is configured, based on the following RAN2 131 meeting agreement:

The exit condition for RRM relaxation is defined as ‘not fulfilling the entry condition’.

If the exit from case 3 to legacy is triggered by LR, considering the DRX cycle with a scaling factor 16, there could a few case where the exiting happens between two MR measurement occasions and it is also likely that the MR goes into deep sleep mode between two MR measurement occasions, as illustrated.

Figure 1 MR wake up time after exiting RRM relaxation

Since the MR wake up delay has already been used in the scenario when exiting from case 1 and exiting from LP-WUS monitoring, it is reasonable to apply the MR wake up delay when exiting from case 3 to align the UE behaviour. After the MR wake up delay, the MR can simply follow the legacy measurement requirements.

Proposal 3: When the UE is in RRM relaxation (case 3) state, and the UE exits from case 3 due to the configured LR conditions of RRM relaxation is not met, MR reuse the existing wake up delay.

Issue 1-1-7: On MR measurement behaviour after exiting case 1 (case 3) due to corresponding conditions cannot be met

- Proposals:

  - Scenario 1: exit from case 1 due to corresponding condition cannot be met:

    - Option 1: UE follows legacy measurement and meet the corresponding requirements. (oppo Huawei)

    - Option 2: Up to UE implementation and no requirements apply. (oppo)

    - Option 3: UE shall meet the requirements for Case 3 (if the related relaxation criteria are configured); (xiaomi)

  - Scenario 2: exit from case 3 due to corresponding condition cannot be met:

    - Option 1: After existing Case 3, UE shall meet the legacy requirements. (xiaomi vivo)

  - P1: RAN4 to discuss transition requirement among normal measurement(Rel-15, Rel-16 measurement) and Rel-19 MR RRM measurement relaxation, offloading (Ericsson)

Recommendations:

For this issue, for scenario 2, option 1 is the only choice. For scenario 1, when a UE exits from offloading mode, it cannot know whether the RRM relaxation conditions, if configured, are met or not due to lack of MR measurement results. A reasonable way is the MR follow the legacy measurement cycle and after the MR collects a few samples, it can determine whether the RRM relaxation conditions, if configured, are met or not. Hence, option 1 is supported.

Proposal 4:

- When a UE exits from case 1 due to corresponding condition cannot be met, after exiting case 1, UE follows legacy measurement requirements.

- When a UE exit from case 3 due to corresponding condition cannot be met, after exiting case 3, UE follows legacy measurement requirements.

Issue 1-1-2: LP-WUR operation with SDT

- Proposals

  - P1: SDT requirements apply also for UE supporting LP-WUS.

  - P1-1: The UE supporting SDT and LP-WUS uses LR-RSRP measurements for SDT condition evaluation when the evaluation is started while the UE is in MR offloading. (Nokia)

Recommendations:

For the SDT, current in the TS38.133 the following requirements are available:

5.5.2	Requirements on UE synchronization for small data transmissions

The UE is allowed to transmit using the configured uplink resources provided that the UE is synchronized towards (i.e. using the timing derived using the latest available NTA value as specified in subclause 7.1.2) the serving cell prior to transmission. If the UE is not able to obtain the synchronization towards the serving cell, then the UE shall drop the small data transmission.

5.5.3	TA validation requirements

If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions, then the UE shall not validate the CG-SDT using RSRP1 and RSRP2 and shall not transmit using CG-SDT.

Based on above requirements, it is up to UE to wake up from offloading mode in time if that UE prefer to perform validation and transmit its data. Otherwise if the validation conditions are not met the UE simply drop the small data transmission. Hence there is no impact on LP-WUR requirements.

Proposal 5: No impact on LP-WUR requirements for LP-WUR operation with SDT.

Conclusion

In this contribution, we provide our considerations on the LP-WUS/WUR WI and have the following observations and proposals.

Proposal 1: For the interworking between Rel-18 EMR and LP-WUR, besides existing agreement at RAN4 116, there is no further specs impact.

Proposal 2:

- UE is allowed to follow the most relaxation requirements when both Rel-16 criteria are configured, and both Rel-16 and Rel-19 LP-WUR offloading/relaxation are both satisfied;

- UE is allowed to follow Rel-19 relaxation requirements when both Rel-16 criteria and Rel-19 criteria are configured, and Rel-19 LP-WUR offloading/relaxation criteria are satisfied however Rel-16 low mobility criteria is not satisfied.

Proposal 3: When the UE is in RRM relaxation (case 3) state, and the UE exits from case 3 due to the configured LR conditions of RRM relaxation is not met, MR reuse the existing wake up delay.

Proposal 4:

- When a UE exits from case 1 due to corresponding condition cannot be met, after exiting case 1, UE follows legacy measurement requirements.

- When a UE exit from case 3 due to corresponding condition cannot be met, after exiting case 3, UE follows legacy measurement requirements.

Proposal 5: No impact on LP-WUR requirements for LP-WUR operation with SDT.

- References

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 102

[2] RP-240801, Revised WID: Low-power wake-up signal and receiver for NR ((LP-WUS/WUR), RAN 103

[3] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 105, vivo, NTT DOCOMO

[4] R4-2514811, WF on NR_LPWUS_RRM, RAN4 116bis, vivo

[5] R4-2513516,	Topic summary for [116bis][210] NR_LPWUS_RRM, RAN4 116bis, vivo