3GPP TSG-RAN WG4 Meeting #117			                        R4-2520436

Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.4.3

Source: 	CMCC

Title: 	Discussion on 6G BS RF coexistence

Document for:	Discussion

# 1. Introduction

The last meeting discussed some aspects of 6G BS coexistence. Apart from the candidate frequencies, there was not much consensus reached among the companies. Therefore, the priority of coexistence research should be considered first at this meeting.

This contribution will analyze the key points of 6G coexistence research from the perspective of operators.

- Discussion

### 2.1 General consideration on 6GR coexistence studies

During last meeting, RAN4 has achieved the following agreement:

Agreement List of candidate frequencies: 700 MHz, 2 GHz, 4 GHz, 7 GHz, 15 GHz and 30 GHz. For each exemplary frequency, companies proposing to redo coexistence studies should: - Identify the key assumptions differences (comparing to the assumptions used when RAN4 did the coex study e.g. the work done in TR 38.921 for 6.425-7.125GHz - Identify the potential impacts on the conclusions of previous coex studies (e.g. how much ACLR/ACS difference is expected). - Identify the next steps (update existing requirements? New requirements/new type? Regulation impacts?).

The last meeting listed some candidate frequencies for 6GR coexistence studies. From our perspective, re-evaluating all frequency bands implies a huge amount of work. Moreover, compared to the existing NR or LTE network deployment frequencies, the deployment of 6GR networks may preferentially favor frequencies that have not yet been deployed, such as the frequency around 7GHz. Therefore, it is recommended to set priorities for the candidate frequencies.

Proposal 1: It is proposed to set priorities for the candidate frequencies for coexistence studies.

Among the above candidate frequencies, from the perspective of operators, the 7GHz frequency band should be listed as a high priority. The 7GHz frequency band includes the 5925-7125MHz frequency range that has already completed IMT division. This band has not yet deployed other networks and can deploy 6GR networks. While the 7125-8400MHz frequency band has not yet completed IMT division, it also requires coexistence studies.

Proposal 2: It is suggested to list the frequency around 7GHz as a high priority among the candidate frequencies.

For the coexistence study in the frequency around 7GHz, since it is close to the band n104, and the IMT study in the 6.425-7.125GHz has already been completed in TR 38.921. Compared to the assumptions used in TR 38.921 for the 6.425-7.125GHz range, most of the simulation assumptions can be reused.

Proposal 3: Simulation assumptions in TR 38.921 could be set as starting point for frequency around 7GHz.

For the Network layout model, the urban macro, indoor hotspots, and other scenarios listed in TR 38.914 are typical models. Considering that the 6G network has not yet been deployed, it is recommended to give priority to considering these scenarios. Among them, the urban macro scenario should be classified as having a high priority.

Observation 1: Deployment scenarios in TR 38.914 are typical network layout models.

For the modelling of antenna and beamforming patterns, the current issue lies in the number of antenna elements in 6GR BS. More studies are still needed in this part. In addition, the modelling of ACLR and ACS has not been determined yet, and further study is also required in these two parts.

Observation 2: More discussion is need for antenna and beam forming pattern modelling, and ACLR and ACS modelling.

Proposal 4: General consideration on frequency around 7GHz coexistence study is listed in table 1.

Table 1. General parameters for frequency around 7GHz coexistence study

| Frequency Range | Around 7GHz |
| --- | --- |
| Network layout model | Urban macro (high priority), Indoor hotspot, Dense urban, Rural |
| Propagation model | assumptions in TR 38.921 as starting point |
| Antenna and beam forming pattern modelling | Further discussion |
| Received power model | assumptions in TR 38.921 as starting point |
| Transmission power control | assumptions in TR 38.921 as starting point |
| ACLR and ACS modelling | Further discussion |
| Link level performance for 6GR coexistence | assumptions in TR 38.921 as starting point |
| Coexistence simulation methodology | assumptions in TR 38.921 as starting point |

# 3. Conclusions

Proposal 1: It is proposed to set priorities for the candidate frequencies for coexistence studies.

Proposal 2: It is suggested to list the frequency around 7GHz as a high priority among the candidate frequencies.

Proposal 3: Simulation assumptions in TR 38.921 could be set as starting point for frequency around 7GHz.

Observation 1: Deployment scenarios in TR 38.914 are typical network layout models.

Observation 2: More discussion is need for antenna and beam forming pattern modelling, and ACLR and ACS modelling.

Proposal 4: General consideration on frequency around 7GHz coexistence study is listed in table 1.

Table 1. General parameters for frequency around 7GHz coexistence study

| Frequency Range | Around 7GHz |
| --- | --- |
| Network layout model | Urban macro (high priority), Indoor hotspot, Dense urban, Rural |
| Propagation model | assumptions in TR 38.921 as starting point |
| Antenna and beam forming pattern modelling | Further discussion |
| Received power model | assumptions in TR 38.921 as starting point |
| Transmission power control | assumptions in TR 38.921 as starting point |
| ACLR and ACS modelling | Further discussion |
| Link level performance for 6GR coexistence | assumptions in TR 38.921 as starting point |
| Coexistence simulation methodology | assumptions in TR 38.921 as starting point |

# 4. Reference

- R4-2514638, WF on 6G BS RF and co-existence, Feature Lead (Ericsson)