# 3GPP TSG-RAN WG4 Meeting #117                             R4-2520457

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	6.11.3

Source: 	CMCC

Title: 			 Discussion on CSI reporting requirement and testing framework for CSI prediction

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion core requirements for CSI prediction.

- Discussion

Issue 1-1: Requirements for performance monitoring Agreement: No performance monitoring delay requirements are specified in RAN4 for CSI prediction. Issue 1-4: Core requirements for PMI prediction reporting Agreement: Introduce a new clause under Clause 6 (6.X) in 38.101-4 as “Reporting of predicted PMI” Issue 1-9: Activation Agreement: No activation requirement in RAN4 spec for CSI prediction will be introduced. - Note: it is a general understanding that UE has to be ready to start measurements for inference after sending RRC reconfiguration complete. No details will be specified in RAN4.

For CSI prediction, it was agreed not to define performance monitoring delay requirement. While whether to define reporting delay for inference is still open.

In RAN1 #121 meeting, it was agreed that for CSI prediction using UE-side model, for inference, in addition to legacy Z/Z’ for doppler codebook, UE may report the value of t per SCS. Based on RAN1 agreements, the inference reporting delay shall be at least legacy Z/Z’ plus t, t is reported by UE.

RAN1 agreements in RAN1#121 Agreement For CSI prediction using UE-side model, for inference, in addition to legacy Z/Z’ for doppler codebook, UE may report the value of t per SCS - Detailed value of t can be discussed in UE feature

Proposal 1: based on RAN1 agreements, for CSI prediction, the inference reporting delay is legacy Z/Z’ plus t, where t is reported by UE.

For performance monitoring, RAN1 agreed to support SGCS as a performance metric for performance monitoring type 3 (i.e. UE reports performance metric to NW). In detail, reporting contents of UE assisted performance monitoring includes two SGCS: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

RAN1 agreements in RAN1#120bis meeting Agreement For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric. RAN1 agreements in RAN1#121 meeting Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, - one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCS is reported - wideband frequency granularity - only for one prediction instance configured by NW - per layer, and the total number of layers is the same as the reported RI in the associated inference report - Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’ - Each SGCS value is quantized with 4-bit - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale. - One codepoint is used to indicate the value range of (0 0.3] - Only one monitoring resource set is configured in the CSI-ReportConfig - The one configured time instance (i.e. f-th doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1 Agreement For CSI prediction using UE-side model, support single UCI part with the following report format (i.e., CSI field mapping order) among a set of SGCSs. SGCS1 #1. SGCS1 #2. …. SGCS1 #v. SGCS2 #1. SGCS2 #2. …. SGCS2 #v. - SGCS1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI, - SGCS2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCSi #k is the SGCS of k-th layer of i-th SGCS where k ={1, …., v}, and i ={1, 2}. - v is the value of the reported RI in the associated inference report. RAN1 agreements in RAN1#122 meeting Agreement Adopt following TPs for TS38.214 Section 5.2.1.4.6. Reason for change; In current specification, there is no SGCS quantization mapping table.. Summary of change; SGCS quantization mapping table is added.. Consequences if not approved; Incomplete capture of the agreement.. TS38.214 5.2.1.4.6 CSI-PAI reporting << Unchanged parts are omitted >> - each calculated SGCS value is quantized to a 4-bit field, where the first codepoint ‘0000’ is used to indicate (0 0.3] value range, and the value range [0.3, 1] is quantized in a linear scale with 0.05 step size. Detailed mapping is specified in [xx, TS 38.abc]Table 5.2.1.4.6-1. Table 5.2.1.4.6-1: SGCS Quantization Mapping Code Point; Range. 0; [0.0000, 0.3000]. 1; (0.3000, 0.3500]. 2; (0.3500, 0.4000]. 3; (0.4000, 0.4500]. 4; (0.4500, 0.5000]. 5; (0.5000, 0.5500]. 6; (0.5500, 0.6000]. 7; (0.6000, 0.6500]. 8; (0.6500, 0.7000]. 9; (0.7000, 0.7500]. 10; (0.7500, 0.8000]. 11; (0.8000, 0.8500]. 12; (0.8500, 0.9000]. 13; (0.9000, 0.9500]. 14; (0.9500, 1.0000]. 15; Reserved. << Unchanged parts are omitted >>.

Normally, RAN4 need to define reporting mapping for the reported quantity. SGCS is newly introduced, and reporting mapping is required. According to RAN1 agreements, the reported SGCS value is quantized with 4-bit. 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale, and one codepoint is used to indicate the value range of (0 0.3]. According to RAN1 latest agreements, SGCS quantization mapping table is added to TS 38.214. RAN4 no need to define the reporting mapping for SGCS.

Proposal 2: for CSI prediction, RAN4 no need to define report mapping for SGCS, since RAN1 has defined the SGCS quantization mapping table in TS38.214.

- Conclusion

This contribution provides discussion on CSI prediction. The proposals are:

Proposal 1: based on RAN1 agreements, for CSI prediction, the inference reporting delay is legacy Z/Z’ plus t, where t is reported by UE.

Proposal 2: for CSI prediction, RAN4 no need to define report mapping for SGCS, since RAN1 has defined the SGCS quantization mapping table in TS38.214.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2514631, WF on [116bis][112] R19 AI for air interface, Qualcomm Incorporated

- R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated

- R4-2505105, WF on requirements for AI/ML air interface normative work, Qualcomm Incorporated

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2502970, Baseline Simulation assumptions for AIML based CSI prediction