3GPP TSG-RAN WG4 Meeting #117														R4-2520493

Dallas, US, 17 – 21 November, 2025

Agenda Item:	8.11

Source: 	Xiaomi

Title: 	Discussion on 6GR testability and OTA

Document for:	Discussion

- Introduction

RAN4 had some preliminary discussion on the testability methodology framework and key assumption for 6G system. The related WF was agreed in [1]. In this contribution, we continue to provide our preliminary views on the testability methodology framework and key assumption for 6G system.

- Discussion

Improved testability of Conducted Requirements

Sub-topic 1-2 Improve traditional conducted test to OTA Issue 1-2-1: Whether it is valuable to improve some conducted requirements to be verified via OTA approach Agreements: - RAN4 can further study the testability limitations of some conducted test cases, consider the following aspects as starting point, e.g., - FR1 conducted test cases already consider antenna-performance impacts but not verified via radiated approach in a case-by-case manner, e.g. MSD - FR1 test case simplification with both conducted and radiated considered, e.g., spurious emission - Discuss which conducted conformance testing could either yield testability issues, e.g., lack of physical Tx/Rx ports, and/or highly dynamic nature of multiple Tx/Rx operation, including antenna tuning for impedance and/or pattern.

MSD aims to evaluate receiver sensitivity degradation under simultaneous uplink transmission, typically in FDD CA scenarios. In NR conformance testing, MSD is measured using a conducted test setup where the UE’s Tx/Rx ports are connected to the tester via cables, providing a controlled and isolated RF environment. However, for NR MSD conformance testing, RAN4 identified several limitations in CA scenarios. Conducted testing assumes ideal impedance matching and perfect duplexer isolation, excluding antenna coupling. This results in underestimation of real-world self-interference and desensitization. Consequently, MSD conducted testing may not fully represent real-word end-to-end UE performance.

Observation 1: The existing MSD conformance testing in NR underestimates the real-world self-interference by excluding antenna coupling.

However, from our perspective, introducing OTA-based MSD testing is not recommended which will introduce significant complexity and uncertainty without proportional benefits. In our understanding, the new issues will be expected with introducing MSD OTA testing:

- Repeatability and inter-lab consistency: OTA measurements heavily depend on chamber setup, probe 	calibration, and device orientation, which can vary across test labs. These will cause the repeatability issues and 	comparability issues.

- Cost and test duration: MSD OTA testing with a large number of carrier aggregation combinations would 	significantly increase the test cost, test time, and the chamber resource burden.

- Traceability to design: The MSD conducted tests are closely linked to the transceiver design (chip, duplexer, 	leakage paths), 	which can enable direct root-cause analysis. However, the MSD OTA testing blurs this 	boundary by mixing antenna coupling effect and RF impact, which will 	result the debugging more difficult.

- Ambiguity of failure causes: The MSD OTA testing results do not easily distinguish whether the sensitivity 	degradation is caused by transceiver leakage, insufficient isolation, or antenna coupling impact.

Observation 2: Introducing MSD OTA testing will introduce significant uncertainty, complexity and cost without the proportional benefit.

In addition, RAN4 is discussing the improvement of MSD framework in UE RF thread in [2],  wherein some potential enhancement was proposed based on  the conducted testing framework, e.g. MSD reporting. It is premature to directly pursue OTA-based MSD testing, and we can instead wait for the consensus on the overall MSD framework design.

Observation 3: It is premature to directly study the OTA-based MSD testing unless there is sufficient consensus on the overall MSD framework design.

Testability for new 6GR frequency

Issue 2-1-1: Testability for New Frequencies between FR1 and FR2 Agreements: RAN4 study the testability including conducted testing and radiated testing for the new frequencies. The following aspects can be considered: - The frequency range definition will be discussed under the spectrum agenda. - For conducted test, use existing test method as a starting point - For OTA test - Further study potential test methodologies including the full-package, i.e., supported frequency range, test setup, configuration, positioning, procedure, validation/calibration, quiet-zone/test zone, MU, testing time reduction

In last meeting, RAN4 had some discussion on the frequency break-point of the new 6GR frequency range, which is important to guide which test methodology, e.g. conducted or OTA will be defined. And from our perspective, it is important to have a preliminary assumption of frequency break-point for the testability discussion. For the new 6GR frequency range, we propose to use FR1-style conducted testing for up to 8.4 GHz. And regarding above 15GHz, we think the FR2-style OTA testing can be reused and as a starting point.

Observation 4: From testability perspective, it is assumed that the frequency break-point of FR1-style conducted can be up to 8.4GHz.

OTA Testability for Different UE Types

Sub-topic 4-1 OTA testability for different device types Issue 4-1-1: OTA testability applicability for different UE types in 6GR day-1 Agreements: - RAN4 consider the study of OTA test system to better accommodate different UE types, new feature/functionalities, new performance metrics and test cases. - The UE types and form factor considered in testability can be further discussed in this SI. - RAN4 study the feasibility of developing a single system to cover above aspects.

From our perspective, it is most practical to use handheld UEs as the baseline for developing OTA test systems. These devices are well understood, widely used, and have established radiating characteristics from NR FR1/FR2 OTA testing. By starting with a handheld baseline, RAN4 can calibrate and validate OTA chambers, probe layouts, and reference configurations with a stable and representative device.

Observation 5: It is practical to to use handheld UEs as the baseline for developing OTA test systems in 6GR day-1.

Once the handheld baseline is mature and well understood, RAN4 can study the extension to other UE types and/or new feature/functionalities by considering the adaptation factors, e.g., antenna count, form factor, antenna radiated orientation, including:

- Tablets and laptops with larger form factor and different antenna layouts

- Wearable or IoT devices, potentially with very different radiation behavior

- CPE or FWA, which may have externally mounted or directional antennas

- NTN UEs/VSATs

Observation 6: RAN4 can study the extension of OTA test systems to other UE types and/or new feature/functionalities by considering the adaptation factors, e.g., antenna count, form factor, antenna radiated orientation.

Improved test methods and metric for SISO OTA

Sub-topic 7-1 Enhanced OTA test method for TRP/TRS Issue 7-1-1: Enhanced OTA test method for TRP/TRS Agreements: - RAN4 further study the feasibility of incorporating real-world usage scenarios more closely into the 6GR OTA testing methodology. - FFS on OTA configurations/parameters to reflect real-world scenarios, e.g., support antenna switching, and other aspects.

From our perspective, it is necessary to study OTA test configurations that reflect real-world usage scenarios, e.g. supporting antenna switching.  UE may use multi-antenna switching (e.g., main/diversity antennas, different frequency band antennas) to adapt to diverse scenarios (e.g., low signal strength, multi-band concurrent transmission). OTA tests should simulate this switching behavior to evaluate TRP/TRS performance during transitions. For example, a terminal may switch from a Sub-6GHz antenna to a 7GHz antenna when moving to a high-frequency coverage area; OTA tests need to verify that this switch does not cause unexpected power loss.

Observation 7: it is necessary to study OTA test configurations that reflect real-world usage scenarios, e.g. supporting antenna switching.

In addition, regarding the OTA test configuration for 6GR, from our perspective, it is benefit and efficiency that OTA test configurations can be upgraded based on the existing 5G OTA system instead of being completely reconstructed. For example:

- Upgrade existing multi-probe anechoic chambers with software-defined capabilities, and realize compatibility with 6G scenarios by adding “antenna switching control modules”  to reduce hardware replacement costs.

- Reuse some 5G test procedures (e.g., basic TRP measurement procedures) and only add scenario-based configuration parameters to reduce the cost of test.

Observation 8: For the SISO OTA test configuration in 6GR, it is benefit and efficiency that OTA test configurations can be upgraded based on the existing 5G OTA system.

- Conclusion

In this contribution, we provided our preliminary view and observations on the testability methodology framework and key assumption for 6G system as follows:

Observation 1: The existing MSD conformance testing in NR underestimates the real-world self-interference by excluding antenna coupling.

Observation 2: Introducing MSD OTA testing will introduce significant uncertainty, complexity and cost without the proportional benefit.

Observation 3: It is premature to directly study the OTA-based MSD testing unless there is sufficient consensus on the overall MSD framework design.

Observation 4: From testability perspective, it is assumed that the frequency break-point of FR1-style conducted can be up to 8.4GHz.

Observation 5: It is practical to to use handheld UEs as the baseline for developing OTA test systems in 6GR day-1.

Observation 6: RAN4 can study the extension of OTA test systems to other UE types and/or new feature/functionalities by considering the adaptation factors, e.g., antenna count, form factor, antenna radiated orientation.

Observation 7: it is necessary to study OTA test configurations that reflect real-world usage scenarios, e.g. supporting antenna switching.

Observation 8: For the SISO OTA test configuration in 6GR, it is benefit and efficiency that OTA test configurations can be upgraded based on the existing 5G OTA system.

Reference

- R4-2514647	WF for [116bis][110] 6G testability and OTA, vivo

- R4-2514509	Topic Summary for [116bis][102] 6G general RF and UE RF, Qualcomm