# 3GPP TSG-RAN WG4 Meeting #117	R4-2520506

# Dallas, US, 17-21 Nov, 2025

Agenda Item:	8.2.5.1

Source: 	Xiaomi

Title: 	View on 6GR waveform

Document for:	Discussion

- Introduction

According to the endorsed 6G work arrangement from RAN4 chair (R4-2511652), RAN4 divides 6G study into 11 separate topic areas in initial stage. System parameter is one of major agenda with following candidate scope:

In this contribution, our preliminary view on waveform and PA model are provided.

- Discussion

PA model

Regarding PA model, last RAN4 meeting has following agreements:

#### PA model Agreement - Consider PA modelling at least for the following aspects for RAN4 discussion - Memory effects for UE supporting larger CBW - Different PA models for different sub-frequency ranges, e.g. around 7GHz (high priority, PC3/PC2) and lower frequency bands (PC3) - Calibration conditions - RF impairments used for MPR evaluation, e.g. carrier leakage, I/Q imbalance, etc. - PA models for different device types, handheld UE is prioritized - Applicable requirements - 5G-A requirements as starting point - PA models for RAN1 waveform evaluation and RAN4 requirements discussion can be decoupled

PA model can be served for initial evaulation work on waveform and modulation orders and also needs to be used to develop RAN4 requirements e.g. Tx EVM, MPR and A-MPR. The urgency of above purpose is different, waveform needs to be decided initial stage of 6G study and RAN4 requirements will be continued studed even under WI phase.

Also the usage of PA model for waveform/modulation evalution was to evaluate relative performance gain compared reference waveform. For RAN4 requirements (MPR, and Tx EVM), PA model was used to develop absolute performance.

Given the urgency of PA model on waveform evalution, we sugguest to set-up deadline on PA model of RAN1 waveform evalution purpose.

Based on initial survey of availabel commercial PA in  ~7GHz with wide BW, the avaliable commercial PA is quite limited and performance not well optimized. Also under ~7GHz wit wide BW, memeory effect will be important. In TR 38.804, both reference PA model with memory and memoryless was provided. We sugguest to only consider PA model with memory effect for 6GR evaluation.

Obseravtion 1: PA model for waveform evalution was to evalaute relative performance compared to reference waveform and PA model for RAN4 requirement  was used to develop absolute performance.

Proposal 1: Decouple PA models for RAN1/RAN4 waveform evaluation and PA model used for RAN4 requirement development.

Propsoal 2: For PA Model of RAN1/RAN4 wavefrom evaluation, deadline as Q1’26 and only PA model with consideration of memory effect shall be considered.

Proposal 3: PA model from TR 38.803 with memory effect can be considered as default one for RAN1 evaluation  if no avaliable commercial PA model collected till Q1’26

Waveform evalaution

#### Waveform Agreement - The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1 - To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects - Tx assumption including PA model - Related RF requirements which should be taken into consideration - Both UL and DL are considered - Take the candidate waveforms identified/discussed in RAN1 as the baseline - Identify the evaluation metric, e.g. net gain - Implementation constraints - Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs - Evaluation cases could be selected step-by-step, based on RAN1 progress - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement

Regarding on the evaulation of waveform, tight cooperation between RAN1 and RAN4 was required. Net gain is agreed in RAN1 as metric for performance comparsion:

  - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER

Tx power gain can be evaluated in RAN4, and SNR degradation can be evaluated in RAN1.

Proposal 4: RAN4 evaluation work focus on Tx power gain and RAN1 evaluation work focus on SNR degradation with same PA model applied.

On the basic RF requirements assumption, relevant requitements is still on discussion especially on ~7GHz for 6GR.

We suggest to consider existing NR requirements as baseline for initial evaluation purpose.

Proposal 5: Using existing NR requirements as baseline for initial evaluation purpose including:

- ACLR/ACS

- Tx EVM

- Image rejection

- Tx Leakage

- CIM3/CIM5

- Conclusion

In this contribution, our preliminary view on waveform and PA model are provided.

Obseravtion 1: PA model for waveform evalution was to evalaute relative performance compared to reference waveform (urgent for intial stage/milestone) and PA model for RAN4 requirement  was used to develop absolute performance (can be discussed over SI and WI).

Proposal 1: Decouple PA models for RAN1/RAN4 waveform evaluation and PA model used for RAN4 requirement development.

Propsoal 2: For PA Model of RAN1/RAN4 wavefrom evaluation, deadline as Q1’26 and only PA model with consideration of memory effect shall be considered.

Proposal 3: PA model from TR 38.803 with memory effect can be considered as default one for RAN1 evaluation  if no avaliable commercial PA model collected till Q1’26

Proposal 4: RAN4 evaluation work for waveform focus on Tx power gain and RAN1 evaluation work focus on SNR degradation with same PA model applied.

Proposal 5: Using existing NR requirements as baseline for initial evaluation purpose including:

- ACLR/ACS

- Tx EVM

- Image rejection

- Tx Leakage

- CIM3/CIM5

Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] R4-2514650,“WF on 6GR system parameter”, Feature Lead (Huawei)