3GPP TSG-RAN WG4 Meeting #117									  R4-2520524

Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.11

Source:	CAICT

Title:	Views on Testability and OTA for 6GR

Document for:	Approval

- Introduction

In RAN#108, the 6G SI Study on 6G Radio was approved, with one of its core objectives being the investigation of testability and OTA aspects for 6GR [1]. During RAN4#116bis meeting, discussions on 6G testability and OTA led to the categorization of proposals into the following key topics [2]:

1)Improved testability of Conducted Requirements

2)New test methodologies for new 6GR frequencies

3)OTA test methods for Multi-Tx and CA

4)Testability for different Device types

5)AI/ML OTA testability

6)Harmonized testing for TN and NTN

7)Improved test methods and metric for SISO OTA

8)Improved test methods and metric for DL MIMO OTA

9)Test efficiency improvement

10)Testability for BS requirements

11)others

High-level agreements were also captured in the WF [3]. In this contribution, we provide our initial views on testability and OTA for 6GR.

- Discussion

  - Improved testability of Conducted Requirements

Sub-topic 1-1 Antenna assumption of FR1 transmitter and receiver Issue 1-1-1: Whether RAN4 should consider a More realistic antenna efficiency assumption for FR1 Agreements: - move this discussion to general RF/UE RF from next meeting. Sub-topic 1-2 Improve traditional conducted test to OTA Issue 1-2-1: Whether it is valuable to improve some conducted requirements to be verified via OTA approach Agreements: - RAN4 can further study the testability limitations of some conducted test cases, consider the following aspects as starting point, e.g., - FR1 conducted test cases already consider antenna-performance impacts but not verified via radiated approach in a case-by-case manner, e.g. MSD - FR1 test case simplification with both conducted and radiated considered, e.g., spurious emission - Discuss which conducted conformance testing could either yield testability issues, e.g., lack of physical Tx/Rx ports, and/or highly dynamic nature of multiple Tx/Rx operation, including antenna tuning for impedance and/or pattern.

- It is recognized that OTA testing provides a more realistic assessment of UE performance compare to conducted test,  however, it also introduces test complexity, cost, and test time at the same time. Therefore, it is unrealistic to transfer all conducted requirements with demands to OTA. A principled and selective approach is necessary. The key is to identify specific requirements where the added value of OTA testing outweighs its drawbacks, particularly for cases where conducted testing faces fundamental testability challenges or fails to capture critical real-world performance aspects.

- The evaluation under this framework should be considered, but not be limited to, the following criteria:

- Technical Necessity: Whether the OTA method provides a critically more realistic performance assessment that conducted testing cannot adequately capture.

- Testability Gap: Whether the conducted test faces inherent feasibility issues.

- Test Repeatability & Stability: Whether a stable, repeatable, and standardized OTA test setup and procedure can be defined.

- Cost & Complexity Impact: Whether the added value in performance realism justifies the anticipated increase in test complexity, cost and duration.

- Proposal 1: Consider technical necessity, testability gap, test repeatability & stability, and cost & complexity impact when selecting candidate testing for migration from conducted to OTA testing.

  - New test methodologies for new 6GR frequencies

Sub-topic 2-1 Testability for new 6GR frequency Issue 2-1-1: Testability for New Frequencies between FR1 and FR2 Agreements: RAN4 study the testability including conducted testing and radiated testing for the new frequencies. The following aspects can be considered: - The frequency range definition will be discussed under the spectrum agenda. - For conducted test, use existing test method as a starting point - For OTA test - Further study potential test methodologies including the full-package, i.e., supported frequency range, test setup, configuration, positioning, procedure, validation/calibration, quiet-zone/test zone, MU, testing time reduction

- The formal definition of frequency ranges is handled by the spectrum agenda. However, RAN4 must define the OTA methodology break-point to avoid a mismatch between spectrum definitions and test methods. This break-point marks the transition frequency from FR1-style to FR2-style testing. A key risk is that a band defined as FR1 might not be testable in FR1 OTA systems, forcing a shift to FR2 methods. This break-point will be set based on the upper frequency limit of FR1-style test systems and the lower frequency limit of FR2-style systems.

- For frequencies overlapping with 5G, existing NR test methods should be reused as the baseline.

- Proposal 2: For 6G frequency bands that overlap with existing 5G NR bands, the corresponding 5G FR1 and FR2 test methodologies shall be reused as the baseline.

- Proposal 3: RAN4 to evaluate the extension of existing FR1 and FR2 OTA test systems to cover new 6G frequencies, with the objective of identifying any coverage gaps or overlaps. This evaluation should address:

- - the practical upper-frequency limit of FR1 OTA test systems

- - the practical lower-frequency limit of FR2 OTA test systems

  - Improved test methods and metric for DL MIMO OTA

Sub-topic 8-1 MIMO OTA for 6GR Issue 8-1-1: RAN4 consider MIMO OTA for 6GR day-1 Agreements： RAN4 will study the dynamic MIMO OTA (at least dynamic channel model and link adaptation) for 6G. static MIMO is not precluded. The following can be considered as starting point: - 2D or 3D channel model - FFS phantom involved testing - FFS multi-TRP - FFS focus on FR1 as first priority - FR2 and new frequency range may also be considered

- The evolution from static to dynamic MIMO OTA in 5G, which incorporates dynamic channel models and link adaptation, has been widely adopted by standards bodies (e.g., 3GPP, CTIA, CCSA) as it provides a more accurate assessment of real-world performance. For 6G, there is no justified rationale to revert to a static methodology. Therefore, dynamic MIMO OTA should be established as the baseline, with static methods considered only as a fallback if critical issues are identified.

- Proposal 4: Dynamic MIMO OTA (at least dynamic channel model and link adaptation) is established as the baseline methodology for 6G.

- Furthermore, while MIMO OTA conformance testing for handheld UEs has been limited to free-space conditions, the TRP/TRS testing rightly emphasizes the importance of hand and head-hand phantom conditions. To maintain consistency and ensure real-world relevance, the impact of the hand phantom must be evaluated in 6G MIMO OTA test cases.

- Proposal 5: The study of 6G dynamic MIMO OTA should include the evaluation of hand model conditions. The test zone should be dimensioned accordingly to accommodate the phantom.

3	Conclusion

This contribution makes the following observations and proposals.

- Proposal 1: Consider technical necessity, testability gap, test repeatability & stability, and cost & complexity impact when selecting candidate testing for migration from conducted to OTA testing.

- Proposal 2: For 6G frequency bands that overlap with existing 5G NR bands, the corresponding 5G FR1 and FR2 test methodologies shall be reused as the baseline.

- Proposal 3: RAN4 to evaluate the extension of existing FR1 and FR2 OTA test systems to cover new 6G frequencies, with the objective of identifying any coverage gaps or overlaps. This evaluation should address:

- - the practical upper-frequency limit of FR1 OTA test systems

- - the practical lower-frequency limit of FR2 OTA test systems

- Proposal 4: Dynamic MIMO OTA (at least dynamic channel model and link adaptation) is established as the baseline methodology for 6G.

- Proposal 5: The study of 6G dynamic MIMO OTA should include the evaluation of hand model conditions. The test zone should be dimensioned accordingly to accommodate the phantom.

References

- RP-252912 Revised SID: Study on 6G Radio, RAN#109

- R4-2514517 Topic summary for [116bis][110] 6G testability and OTA, RAN4#116bis, vivo

- R4-25114640 WF for [116bis][110] 6G testability and OTA, RAN4#116bis, vivo