3GPP TSG RAN4 Meeting #117							R4-2520553

Dallas, Texas US, 17th – 21st November 2025

Source:	Nokia

Title:	6GR System parameters - Waveform

Agenda item:	8.2.1

Document for:	Approval

Introduction

Under 6G system parameters topic following items are to be discussed during the Q4 2025 [1].

Waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types.

At this meeting a sperate agenda have been set for the Waveform discussion which is discussed in this paper.

WF from last meeting

Discussion

Waveform

August RAN1#122 made the following agreement on waveform:

Agreement CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. Agreement CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc.

While additions to the above RAN1 agreement on 6G waveforms maybe done in the future, CP-OFDM for DL and UL as well as DFT-s-OFDM for UL should be used as RAN4 assumption for system parameters.

Support CP-OFDM waveform for downlink

Support CP-OFDM and DFT-s-OFDM for uplink

For further waveform related work in RAN1, RAN4 should discuss and agree on the PA modelling approach that RAN1 should use, and provide the information to RAN1 with a liaison statement:

Ask RAN1 to evaluate waveforms with a PA modelling approach and provide the related PA model to RAN1

PA model

Observed electrical memory effects are a function of channel center frequency. Since the same MPR applies to the entire frequency range (A-MPR applies to entire operating band), a PA model used in simulations should be generalized to represent the entire frequency range (operating band). Such generalization tends to reduce the memory effects of the PA model. Variations based on absolute frequency should be removed; the PA model should cover them in an upper-bound manner. Common memory effects present at all center frequencies can be retained. In the extreme case, a memoryless model could be used as an upper-bound model.

PAs differ by their memory effects. The specified MPR or A-MPR should be sufficient for all "reasonably good" PAs, hence the specified MPR should be the maximum of the back-offs needed by such PAs. The problem is how to define "reasonably good".

It would be beneficial if PA model discussed in this agenda can also be used in Tx EVM relaxation work. However if PA model targeted to this Tx EVM relaxation is not readily available then especially RAN1 cannot stop work on waveform evaluation. RAN4 should check what is the situation in PA model situation in Q1 2026 and discuss if PA model for this feature is ready to be shared with RAN1.

Aim to use the same PA model for RAN1/RAN4 waveform evaluations and RAN4 Tx requirement evaluation including Tx EVM relaxation.

The PA model calibration approach used in LTE and NR can be applied also to PA models with memory.

FDSS-SE

## Framework study for waveform - Main proposals - Close collaboration with RAN1 - RAN4 must work closely with RAN1 to define evaluation methodologies, assumptions, and a common PA model for simulations. - Some proposals suggest postponing detailed RAN4 waveform discussions until 2026, after RAN1 has made sufficient progress. - Key Performance Indicators (KPIs) - Waveform evaluation should include KPIs like PAPR, Cubic Metric (CM), SEM, ACLR, OBE, EVM, implementation complexity, and compatibility with 5G band combinations. - Evaluation metric - Propose using "Net Gain" as a starting point for evaluating low-PAPR schemes. - Specific studies include: - Study Low-PAPR techniques: Investigate both transparent and non-transparent techniques (like CFR-SE) to reduce PAPR and assess their impact on RF requirements. - Explore new waveforms for new use cases: Study the feasibility and impact of supporting DFT-s-OFDM in the DL (e.g., for massive IoT) and new waveforms for Integrated Sensing and Communications (ISAC). - Agreement - The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1 - To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects - Tx assumption including PA model - Related RF requirements which should be taken into consideration - Both UL and DL are considered - Take the candidate waveforms identified/discussed in RAN1 as the baseline - Identify the evaluation metric, e.g. net gain - Implementation constraints - Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs - Evaluation cases could be selected step-by-step, based on RAN1 progress Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement

The following has been agreed in RAN1 #122bis:

Agreement Final LS R1-2508069 is endorsed. Agreement - For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - FFS: Other evaluation metrics - Note: Companies to report how to calculate the Tx power gain, modulation and coding

Effect of lower PAPR waveform to coverage is due to increased UE maximum transmission power, however there may be some loss in link performance. Hence the actual effect to link budget can be described using net gain, which has been agreed to be used as performance evaluation criterion in RAN1 #122bis. Use of net gain requires PA model and hence also LS has been sent from RAN1 to RAN4 to request information of PA models especially for new 7GHz range.A low PAPR method used already in 5G is frequency domain spectrum shaping (FDSS). FDSS is a method where spectrum of DFT-s-OFDM signal is shaped by utilizing frequency domain shaping filter resulting to low complexity and good reduction in PAPR. FDSS was specified already in Rel-15 for pi/2 BPSK and later in Rel-18 with slightly different requirements for both pi/2 BPSK and QPSK. In Rel-18 timeframe also frequency domain spectrum shaping with spectrum extension (FDSS-SE) was discussed. FDSS-SE provides more enhancement compared to FDSS by adding excess bandwidth where data is copied from inband, see figure 2.3.2-1 below, where the symmetric extension stage copies the edges of the nominal frequency allocation to the extended allocation before the frequency domain spectrum shaping stage.

Spectrum shaping filtering could be done for FDSS/FDSS-SE either in transparent or non-transparent way. In transparent approach the same filter is used for both DMRS and data, hence the effect of the filter gets compensated in the receiver and the receiver does not need to be aware of the filter parameters. There is no need to specify anything related to filter except relaxed spectrum flatness requirement as done in NR (TS 38.101-1, sections 6.4.2.4.1 and 6.4.2.4.2). Spectrum flatness is a requirement for Tx signal defined by RAN4 to facilitate frequency domain equalization in base station receiver. Specifying filter parameters would require extensive discussions in 3GPP as the optimum filter depends also on implementation. Together with the fact that there are no clear benefits of non-transparent approach we believe that transparent approach should be followed also in 6G.

Both FDSS and FDSS-SE should be supported in 6G to provide enhanced coverage compared to 5G.

Figure 2.3.2-1. FDSS-SE

Frequency Domain Spectrum shaping (FDSS) and FDSS with spectrum extension (FDSS-SE) are supported in 6G Radio.

Transparent filtering approach is used for FDSS and FDSS-SE in 6G Radio.

Net gains of FDSS and FDSS-SE are shown in Figure 2.3.2-2 and Figure 2.3.2-3 for outer and inner RB allocations. In these results Tx power gains have been evaluated by RF simulations. Link simulation assumptions including MCSs used are as agreed in Rel-18 work item (RAN1 #111). As the results show, the net gains of FDSS-SE are much higher than FDSS. This is especially true for the inner allocations where the FDSS gains are modest.

Similar results obtained in Rel-18 work item and averaged over all the companies results can be found in [5]. Despite differences in different companies results the average gains seem to be in the same ballpark as in Figures 2.3.2- 2 and 2.3.2- 3.

Figure 2.3.2-2. Net gains of FDSS and FDSS-SE for outer RB allocations

Figure 2.3.2-3. Net gains of FDSS and FDSS-SE for inner RB allocations

Conclusion

In this contribution we present our view on 6GR General RF and UE RF. The following observations have been made:

Proposal 1: Support CP-OFDM waveform for downlink

Proposal 2: Support CP-OFDM and DFT-s-OFDM for uplink

Proposal 3: Ask RAN1 to evaluate waveforms with a PA modelling approach and provide the related PA model to RAN1

Proposal 4: Aim to use the same PA model for RAN1/RAN4 waveform evaluations and RAN4 Tx requirement evaluation including Tx EVM relaxation.

Proposal 5: Frequency Domain Spectrum shaping (FDSS) and FDSS with spectrum extension (FDSS-SE) are supported in 6G Radio.

Proposal 6: Transparent filtering approach is used for FDSS and FDSS-SE in 6G Radio.

- 4	References

R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4#116, RAN4 Chair

A. Melendez-Cano et al., "Behavioral Modeling for Power Amplifiers Comparing MPM, Wiener and Hammerstein with FPGA-Based Implementation," 2016 International Conference on Mechatronics, Electronics and Automotive Engineering (ICMEAE), Cuernavaca, Mexico, 2016, pp. 119-124, doi: 10.1109/ICMEAE.2016.030.