3GPP TSG-RAN WG4 Meeting #117	R4-2520707

Dallas, USA, November 17th – 21st, 2025

Source:	Nokia

Title:	On RRM measurement prediction (UE sided model)

Agenda item:	7.12.3

Document for:	Discussion

Introduction

The study on AI/ML Mobility was concluded in RAN4#116 in the scope of Rel-19 SI ‎[1]. Agreements achieved during this meeting are summarized in the WF ‎[2] and [3]. The new WI of Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR [4] was approved in RAN#109 meeting.

In this paper, we provide our views on issues related to RRM requirements for RRM measurement prediction use case, as listed below:

- Simulation assumptions

- Impact of measurement error on prediction accuracy

- Ground truth definition for RSRP accuracy for FR1

- Discussion on Prediction Accuracy for Temporal Domain Prediction

- Inter-frequency prediction scenarios

General aspects and Measurement event prediction use case are discussed in our accompanying papers [5] and [6].

Discussion

  - Simulation Assumptions

The following agreement on simulation assumptions for both RRM measurement prediction and indirect event prediction was made in RAN4#116bis.

Agreement: Take the simulation procedures used in AI/ML beam management as starting point for the simulation discussion of measurement prediction and [indirect measurement event prediction] to evaluate the predicted L3-RSRP accuracy considering the impact of measurement error.

Regarding scenarios and sub-use cases, following agreements were made online.

Agreement: RAN4 to discuss the potential RRM requirements for cell-level based prediction for the following scenarios: Evaluation scenario; Methodology. FR1 to FR1 intra-frequency temporal domain case A; intra-cell. FR1 to FR1 intra-frequency temporal domain case B; Intra-cell. FR1 to FR1 inter-frequency (frequency domain) (co-located case); Inter-cell. FR2 to FR2 intra-frequency temporal domain case A; Intra-cell. FR2 to FR2 intra-frequency temporal domain case B; Intra-cell. FFS whether to discuss the requirements for the following scenario: Evaluation scenario; Methodology. FR2 to FR2 inter-frequency (frequency domain) (co-located case); Inter-cell.

Agreement: - As starting point, evaluate sub-case 2, L3-L3 measurement prediction as baseline for both FR1 and FR2. - Interested companies can also provide results for other sub-cases. - It is not precluded to add additional sub-case for FR2 in future meetings. - Further discuss how to define requirements based on the available simulation results.

Agreement: As starting point, evaluate non-cluster (single-cell input and single-cell output) approach for frequency domain prediction as baseline. - Interested companies can also provide results for cluster approach. Further discuss the detailed simulation assumptions. - Further discuss how to define requirements based on the available simulation results.

Based on the agreements, we provide our observations and proposals in the following.

As a starting point, we recommended to use same system-level simulation (SLS) parameters as those used by RAN2 AIML Mobility for the evaluation of each sub-use case. This will help to simplify the efforts in RAN4. The detailed parameters are provided in TR 38.744 V19.0.0.

RAN4 AIML Mobility to consider same system-level simulation (SLS) parameters as used by RAN2 for the evaluation of each sub-use case.

RAN2 observation (TR 38.744 V19.0.0), e.g., for FR2 intra-frequency temporal domain case A concludes that increasing the observation window (OW) length can improve the prediction accuracy, especially when the OW length is relatively short. However, once the OW length exceeds a certain value, further increase of the OW length does not yield significant benefit. Therefore, in RAN4 AIML Mobility, the length of observation window OW and prediction window (PW) can be chosen based on RAN2’s evaluations, i.e., OW and PW that have shown the better performance in RAN2 study can be selected in RAN4 AIML mobility. This will simplify/reduce the simulation efforts in RAN4.

RAN4 to consider length of OW and PW based on RAN2 evaluations. The values could be different for each sub-use case.

Generally, in AIML, length of OW is double the length of PW due to several factors. For example, a larger OW allows AIML model to capture historical patterns (correlation between two or more input time instances etc.), enables the extraction of more robust and meaningful features (standard deviation, moving average etc.), and stability of AIML model. Besides, in RAN2 simulations [7], a comparison of various OW and PW configurations is provided. For example, in Figure 1, it can be observed that doubling the OW length compared to PW brings some gain.

Figure 1: Average MAE comparisons for different OW & PW and different models, temporal domain case A.

Doubling the length of observation window (OW) compared to prediction (PW) brings gains and considered as a general guideline in AIML model implementation due to various reasons, e.g., better capturing historical patterns.

For simulation assumptions, RAN4 to consider observation window (OW) length of 200ms and prediction window (PW) length of 100ms with a sampling period of 40ms for both FR1 and FR2.

- Regarding the link-level simulation (LLS) parameters for BB error, we support using similar parameters as used by RAN4 AIML beam management (BM) [8]. For example, error model and the corresponding values can be used as agreed in RAN4 AIML BM. However, AIML BM parameters are only defined for FR2. Therefore, RAN4 AIML Mobility needs to define the parameters for FR1.

For FR2, RAN4 to consider similar link-level simulation (LLS) parameters as used by RAN4 AIML BM for the BB error. For example, error model and corresponding values could be used similar as agreed in RAN4 AIML BM.

We provide more details regarding measurement error models and the corresponding values in the following section.

  - Impacts of measurement error on prediction accuracy

Regarding the impacts of measurement error on prediction accuracy following agreement was made in RAN4#116.

Agreement: RAN4 to capture the following to model measurement error to consider the impact on prediction accuracy: - As a baseline: - For BB error, use link level simulation to generate L3-RSRP difference as baseband error. - For RF error model, use a truncated Gaussian distribution. - Further discuss in the WI phase either the same or different RF error value may be considered at least for different frequency layers and/or different RF chains.

In AIML beam management (BM) use case, the results of evaluation with BB and RF errors are captured in [9]. In these evaluations, spatial domain prediction is considered with 32 beams in Set A, Set B contains 8 beams and is subset of Set A. AIML model predicts Top-K beam IDs. Also, measured RSRP (Ideal SLS RSRP+BB error + RF error). RSRP error is calculated as L1-RSRP difference between the maximum RSRP of the Top-1 predicted beam and the RSRP of the genie aided strongest beam (without considering measurement error at the output). In the evaluation, it has been observed that Gaussian distribution provides the highest performance degradation in terms of Top-K and RSRP error. It can also be observed that RF error has the highest impact on prediction accuracy, where RF errors are independently added to all TX beams.

Besides, BB and RF error impact has also been evaluated for low and high SNR UEs. Compared to no measurement error, low SNR UEs experience high RSRP difference and high SNR UEs experience low RSRP difference. Furthermore, considerable drop in Top-1 accuracy can be observed for low SNR UEs. When considering different BB+RF measurement errors, prediction accuracy related KPIs drop regardless of the SNR condition of the UE. However, drop in performance in AI/ML (With error) compared to AI/ML (Without error) demonstrates the impact of measurement error.

In AIML BM use case, the impact of BB and RF error on prediction accuracy shows that RF error is prominent compared to BB error, which is mainly due to high variation of error samples.

In AIML BM use case, with measurement error, all UEs experience degradation in beam prediction-related KPIs. However, a severe impact is observed with high SNR UEs compared to low SNR UEs.

In AIML BM use case, addition of constant RF error across all TX beams did not show much performance degradation in terms of prediction KPIs. However, addition of independent RF errors for all TX beams showed the worst degradation.

In the case of AIML BM, it has been observed that RF error has a significant impact on the performance of AIML model. In RAN4#116, simulation assumptions for BM use case were revisited [8]. Specifically, truncated Gaussian distribution under ±4.5 dB RF error (μ=0,  σ2=4.5) is used.

In the AIML for mobility, similar assumptions as in AIML BM use case can be considered to evaluate the impact on prediction accuracy. Also, due to the mobile nature of the UE, it is recommended to consider independent RF error values for different frequency layers and RF chains to evaluate the impact on prediction accuracy as different error consideration leads to realistic scenario. If we assume same RF error, it may just be an offset compared to training data so it might be easy for AIML model to learn the offset and even if offset is not learnt, AIML model can predict accurately as difference is little offset. However, with independent RF error values there will be no offset, which reflects a realistic scenario.

To evaluate the impact of measurement error on prediction accuracy, RAN4 to consider independent RF error values for different frequency layers and RF chains.

RAN4 to consider AIML-based beam management (BM) simulation assumptions for the measurement error model in order to evaluate the impact on prediction accuracy. In particular, for FR2, the RF error could be modelled as a truncated Gaussian distribution with mean μ = 0 and variance σ² = 4.5, corresponding to an error range of ±4.5 dB.

In FR1 and FR2, we have two different RF architectures and RF receivers. Besides, for measurements, we define significantly different accuracy requirements for FR1 and FR2 because of RF and BB errors. Since accuracy requirements are different for FR1 and FR2; therefore, modeling values (mean and variance) should be different for FR1 and FR2.

It is proposed to consider same RF error model (truncated Gaussian) for FR1 and FR2 with potentially different mean and variance.

Ground truth definition for FR1

In the RAN4#114 meeting, the following agreements were made for FR1 Ground truth definition for RSRP accuracy:

Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: - The way the ground truth is extracted in the testing may differentiate between FR1 and FR2.

Issue 2-1-2: Ground Truth Definition for RSRP accuracy - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

Following agreement was reached in RAN4#114bis meeting:

Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1 Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

Following is the RAN4#114 agreement for AIML Beam management use case:

Agreement: - Use CDL-based channel model as starting point

Alternative 1, the ground truth for FR1 can be derived by TE based on the transmitted or received power:

- This is aligned with the legacy requirement for L3 RSRP accuracy. However, the legacy test is based on AWGN channel model. The AWGN channel is a deterministic channel, where signal power will be constant for both spatial and time domain.

- Derivation of ground truth based on the transmitted or received power is not suitable for the fading channel since it is challenging to identify the exact time period of the measured and the predicted samples and this may lead to inaccuracies of ground truth derivation.

- The L1 filtering method, filtering co-efficient and the number of samples for filtering are not known by the TE since it is UE proprietary implementation, hence it will impact the derivation of the ground truth at TE.

- The advantage with this alternative is that there will be no issue of UE cheating since the ground truth is derived at TE.

Alternative 1 where the FR1 ground truth can be derived by TE based on the transmitted or received power (legacy approach) is not suitable for AIML Mobility due to the fading channel and due to the L1 filtering which is UE implementation specific and not known to the TE.

Alternative 2, the ground truth for FR1 is derived based on the measurements reported by UE based on certain conditions:

- With this approach, there is no complexity of considering the channel variations and the parameters that are UE implementation specific (e.g.: L1 filtering related information) since these would be considered at UE while reporting the measurements.

- If the timing window is configured, some UEs may not be able to measure and predict at the same time (depending upon the UE capabilities). However, this can be handled during testing as described below:

  - For temporal domain case A, measurement is always performed (i.e. there is no measurement reduction), hence UE can always report the measured values that will serve as ground truth.

  - For temporal domain case B, few measurements are skipped in the observation window and the continuous measurement values will not be available at UE to report to TE. Hence there is a need for methods to extract ground truth during testing.

- S(I)NR variations in the test at different time instances may pose the challenge for this alternative. However, this can be handled in multiple ways. For example: One option is to maintain average SINR value during the test however measure at higher SINR level for more accurate ground truth and the SINR may be lower during the prediction with reference to the average SINR.

- Another important aspect is to ensure that UE cheating does not occur with this alternative. This can be handled via various test procedures. Test procedures to identify UE cheating have been described in our papers in previous RAN4 meetings.

Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is more suitable for AIML Mobility since the channel variations and L1 filtering are already considered by UE while performing measurements that serve as ground truth.

Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is the preferred option for ground truth definition for RSRP accuracy even though there are inaccuracies in the UE measurements, since it is more feasible option for the dynamic channel conditions.

For alternative 2 where the FR1 ground truth is derived based on the measurements reported by UE, the following are the different methods of ground truth extraction for evaluation:

- Iterative/ repeated test: The channel conditions (transmitted power, SINR etc.) and other attributes like UE movement, speed etc., should be same during the iterative test. UE can be configured to perform the test with iteration 1 configured for measurement reporting (non AIML method) where the reported L3-RSRP measurements serve as ground truth. In the next test iteration, UE can be configured for reporting AIML predicted L3-RSRP under same channel conditions as the first iteration. The number of test iterations can be more than two with different configurations to have more robust evaluation.

For the example shown in Figure 2, UE may be configured to report the measurement periodically in iteration 1. In iteration 2, UE may be configured to report the L3-RSRP prediction results for the future time instances i.e. (t2+t3) along with measurement report for time period (t0+t1).  The iteration 1 measurement report at t3 can serve as ground truth for iteration 2 prediction result reported at t1. It should be noted that the time instance at which the reporting is done may vary depending upon the OW/ PW/ reporting configurations and this should not impact the ground truth extraction.

This is the preferred option as it is simpler in terms of testing. The test setup configuration for the channel conditions remains same and the test can be repeated by simply enabling/disabling AIML prediction in different iterations for measurements at UE. The possibility of UE cheating is less compared to other options since UE will not be aware of the measurements reported in the other iteration.

Figure 2: Iterative test

- Parallel test: Configure UE to perform measurements as well as AIML predictions parallelly during the prediction window and report both the measured and predicted values to TE depending upon the UE capability (note that the configuration for performing measurement and prediction for the same time period is used only for testing purposes). In this case, the received measurement report can be used as ground truth for the predicted report. The main issue with this option is that it is difficult to identify if UE is cheating as UE is reporting both prediction and measurement results at the same time for the same time period.

For alternative 2 where the FR1 ground truth is derived based on the measurements reported by UE, the ground truth can be extracted for measurement prediction evaluation using iterative option or parallel test option.

For alternative 2 where the FR1 ground truth is derived based on the measurements reported by UE, RAN4 should define the method (i.e. iterative or parallel method) for extraction of the ground truth for measurement prediction evaluation considering the ease of test configuration.

Discussion on Prediction Accuracy for Temporal Domain Prediction

Regarding the impacts of observation and prediction windows, agreements reached in RAN4#114 meeting were copied below:

RAN4#114 Agreement: - RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions. - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

In addition, it was agreed in RAN2#130:

RAN2#130 Agreement: For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping.

Agreements/Observations to be captured in TR - The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases. - For temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance. Companies may report the adopted skipping pattern when providing simulation results. Companies are not required to run new simulations but can clarify in the spreadsheet. We can capture in the TR how the skipping pattern affects the performance. - For the inter-frequency prediction, the evaluation results show that the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy. FFS on observations on low correlations. - For inter-frequency, cluster as input (i.e. measurements from different cells as inputs )can improve the prediction accuracy than single cell as input. For temporal domain, the gains are unclear. RAN2 will focus on frequency domain for cluster based approach. - In cluster approach the model takes measurements from more than one cell as inputs.

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

RAN4 agreed to consider the impact of UE speed, MRRT and OW and PW lengths on RRM requirements for temporal predictions.

Also, lately, RAN2 has confirmed that the skipping pattern in temporal domain case B refers to the SSB configuration only (the timing that SSBs are transmitted by the NW) and do not correspond to the UEs measurement or skipping. In addition, in RAN2, potential inference parameters to be shared with the UE includes skipping pattern optionally and MRRT is not therefore discussed in study item. It can then be understood that the UE would need either skipping pattern or MRRT information for the temporal domain case B.

On the other hand, RAN2 has confirmed that for temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance. For this reason, within RAN2, the different simulations results provided by the companies should clearly indicate the used skipping pattern. With this assumption, it appears that the MRRT information alone is not sufficient, and the testing procedure should specify the skipping pattern to use.

Note that the skipping pattern (as pointed by the RAN2) is NW specific configuration. Similarly, the MRRT is also configured by NW. However, it is up to UE implementation, to select what to measure and what to skip among the measurement opportunities (referred as UE measurement reduction).

Therefore, from RAN4 perspective, two options can be foreseen, based on which the requirement on prediction accuracy should be setup:

- Option 1: The UE measurement reduction is UE implementation. This information is not shared with the NW and could not be part of the testing conditions

- Option 2: The UE measurement reduction information can be shared with the NW and therefore can be part of the testing conditions. Note that the UE measurement reduction accounts obviously on the selected MRRT as well as the skipping pattern.

As the prediction accuracy for temporal domain case B for a given MRRT vary as function of the skipping pattern selected by the NW and possibly as function of the UE measurement reduction, the requirement for the prediction accuracy can be defined with a minimum required accuracy level that should be verified for any configuration.

To handle different impacting factors on the temporal prediction case B, RAN4 can define a lower bound threshold for the prediction accuracy which should be verified irrespective of selected MRRT and skipping pattern.

For temporal domain prediction case B, the UE is not required to measure all the time and may use instead predicted measurements. The degree of measurement reduction can be selected through the Measurement reduction rate in temporal domain (MRRT) parameter.

For the testing, the MRRT can be set to [50, 60, 66, 75, 80] % and UE speed can be set to [30, 60, 90, 120] km/h, resulting in a total number of 20 combinations. The number of test configurations can further increase with different OW and PW values as well (and even more if skipping pattern should be specified as well).

The number of possible test cases can increase considerably for a set of selected OW, PW and MRRT (possibly with skipping pattern) considering multiple UE speed cases.

For temporal domain prediction case B, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT (possibly with skipping pattern) as well as UE speed.

Inter-frequency prediction scenarios

One of the high-priority study items for AI/ML in NR inter-cell mobility is inter-cell inter-frequency measurement prediction. Inter-frequency measurements require configuring measurement gaps, during which the UE must temporarily stop communicating with the serving cell to measure other frequencies. This process can lead to service interruptions and increased power consumption. AI/ML techniques can help reduce the number of inter-frequency measurements a UE needs to perform, thereby minimizing the need for continuous measurement and enhancing UE energy efficiency. An additional advantage of reducing inter-frequency measurements is that it can also reduce the number of configured measurement gaps, which can improve UE throughput and reduce latency.

The following agreement was reached in RAN4#115 meeting:

Agreement: RAN4 to study whether and how correlation coefficient can be considered in simulation assumption for RAN4 requirements.

Furthermore, the following agreement was reached in RAN4#114bis meeting:

Agreement: - RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects: - the side condition of frequency prediction (e.g., EPRE difference) - the impact of correlation coefficient between measured and predicted frequency layers - impact of cluster approach, e.g., - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

In the following, we address the impact of correlation coefficient, carrier specific scaling factor (CSSF), cluster approach, and measurement gap (MG) due to inter-frequency prediction.

Impact of Correlation Coefficient

In RAN4, there is no clear definition of correlation coefficient. However, RAN2 has used Pearson Correlation Coefficient (PCC) to evaluate the results. As discussed in the previous subsection, RAN2 agreed that the PCC would be used to measure the correlation between two measured and predicted frequency layers [TR 38.744 V0.0.6 7].  For the sake of simplicity, it is better to use the same definition in both RAN2 and RAN4 WGs.

For inter-frequency prediction scenario, RAN4 to consider the RAN2 definition of correlation coefficient as a starting point.

However, the current definition for correlation coefficient is simple and it might get complicated when different aspects such as beam pattern, non-co-location scenario, channel model in each frequency, are taken into consideration. Therefore, it might be required to redefine the definition for correlation coefficient.

Different aspects, e.g., beam pattern/configuration, channel model in each frequency, might have an impact on the currently used definition for correlation coefficient. Hence, it might be necessary to redefine correlation coefficient when such aspects are taken into consideration.

In RAN4, there is no clear definition of correlation coefficient. Unless RAN4 defines a common dataset, it is not feasible to study the impact of correlation coefficient. Hence, it’s important to define correlation coefficient for RAN4 requirements.

To start discussion on RAN4 requirements, it is recommended to first define the requirements for the agreed scenario in RAN2. For example, RAN4 requirements must be first studied for co-located scenario. Besides, it is possible that different/same frequency pairs might have different correlation coefficients for co-located and non-co-located scenarios. Hence, RAN4 might need to define requirements accordingly.

In RAN2, inter-frequency prediction focuses on the co-located scenario. Hence, RAN4 can take the same assumption to define the requirements.

In inter-frequency prediction scenario, it is possible that different/same frequency pairs might have different correlation coefficients in co-located and/or non-co-located scenarios. Hence, RAN4 requirements can be defined accordingly.

Besides, in RAN2 evaluations, same beam pattern (beam configuration, grid of beams) is assumed [TR 38.744 V0.0.6 7]. Beam pattern might impact the prediction performance or the correlation coefficient. It is expected that with different grid of beams, prediction performance might degrade compared to same beam pattern. In our view, different gird of beams might have different impact; hence, RAN4 requirements must be defined accordingly.

For inter-frequency prediction scenario, it is possible that different beam patterns might show different performance. Therefore, RAN4 need to take this into account when defining the requirements.

In RAN2, the definition of correlation coefficient has been provided. However, if there is need to provide an alternative definition then one possibility could be the correlation between power-delay profiles (PDPs) of different frequency layers. For instance, for two different channel profiles of frequency 1 and frequency 2, PDP can be calculated for each, where number of delay taps can be predefined. Lastly, the correlation coefficient can be computed between two PDPs. This could be beneficial as PDP helps distinguish fading due to movement versus frequency selectivity.

An alternative way to define correlation coefficient for inter-frequency predictions could be the correlation between PDPs of different frequency layers channel profiles.

Impact on Measurement Gap

AI/ML-based inter-frequency prediction can have significant impact on measurements gap (MG) configuration. The following aspects can be considered by RAN4 when studying the impact on MG.

- Reduction in Measurement Gaps Configurations: With the help of AIML predictions, less measurement gaps configurations will be required.

- Simplifications of Measurement Gap Patterns (Table 9.1.2-1 (TS 38.133):

  - Measurement Gap length (MGL) and Measurement Gap Repetition Period (MGRP) can be impacted.

  - MG timing advance: There might be an impact.

For inter-frequency prediction scenario, measurement gap (MG) can be impacted in several ways. For example, measurement gap length (MGL), Measurement Gap Repetition Period (MGRP), and measurement gap configuration (MGC), might be revised.

For inter-frequency predictions, RAN4 to consider the impact on measurement gap (MG), measurement gap length (MGL), Measurement Gap Repetition Period (MGRP), and measurement gap configuration (MGC).

Conclusion

In this paper we provide our further view on the RRM Measurement prediction use case and RRM requirements to be studied, we particularly addressed the following topics:

- Simulation assumptions

- Impact of measurement error on prediction accuracy

- Ground truth definition for RSRP accuracy for FR1

- Discussion on Prediction Accuracy for Temporal Domain Prediction

- Inter-frequency prediction scenario

The following Observations and Proposals were made:

Observation 1: Doubling the length of observation window (OW) compared to prediction (PW) brings gains and considered as a general guideline in AIML model implementation due to various reasons, e.g., better capturing historical patterns.

Observation 2: In AIML BM use case, the impact of BB and RF error on prediction accuracy shows that RF error is prominent compared to BB error, which is mainly due to high variation of error samples.

Observation 3: In AIML BM use case, with measurement error, all UEs experience degradation in beam prediction-related KPIs. However, a severe impact is observed with high SNR UEs compared to low SNR UEs.

Observation 4: In AIML BM use case, addition of constant RF error across all TX beams did not show much performance degradation in terms of prediction KPIs. However, addition of independent RF errors for all TX beams showed the worst degradation.

Observation 5: Alternative 1 where the FR1 ground truth can be derived by TE based on the transmitted or received power (legacy approach) is not suitable for AIML Mobility due to the fading channel and due to the L1 filtering which is UE implementation specific and not known to the TE.

Observation 6: Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is more suitable for AIML Mobility since the channel variations and L1 filtering are already considered by UE while performing measurements that serve as ground truth.

Observation 7: For alternative 2 where the FR1 ground truth is derived based on the measurements reported by UE, ground truth can be extracted for measurement prediction evaluation using iterative option or parallel test option.

Observation 8: The number of possible test cases can increase considerably for a set of selected OW, PW and MRRT (possibly with skipping pattern) considering multiple UE speed cases.

Observation 9: Different aspects, e.g., beam pattern/configuration, channel model in each frequency, might have an impact on the currently used definition for correlation coefficient. Hence, it might be necessary to redefine correlation coefficient when such aspects are taken into consideration.

Observation 10: In RAN4, there is no clear definition of correlation coefficient. Unless RAN4 defines a common dataset, it is not feasible to study the impact of correlation coefficient. Hence, it’s important to define correlation coefficient for RAN4 requirements.

Observation 11: For inter-frequency prediction scenario, it is possible that different beam patterns might show different performance. Therefore, RAN4 need to take this into account when defining the requirements.

Observation 12: For inter-frequency prediction scenario, measurement gap (MG) can be impacted in several ways. For example, measurement gap length (MGL), Measurement Gap Repetition Period (MGRP), and measurement gap configuration (MGC), might be revised.

Observation 13: Feeding the input from various sources might enhance AIML model performance.

Proposal 1: RAN4 AIML Mobility to consider same system-level simulation (SLS) parameters as used by RAN2 for the evaluation of each sub-use case.

Proposal 2: RAN4 to consider length of OW and PW based on RAN2 evaluations. The values could be different for each sub-use case.

Proposal 3: For simulation assumptions, RAN4 to consider observation window (OW) length of 200ms and prediction window (PW) length of 100ms with a sampling period of 40ms for both FR1 and FR2.

Proposal 4: For FR2, RAN4 to consider similar link-level simulation (LLS) parameters as used by RAN4 AIML BM for the BB error. For example, error model and corresponding values could be used similar as agreed in RAN4 AIML BM.

Proposal 5: To evaluate the impact of measurement error on prediction accuracy, RAN4 to consider independent RF error values for different frequency layers and RF chains.

Proposal 6: RAN4 to consider AIML-based beam management (BM) simulation assumptions for the measurement error model in order to evaluate the impact on prediction accuracy. In particular, for FR2, the RF error could be modelled as a truncated Gaussian distribution with mean μ = 0 and variance σ² = 4.5, corresponding to an error range of ±4.5 dB.

Proposal 7: It is proposed to consider same RF error model (truncated Gaussian) for FR1 and FR2 with potentially different mean and variance.

Proposal 8: Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is the preferred option for ground truth definition for RSRP accuracy even though there are inaccuracies in the UE measurements, since it is more feasible option for the dynamic channel conditions.

Proposal 9: For alternative 2 where the FR1 ground truth is derived based on the measurements reported by UE, RAN4 should define the method (i.e. iterative or parallel method) for extraction of the ground truth for measurement prediction evaluation considering the ease of test configuration.

Proposal 10: To handle different impacting factors on the temporal prediction case B, RAN4 can define a lower bound threshold for the prediction accuracy which should be verified irrespective of selected MRRT and skipping pattern.

Proposal 11: For temporal domain prediction case B, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT (possibly with skipping pattern) as well as UE speed.

Proposal 12: For inter-frequency prediction scenario, RAN4 to consider the RAN2 definition of correlation coefficient as a starting point.

Proposal 13: In RAN2, inter-frequency prediction focuses on the co-located scenario. Hence, RAN4 can take the same assumption to define the requirements.

Proposal 14: In inter-frequency prediction scenario, it is possible that different/same frequency pairs might have different correlation coefficients in co-located and/or non-co-located scenarios. Hence, RAN4 requirements can be defined accordingly.

Proposal 15: An alternative way to define correlation coefficient for inter-frequency predictions could be the correlation between PDPs of different frequency layers channel profiles.

Proposal 16: For inter-frequency predictions, RAN4 to consider the impact on measurement gap (MG), measurement gap length (MGL), Measurement Gap Repetition Period (MGRP), and measurement gap configuration (MGC).

- References

- RP-242393, “Revised SID on AIML for mobility in NR”, OPPO, RAN#105 meeting, 9 – 12 September 2024, Melbourne, Australia.

- R4-2512127, WF on AI/ML Mobility Part 1, Nokia, RAN4#116, Bangalore, India, 25th – 29th August 2025.

- R4-2512128, WF on FS_NR_AI/ML_Mob_Part 2, OPPO, RAN4#116, Bangalore, India, 25th – 29th August 2025.

- RP-252899, “Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”, OPPO, Interdigital, RAN#109 meeting, 15 – 18 September 2025, Beijing, China

- R4-2520708, “On General aspects in AIML Mobility”, Nokia, RAN4#117 meeting, Dallas, USA, 17th – 21st November 2025.

- R4-2521900, “On RAN4 Impacts for Measurement Event Prediction in AIML Mobility”, Nokia, RAN4#117 meeting, Dallas, USA, 17th – 21st November 2025.

- R2-2500946, “Simulation results for RRM measurement predictions”, Nokia, RAN2#129, Athens, Greece, 17th ‒ 21st February 2025.

- R4-2511797, WF on updated simulation assumptions for AIML BM, vivo et al., RAN4#116, Bengaluru, India, 25th ‒ 29th August, 2025.

- R4-2511865, “Summary of beam management simulation results”, vivo, RAN4#116, Bengaluru, India, 25th ‒ 29th August 2025.