3GPP TSG-RAN WG4 Meeting # 117 	R4-2520722

Dallas, USA, Nov. 17-21, 2025

Source: 	vivo

Title: 	Further views on the co-existence assumption for outdoor active device

Agenda Item:	7.11.3

Document for:	Approval

- Introduction

In the last meeting, it was agreed that the calibration is only for standalone operation and corresponding simulation assumptions are finished [1]. However, the simulation assumption for final simulation campaign is not decided yet, especially for in-band operation. In this contribution, we provide our views on the pending assumptions.

- Discussion

  - Evaluation cases

The following scenarios were agreed for simulation:

| Operation mode | Device type | Evaluation case number | Aggressor | Victim | Direction |
| --- | --- | --- | --- | --- | --- |
| Standalone | Device 2b | 1 | Device | NR UL | D2R |
|  |  | 2 | NR UL | reader | D2R |
|  |  | 3 | Reader | NR DL | R2D |
|  |  | 4 | NR DL | device | R2D |
|  | Device C | 5 | Device | NR UL | D2R |
|  |  | 6 | NR UL | reader | D2R |
|  |  | 7 | Reader | NR DL | R2D |
|  |  | 8 | NR DL | device | R2D |
| In-band | Device 2b | 9 | Device | NR UL | D2R |
|  |  | 10 | NR UL | reader | D2R |
|  |  | 11 | Reader | NR DL | R2D |
|  |  | 12 | NR DL | device | R2D |
|  | Device C | 13 | Device | NR UL | D2R |
|  |  | 14 | NR UL | reader | D2R |
|  |  | 15 | Reader | NR DL | R2D |
|  |  | 16 | NR DL | device | R2D |

For standalone, all cases listed above are valid but for in-band operation, some cases could be unnecessary. As we analyzed in the last meeting, for case 9/13, when D2R interfere NR UL, the interference condition is exactly same as standalone, i.e., case1/5, since the BS in-band selectivity is assumed same as the adjacent channel selectivity. For case 11/15, the R2D and NR DL signal are orthogonal while the victim is NR UE which is equipped with OFDM receiver, so the interference is not need to be considered.

Observation 1: For device D2R interferer NR UL (case 9&case13), the simulation would be same for in-band operation and standalone since the BS in-band selectivity is assumed same as the adjacent channel selectivity.

Observation 2: For reader R2D interferer NR DL under in-band operation (case 11&case15), the interference doesn’t exist since the R2D and NR DL are orthogonal while the victim NR UE equip with OFDM receiver.

To further reduce the simulation workload, it is suggested to remove these cases.

Proposal 1: The following cases are removed from the simulation campaign.

| In-band | Device 2b | 9 | Device | NR UL | D2R |
| --- | --- | --- | --- | --- | --- |
|  |  | 11 | Reader | NR DL | R2D |
|  | Device C | 13 | Device | NR UL | D2R |
|  |  | 15 | Reader | NR DL | R2D |

  - Simulation assumption

2.2.1 Maximum output power

The maximum output power for in-band operation was discussed in the last meeting but no agreement was achieved. The main controversy behind is the architecture assumption for the reader. If the reader and NR BS share same hardware, there could be the limitation on the PSD and the reader output power should be scaled based on the BW ration between NR and AIoT. However, if the reader and NR BS doesn’t share hardware, such limitation can be removed and the output power informed by RAN1 can be used.

In our understanding, it is unnecessary to discuss and make decision on BS/reader architecture during SI stage since only 2 meeting cycles are left. We can keep both options and companies can provide input based on their own interests.

Observation 3: The main target for RAN4 SI is to verify the feasibility from co-existence perspective, and it is unnecessary to discuss and make decision on BS/reader architecture during SI stage.

Proposal 2: Keep both following options of reader maximum output power for simulation and companies can input based on their interests.

Option 1: Fixed power as informed by RAN1, i.e., 33 dBm and 38 dBm

Option 2: Power scaling based on BW ratio between NR and AIoT, i.e., 28.8 dBm

2.2.2 Power control

In [2], the following agreement was agreed in RAN1:

Agreement

Study the necessity of D2R Tx power control for Device 2b and for Device C for outdoor scenarios

- Target to agree on observations on pros and cons of D2R Tx power control vs. no Tx power control at RAN1#123

Continue studying feasibility of the following open loop power control methods considering device complexity:

- Methods of open loop power control

  - Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader

    - Required information at the device side at least includes reader Tx power, received target power at the reader, etc.

  - Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range.

    - The RSRP range is based on RSRP threshold(s) configured by reader or predefined

    - Required information at the device side at least includes RSRP threshold(s), corresponding D2R Tx power related information per each RSRP range defined by the RSRP threshold(s)

  - FFS details (e.g., required measurements, required information)

Continue studying the necessity and feasibility of closed loop power control for D2R.

Unfortunately, the power control is still discussed in RAN1 and no clear agreement is done. Considering the next February meeting will be the last meeting for SI, there is no time to wait RAN1 decision and RAN4 should provide simulation results based on both with and without power control.

Observation 4: There is no time to wait for RAN1’s decision on power control.

For power control algorithm, the main change for device is the target SNR which is 15 dB, and the reason behind could be the link level performance as shown in Table I, which comes from TR 36.942.

Table I Look-Up-Table of UL and DL Throughput vs. SNIR for Baseline E-UTRA Coexistence Studies [3]

|  | Throughput |  |  |  |  |  | Throughput |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNIR | bps/Hz |  | kbps per 375kHz RB |  |  | SNIR | bps/Hz |  | kbps per 375kHz RB |  |
| dB | DL | UL | DL | UL |  | dB | DL | UL | DL | UL |
| -15 | 0 | 0 | 0 | 0 |  | 6 | 1.39 | 0.93 | 521 | 347 |
| -14 | 0 | 0 | 0 | 0 |  | 7 | 1.55 | 1.04 | 582 | 388 |
| -13 | 0 | 0 | 0 | 0 |  | 8 | 1.72 | 1.15 | 646 | 430 |
| -12 | 0 | 0 | 0 | 0 |  | 9 | 1.90 | 1.26 | 711 | 474 |
| -11 | 0 | 0 | 0 | 0 |  | 10 | 2.08 | 1.38 | 778 | 519 |
| -10 | 0.08 | 0.06 | 31 | 21 |  | 11 | 2.26 | 1.51 | 847 | 565 |
| -9 | 0.10 | 0.07 | 38 | 26 |  | 12 | 2.44 | 1.63 | 917 | 611 |
| -8 | 0.13 | 0.08 | 48 | 32 |  | 13 | 2.63 | 1.76 | 988 | 658 |
| -7 | 0.16 | 0.10 | 59 | 39 |  | 14 | 2.82 | 1.88 | 1059 | 706 |
| -6 | 0.19 | 0.13 | 73 | 48 |  | 15 | 3.02 | 2.00 | 1131 | 750 |
| -5 | 0.24 | 0.16 | 89 | 59 |  | 16 | 3.21 | 2.00 | 1204 | 750 |
| -4 | 0.29 | 0.19 | 109 | 73 |  | 17 | 3.41 | 2.00 | 1277 | 750 |
| -3 | 0.35 | 0.23 | 132 | 88 |  | 18 | 3.60 | 2.00 | 1350 | 750 |
| -2 | 0.42 | 0.28 | 159 | 106 |  | 19 | 3.80 | 2.00 | 1424 | 750 |
| -1 | 0.51 | 0.34 | 190 | 127 |  | 20 | 3.99 | 2.00 | 1498 | 750 |
| 0 | 0.60 | 0.40 | 225 | 150 |  | 21 | 4.19 | 2.00 | 1572 | 750 |
| 1 | 0.71 | 0.47 | 265 | 176 |  | 22 | 4.39 | 2.00 | 1646 | 750 |
| 2 | 0.82 | 0.55 | 308 | 206 |  | 23 | 4.40 | 2.00 | 1650 | 750 |
| 3 | 0.95 | 0.63 | 356 | 237 |  | 24 | 4.40 | 2.00 | 1650 | 750 |
| 4 | 1.09 | 0.72 | 408 | 272 |  | 25 | 4.40 | 2.00 | 1650 | 750 |
| 5 | 1.23 | 0.82 | 463 | 309 |  |  |  |  |  |  |

For SINR larger than 15 dB, the throughput keeps same and no need to further increase the UE power. For device D2R, based on the LLS results across companies, the SNR is around -6 dB to 0 dB, so it is suggested to use 0 dB as the target SNR for device power control.

Another parameter needs to be updated is the output dynamic. For NR PC3, the dynamic range is 63 dB with -40 dBm minimum output power control. To prevent possible interference from device to NR at the position near the BS/reader, we suggest use same minimum output power for device, and the dynamic range for device 2b is 30 dB while for device c is 45 dB

Proposal 3: Both with/without power control can be provided based on companies’ interests, and the following power control algorithm and parameter can be used, which is based on 0 dB target SNR:

| Gamma | CLx-ile (15 kHz bandwidth) |  | Rmin |  |
| --- | --- | --- | --- | --- |
|  | Device 2b | Device c | Device 2b | Device c |
| 1 | 117 | 132 | 30 | 45 |

2.2.3 Clarification on ACLR/ACS model

During the offline discussion, it seems that the ACLR/ACS model is not aligned across companies, and in this section, we provide our views on the detailed modelling in Table below.

Table II Clarification on ACLR/ACS model

| Aggressor | Victim | ACLR | ACS |
| --- | --- | --- | --- |
| Device D2R | NR UL |  |  |
| NR UL | Reader D2R (standalone) | ACLRe = X + 10*log10(BWUE/BWD2R) for all 3 UEs | ACSe = X + 10*log10(BWUE/BWD2R) for UE3 No interference from UE1/UE2 |
| Reader R2D | NR DL |  |  |
| NR DL | Device R2D (standalone) | ACLRe = X + 10*log10(BWUE/BWD2R) | ACSe = X + 10*log10(BWUE/BWD2R) |
| NR UL | Reader D2R (in-band) | ACLRe = IBE + 10*log10(180kHz/15kHz) | ACSe = X + 10*log10(180kHz/15kHz) |
| NR DL | Device R2D (in-band) | ACLRe = X + 10*log10(BWUE/BWD2R) | ACSe = X + 10*log10(BWUE/BWD2R) |

Proposal 4: Take clarification in Table II for ACLR/ACS modelling.

  - Performance metric

For NR, 5% throughput loss can be the performance metric as usual, and the metric for AIoT system is still pending. In our understanding, it is reasonable to have a absolute SINR value to check the outage probability but it would be hard to decide the value for this short SI, since the SINR value could depends on the coverage evaluation which is parallelly discussed in RAN1.

Observation 5: It is difficult to give a reasonable absolute SINR level as performance metric during SI, since it could depend on RAN1 coverage study.

For simplicity, we can still use 1 dB SINR degradation as the metric which is already used in R19 SI and encourage companies to provide the absolute SINR value for comparison.

Proposal 5: For AIoT system, take 1 dB SINR degradation at 5% and 50% CDF as performance metric for inter system interference.

- Companies are encouraged to provide the absolute SINR at 5% and 50% CDF for comparison.

- Conclusion

In this contribution, we provide our views on update the simulation assumptions.

Observation 1: For device D2R interferer NR UL (case 9&case13), the simulation would be same for in-band operation and standalone since the BS in-band selectivity is assumed same as the adjacent channel selectivity.

Observation 2: For reader R2D interferer NR DL under in-band operation (case 11&case15), the interference doesn’t exist since the R2D and NR DL are orthogonal while the victim NR UE equip with OFDM receiver.

Proposal 1: The following cases are removed from the simulation campaign.

Observation 3: The main target for RAN4 SI is to verify the feasibility from co-existence perspective, and it is unnecessary to discuss and make decision on BS/reader architecture during SI stage.

Proposal 2: Keep both following options of reader maximum output power for simulation and companies can input based on their interests.

Option 1: Fixed power as informed by RAN1, i.e., 33 dBm and 38 dBm

Option 2: Power scaling based on BW ratio between NR and AIoT, i.e., 28.8 dBm

Observation 4: There is no time to wait for RAN1’s decision on power control.

Proposal 3: Both with/without power control can be provided based on companies’ interests, and the following power control algorithm and parameter can be used which is based on 0 dB target SNR:

| Gamma | CLx-ile (15 kHz bandwidth) |  | Rmin |  |
| --- | --- | --- | --- | --- |
|  | Device 2b | Device c | Device 2b | Device c |
| 1 | 117 | 132 | 30 | 45 |

Proposal 4: Take clarification in Table II for ACLR/ACS modelling.

Table II Clarification on ACLR/ACS model

| Aggressor | Victim | ACLR | ACS |
| --- | --- | --- | --- |
| Device D2R | NR UL |  |  |
| NR UL | Reader D2R (standalone) | ACLRe = X + 10*log10(BWUE/BWD2R) for all 3 UEs | ACSe = X + 10*log10(BWUE/BWD2R) for UE3 No interference from UE1/UE2 |
| Reader R2D | NR DL |  |  |
| NR DL | Device R2D (standalone) | ACLRe = X + 10*log10(BWUE/BWD2R) | ACSe = X + 10*log10(BWUE/BWD2R) |
| NR UL | Reader D2R (in-band) | ACLRe = IBE + 10*log10(180kHz/15kHz) | ACSe = X + 10*log10(180kHz/15kHz) |
| NR DL | Device R2D (in-band) | ACLRe = X + 10*log10(BWUE/BWD2R) | ACSe = X + 10*log10(BWUE/BWD2R) |

Observation 5: It is difficult to give a reasonable absolute SINR level as performance metric during SI, since it could depend on RAN1 coverage study.

Proposal 5: For AIoT system, take 1 dB SINR degradation at 5% and 50% CDF as performance metric for inter system interference.

- Companies are encouraged to provide the absolute SINR at 5% and 50% CDF for comparison.

- References

[1] R4-2515152, WF for [116bis][321] Study of A-IoT for outdoor, LGE

[2] R1-2508129, FL summary #7 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

[3] 3GPP TR 36.942