3GPP TSG-RAN WG4 Meeting #117	R4-2521043

Dallas, USA, Nov. 17-21, 2025

Agenda item:	7.5.4

Source:	Samsung

Title:	Discussion on Rel-20 performance requirements for 6Rx UE with interference

Document for:	Discussion

- Introduction

In the RAN#109 meeting, a new work item for “NR demodulation performance: Phase 6” has been approved [1], in which additional scenarios with interference for 6Rx UE are important objectives for the enhancement of UE demodulation performance parts. The detailed objective descriptions of 6Rx UE with interference are as below:

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver - Use the Rel-17 interference model for 2/4Rx UE as a baseline - Up to 4 MIMO layers for target cell - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Note 3: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference. - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Use the existing interference model for 2/4Rx UE as a baseline - Up to 2 MIMO layers for each user

Generally, the enhancement focuses on two scenarios, the first one is the inter-cell interference with MMSE-IRC receiver scenario, the second one is the intra-cell inter-user interference with MMSE-IRC receiver scenario. In this contribution, we provide our analysis and views on the test scopes and simulation assumptions for 6Rx with interference.

- Discussion

  - General

In last RAN4#116bis meeting, the test scope, receiver assumption, Duplex mode and  SCS reached agreements [2]. Only two open issues left for general scope as below.

Coverage of UE types - Observation: - It is clearly stated in the approved WID that the 6Rx requirements with interference will cover both FWA and handheld UEs. - Proposals: - Proposal 1: FFS whether to define one set or two sets of requirements for handheld and FWA UEs until there is clear agreement in Rel-19. - Proposal 2: Define only a single requirement for both handheld and FWA devices. - Proposal 3: Define separate requirements for handheld and FWA devices Antenna correlation - Candidate options: - Proposal 1: Follow the decision in the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’. - Proposal 2: Evaluate the performance with α = 0 and β = 0.07179 for handheld UE and low correlation for FWA - Proposal 3: For initial simulation alignment, cover both ULA Low and ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx.

Coverage of UE types

Take the agreements for Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’ [3] as below as reference, it is reasonable to define one set of requirements for handheld and FWA UEs in order to ensure the consistency of RAN4 performance requirements.

Agreement for Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’

Issue 2-2-1: Test cases and MIMO correlation for 4 MIMO layers for FWA and Handheld UE - Agreement - One single test case that is applicable for both FWA and Handheld UE with α = 0 and [β = 0.07179]

Proposal 1: Define one set of requirements for handheld and FWA UEs in order to ensure the consistency of RAN4 performance requirements.

Antenna correlation

For the same reason as above, follow the decision in the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’ on antenna correlation.

Proposal 2: Evaluate the performance with α = 0 and β = 0.07179 for both handheld and FWA UEs.

  - PDSCH requirements with inter-cell interference

Propagation condition

Since WID mentioned “Use the Rel-17 interference model for 2/4Rx UE as a baseline”, taking the propagation condition for 2/4Rx UE as baseline, which corresponding to TDLC300-100 for 2 interfering cells and TDLA30-10 for 1 interfering cell, it’s a good selection to reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells and TDLA30-10 for 1 interfering cell.

Proposal 3: For propagation condition of 6Rx UE with inter-cell interference, reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells and TDLA30-10 for 1 interfering cell.

MCS values

According the experience on Rel-17 2/4Rx demodulation requirements with inter-cell interference and Rel-19 8Rx demodulation requirements with inter-cell interference, we would like to propose MCS17 (Table 1) for serving cell Rank 2 case, and MCS13 (Table 1) for serving cell Rank 4 case.

Proposal 4: For the serving cell MCS values of 6Rx UE with inter-cell interference, prefer to use MCS17 (Table 1) for serving cell Rank 2 case, and MCS13 (Table 1) for serving cell Rank 4 case.

  - PDSCH requirements with intra-cell inter-user interference

Propagation condition

For the propagation condition, reuse the same configuration as Rel-17 is a suitable selection.

Proposal 5: For 6Rx intra-cell inter-user interference scenario, reuse the same propagation condition as Rel-17 is a suitable selection, i.e., TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2.

Precoding method for the co-scheduled UE

Considering the MU-MIMO pairing scheduling strategy in the network is highly related with the long-term beam weights but not with the short-term weight i.e., PMIs, it seems it is too idealization to use orthogonal PMI selection for co-scheduled UE with the target UE. Thus, it is better to use the randomized precoder which is not equal to any column of the precoder matrix of target UE. However, due to Rel-17 MU-MIMO requirements are based on both random and orthogonal PMI selection, we think it’s an acceptable way to proceed.

Proposal 6: For 6Rx intra-cell inter-user interference scenario, reuse the same precoding method as Rel-17 MMSE-IRC, i.e., random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests.

MCS value

For MCS value of the target UE, Rel-17 2Rx/4Rx MU-MIMO requirements select MCS13 (Table 1), while Rel-19 8Rx MU-MIMO requirements select MCS17 (Table 1). Considering more correlation are introduced by the antenna correlation matrixes compared with 2Rx/4Rx/8Rx cases, MCS17 (Table 1) for 1+1 case and MCS 13 (Table 1) for 2+2 case should be more suitable.

Proposal 7: For 6Rx intra-cell inter-user interference scenario, MCS 13 (Table 1) for 2+2 case, and MCS17 (Table 1) for 1+1 case.

  - Applicability rule and release independent

Test applicability

In order to balance the test coverage and test workload, we prefer to define a test applicability rule similar as 8Rx defined in sub-section 5.1.1.2 to skip 2/4Rx requirements if 6Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 8: For 6Rx inter-cell interference scenario and intra-cell inter-user interference scenario, prefer to define a test applicability rule similar as 8Rx defined in sub-section 5.1.1.2 to skip 2/4Rx requirements if 6Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

In addition, for UEs could support both 6Rx and 8Rx bands, the test cases to be conducted need to be clarified.

Proposal 9: For 6Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for UEs could support both 6Rx and 8Rx bands, the test cases to be conducted need to be clarified.

Release independent

Since the release independent issue about 6Rx is still no agreement on Main session, it is reasonable to wait the conclusion and use the same conclusion for 6Rx demodulation requirements with interference.

Proposal 10: For 6Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the release independent should follow the same conclusion about 6Rx in NR_ENDC_RF_Ph4 WI.

- Conclusion

In this contribution, we provide analysis and views on the 6Rx UE demodulation performance with interference objective of NR demodulation performance: Phase 5.

The observations and proposals could be summarized as:

Proposal 1: Define one set of requirements for handheld and FWA UEs in order to ensure the consistency of RAN4 performance requirements.

Proposal 2: Evaluate the performance with α = 0 and β = 0.07179 for both handheld and FWA UEs.

Proposal 3: For propagation condition of 6Rx UE with inter-cell interference, reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells and TDLA30-10 for 1 interfering cell.

Proposal 4: For the serving cell MCS values of 6Rx UE with inter-cell interference, prefer to use MCS17 (Table 1) for serving cell Rank 2 case, and MCS13 (Table 1) for serving cell Rank 4 case.

Proposal 5: For 6Rx intra-cell inter-user interference scenario, reuse the same propagation condition as Rel-17 is a suitable selection, i.e., TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2.

Proposal 6: For 6Rx intra-cell inter-user interference scenario, reuse the same precoding method as Rel-17 MMSE-IRC, i.e., random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests.

Proposal 7: For 6Rx intra-cell inter-user interference scenario, MCS 13 (Table 1) for 2+2 case, and MCS17 (Table 1) for 1+1 case.

Proposal 8: For 6Rx inter-cell interference scenario and intra-cell inter-user interference scenario, prefer to define a test applicability rule similar as 8Rx defined in sub-section 5.1.1.2 to skip 2/4Rx requirements if 6Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 9: For 6Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for UEs could support both 6Rx and 8Rx bands, the test cases to be conducted need to be clarified.

Proposal 10: For 6Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the release independent should follow the same conclusion about 6Rx in NR_ENDC_RF_Ph4 WI.

- Reference

[1] RP-252957, WID: NR demodulation performance: Phase 6, RAN#109 meeting, Apple

[2] R4-2514823, WF on NR_demod_Ph6_Ph5, RAN4#116bis meeting, China Telecom

[3] R4-2514788, Way Forward for [116bis][215] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#116bis meeting, Huawei, HiSilicon