3GPP TSG-RAN WG4 Meeting #117              	                    R4-2521046

Dallas, US, 17th – 21st Oct 2025

Title:	RRM performance requirements on conditional Intra-CU LTM

Source:	China Telecom

Agenda item:			6.17.2.3

Document for:	Discussion

- Introduction

In the RAN4 #116bis meeting, the performance requirements were discussed and WF [1] has been approved. In this paper, we will continue to discuss the RRM performance requirements on conditional Intra-CU LTM.

- Discussion

Issue 1-7: early DL sync

< Way forward > - Option 1: With early DL sync. - Option 2: With and without early DL sync. - Option 3: For each FR, one test case with and without early DL sync, with early DL sync only for other test cases.

For R18 LTM, every test case covers both with early DL sync and without early DL sync. And during core part, RAN2 agreed that early DL sync is supported. So, we think it is better to cover with and without early DL sync for CLTM.

Proposal 1: Cover with and without early DL sync for CLTM.

Issue 2-1: Test case list for Subsequent CLTM

< Way forward > - Option 1: Define test cases for subsequent CLTM. - For FR1: - If UE support 2 or more test cases for FR1, UE to pass one test case for subsequent CLTM which is different from CLTM. - If UE support 1 test case for FR1, UE is only required to pass the test case for CLTM, and not required to pass the test case for subsequent CLTM. - For FR2: the same proposal as FR1. - Option 2: Not define test case for subsequent CLTM as CLTM.

In previous meetings, we have defined the test cases for initial CLTM as follows,

- RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

- RACH-based inter-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

- RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

- RACH-less intra-frequency PCell L3 triggered CLTM cell switch from FR1 to FR1

- RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2.

- RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2.

UE needs to pass one test case for FR1/FR2 based on the supported UE capability.

For subsequent CLTM, there are 2 options. And the same between op1 and op2 is that if UE only supports 1 test case, it is not required to pass the test case for subsequent CLTM. Only for 2 or more test cases, the test case for subsequent CLTM is needed in op1. We think it is better that either define test case for subsequent CLTM in all cases or not define any test case for subsequent CLTM. So, in this case, we prefer not to define test case for subsequent CLTM.

Proposal 2: Not to define test case for subsequent CLTM.

- Summary

In this paper, we provide our views on measurements related enhancements for LTM. From this discussion we have derived the following proposals:

Proposal 1: Cover with and without early DL sync for CLTM.

Proposal 2: Not to define test case for subsequent CLTM.

- References

[1] R4-2514816, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom.