# 3GPP TSG-RAN WG4 Meeting #117		  R4-2521254

Dallas, USA, Nov. 17-21, 2025

Agenda item:	7.7.3

Source:		Qualcomm Incorporated

Title:			Discussion on UAV coexistence

Document for:	Approval

# 1  Introduction

New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV) has been approved in RAN#109 [1] with the objective of analyzing the coexistence for UAV with higher transmit power.

- Specify RF requirements for UAV UE to support higher UL output power. - Example bands: - TDD with same TDD pattern, including n41, n77, n78, n79, - FDD: n3 - Consider UAV UE with omni-direction antenna or with antenna array - Consider UAV UE with up to 33dBm max total conductive output power - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed. - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM.

In RAN4#116bis, the UAV coexistence has been initially discussed, and the agreements are captured in WF [2]. In this paper, we provide our further views on UAV coexistence.

# 2  Discussion

## 2.1 Coexistence scenarios

It was agreed in [2] that RAN4 will consider the synchronized coexistence scenarios listed in Table 1 as the starting point. Further down-selection of simulation scenarios is not precluded.

Table 2.1-1: Co-existence scenarios between UAV and TN network

|  | Aggressor |  | Victim |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | deployment scenario UL/DL | System BW duplex mode | deployment scenario UL/DL | System BW duplex mode | Simulation frequency | ACLR/ACS requirements |
| 1 | TN UL | 100MHz TDD | UAV UL | 100MHz TDD | 4 GHz | UAV BS ACS |
| 2 | UAV DL | 100MHz TDD | TN DL | 100MHz TDD | 4 GHz | UAV BS ACLR |
| 3 | TN DL | 100MHz TDD | UAV DL | 100MHz TDD | 4 GHz | UAV UE ACS |
| 4 | UAV UL | 100MHz TDD | TN UL | 100MHz TDD | 4 GHz | UAV UE ACLR |
| 5 | TN UL | [20MHz] FDD | UAV UL | [20MHz] FDD | 2 GHz | UAV BS ACS |
| 6 | UAV DL | [20MHz] FDD | TN DL | [20MHz] FDD | 2 GHz | UAV BS ACLR |
| 7 | TN DL | [20MHz] FDD | UAV DL | [20MHz] FDD | 2 GHz | UAV UE ACS |
| 8 | UAV UL | [20MHz] FDD | TN UL | [20MHz] FDD | 2 GHz | UAV UE ACLR |

ACLR and ACS for UAV UE with PC3 has been specified in Rel-18. As the objective of Rel-20 UAV enhanceent is to specify the UE RF requiremetns with higher power, therefore it is not expected to have any impact on legacy UAV UE ACS and UAV BS ACLR/ACS requirements that is not impacted by higher transmit power from UAV UE. Hence, we suggest to focus on Scenario 4 and Scenario 8 in Rel-20 UAV coexistence study.

ACLR and ACS for UAV UEs with PC3 have been specified in Release 18. Since the objective of Release 20 UAV enhancement is to define UE RF requirements for higher transmit power, it is not expected to impact legacy UAV UE ACS or UAV BS ACLR/ACS requirements, which should remain unaffected by higher UAV UE transmit power. Therefore, we propose focusing on Scenario 4 and Scenario 8 in the Release 20 UAV coexistence study.

Proposal 1: RAN4 to prioritize Scenario 4 and Scenario 8 in the coexistence study.

## 2.2 Assumptions for coexistence

### 2.2.1 Deployment scenario

As agreed in WF [2], RAN4 needs to further discuss deployment for coexistence study from the below candidate options:

- Option 1: UMa

- Option 2: RMa

- Option 3: UMa and RMa

From a coexistence perspective, interference in the UMa scenario is likely more severe than in the RMa scenario, as more victim TN cells will be affected by the aggressor UAV UE due to the smaller ISD in UMa. Therefore, it is suggested to consider only the UMa scenario in the coexistence study.

Proposal 2: RAN4 to consider only the UMa scenario in the UAV coexistence study.

### 2.2.2 Network layout

In RAN4#116bis, two potential network layouts including co-located and non-collocated were discussed. From the view of protection between two operators, the non-collocated network layout is more realistic. In addition, due to  near-far effect, the non-collocated network layout is usually the worst case scenario.

In RAN4#116bis, two potential network layouts—co-located and non-collocated—were discussed. From the perspective of inter-operator protection, the non-collocated layout is considered more realistic. Furthermore, due to the near-far effect, the non-collocated layout typically represents the worst-case scenario.

Figure 2.2.2-1: Illustration of co-located network layout

Figure 2.2.2-2: Illustration of non-collocated network layout

Another scenario, referred to as UAV routes, involves UAV networks providing coverage along specific routes rather than continuous coverage. Compared to co-located and non-collocated layouts, the number of victim TN BSs in Scenario 4 and Scenario 8 for UAV route layouts is smaller. Therefore, it is suggested to adopt the non-collocated layout with ISDs of 4 km for the UAV network and 500 m for TN UMa as the starting point.

Figure 2.2.2-3: Illustration of UAV routes network layout

Proposal 3: RAN4 to consider the non-collocated network layout with ISDs of 4 km for the UAV network and 500 m for TN UMa as the starting point.

### 2.2.3 UAV and TN BS antenna assumptions

The open issue for UAV and TN BS assumption whether the deployment of using one antenna to serve both UAV and TN. That says the main lobe of the BS antenna points toward the ground, and UAV coverage mainly relies on side lobes, which leads to frequent handovers and severe interference, making it difficult to meet low-altitude requirements.

An open issue remains regarding the assumption for UAV and TN BS deployment—specifically, whether a single antenna is used to serve both UAV and TN. As shown in Figure 2.2.2-5, in such a configuration, the BS antenna’s main lobe points toward the ground, and UAV coverage primarily relies on side lobes. This results in frequent handovers and significant interference, making it challenging to meet low-altitude requirements.

Figure 2.2.2-4:  Use two separate antennas or beams to serve UAV and TN

Figure 2.2.2-5:  Use single antenna serve UAV and TN (Main lobe to serve UAV, and Side lobe to serve TN)

Proposal 4: RAN4 to consider using two separate antennas or beams to serve UAV and TN in the coexistence study.

In [2], the antenna configuraions and antenna gain assumptions for both AAS model and non-AAS model have been agreed. Input from operators is encouraged regarding whether the non-AAS BS model will be used for UAV deployment.

Proposal 5: Operators are encouraged to provide input on whether the non-AAS BS model will be used for UAV deployment. If non-AAS is not used, RAN4 can focus on the AAS BS model in the coexistence study.

### 2.2.4 UAV UE distribution

Table 2.2.4-1 provides the typical UAV UE altitudes in different application scenarios.

| Application Scenario | Typical UAV UE Altitude |
| --- | --- |
| Urban Logistics / Delivery | 50–120 m |
| Agricultural Monitoring / Plant Protection | 30–100 m |
| Geographic Mapping / Surveying | 100–300 m |
| Infrastructure Inspection (Power Lines, Bridges) | 20–80 m |
| Security & Surveillance | 50–150 m |
| Urban Air Mobility (eVTOL) | 300–600 m |

To fully analyze the impact of terminal distribution on coexistence results, we propose considering two typical application scenarios and their corresponding UAV UE distributions in the coexistence study:

- Urban Logistics / Delivery: UAV UEs deployed with a uniform distribution between 50 m and 120 m altitude.

- Urban Air Mobility (e.g., eVTOL): UAV UEs deployed with a uniform distribution between 300 m and 600 m altitude.

Proposal 6: RAN4 to include the following two application scenarios and UAV UE distributions in the coexistence study:

- Urban Logistics / Delivery: Uniform distribution between 50 m and 120 m.

- Urban Air Mobility (e.g., eVTOL): Uniform distribution between 300 m and 600 m.

# 3. Conclusion

In summary, we have following proposals.

Proposal 1: RAN4 to prioritize Scenario 4 and Scenario 8 in the coexistence study.

Proposal 2: RAN4 to consider only the UMa scenario in the UAV coexistence study.

Proposal 3: RAN4 to consider the non-collocated network layout with ISDs of 4 km for the UAV network and 500 m for TN UMa as the starting point.

Proposal 4: RAN4 to consider using two separate antennas or beams to serve UAV and TN in the coexistence study.

Proposal 5: Operators are encouraged to provide input on whether the non-AAS BS model will be used for UAV deployment. If non-AAS is not used, RAN4 can focus on the AAS BS model in the coexistence study.

Proposal 6: RAN4 to include the following two application scenarios and UAV UE distributions in the coexistence study:

- Urban Logistics / Delivery: Uniform distribution between 50 m and 120 m.

- Urban Air Mobility (e.g., eVTOL): Uniform distribution between 300 m and 600 m.

# 4  References

- RP-252908, New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV), RAN #109, September 2025

- R4-2515159, Way Forward for [116bis][319] NR_UAV, RAN4 #116bis, October 2025