3GPP TSG-RAN WG4 Meeting #117 	R4-2521323

# Dallas, USA,  17th - 21st November, 2025

Source:	Ericsson

Title:	Simulation results for FR1 3Tx demodulation requirements

Agenda item:	6.12.3

Document for:	Information

# Introduction

In the last RAN4 meeting, companies agreed on parameters for the FR1 3Tx demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered for alignment.

# Simulation result

## 2.1 Simulation assumptions

Table 2.1-1 Simulation parameters for normal PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
|  | DM-RS port | {0, 1, 2} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth 15kHz SCS: 5 and 50MHz 30kHz SCS: 10 and 100MHz |
| assignment | Frequency hopping | Disabled |
| TPMI index for 3Tx three-layer spatial multiplexing transmission |  | 0 |
| Channel model and MCS |  | TDLB100-400: MCS2 TDLC300-100: MCS16 TDLA30-10: MCS20 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

## 2.2 Simulation results

Table 2.2-1 Simulation results with 70% of normalized throughput for 15kHz SCS, 5MHz CBW

| Tx | Rx | Channel model | MCS | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 3 | 4 | TDLB100-400 | 2 | A | -2.71 |
|  |  |  |  | B | -2.59 |
|  |  | TDLC300-100 | 16 | A | 12.78 |
|  |  |  |  | B | 12.86 |
|  |  | TDLA30-10 | 20 | A | 14.29 |
|  |  |  |  | B | 14.28 |
| 3 | 8 | TDLB100-400 | 2 | A | -5.99 |
|  |  |  |  | B | -5.88 |
|  |  | TDLC300-100 | 16 | A | 7.13 |
|  |  |  |  | B | 7.13 |
|  |  | TDLA30-10 | 20 | A | 9.08 |
|  |  |  |  | B | 9.08 |

Table 2.2-2 Simulation results with 70% of normalized throughput for 15kHz SCS, 50MHz CBW

| Tx | Rx | Channel model | MCS | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 3 | 4 | TDLB100-400 | 2 | A | -2.73 |
|  |  |  |  | B | -2.54 |
|  |  | TDLC300-100 | 16 | A | 13.19 |
|  |  |  |  | B | 13.31 |
|  |  | TDLA30-10 | 20 | A | 16.15 |
|  |  |  |  | B | 16.14 |
| 3 | 8 | TDLB100-400 | 2 | A | -6.29 |
|  |  |  |  | B | -5.95 |
|  |  | TDLC300-100 | 16 | A | 7.33 |
|  |  |  |  | B | 7.35 |
|  |  | TDLA30-10 | 20 | A | 9.75 |
|  |  |  |  | B | 9.75 |

Table 2.2-3 Simulation results with 70% of normalized throughput for 30kHz SCS, 10MHz CBW

| Tx | Rx | Channel model | MCS | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 3 | 4 | TDLB100-400 | 2 | A | -2.74 |
|  |  |  |  | B | -2.72 |
|  |  | TDLC300-100 | 16 | A | 13.29 |
|  |  |  |  | B | 13.44 |
|  |  | TDLA30-10 | 20 | A | 14.43 |
|  |  |  |  | B | 14.43 |
| 3 | 8 | TDLB100-400 | 2 | A | -5.88 |
|  |  |  |  | B | -5.85 |
|  |  | TDLC300-100 | 16 | A | 7.25 |
|  |  |  |  | B | 7.28 |
|  |  | TDLA30-10 | 20 | A | 9.24 |
|  |  |  |  | B | 9.24 |

Table 2.2-4 Simulation results with 70% of normalized throughput for 30kHz SCS, 100MHz CBW

| Tx | Rx | Channel model | MCS | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 3 | 4 | TDLB100-400 | 2 | A | -3.02 |
|  |  |  |  | B | -2.99 |
|  |  | TDLC300-100 | 16 | A | 14.25 |
|  |  |  |  | B | 14.37 |
|  |  | TDLA30-10 | 20 | A | 16.15 |
|  |  |  |  | B | 16.15 |
| 3 | 8 | TDLB100-400 | 2 | A | -5.94 |
|  |  |  |  | B | -6.38 |
|  |  | TDLC300-100 | 16 | A | 7.61 |
|  |  |  |  | B | 7.67 |
|  |  | TDLA30-10 | 20 | A | 9.94 |
|  |  |  |  | B | 9.94 |

# Conclusions

Simulation results for FR1 3Tx demodulation requirements are delivered.

# References

- [1]	R4-2514806, “WF on NR_MIMO_demod”, Samsung, RAN4#116bis, Prague.