3GPP TSG-RAN WG4 Meeting #117	R4-2521369

# Dallas, US, November 17th-21st, 2025

Title: 	Discussion on performance requirements for R19 MIMO evolution

Source: 	Huawei, HiSilicon

Agenda item:	6.12.3

Document for:	Discussion

Background

A WF[1]  on R19 MIMO demod was agreed in last meeting.  In this contribution we provide our views on open issues for performance requirements for R19 MIMO evolution.

- UE performance requirements

Test metric for 64 CSI-RS ports

One issue is test metric for 64 CSI-RS ports, we have following options to address the infinite Gama issue:

Proposals: - Option 1: Modify the legacy test metric to following: - Option 1A: X=5dB γ=tuetrnd tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trndis the throughput measured at SNRue+[XdB] with random precoding. X could be specified to make the γ value in the reasonable range.. - Option 2: Consider possibility to define requirement only based on SNR of 90% of the maximum throughput when PMI is followed Way forward: - FFS based on simulation results provided in following meetings

Based on our understanding, option 2 is straighter and simper than option 1. However, RAN4 have never aligned the SNR point for PMI test before, the reason is that PMI calculation may be diverse especially for eType II codebook, which could lead to non-aligned results across companies. We propose to increase the target SNR span threshold used to judge the result alignment, e,g, 2.5dB->3.5dB and add additional margin on the top of average results to derive final requirements if ideal results are not well aligned.

Proposal 1: RAN4 to adopt option2, define requirements only based on SNR@90% of the maximum throughput when PMI is followed.

Observation 1: It would be difficult to align the results based on legacy criteria used to judge the simulation results alignment, especially for eType II codebook test.

Proposal 2: Increase the target SNR span threshold used to judge the result alignment, e,g, 2.5dB->3.5dB and add additional margin on the top of average results to derive final requirements if ideal results are not well aligned.

Scaling factor for MIMO correlation matrices

One issue is scaling factor for MIMO correlation matrices

Round-off operation of MIMO correlation matrices Agreement: - Companies to provide simulation results based on either option 1 or option 2, the results based on both options will be taken into account for deriving the requirements. - Further discuss and decide which option to be captured in the CR in next meeting - Option 1: Include round-off to four digits precision operation - Option 2: Not include round-off to four digits precision operation Scaling factors for MIMO correlation matrices Agreement: - For option 1 in Issue 1-2-9, as baseline, adopt the following scaling factors for 64 CSI-RS ports XP-Medium correlation level matrices to ensure PSD matrices with a measure of numerical stability Array Type; Antenna Configuration; Correlation Level; Scaling Factor. Cross-polarized; 64 (8,4,2)×2; Medium; 0.00065. Cross-polarized; 64 (8,4,2)×4; Medium; 0.00092. - Further update on the scaling factor is not precluded.

To get correlated signal, square root operation on the MIMO correlation matrix is needed. SVD decomposition and Cholesky decomposition are widely used to derive the square root of matrix. Cholesky decomposition requires the matrix to be positive semi-definite. SVD decomposition doesn’t need such condition.  Based on our calculation, without Round-off operation, correlation matrices for both 2Rx and 4Rx are positive semi-definite, we believe both SVD and Cholesky works under such condition. Hence we propose not to specify the round-off operation and scaling factor.

Proposal 3: Not to specify round-off operation and scaling factor for 64 Tx

- BS performance requirements

It was agreed to introduce PUSCH requirements with 3Tx codebook based on UL transmission. The applicability rules are FFS for discussion.

Clarify if demodulation requirements should be specified for 3-antenna-port codebook-based UL transmissions Agreement: - RAN4 should specify PUSCH demodulation requirements for 3-antenna-port codebook-based UL transmissions. - FFS on whether to introduce test applicability rule for 3Tx

4Tx/4layers tests have exact same test parameters including MCS, bandwidth channel model as 3TX/3layers test except for codebook. However, considering the test purpose is to verify demodulation processing under open-loop condition rather than precoder calculation and TPMI is fixed in the test, codebook doesn’t play any role in the BS baseband processing. Hence 4Tx/4layers tests and 3Tx/3layers fundamentally verify the same baseband processing behaviour. We don’t see any need to test both.

Observation 2: 4Tx/4layers have exact same test parameters including MCS, bandwidth, channel model as 4Tx/3layers except for codebook.

Observation 3: Codebook doesn’t play any role in the BS baseband processing in open-loop PUSCH testing.

Observation 4: Supporting 3 layers is not a new feature, there is no need to test both 3 layers and 4 layers requirements.

Proposal 4: Introduce the applicability rules that if Base station passes the tests with 4 layers, the tests with 3 layers can be skipped.

- Conclusion

In this contribution we provide our views on open issues for performance requirements for R19 MIMO evolution. The proposals and observations are

Proposal 1: RAN4 to adopt option2, define requirements only based on SNR@90% of the maximum throughput when PMI is followed.

Observation 1: It would be difficult to align the results based on legacy criteria used to judge the simulation results alignment, especially for eType II codebook test.

Proposal 2: Increase the target SNR span threshold used to judge the result alignment, e,g, 2.5dB->3.5dB and add additional margin on the top of average results to derive final requirements if ideal results are not well aligned.

Proposal 3: Not to specify round-off operation and scaling factor for 64 Tx

Observation 2: 4Tx/4layers have exact same test parameters including MCS, bandwidth, channel model as 4Tx/3layers except for codebook.

Observation 3: Codebook doesn’t play any role in the BS baseband processing in open-loop PUSCH testing.

Observation 4: Supporting 3 layers is not a new feature, there is no need to test both 3 layers and 4 layers requirements.

Proposal 4: Introduce the applicability rules that if Base station passes the tests with 4 layers, the tests with 3 layers can be skipped.

- Reference

[1] R4-2512527-Way Forward for [116][327] NR_MIMO_demod