3GPP TSG-RAN WG4 Meeting #117	R4-2521380

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	6.11.3

Source:	Huawei, HiSilicon

Title:	Discussion on CSI reporting requirement and testing framework for CSI prediction

Document for:	Discussion

# Introduction

According to WF in RAN4 #116 bis [1], we discuss the remaining issues related to CSI reporting requirement and testing framework for CSI prediction.

# Testing setup

RAN4 #113bis: Channel model for CSI prediction tests For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test. RAN4 #115 Updated simulation assumptions for AIML CSI prediction [2]: Clarifications on the SGCS calculation: - for step-1, SGCS1 is calculated by comparing the SVD of model output (before feedback) with the SVD of the ground truth Raw channel - for step-2, Companies are encouraged to provide the following two SGCS metrics: - SGCS 2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD. - SGCS 3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth. Way forward for next step simulation for CSI prediction For step-1 and step-2, - Interested companies are encouraged to provide more CSI prediction simulation results in RAN4#116 meeting. For step-3, - Interested companies are encouraged to provide step-3 CSI prediction simulation results in RAN4#116 meeting.

It is observed that in RAN4 legacy, TDL channel models are used for PMI reporting testing. Spatial channel model for demodulation performance requirements is still under study in RAN4. It is suggested that RAN4 will not consider CDL channel model in AI CSI prediction.

Proposal 1: CDL channel model is not used in RAN4 for AI CSI prediction.

For generalization test, there are two options to verify the performance of AI CSI prediction. One is to define several requirements under different test configurations/conditions. If the DUT is able to pass all the tests, then it means that the performance of CSI prediction can be guaranteed in a generalized scenario consisting of these configurations/conditions. This method is what RAN4 legacy uses.

RAN4 #113 WF: Testing conditions Use static conditions as baseline. - FFS on whether to have non-static condition

Another is that the test configurations/conditions vary over time during testing. This method has never been used in RAN4 before. Firstly, the intention is not clear to introduce more complicated testing compared legacy for generalized performance verification. Secondly, when model switching is involved under non-static condition to ensure the performance, LCM procedure testing and generalized performance testing are coupled together in one testing.

Proposal 2: Non-static condition is not used for generalized performance testing.

For LCM procedure, if model/performance monitoring and management are performed by NW, there is no need to introduce non-static condition for UE testing either. When to activate/deactivate AI is up to NW implementation.

For UE-side monitoring, two types are on the table in RAN1. One is to report performance metric, another is to report performance monitoring output. The former one needs to define the performance metric and corresponding report mechanism. The latter one needs to define the performance metric, threshold criterion and corresponding report mechanism. The performance metric and threshold definition may be subject to configurations/conditions.

TR 38.843: Performance monitoring: For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 1: - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculates the performance metric(s) - UE reports performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). RAN1 #120bis: Agreement For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric. Agreement For the definition of SGCS, Note: How to handle layer mapping mismatch, if any, is up to UE implementation. Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121. - Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI - Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. - Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window - Monitoring window can be configured by NW - FFS on signalling details - FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances - FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband) - FFS on whether to report per layer, the first layer, averaged over layer - FFS on how to quantize and report mechanism RAN1 #121bis: Agreement For CSI prediction using UE-side model, for UE assisted performance monitoring, - Support to reuse CSI framework for the configuration for monitoring result report in L1 signaling - Dedicated resource set for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - For monitoring report type, semi-persistent and aperiodic are supported - The following combination for inference report type and monitoring report type are supported Monitoring report type Inference report type; SP report; AP report. AP report; Not support; Support. SP report; Support; Support. - For monitoring resource type for measurement, periodic, semi-persistent and aperiodic are supported. Agreement - For CSI prediction using UE-side model, for UE assisted performance monitoring, - If the inference measurement resource is aperiodic CSI-RS, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of K inference measurement resources that are no later than the CSI reference resource of the associated inference report; - Additional active resource/port counting time for the aperiodic resource of the inference report is from the end of the inference report to the end of the triggered monitoring report - If inference report is aperiodic and the inference measurement resource is periodic or semi-persistent, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of the most recent Kp inference measurement resource transmission occasions that are no later than the CSI reference resource of the associated inference report

It is observed that for monitoring methods, both UE-assisted performance monitoring Type-3 and NW-side performance monitoring are supported. RAN1 has achieved a list of agreements on specifications for supporting Type-3. Since Type-3 is an additional UE capability while Type-1 can be tested by reusing current measurement and reporting framework, RAN4 will focus on Type-1 monitoring where legacy requirements can be reused.

Proposal 4: RAN4 will focus on NW-side monitoring for AI CSI prediction and legacy measurements reporting requirements are reused.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: CDL channel model is not used in RAN4 for AI CSI prediction.

Proposal 2: For generalization testing, there is no need to use non-static condition.

Proposal 3: RAN4 will start to discuss whether/how to define requirements for UE-side monitoring if other WGs achieve sufficient progress.

Proposal 4: RAN4 will focus on NW-side monitoring for AI CSI prediction and legacy measurements reporting requirements are reused.

# References

- R4-2514631, “WF on AI ML for air interface study”, 3GPP RAN4 #116bis, 2025.

- R4-2508102, “Updated simulation assumptions for AIML CSI prediction”, 3GPP RAN4 #115, 2025.