3GPP TSG-RAN WG4 Meeting #117	R4-2521388

Dallas, USA, Nov 17 – 21, 2025

Agenda Item:	7.9.4

Source:	Huawei, HiSilicon

Title:	Discussion on RRM core requirement for AI-CSI compression

Document for:	Discussion and Decision

# Introduction

According to new WI on AI/ML for NR air interface [1], the core part work of interoperability and RRM requirement in RAN4 includes:

Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability o The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In this contribution, we discuss RRM core requirement for AI-CSI compression.

# Discussion on LCM for AI-CSI compression

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

In RAN4 legacy PMI reporting, no RRM requirements are involved. However, in AI-CSI compression, since UE will either implement a specified model or a private model trained by UE vendors (based on the training dataset shared by NW), there is an AI model located at the UE that will be managed by the NW. Therefore, RRM requirements may be involved. In R19, potential impacts on RRM specifications have been identified for AI-CSI prediction, AI-BM and AI-positioning. It is suggested that RAN4 should leverage the testing framework defined for other use cases related to LCM as much as possible. For example, discussions could be held on whether to introduce or reuse the definitions of activation delay and reporting delay. Additionally, if requirements are to be defined, the requirement for activation delay could be derived on a case-by-case basis.

Observation 1: No RRM requirements are involved in legacy PMI reporting.

Proposal 1: Reuse LCM testing framework defined in R19 as a starting point for AI-CSI compression, if impact on RRM requirement is identified.

# Conclusions

According to the discussion, following proposals and observations are provided:

Observation 1: No RRM requirements are involved in legacy PMI reporting.

Proposal 1: Reuse LCM testing framework defined in R19 as a starting point for AI-CSI compression, if impact on RRM requirement is identified.

# References

- RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, 3GPP RAN #108, 2025.