3GPP TSG-RAN WG4 Meeting #117	R4-2521408

Dallas, USA, November 17 – 21, 2025

Source:	Nokia

Title:	On Enhancements for Air-to-Ground Network for NR

Agenda item:	6.7.4

Document for:	Discussion

Introduction

The Way Forward (WF) from RAN4#116bis discussions on Rel-19 Enhancements for Air-to-ground network for NR is documented in [1]. The discussions covered 3 topics, namely, DL CA, DL MIMO and UL MIMO, with issues which have agreements are listed below:

<Topic 1> DL CA Issue 1-1: TDD pattern Issue 1-2: HARQ configuration for TDD pattern 30D4S6U in CA Issue 1-3: CA configuration selection rules Issue 1-4: Test applicability rules # <Topic 2> DL MIMO Issue 2-1: TDD pattern Issue 2-2: MCS

In this contribution, we discuss one minor issue on one of the agreements made in RAN4#116bis.

Discussion

One of the agreements made in RAN4#116bis as captured in [1] in on the HARQ configuration for TDD pattern 30D4S6U in CA as below.

Issue 1-2: HARQ configuration for TDD pattern 30D4S6U in CA Agreement: - Following the number of HARQ process and K1 values Test parameters for number of HARQ processes for ATG CA requirements HARQ process number; CCs with the same duplex mode & SCS with Pcell; CCs with different duplex mode / SCS with Pcell. FDD 15 kHz + TDD 30 kHz CA; FDD PCell; 4; 13. TDD 30 kHz + TDD 30 kHz CA; TDD PCell; 13; N/A. Test parameters for K1 values for ATG CA requirements The number of slots between PDSCH and corresponding HARQ-ACK information; CCs with the same duplex mode and SCS with Pcell; CCs with different duplex mode and/or SCS with Pcell. FDD 15 kHz + TDD 30 kHz CA; FDD PCell; {2}; 2 if mod(i,40) = {1,4,5,9,10,14,15,19,20,24,25,28,29}. TDD 30 kHz + TDD 30 kHz CA; TDD PCell; 33 if mod(i,40) = 1 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 10 if mod(i,40) = 29; N/A.

As highlighted in the Tables above, the “CCs with the same duplex mode & SCS with Pcell” should only apply for TDD 30 kHz + TDD 30 kHz CA case. Nonetheless, in the Tables under “CCs with the same duplex mode & SCS with Pcell”, there are values “4” for number of HARQ processes and “{2}” for K1 values being written in the cells for FDD 15 kHz + TDD 30 kHz CA, when it is a different duplex mode (FDD and TDD) and SCS (15 KHz and 30 KHz).

The previously agreed Tables on number of HARQ processes and K1 values contain values under “CCs with the same duplex mode & SCS with Pcell” for different duplex and SCS (FDD 15 kHz + TDD 30 kHz CA) case.

RAN4 to update the agreed Tables on number of HARQ processes and K1 values under “CCs with the same duplex mode & SCS with Pcell” to void the values for FDD 15 kHz + TDD 30 kHz CA.

Conclusion

In this contribution, we discuss one minor issue on the agreements made during RAN4#116bis. The following Observations and Proposals were made:

Observation 1: The previously agreed Tables on number of HARQ processes and K1 values contain values under “CCs with the same duplex mode & SCS with Pcell” for different duplex and SCS (FDD 15 kHz + TDD 30 kHz CA) case.

Proposal 1: RAN4 to update the agreed Tables on number of HARQ processes and K1 values under “CCs with the same duplex mode & SCS with Pcell” to void the values for FDD 15 kHz + TDD 30 kHz CA.

- References

- R4-2514796, WF on NR_ATG_enh_demod, 3GPP RAN4#117, CMCC, Prague, Czech Republic, October 13-17, 2025