# 3GPP TSG-RAN WG4 Meeting #117	R4-2521421

# Dallas, USA, Nov. 17-21, 2025

Title:	Discussion on TN BS RF for 6GR

Source:	Samsung

Agenda item:	8.4.1

Document for:	Approval

Introduction

In last RAN4#116bis meeting, RAN4 had initially discussed the contributions for 6GR BS RF requirements and co-existence from multiple companies. The meeting had agreed on some high-level guidance on some aspects and documented in [1], but the detailed proposals from each company still require more discussion. In this paper, we will continue to provide our views on the TN BS RF aspects for 6GR.

Discussion

Previously in 6G BS RF

In #116bis meeting, in our paper [2], we have discussed the challenges and advancements in 6G base station (BS) RF requirements, particularly for new carriers, emphasizing the need for hybrid beamforming, massive MIMO (NeX-MIMO), and energy-efficient antenna structures. It highlights the importance of feasibility and co-existence studies for around 7 and/or around 15GHz carriers, to determine RF requirements and their potential impact on legacy carriers under 6GHz in 6GR. And in that paper, we have proposed following observations and proposals in BS RF aspects.

R4-2513045 [2] Observation 19: The 6G BS would support 7-24 GHz carrier with potentially more ports and MIMO schemes, and likely in hybrid beamforming scheme. Observation 20: The feasibility and co-ex studies on FR3 carriers, e.g. 7GHz, are the basis of 6G BS RF requirements for 6G Radio. And the study outcomes would also impact the RF requirements in the 6G BS RF requirements in FR1 carriers. Proposal 12: The new 6G BS RF requirements should be discussed based on feasibility and new co-ex studies at least on 7GHz carrier. Proposal 13: To be efficient and not duplicating work, the Rel-20 BS evolution WI outcomes, e.g. SBFD-capable gNB unwanted emission and Rx in-band blocking, can be taken as starting point for 6G BS requirements discussions in normative phase. Observation 21: The agreed 6G BS requirements from 7GHz could then be evaluated to check if they can be expanded to other carriers < 6GHz. Observation 22: It’s worth to discuss whether and how to apply RF requirements on “energy saving mode” when only part of the element arrays and their associated PA is activated.

For this BS RF parts, in last meeting WF[1], companies had agreed that given the existing deployment for both FR1 and FR2 base station and their associated hardware, and the experience with existing 5G spec, RAN4 would take 5G BS RF requirements as starting point when defining 6G BS RF requirements. And the importance of the hybrid beamforming were also brought up by multiple BS vendors, and it was then agreed to be studied. Moreover, meeting also agreed that 6G SBFD BS RF requirement would leverage the outcomes from Rel-19 and on-going Rel-20, and potential enhancement can be discussed later with more stable Rel-20 outcomes. In last, [1] agreed on the template to collect views for individual requirements from companies.

Discussions on 6G BS RF requirements

Following the example provided in [1], in this section, we provided our initial views on the potential impacts on 6G BS RF requirements. However, it should be noted that many of these initial analyses are subject to change given the 6G SI is still on-going and in early phase.

Among all the existing BS RF requirements, we believe following are some factors to be considered for 6G at this stage. Of course, there could be more impacts from 6G studies, and we should always review the potential impacts along with the 6G discussions in other topics and working groups.

As the plenary meeting is still discussing about the 6G, including the system parameters, scenarios, which include the high level design over some important aspects of the 6G BS, especially on the BS AAS array size and its coverage. And also the new chnanel bandwidth would also need to be scaled/supported in the new 6G BS requirements. Our initial review is that the output power requirements would depend on the 6G designs, and also need to consider the energy saving or energy efficiency techniques. Also, in our last paper [2], we also brought up the potential impacts from energy saving techniques adopted in 6G.

R4-2513045 [2] Another key technology/topic to be considered in 6G BS is energy efficiency topic, in which, RAN1 is discussing about to utilize part of the antenna elements and associated power amplifiers. The candidate 6GR spectrum such as around 7 GHz would make the spatial and power adaptation effective where the power usage scales with the number of antennas and associated power amplifiers. One of the potential solutions is depicted as follows: Figure 3-1: Spatial and power adaptation (left), Joint network and UE energy efficiency (right) By adopting the ‘energy saving mode’ as shown above, one thing that RAN4 should discuss is whether and how to apply RF requirements on the partially activated 6G BS. Especially, considering the beam would usually grow “fatter” when adopting less elements under such “energy saving mode”, and also the Tx power would be less, then whether different ACLR/ACS is needed for it could also be discussed.

Observation 1: The base station output power requirements may subject to change depending on the 6G discussions on deployment scenario, typical antenna size, energy saving, etc.

Moreover, in our last paper [2], the on-going BS RF evolution WI in RAN4 was mentioned, and we proposed that the the outcomes of this WI should be leveraged as much as possible. More explicitly, the discussions and outcomes for the RF requirements, like spurious emission, intermodulation requirements, out-of-band blocking requirements, along with the assumed coupling loss between base stations in new 6G frequency or frequency ranges should be leveraged as much as possible in 6G BS RF.

The SBFD-SBFD adjacent channel co-existence study and its impacts on Tx unwanted emission and Rx in-band blocking requirements should follow the same principle as above. And that was agreed in last meeting in [1].

R4-2513045 [2] Given the overlapping timeline, the RAN4 ongoing Rel-20 WI of BS RF evolution for FR1 and testing phase 2 [12] should be leveraged as much as possible. And the outcomes of this WI can be taken as starting point for the 6G BS requirements discussion. Enhancement of co-location requirements - Enhancement of co-location requirements for BS type 1-O for upper frequencies of FR1 targeting above 3.3GHz - Evaluate impact of Rel-19 BS-to-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements. - Consider impact on the following OTA co-location requirements: - Transmitter OFF power requirement - Transmitter co-location spurious emission requirement - Transmitter intermodulation requirement - Co-location OOBB requirement - Enhance OTA co-location requirements - Improve the core requirements with reasonable expected BS-to-BS isolation for the upper FR1 frequencies. Two options will be considered - Option 1: Investigate and improve CLRA concept - Option 2: Investigate possibility to define requirements using the existing OTA metrics (such as TRP, EIRP, EIS), if any SBFD BS to SBFD BS adjacent channel co-existence - SBFD BS to SBFD BS adjacent channel coexistence - Focus on FR1 WA. - Consider both scenarios including SBFD capable BS being co-located and non-collocated. - Focus on bands: n104, n79, n41. - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include - TX unwanted emission - RX in-band blocking - Note 1: whether to comply with requirements is declared by BS. - Note 2: The existing TDD requirements apply for non-SBFD BSs.

Observation 2: The on-going Rel-20 BS evolution WI discussions and/or outcomes should be leveraged in 6G BS RF, which may impacts spurious emission, intermodulation requirements, out-of-band blocking requirements, along with the assumed coupling loss between base stations in new 6G frequency or frequency ranges.

The overal co-existence study discussion falls into another sub-agenda under this topic, which this paper will not cover much about the details. It’s quite straightforward, that if there is new co-ex study to be established for 6GR in RAN4, there are numbers of requirements would be impacted depending on the findings from the co-ex study, e.g. ACLR, ACS, etc.

Observation 3: The co-ex study findings would impact the ACLR, ACS requirements, if new study is conducted.

Considerations for hybrid type BS requirements

In our last paper [2], we have described the importantce and emerging adoption of larger array and the hybrid beamforming for 6G BS. RAN4#116bis meeting had agreed to review and study the existing 1-H requirements in [1].

R4-2513045 [2] Supporting FR2 in NR posed challenges due to significant pathloss, which reduced coverage. Analog beams were therefore implemented to enhance coverage. Efforts to combine the benefits of digital beams have further led to hybrid beamforming, integrating existing MIMO and beam technologies. In 6GR, exploring the optimal system structure for FR3, which is crucial for both 6G BS and UE antenna architecture, should be prioritized. In 6GR FR3 band (7-24 GHz), 6G BS utilize energy-efficient antenna structures with up to 256 ports to support massive MIMO (NeX-MIMO). The subarray structure is essential to support 256 ports by utilizing more antenna elements but various aspects such as the performance and complexity of the power splitting network for the subarray network, resolution for phase array operation, and UE’s channel measurement complexity must be considered. Use of AI/ML for scalable CSI feedback reduction, dynamic spatial adaptation, and channel reconstruction, minimizing overhead while maximizing coverage should be studied.

In our view, considering that the OTA performance of the BS is of course closely related to the actual performance in deployment, we would like to study the OTA requirements for 1-H BS or enhanced type of 1-H BS whichever would be agreed for 6G. However, given the testing efforts and costs for OTA requirements is usually higher than those for conducted requirements, we suggest to carefully make decision to increase OTA requirements after study. Especially, if both OTA and conducted options are studied and then specified for the same requirement, we believe it should up to the manufacturer to select one of them other than test both.

Observation 4: RAN4 can study the OTA options for hybrid requirements but should also carefully decide whether to inrtoduce them, even as optional, after study considering the OTA testing costs and time.

Initial views provided in table

Based on above discussions, we provide our initial views in the template provided in [1]. It worth to be explicitly mentioned that we added several SBFD gNB requirements into the table, and we would like them to be included.

Proposal 1: Propose the following initial views on 6G RF requirements.

| BS RF requirements | No impact expected, might be reused as is for 6G | To be re-evaluated based on system parameters decision for 6G (SU, channel BW, …), following TR 38.817-02 principles/formula | Would need to be studied in the 6G SI scope |
| --- | --- | --- | --- |
| Base station output power |  | This may subject to change depending on the 6G deployment scenario (e.g. coverage), energy saving/efficiency considerations and other aspects. |  |
| Output power dynamics |  |  |  |
| RE power control dynamic range |  |  |  |
| Total power dynamic range |  |  |  |
| Transmit ON/OFF |  |  |  |
| Transmitted signal quality |  |  |  |
| Frequency error |  |  |  |
| Modulation quality |  |  |  |
| Time alignment error |  |  |  |
| Unwanted emissions |  | Leverage the outcomes of Rel-19 and Rel-20 SBFD requirements, potential enhancement can be studied depending on 6G study progress. |  |
| Occupied bandwidth |  |  |  |
| ACLR |  | Re-evaluate the ACLR from co-existence study. This depends on the typical 6G scenario and BS/UE parameters and also if critical difference is identified. |  |
| Operating Band Unwanted Emissions |  |  |  |
| Transmitter spurious emission |  |  |  |
| General requirement |  |  |  |
| Protection of BS receiver of own or different BS |  |  |  |
| Additional spurious (coex) |  |  |  |
| Co-location |  |  |  |
| Transmitter intermodulation |  | Leverage the outcomes of Rel-20 new co-location framework, especially to re-evaluate the coupling loss assumption for 6G new frequency or frequency range. |  |
| Transient period between SBFD and non-SBFD operation |  | Leverage the outcomes of Rel-19 and Rel-20 SBFD requirements, potential enhancement can be studied depending on 6G study progress. |  |
| In-channel adjacent subband leakage for SBFD |  |  |  |
| Receiver sensitivity level |  |  |  |
| Dynamic range |  |  |  |
| In-band selectivity and blocking |  |  |  |
| ACS |  | Re-evaluate the ACS from co-existence study. This depends on the typical 6G scenario and BS/UE parameters and also if critical difference is identified. |  |
| In-band blocking |  | Leverage the outcomes of Rel-19 and Rel-20 SBFD requirements, potential enhancement can be studied depending on 6G study progress. |  |
| Out of band blocking |  | Leverage the outcomes of Rel-20 new co-location framework. |  |
| Receiver spurious emissions |  |  |  |
| Receiver intermodulation |  |  |  |
| In channel selectivity |  |  |  |
| In-channel adjacent subband blocking for SBFD. |  | Leverage the outcomes of Rel-19 and Rel-20 SBFD requirements, potential enhancement can be studied depending on 6G study progress. |  |

Conclusion

In this contribution, we proposed following for the 6G radio TN BS RF requirements:

Observation 1: The base station output power requirements may subject to change depending on the 6G discussions on deployment scenario, typical antenna size, energy saving, etc.

Observation 2: The on-going Rel-20 BS evolution WI discussions and/or outcomes should be leveraged in 6G BS RF, which may impacts spurious emission, intermodulation requirements, out-of-band blocking requirements, along with the assumed coupling loss between base stations in new 6G frequency or frequency ranges.

Observation 3: The co-ex study findings would impact the ACLR, ACS requirements, if new study is conducted.

Observation 4: RAN4 can study the OTA options for hybrid requirements but should also carefully decide whether to inrtoduce them even as optional after study considering the testing costs and time.

Proposal 1: Propose the following initial views on 6G RF requirements.

Reference

[1] R4-2514638, WF on 6G BS RF and co-existence, Feature Lead (Ericsson)

[2] R4-2513045, Discussion on BS RF and co-existence for 6GR, Samsung