3GPP TSG-RAN4 Meeting #117	R4-2521429

Dallas, USA, 17 November – 21 November 2025

Source:			Ericsson

Title:			6 MHz channel bandwidth – system and UE RF impacts

Agenda item:		7.1.5.2

Document for:		Approval

# Introduction

In last RAN#109, a new WI ([1]) has been approved to enhance UE RF requirements for the deployment in NR FR1. One of the objectives of this WI is to specify 6 MHz channel bandwidth for NR bands n5, n8, n12, n85 and n29.

Based on the agreed Way Forward ([4]) from last RAN4#116bis meeting, this contribution is discussing the remaining open issues related to UE RF requirements.

# Discussion

## Background

In last RAN4#116bis meeting, the maximum transmission bandwidth for 6 MHz channel bandwidth was agreed (30 RBs) and most of the UE RF requirements impacst related to the introduction of 6 MHz channel bandwidth were also agreed ([4]):

- Tx requirements:

  - SEM.

  - MPR.

  - Minimum Output power.

  - Transmit Off power.

  - Occupied bandwidth.

  - ACLR.

  - General spurious.

- Rx requirements:

  - REFSENS and UL configurations for the considered bands (n5, n8, n12, n85 and n29).

  - Maximum input level.

  - ACS.

  - In-band blocking.

  - Out of band blocking.

  - Narrow Band blocking.

  - Intermodulation.

  - Spurious response.

The only remaining open issue on UE RF side is related to A-MPR.

## NSs and A-MPR

### Introduction

In last RAN4#116bis meeting, regarding NSs, it was already agreed that no A-MPR evaluation would be needed for 6 MHz channel bandwidth and:

- NS_43 and NS_43U:

- NS_14:

The following NS and associated A-MPRs were left for further study:

- NS_06 (PC3 and PC2)

- NS_15 (PC3 and PC2).

### NS_06

For PC3 operation in bands n12 and n85, TS 38.101-1 already specifies that no A-MPR is applicable for the currently supported channel bandwidth.

Considering that 5 and 10 MHz channel bandwidths are already supported in those 2 bands, there is no reason that A-MPR would be needed for PC3 and 6 MHz channel bandwidth.

For PC2 operation in band n85, the A-MPR is specified identically for all supported channel bandwidth, the A-MPR values are channel bandwidth agnostic.

Again, considering that 5 and 10 MHz channel bandwidths are already supported in n85, there is so no reason that A-MPR should be re-evaluated for PC2 and 6 MHz channel bandwidth.

Proposal 1: No A-MPR simulations is needed for NS_06 when introducing 6 MHz channel bandwidth, the current A-MPR requirements for PC2 and PC3 are still valid.

### NS_15

After the RAN4#116bis meeting, Verizon confirmed that 6 MHz channel bandwidth would be of interest for n5, meaning RAN4 should evaluate A-MPR for NS_15 and 6 MHz channel bandwidth.

Reusing the assumptions taken for 7 MHz channel bandwidth ([5]), we propose to evaluation the needed NS_15 A-MPR considering that 6 MHz channel bandwidth would be deployed within 814-845 MHz or within 841.5-849 MHz.

Proposal 2: Evaluation A-MPR for NS_15 and 6 MHz channel bandwidth assuming deployment within 814-845 MHz or within 841.5-849 MHz.

The following usual A-MPR assumption should also be considered:

- Channel Bandwidth = 6 MHz

- SCS = 15 kHz

- Maximum # of RBs = 30

- Band of Operation : n5 (Uplink : 824 – 849 MHz)

- LO leakage = 28 dBc

- Image = 28 dBc

- CIM3 = 60 dBc

- Waveforms : CP-OFDM and DFT-s-OFDM

- Modulation : QPSK, 16 QAM, 64 QAM and 256 QAM

- Calibration: PC3, 1dB MPR with DFT-s-OFDM QPSK 20MHz, L_CRB=100 and Start_RB=0

Also, to leverage the work already done for 7 MHz channel bandwidth, we would propose to reuse as starting point the same A-MPR regions definition, as captured below in Table 1 for PC3 and Table 2 for PC2, the corresponding A-MPR values shall be evaluated via simulations. Note that, if needed, those regions could be re-defined in a second step, if some parts of those regions don’t need any A-MPR.

Table 1: A-MPR regions for NS_15 and PC3

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 841.5 < Fc ≤ 845.5 | ≥ 4.5 | > 0 | A7 |
|  |  | ≥ 2.16 , < 4.5 | ≥ 1.8 | A2 |
|  |  | ≤ 1.44 | ≤ 0.36 | A8 |
|  | 838 < Fc ≤ 841.5 | > 0 | > 2.7 | A9 |
|  |  | ≥ 5.94 | ≤ 2.7 | A10 |

Table 2:  A-MPR regions for NS_15 and PC2

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 841.5 < Fc ≤ 845.5 | ≥ 4.5 | > 0 | A8 |
|  |  | ≥ 2.16, < 4.5 | ≥ 1.8 | A9 |
|  |  | ≥ 2.16, < 4.5 | ≥ 1.44, < 1.8 | A10 |
|  |  | < 1.44 | ≤ 0.36 | A11 |
|  | 837 < Fc ≤ 841.5 | ≥ 0 | > 2.7 | A12 |
|  |  | ≥ 5.94 | ≤ 2.7 | A13 |

Proposal 3: When doing the A-MPR evaluation, reuse as starting point the regions defined for 7 MHz channel bandwidth (Table 1 and Table 2). Those regions might be re-tuned depending on A-MPR results.

# Conclusion

In this contribution, we discussed the remaining open issues related to UE RF when introducing 6 MHz channel bandwidth. We made the following proposals:

Proposal 1: No A-MPR simulations is needed for NS_06 when introducing 6 MHz channel bandwidth, the current A-MPR requirements for PC2 and PC3 are still valid.

Proposal 2: Evaluation A-MPR for NS_15 and 6 MHz channel bandwidth assuming deployment within 814-845 MHz or within 841.5-849 MHz.

Proposal 3: When doing the A-MPR evaluation, reuse as starting point the regions defined for 7 MHz channel bandwidth (Table 1 and Table 2). Those regions might be re-tuned depending on A-MPR results.

# References

- RP-252901, UE RF enhancements for NR FR1, Phase 5

- R4-2501361, 7 MHz channel bandwidth – system impacts, Ericsson

- R4-2513833, 7 MHz channel bandwidth – BS impacts, Ericsson

- R4-2515084, Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 -- 6 MHz channel, Samsung, Nokia, Ericsson, Skyworks, Apple, Huawei, CATT, ZTE, Qualcomm

- R4-2505098, WF on requirements for 7MHz channel bandwidth, Ericsson