3GPP TSG-RAN4 Meeting #117	R4-2521432

Dallas, USA, 17 November – 21 November 2025

Source:			Ericsson, FirstNet, APCO

Title:			US 4.9 GHz band – BS RF aspects

Agenda item:		5.11.3

Document for:		Approval

# Introduction

In RAN#108, a WI ([1]) has been approved to introduce a new 4.9GHz band for US.

Based on the Way Forward ([3]) agreed in last RAN4#116bis meeting, this contribution is discussing the remaining BS RF open issues related to the introduction of the US 4.9GHz band.

# Discussion

## Background

In last RAN4#116bis meeting, RAN4 agreed ([3]) on the following BS RF aspects:

- Specify an additional OBUE requirement for the US 4.9 GHz band based on FCC mask M.

- Specify an additional spurious requirement for the US 4.9 GHz band.

- Coexistence and coexistence requirements for the US 4.9 GHz band should not be captured as there is an on-going work to make those requirements more generic, avoiding introducing new requirements each time a new band is added.

Nevertheless, there was no agreement on the BS maximum output power limits, not the BS classes to be introduced.

## FCC §90.1215 - Power limits

FCC specified the following maximum conducted output power (below Table 1), mentioning that higher power devices, using another channel bandwidth or not, are limited to a peak PSD of 21dBm/MHz.

Table 1: Maximum output power per channel bandwidth - §90.1215(a)(1)

According to §90.1215 (2), for “high power point-to-point and point-to-multipoint operations (both fixed and temporary-fixed rapid deployment) may employ transmitting antennas with directional gain up to 26 dBi without any corresponding reduction in the maximum conducted output power or spectral density”, this would correspond to a maximum output power of 63 dBm EIRP for 50MHz channel BW, still keeping the PSD limit of 21dBm/MHz.

Also, as stated in §90.1215 (f), the maximum EIRP must not exceed 85 dBm for permanent point-to-point and point-to-multipoint stations (FCC gives the following definition of a station: a station equipped to engage in radio communication or radio transmission of energy). There is an additional restriction when the path length is less than 17 km (New EIRP limit = 55 dBW-40*log(17/B) dBW, where B = the actual path length in kilometres) but as RAN4 doesn’t capture requirement which might be deployment related, this restriction should not be captured in 3GPP specifications (the restriction still applies of course).

It should be noted that, in §90.1215(f), FCC doesn’t mention if the “point-to-point” and “point-to-multipoint” stations are fixed or not.

Observation 1: In §90.1215(f), FCC doesn’t indicate if the “point-to-point” and “point-to-multipoint” stations are fixed or not.

In last RAN4#116bis meeting, RAN4 discussed which BS classes should be specified for this 4.9 GHz band. Looking at §90.1215(a)(1) and the maximum output power per channel bandwidth table, it seems that only Local Area and Medium Range classes of BS should be considered as the maximum output power for 50MHz channel BW is limited to 37dBm.

To our view, as §90.1215(f) doesn’t clearly mention if the 85dBm EIRP limit is for fixed station or not, while it’s explicitly mentioned in other subclauses, we consider this limit could be applicable to Base Stations as well. And so, RAN4 should also consider Wide Area BS class for the 4.9GHz band, specifying the 85dBm maximum EIRP limit. It would then be left up to the operator to consider this 85 dBm EIRP limit or the maximum output power per channel bandwidth (Table 1) depending on the considered deployment.

Observation 2: Depending on the considered deployment, the operator shall either consider the 85dBm maximum EIRP limit or the maximum output power per channel bandwidth (Table 1).

Proposal 1: To not restrict any type of deployment, RAN4 should specify the 85dBm maximum EIRP limit and consider Wide Area BS class for the US 4.9 GHz band.

# Conclusion

In this contribution, we discussed the remaining BS RF open issues when introducing the new US 4.9 GHz band and made the following observations and proposals related to the BS RF impacts:

Observation 1: In §90.1215(f), FCC doesn’t indicate if the “point-to-point” and “point-to-multipoint” stations are fixed or not.

Observation 2: Depending on the considered deployment, the operator shall either consider the 85dBm maximum EIRP limit or the maximum output power per channel bandwidth (Table 1).

Proposal: To not restrict any type of deployment, RAN4 should specify the 85dBm maximum EIRP limit and consider Wide Area BS class for the US 4.9 GHz band.

# References

- RP-251887, New WID on Introduction of NR TDD 4.9GHz Band for US Operation, Fujitsu

- Eight Report and Order, WP Docket No. 07-100, 22 October 2024.

- R4-2515103, Way Forward for [116bis][322] NR_TDD_4.9GHz_band, Fujitsu