3GPP TSG-RAN WG4 Meeting #117		R4-2521453
Dallas, USA, Nov. 17-21, 2025

Title:	Discussion on BC Basket procedure simplification

Source:	OPPO

Agenda item:	8.12.1

Document for:	Approval

Introduction

In this contribution, we discuss how to simplify the BC basket procedure.

Discussion

In LTE and 5G NR, CA was introduced to aggregate multiple contiguous or non-contiguous carriers in single band or in multiple bands. The original intention of CA is to enable multiple UL carriers to work simultaneously for transmission and multiple DL carriers to work simultaneously for reception, in order to achieve a higher data rate in UL and DL. With the application of CA, on one hand, the number of aggregated bands and aggregated carriers is constantly increasing; On the other hand, to meet some deployment requests, more different working modes have been introduced, especially in 5G NR, such as Tx switching and SDL switching.

In 5G NR tens of thousands of band combination has been defined. The band combination definition and related requirements for CA take up a significant portion in Spec. If no any limitation, this trend is unsustainable in the future.

In 6G, although some new technologies to aggregate multiple carriers are raised in RAN1, CA is expected to be supported. From RAN4 perspective, one of important task is how to simplify band combination.

In 5G NR, to introduce new band combinations, operators need initiate the request and then RAN4 define the requirements following the workflow in TR 38. 862 as shown in Figure 1:

Figure 1 The workflow on the introduction of band combinations for block approval

From the workflow, we can see at least two RAN4 meetings and two RAN meetings are needed from initiating request to introducing the band combination into the Spec. That is, this will take at least six months.

Observation 1: It takes at least two RAN4 meetings and two RAN meetings to complete one band combination from request to introduce into the spec.

In order to introduce band combinations more efficiently, the current request procedure can be further optimised. E.g., one special band combination request procedure can be defined

- When operator want to introduce a band combination, it can request it in RAN4 meanwhile provide TP/draft CR at the same RAN4 meeting;

- RAN4 can endorse this TP/draft CR in that meeting;

- In the following RAN plenary, this operator can bring this endorsed TP/CR to RAN plenary for approval directly together with the WID revision;

To obey RAN plenary’s procedure, some limitation is also needed. Like

- This special procedure is limited to at most N times/ M band combination number per operator

- The urgency of this band combination needs to be well justified at the very beginning of RAN4 discussion

- This justification needs to be well accepted in RAN4

- The whole workload is within the TU budget, i.e., it should not be used to overload RAN4

Proposal 1:	Consider introduce “high speed band combination request procedure”:

- Operator request one band combination and provide TR/draft CR at the same RAN4 meeting;

- RAN4 endorse this TP/draft CR in that meeting;

- In the following RAN plenary, operator bring this endorsed TP/CR to RAN plenary for approval directly together with the WID revision.

Proposal 2:	To avoid overload RAN4:

- This special procedure is limited to at most N times with M band combinations requested per operator each time;

- The urgency should be well justified at the very beginning of RAN4 discussion;

- The newly requested band combinations should not exceed the TU limits for basket WI.

Conclusion

This paper discussed BC Basket procedure simplification and proposed:

Observation 1: It takes at least two RAN4 meetings and two RAN meetings to complete one band combination from request to introduce into the spec.

Proposal 1:	Consider introduce “high speed band combination request procedure”:

- Operator request one band combination and provide TR/draft CR at the same RAN4 meeting;

- RAN4 endorse this TP/draft CR in that meeting;

- In the following RAN plenary, operator bring this endorsed TP/CR to RAN plenary for approval directly together with the WID revision.

Proposal 2:	To avoid overload RAN4:

- This special procedure is limited to at most N times with M band combinations requested per operator each time;

- The urgency should be well justified at the very beginning of RAN4 discussion;

- The newly requested band combinations should not exceed the TU limits for basket WI.

Reference