3GPP TSG-RAN WG4 Meeting #117          	R4-2521478

Dallas, USA, 17th-21th, Nov, 2025

Agenda item:	6.13.4

Source:	Samsung

Title:	View on demodulation requirement for SBFD

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the test scope for UE and BS demodualtion requirements with supporting SBFD feature was discussed,  the related agreement was captured into WF in [1] as following

- BS demodulation requirement - If RAN4 to define Rel-19 SBFD PUSCH requirement with UL muting, only considering inter-site gNB-gNB interference, self-interference and intra-site inter sector interference are not taken into account - For inter-site gNB-gNB interference modeling - Considering the RF requirement, the following INR values derived based on link budget analysis can be considered for evaluation. Based on the evaluation results, RAN4 can further discuss whether to introduce the PUSCH requirement with UL muting - Not pursue to derive INR based system level simulation for inter-site gNB-gNB interference modeling when defining PUSCH requirement with UL muting for SBFD in Rel-19 - Whether to introduce PUSCH requirement with UL muting - FFS on whether RAN4 introduce the PUSCH requirement with UL symbols muting - Test setup - Frequency Range - Option 1: cover both FR1 and FR2-1 - Waveform for wanted signal - Option 1: cover both CP-OFDM and DFT-s-OFDM waveform - sub-band pattern - Option 1: considering sub-band configuration as {DU}, where the UL sub-band is located in the lower band - Channel Bandwidth for wanted signal - Option 1: - For FR1: 40MHz CBW (20MHz DL sub-band+ 20MHz UL sub-band) for 30kHz SCS - For FR2-1: 200MHz CBW (100MHz DL sub-band+ 100MHz UL sub-band) for 120kHz SCS - TDD pattern - Option 1: - 30KHz SCS: XXXXXXXXUU - 120Khz SCS: XXXXU - UL sub-band transmission could start from the first F slots - Skip U slots to simply simulation setup - Sub-band configuration in SBFD slot - Option 1: - Take SBFD UL sub-band spans all the symbols with a SBFD slot - Number of symbol muting and symbol configuration - Option 1: 1 - For Mapping type, A - For Mapping type B - Option 2: 2 - For Mapping type A - For Mapping type B - PUSCH mapping type for wanted signal and interferers - Option 1: - Both mapping A and mapping B in FR1 - Mapping B in FR2 - Number of DMRS symbols for wanted signal - Option 1: Configure 2 DMRS symbols for defining SBFD PUSCH demodulation requirement in FR1 and FR2-1 - Channel model - TDLC300-100 for wanted signal in FR1 - TDLC300-0 for interference signal if considering INR based interference modelling in FR1 - MCS - MCS 16 for wanted signal in FR1 and FR2 - 16QAM for interference signal in FR1 - Rank 1 for wanted signal and interference signal - Antenna Configuration - Option 1: Reuse the same antenna configuration assumption for evaluation SBFD PUSCH performance with UL muting as 1Tx/2Rx/4Rx/8Rx in FR1 and 1Tx/2Rx in FR2-1 - Other Assumption - Option 1: Reuse the same assumption for specifying SBFD PUSCH demodulation requirement in FR1 and FR2-1 as referred in Table: 8.2.1.1-1/ Table: 8.2.2.1-1 and Table 11.2.2.1.1-1/ Table 11.2.2.2.1-1 in spec 38.104, respectively - Specification Impact - Option 1: New sub-clause for demodulation requirements under same chapter with RF requirements can be considered. If RAN4 agree Not to introduce new BS demodulation requirements for SBFD, only applicability rule can be captured. - Other options are not precluded

In this contribution, the view on test scope and test setup for BS demodulation requirement with supporting SBFD operation is provided. Meanwhile, the initial simulation results are provided for feasibility evaluation with UL muting

# 2	Simulation Results

In this section, the initial simulation results for UL muting were provided based on agreed simulation assumption. The simulation assumption can be refereed table 1

| Parameter |  |  | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interfer 1 | Interfer 2 |
| Transform precoding |  |  | Disabled |  |  |
| Example TDD UL-DL pattern (Note 1) |  |  | 30 kHz SCS: XXXXXXXXUU  UL sub-band transmission could start from the first F slots  Skip U slots to simply simulation setup |  |  |
| INR (Note3) |  | Set 1(Interf1 > Interf2) | N/A | Case 1: 5 Case 2: 9 | Case 1: -5 Case 2: -1 |
|  |  | Set 2 (Interf1 = Interf2) |  | Case 1: -1 Case 2: 3 | Case 1: -1 Case 2: 3 |
| Cell ID |  |  | 0 | 1 | 2 |
| Interference model |  |  | N/A |  |  |
| HARQ | Maximum number of HARQ transmissions |  | 4 | N/A | N/A |
|  | RV sequence |  | 0, 2, 3, 1 | N/A | N/A |
| DM-RS | DM-RS configuration type |  | 1 |  |  |
|  | DM-RS duration |  | single-symbol DM-RS |  |  |
|  | Additional DM-RS position |  | pos1 |  |  |
|  | Number of DM-RS CDM group(s) without data |  | 2 |  |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |  |  |
|  | DM-RS port(s) |  | {0} |  |  |
|  | DM-RS sequence generation |  | NID0=0, nSCID =0 | NID0=1, nSCID =0 | NID0=2, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type |  | A and B |  |  |
|  | Start symbol |  | 0 |  |  |
|  | Allocation length |  | 14 |  |  |
| Frequency domain resource assignment | RB assignment |  | Full applicable test bandwidth |  |  |
|  | Frequency hopping |  | Disabled |  |  |
| Code block group based PUSCH transmission |  |  | Disabled |  |  |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. NOTE 2: One explicit interferer, i.e., interferer 1, is modelled for tests with 2/4/8 RX antennas. Two explicit interferers are modelled for tests with 2/4/8 RX antennas. NOTE 3: INR is defined in Annex B.X.1. INR set 1 and set 2 are derived respectively in homogeneous and heterogeneous network scenarios. NOTE 4 All cells are time-synchronous. |  |  |  |  |  |

For UL symbol muting pattern, since the interference modeling is considered from neighboring gNB without considering the mobility, there is no performance difference excepted with the UL symbol muting pattern in different symbol location. In current performance evaluation, we consider the following UL symbol muting pattern to check the performance difference with and without using muting REs for interference covariance estimation.

UL symbol muting pattern

Figure 1: Muting symbol configuration with 2 symbols muting

Figure 2: Muting symbol configuration with 1 symbol muting

The simulation results were summarized in table 2

Table 2: Simulation results for PUSCH with UL symbol muting, 2Rx, Mapping A

| Tx | Rx | SCS | BW | Number of muting symbols | Mapping type | Channel model (target cell) | Channel model (interferer cells) | Set | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) without using muting REs | SNR@70% (MMSE-IRC) without using muting REs | gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 13.444 | 13.158 | 0.286 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 14.653 | 14.376 | 0.277 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 12.69 | 12.369 | 0.321 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 13.59 | 13.297 | 0.293 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 12.908 | 12.682 | 0.226 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 14.213 | 14.058 | 0.155 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 12.152 | 11.918 | 0.234 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 13.055 | 12.849 | 0.206 |

Table 3: Simulation results for PUSCH with UL symbol muting, 4Rx, Mapping A

| Tx | Rx | SCS | BW | Number of muting symbols | Mapping type | Channel model (target cell) | Channel model (interferer cells) | Set | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) without using muting REs | SNR@70% (MMSE-IRC) without using muting REs | gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 8.251 | 7.607 | 0.644 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 8.475 | 7.682 | 0.793 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 7.83 | 7.065 | 0.765 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 8.369 | 7.671 | 0.698 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 7.949 | 7.427 | 0.522 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 8.035 | 7.422 | 0.613 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 7.479 | 6.918 | 0.561 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 8.038 | 7.482 | 0.556 |

Table 4: Simulation results for PUSCH with UL symbol muting, 8Rx, Mapping A

| Tx | Rx | SCS | BW | Number of muting symbols | Mapping type | Channel model (target cell) | Channel model (interferer cells) | Set | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) without using muting REs | SNR@70% (MMSE-IRC) without using muting REs | gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 6.946 | 3.868 | 3.078 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 6.886 | 4.047 | 2.839 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 7.035 | 3.908 | 3.127 |
| 1 | 2 | 30KHz | 20MHz | 2 (4,13) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 6.985 | 3.856 | 3.129 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 6.597 | 4.327 | 2.27 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 6.494 | 4.397 | 2.097 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 1 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 6.654 | 4.354 | 2.3 |
| 1 | 2 | 30KHz | 20MHz | 1 (4) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 2 | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 6.615 | 4.295 | 2.32 |

Table 5: Simulation results for PUSCH with UL symbol muting, 2Rx, Mapping B

| Tx | Rx | SCS | BW | Number of muting symbols | Mapping type | Channel model (target cell) | Channel model (interferer cells) | Set | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) without using muting REs | SNR@70% (MMSE-IRC) without using muting REs | gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 30KHz | 20MHz | 2 (2,12) | B | TDLC300-100 | TDLC300-0 | Set 1 | Case 1 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 13.429 | 13.144 | 0.285 |
| 1 | 2 | 30KHz | 20MHz | 2 (2,12) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 2 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 14.589 | 14.3 | 0.289 |
| 1 | 2 | 30KHz | 20MHz | 2 (2,12) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 1 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 12.714 | 12.364 | 0.35 |
| 1 | 2 | 30KHz | 20MHz | 2 (2,12) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 2 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 13.579 | 13.267 | 0.312 |
| 1 | 2 | 30KHz | 20MHz | 1 (2) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 1 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 12.909 | 12.685 | 0.224 |
| 1 | 2 | 30KHz | 20MHz | 1 (2) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 2 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 14.158 | 13.971 | 0.187 |
| 1 | 2 | 30KHz | 20MHz | 1 (2) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 1 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 12.19 | 11.984 | 0.206 |
| 1 | 2 | 30KHz | 20MHz | 1 (2) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 2 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 13.059 | 12.866 | 0.193 |

Table 6: Simulation results for PUSCH with UL symbol muting, 4Rx, Mapping B

| Tx | Rx | SCS | BW | Number of muting symbols | Mapping type | Channel model (target cell) | Channel model (interferer cells) | Set | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) without using muting REs | SNR@70% (MMSE-IRC) without using muting REs | gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 30KHz | 20MHz | 2 (2,12) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 1 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 8.273 | 7.639 | 0.634 |
| 1 | 2 | 30KHz | 20MHz | 2 (2,12) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 2 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 8.526 | 7.695 | 0.831 |
| 1 | 2 | 30KHz | 20MHz | 2 (2,12) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 1 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 7.847 | 7.06 | 0.787 |
| 1 | 2 | 30KHz | 20MHz | 2 (2,12) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 2 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 8.387 | 7.745 | 0.642 |
| 1 | 2 | 30KHz | 20MHz | 1 (2) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 1 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 8.029 | 7.526 | 0.503 |
| 1 | 2 | 30KHz | 20MHz | 1 (2) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 2 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 8.091 | 7.465 | 0.626 |
| 1 | 2 | 30KHz | 20MHz | 1 (2) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 1 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 7.541 | 6.972 | 0.569 |
| 1 | 2 | 30KHz | 20MHz | 1 (2) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 2 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 8.151 | 7.626 | 0.525 |

Table 7: Simulation results for PUSCH with UL symbol muting, 8Rx, Mapping B

| Tx | Rx | SCS | BW | Number of muting symbols | Mapping type | Channel model (target cell) | Channel model (interferer cells) | Set | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) without using muting REs | SNR@70% (MMSE-IRC) without using muting REs | gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 30KHz | 20MHz | 2 (2,12) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 1 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 7.011 | 3.874 | 3.137 |
| 1 | 2 | 30KHz | 20MHz | 2 (2,12) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 2 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 6.87 | 3.995 | 2.875 |
| 1 | 2 | 30KHz | 20MHz | 2 (2,12) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 1 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 7.126 | 3.987 | 3.139 |
| 1 | 2 | 30KHz | 20MHz | 2 (2,12) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 2 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 7.034 | 3.857 | 3.177 |
| 1 | 2 | 30KHz | 20MHz | 1 (2) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 1 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 6.718 | 4.399 | 2.319 |
| 1 | 2 | 30KHz | 20MHz | 1 (2) | A | TDLC300-100 | TDLC300-0 | Set 1 | Case 2 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 6.536 | 4.396 | 2.14 |
| 1 | 2 | 30KHz | 20MHz | 1 (2) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 1 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 6.82 | 4.477 | 2.343 |
| 1 | 2 | 30KHz | 20MHz | 1 (2) | A | TDLC300-100 | TDLC300-0 | Set 2 | Case 2 | 2 | 1+1 (0,10) | MCS 16 | 16QAM | 6.747 | 4.376 | 2.371 |

Based our evaluation, the following observations can be drawn as

- For 2Rx, all the cases evaluated based on the candidate INR values, at least around 0.15dB gain with using muting REs for interference covariance estimation achieved. Around 0.3 dB gain for 2 symbols muting cases

- For 4Rx, all the cases evaluated based on the candidate INR values, at least around 0.5dB gain with using muting REs for interference covariance estimation achieved. Around 0.7-0.8 dB gain for 2 symbols muting cases

- For 8Rx, all the cases evaluated based on the candidate INR values, at least around 2dB gain with using muting REs for interference covariance estimation achieved. Around 3dB gain for 2 symbols muting cases

- Similar performance and gain achieved for both mapping type A and B

- Similar performance and gain achieved for two INR values

Based on our evaluation, we think the feasibility of UL symbol muting has already been verified. Therefore, we propose to introduce PUSCH requirement with UL symbol muting.

Observation 1: For 2Rx, all the cases evaluated based on the candidate INR values, at least around 0.15dB gain with using muting REs for interference covariance estimation achieved. Around 0.3 dB gain for 2 symbols muting cases

Observation 2: For 2Rx, all the cases evaluated based on the candidate INR values, at least around 0.15dB gain with using muting REs for interference covariance estimation achieved. Around 0.3 dB gain for 2 symbols muting cases

Observation 3: For 8Rx, all the cases evaluated based on the candidate INR values, at least around 2dB gain with using muting REs for interference covariance estimation achieved. Around 3dB gain for 2 symbols muting cases

Observation 4: Similar performance and gain achieved for both mapping type A and B

Observation 5: Similar performance and gain achieved for two INR values

Proposal 1: RAN4 introduce the PUSCH requirement with UL symbols muting for SBFD-capable BS

# 3	Discussion

Based on evaluation, it is feasible to define PUSCH requirement with UL symbol muting. Regarding the test setup for requirement, the following simulation assumptions can be considered.

3.1	Test setup

Frequency Rang

Cover both FR1 and FR2-1 if agreed to introduce PUSCH demodulation requirement with UL muting

Waveform for wanted signal

Cover both CP-OFDM and DFT-s-OFDM waveforms if agreed to introduce PUSCH demodulation requirement with UL muting

sub-band pattern

Considering sub-band configuration as {DU}, where the UL sub-band is located in the lower band

Channel Bandwidth for wanted signal

For FR1: 40MHz CBW (20MHz DL sub-band+ 20MHz UL sub-band) for 30kHz SCS

For FR2-1: 200MHz CBW (100MHz DL sub-band+ 100MHz UL sub-band) for 120kHz SCS

TDD patterns

30 kHz SCS:  XXXXXXXXUU

120 kHz SCS: XXXXU

UL sub-band transmission could start from the first F slots

Skip U slots to simply simulation setup

Sub-band configuration in SBFD slot

Take SBFD UL sub-band spans all the symbols with a SBFD slot

Number of symbol muting

Consider 2 symbols muting configuration

Muting symbol configuration

FR1

FR2

PUSCH mapping type for wanted signal and interferers

Both mapping A and mapping B in FR1

Mapping B in FR

Number of DMRS symbols for wanted signal and interfers

Configure 2 DMRS symbols for defining SBFD PUSCH demodulation requirement in FR1 and FR2-1

MCS and Rank

MCS16 for wanted signal in FR1 and FR2

16QAM for interference signal in FR1 and FR2

Rank 1 for wanted signal and interference signal

Antenna Configuration

Reuse the same antenna configuration assumption for evaluation SBFD PUSCH performance with UL muting as 1Tx/2Rx/4Rx/8Rx in FR1 and 1Tx/2Rx in FR2-1

Other Assumption

Reuse the same assumption for specifying SBFD PUSCH demodulation requirement in FR1 and FR2-1 as referred in Table: 8.2.1.1-1/ Table: 8.2.2.1-1 and Table 11.2.2.1.1-1/ Table 11.2.2.2.1-1, respectively.

Channel model

In current evaluation, TDLC channel was used for both target cell and interfered cells in FR1. For FR2, TDLA30-75 and TDLA30-0 can be considered as channel model for target cell and interfered cells, respectively.

Proposal 2: RAN4 introduce the PUSCH requirement with UL symbols muting with the following assumption

Frequency Rang

- Cover both FR1 and FR2-1 if agreed to introduce PUSCH demodulation requirement with UL muting

Waveform for wanted signal

- Cover both CP-OFDM and DFT-s-OFDM waveforms if agreed to introduce PUSCH demodulation requirement with UL muting

sub-band pattern

- Considering sub-band configuration as {DU}, wherethe UL sub-band is located in the lower band

Channel Bandwidth for wanted signal

- For FR1: 40MHz CBW (20MHz DL sub-band+ 20MHz UL sub-band) for 30kHz SCS

- For FR2-1: 200MHz CBW (100MHz DL sub-band+ 100MHz UL sub-band) for 120kHz SCS

TDD patterns

- 30 kHz SCS:  XXXXXXXXUU

- 120 kHz SCS: XXXXU

- UL sub-band transmission could start from the first F slot

- Skip U slots to simply simulation setup

Sub-band configuration in SBFD slot

- Take SBFD UL sub-band spans all the symbols with a SBFD slot

Number of symbol muting

- Consider 2 symbols muting configuration

Muting symbol configuration

- FR1

- FR2

PUSCH mapping type for wanted signal and interferers

- Both mapping A and mapping B in FR1

- Mapping B in FR

Number of DMRS symbols for wanted signal and interferes

- Configure 2 DMRS symbols for defining SBFD PUSCH demodulation requirement in FR1 and FR2-1

MCS and Rank

- MCS16 for wanted signal in FR1 and FR2

- 16QAM for interference signal in FR1 and FR2

- Rank 1 for wanted signal and interference signal

Antenna Configuration

- Reuse the same antenna configuration assumption for evaluation SBFD PUSCH performance with UL muting as 1Tx/2Rx/4Rx/8Rx in FR1 and 1Tx/2Rx in FR2-1

Channel model

- TDLC300-100 for wanted signal in FR1, TDLA30-75 for wanted signal in FR2

- TDLC300-0 for interference signal in FR1

- TDLA30-0 for interference signal in FR2

Other Assumption

- Reuse the same assumption for specifying SBFD PUSCH demodulation requirement in FR1 and FR2-1 as referred in Table: 8.2.1.1-1/ Table: 8.2.2.1-1 and Table 11.2.2.1.1-1/ Table 11.2.2.2.1-1, respectively.

3.2	Interference model for FR2

In current evaluation, we mainly focus on FR1. For FR2, we could also apply the similar approach with considering simple link budget considering the worse cases, i.e., boresight pointing from the aggressing gNB

To align with RF requirement, for interfering signal for FR2 and coupling loss, the following was agreed

Agreement: - On the level of in-channel adjacent subband blocking requirement, - For FR2-1 WA BS: - the way to define the requirement is similar to FR1, i.e., by considering coupling loss between co-site sectors: - Absolute value is -52dBm Agreement: - Adopt the following assumption in the specification, and the detailed wording can be further refined: - For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dBc coupling loss between interfering BS transmitter and SBFD BS receiver. - For FR2-1 WA BS, in-channel adjacent subband blocking requirement assumes 87 dBc coupling loss between interfering BS transmitter and SBFD BS receiver.

Proposal 3: Considering the RF requirement on in-channel adjacent subband blocking requirement and coupling loss, to derive the INR with simple link budget approach.

3.3	Specification Impact

According to current spec of TS 38.104, new section s12.5 and 12.7 are agreed to be introduced for performance requirements of SBFD-capable BS, considering SBFD is a new capability for BS which could depend on declaration.

Based on previous discussion, RAN4 has agreed there is no new PUCCH and PRACH requirement introduced for SBFD-capable BS. Then, the legacy BS demodulation requirement can be reused for tests. Appling the similar approach of RF requirement, RAN4 should add the new applicability rule for SBFD -capable BS to indicate the requirements.

And related with PUSCH requirement, besides the legacy requirement, the applicability should be also added for PUSCH requirement with UL muting for SBFD-capable BS pending on declaration.

In our view, the basic cases defined in Rel-15 should be applied, other cases pending on manufactory declaration as following

For PUSCH

- PUSCH requirement tests in clause 8.2.1, 8.2.2 and 8.2.3 for FR1, and 11.2.2.1, 11.2.2.2, 11.2.2.3 for FR2  shall apply for SBFD-capable BS

- PUSCH requirement with UL muting shall apply only for SBFD-capable BS declared to be supported.

For PUCCH

- PUCCH requirement tests in clauses 8.3.1 to 8.3.6 for FR1, and 11.3.1.1 to 11.3.1.6 shall apply only for the BS declared to be supported

For PRACH

- PRACH requirement tests in clauses 8.4.2.2 for FR1, and 11.4.2.2.2 shall apply only for the BS declared to be supported

Proposal 4: RAN4 should add new applicability rule for SBFD-capable BS to indicate which legacy BS deamidation requirements can be applied.

For PUSCH

- PUSCH requirement tests in clause 8.2.1, 8.2.2 and 8.2.3 for FR1, and 11.2.2.1, 11.2.2.2, 11.2.2.3 for FR2  shall apply for SBFD-capable BS

- PUSCH requirement with UL muting shall apply only for SBFD-capable BS declared to be supported

For PUCCH

- PUCCH requirement tests in clauses 8.3.1 to 8.3.6 for FR1, and 11.3.1.1 to 11.3.1.6 shall apply only for the BS declared to be supported

For PRACH

- PRACH requirement tests in clauses 8.4.2.2 for FR1, and 11.4.2.2.2 shall apply only for the BS declared to be supported

# 4	Conclusion

In this contribution, the view on the demodulation scope for SBFD is provided. And initial results for PUSCH with UL muting are provided for analysis.

Observation 1: For 2Rx, all the cases evaluated based on the candidate INR values, at least around 0.15dB gain with using muting REs for interference covariance estimation achieved. Around 0.3 dB gain for 2 symbols muting cases

Observation 2: For 2Rx, all the cases evaluated based on the candidate INR values, at least around 0.15dB gain with using muting REs for interference covariance estimation achieved. Around 0.3 dB gain for 2 symbols muting cases

Observation 3: For 8Rx, all the cases evaluated based on the candidate INR values, at least around 2dB gain with using muting REs for interference covariance estimation achieved. Around 3dB gain for 2 symbols muting cases

Observation 4: Similar performance and gain achieved for both mapping type A and B

Observation 5: Similar performance and gain achieved for two INR values

Proposal 1: RAN4 introduce the PUSCH requirement with UL symbols muting for SBFD-capable BS

Proposal 2: RAN4 introduce the PUSCH requirement with UL symbols muting with the following assumption

Frequency Rang

- Cover both FR1 and FR2-1 if agreed to introduce PUSCH demodulation requirement with UL muting

Waveform for wanted signal

- Cover both CP-OFDM and DFT-s-OFDM waveforms if agreed to introduce PUSCH demodulation requirement with UL muting

sub-band pattern

- Considering sub-band configuration as {DU}, wherethe UL sub-band is located in the lower band

Channel Bandwidth for wanted signal

- For FR1: 40MHz CBW (20MHz DL sub-band+ 20MHz UL sub-band) for 30kHz SCS

- For FR2-1: 200MHz CBW (100MHz DL sub-band+ 100MHz UL sub-band) for 120kHz SCS

TDD patterns

- 30 kHz SCS:  XXXXXXXXUU

- 120 kHz SCS: XXXXU

- UL sub-band transmission could start from the first F slot

- Skip U slots to simply simulation setup

Sub-band configuration in SBFD slot

- Take SBFD UL sub-band spans all the symbols with a SBFD slot

Number of symbol muting

- Consider 2 symbols muting configuration

Muting symbol configuration

- FR1

- FR2

PUSCH mapping type for wanted signal and interferers

- Both mapping A and mapping B in FR1

- Mapping B in FR

Number of DMRS symbols for wanted signal and interferes

- Configure 2 DMRS symbols for defining SBFD PUSCH demodulation requirement in FR1 and FR2-1

MCS and Rank

- MCS16 for wanted signal in FR1 and FR2

- 16QAM for interference signal in FR1 and FR2

- Rank 1 for wanted signal and interference signal

Antenna Configuration

- Reuse the same antenna configuration assumption for evaluation SBFD PUSCH performance with UL muting as 1Tx/2Rx/4Rx/8Rx in FR1 and 1Tx/2Rx in FR2-1

Channel model

- TDLC300-100 for wanted signal in FR1, TDLA30-75 for wanted signal in FR2

- TDLC300-0 for interference signal in FR1

- TDLA30-0 for interference signal in FR2

Other Assumption

- Reuse the same assumption for specifying SBFD PUSCH demodulation requirement in FR1 and FR2-1 as referred in Table: 8.2.1.1-1/ Table: 8.2.2.1-1 and Table 11.2.2.1.1-1/ Table 11.2.2.2.1-1, respectively.

Proposal 3: Considering the RF requirement on in-channel adjacent subband blocking requirement and coupling loss, to derive the INR with simple link budget approach.

Proposal 4: RAN4 should add new applicability rule for SBFD-capable BS to indicate which legacy BS deamidation requirements can be applied.

For PUSCH

- PUSCH requirement tests in clause 8.2.1, 8.2.2 and 8.2.3 for FR1, and 11.2.2.1, 11.2.2.2, 11.2.2.3 for FR2  shall apply for SBFD-capable BS

- PUSCH requirement with UL muting shall apply only for SBFD-capable BS declared to be supported

For PUCCH

- PUCCH requirement tests in clauses 8.3.1 to 8.3.6 for FR1, and 11.3.1.1 to 11.3.1.6 shall apply only for the BS declared to be supported

For PRACH

- PRACH requirement tests in clauses 8.4.2.2 for FR1, and 11.4.2.2.2 shall apply only for the BS declared to be supported

# References

RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, HiSilicon

R4-2508723, Way Forward for [115][330] NR_duplex_evo_demod, Samsung