3GPP TSG-RAN WG4 Meeting #117		R4-2521512
Dallas, USA, Nov. 17-21, 2025

Title:	R17 n104 MPR update for 26dB ACLR

Source:	OPPO

Agenda item:	10.2.2

Document for:	Approval

Introduction

In Rel-17 maintenance, it was agreed that the MPR for n104 will be reviewed with band specific ACLR = 26dB [1]. This paper shares our simulation results and views on this aspect.

Topic 1: Scope Way forward/Agreement - RAN4 introduces a band specific ACLR of 26 dB to n104 with a dedicated MPR under the following conditions. - PC3 for n104 and CA_n104C and Band Combinations (BCs) including n104 - 1Tx and 2Tx are considered Topic 2: technical aspects Sub-topic 2-1: How to introduce the ACLR Way forward/Agreement Below approaches can be further discussed - FFS on if a band specific ACLR for n104 and CA_n104C is treated with NS signaling like other bands or not. - Other approaches are not precluded Sub-topic 2-2: MSD Way forward/Agreement - FFS on MSD for a new MSD value for CA_n78-n104 with respect to cross band isolation Topic 3: Procedure Way forward/Agreement - This topic is treated under Release 17 maintenance as correction

Discussion

N104 MPR results

To analyze the MPR reduction with the relaxation of ACLR and keep all other PC3 conditions/requirements unchanged. We have done some simulations by using the current PC3 PA. The results can be found in Annex.

It should be noticed that in the simulation we didn’t change the current PC3 PA calibration point, i.e., it still meets current 30dB ACLR with 1dB MPR considering UE usually uses same PA to cover several bands and the main purpose of relaxing this ACLR is to increase UE Tx power.

However, if PA is calibrated specifically for this n104 with 26dB ACLR, then the calibrated PA will be less linear and will cause larger MPR. How to align the understanding may need more discussion.

PA calibration point may need to be aligned for n104 with either current 30dB ACLR or newly introduced 26dB ACLR.

With the simulation results in annex for 1Tx PC3, below table 1 summarized the delta MPR between 30dB ACLR and 26dB ACLR.

Table 1 MPR improvement with 26dB ACLR

| MPR (30dB ACLR) – MPR (26dB ACLR) round up to 0.5 |  |  |  |
| --- | --- | --- | --- |
| DFT | Edge | Outer | Inner |
| Pi/2 bpsk | 0 | 0 | 0 |
| qpsk | 0.5 | 0.5 | 0 |
| 16qam | 0.5 | 0.5 | 0.5 |
| 64qam | 0.5 | 0.5 | 0.5 |
| 256qam | 0 | 0 | 0 |
| CP | Edge | Outer | Inner |  |
| qpsk | 0.5 | 0.5 | 0 |  |
| 16qam | 0 | 0.5 | 0 |  |
| 64qam | 0 | 0 | 0 |  |
| 256qam | 0 | 0 | 0 |  |

The MPR is improved by 0.5dB for the yellow highlighted configurations for 26dB ACLR in n104.

How to introduce the ACLR

Regarding how to introduce the ACLR and corresponding MPR requirements, in our view, this can be treated as a special case and defined in MPR/ACLR section. Currently, there is band specific MPR requirements in the spec like the band n14.

Besides, the ACLR is an important requirement, if define with NS signaling approach, then all NW will need to broadcast this NS value which seems not efficient way. And in case the NW doesn’t indicate this NS value, UE will mandate to meet the legacy MPR requirements which means in implementation UE has to make sure to meet the legacy and new ACLR requirements in n104. This will be also an unnecessary burden from UE side.

26dB ACLR is baseline for n104, if define with NS signaling approach it becomes optional and will cause unnecessary burden from both UE and NW side:

- All NW need to broadcast this signaling and unexpected mistake of not broadcasting it may happen.

- In this case, UE has to implement both the 26dB and 30dB ACLR which is also unnecessary.

Introduce 26dB for n104 as band specific requirement in ACLR and MPR section instead of NS signaling approach.

Conclusion

This paper discussed the ACLR relaxation for band n104, and got below observations and proposals based on the MPR simulation results.

Observation 1: 	PA calibration point may need to be aligned for n104 with either current 30dB ACLR or newly introduced 26dB ACLR.

Proposal 1: 	The MPR is improved by 0.5dB for the yellow highlighted configurations for 26dB ACLR in n104.

Table 1 MPR improvement with 26dB ACLR

| MPR (30dB ACLR) – MPR (26dB ACLR) round up to 0.5 |  |  |  |
| --- | --- | --- | --- |
| DFT | Edge | Outer | Inner |
| Pi/2 bpsk | 0 | 0 | 0 |
| qpsk | 0.5 | 0.5 | 0 |
| 16qam | 0.5 | 0.5 | 0.5 |
| 64qam | 0.5 | 0.5 | 0.5 |
| 256qam | 0 | 0 | 0 |
| CP | Edge | Outer | Inner |  |
| qpsk | 0.5 | 0.5 | 0 |  |
| 16qam | 0 | 0.5 | 0 |  |
| 64qam | 0 | 0 | 0 |  |
| 256qam | 0 | 0 | 0 |  |

Observation 2: 	26dB ACLR is baseline for n104, if define with NS signaling approach it becomes optional and will cause unnecessary burden from both UE and NW side:

- All NW need to broadcast this signaling and unexpected mistake of not broadcasting it may happen.

- In this case, UE has to implement both the 26dB and 30dB ACLR which is also unnecessary.

Proposal 2: 	Introduce 26dB for n104 as band specific requirement in ACLR and MPR section instead of NS signaling approach.

Reference

R4-2511799 Way forward for MPR reductio for n104, Huawei, OPPO, Samsung, CATT, RAN4#106, 2025-08

Annex: MPR results for 26dB ACLR

- CP-OFDM with 26dB ACLR

- DFT-S-OFDM with 26dB ACLR

Annex: MPR results for 30dB ACLR

- CP-OFDM with 26dB ACLR

- DFT-S-OFDM with 26dB ACLR