3GPP TSG-RAN WG4 Meeting #117		R4-2521571
Dallas, USA, 17 – 21 Nov. 2025

Title:	Views on 6G sensing

Source:	OPPO

Agenda item:	8.10

Document for:	Approval

Introduction

It was agreed to keep 6G sensing agenda in RAN4 in RAN4 #116bis meeting [1].

This paper shares our view on this aspect.

Discussion

Regulatory status

Regarding the regulations of ITU's radio service, cellular communication belongs to the category of IMT, while sensing belongs to the category of radar services. And under the current ITU spectrum allocation and utilization rules, IMT services and radar services will be allocated in different frequencies to carry out corresponding services. In other words, currently, there is no permitted frequencies for both cellular communication services and radar/sensing services worldwide from ITU’s perspective. Compared to the less optimistic status of ITU, some countries and regions show their interests in integrated sensing and communication (ISAC), and are positively considering to launch the ISAC as a licensed service.

In ITU, sensing belongs to radar service which is different from IMT services in spectrum allocation.

Currently in ITU, there is no permitted frequencies for both cellular communication services and radar/sensing services worldwide. How to handle the sensing service in IMT spectrum is unclear.

FFS on the available spectrum for sensing service in IMT spectrum based on regulatory status in ITU and region specific requirements.

Sensing mode

As RAN1 and RAN2 have not started the discussion on sensing/ISAC topic, all the sensing modes are still on the table from RAN4 study’s perspective, including monostatic (TRP monostatic, UE monostatic), bistatic (TRP-TRP bistatic, TRP-UE bistatic, UE-TRP bistatic, and UE-UE bistatic) and multi-static. Compared to base station/TRP, there are more challenges of implementation from UE side because of limited PCB layout and antenna volume, differentiated RF front-end components, restricted power consumption and so on. Therefore, UE related sensing modes should be considered with more attention.

For 6G sensing, UE based sensing modes and feasibility/limitations should be carefully studied in RAN4.

Conclusion

This paper shares our views on 6G sensing.

- In ITU, sensing belongs to radar service which is different from IMT services in spectrum allocation.

- Currently in ITU, there is no permitted frequencies for both cellular communication services and radar/sensing services worldwide. How to handle the sensing service in IMT spectrum is unclear.

- FFS on the available spectrum for sensing service in IMT spectrum based on regulatory status in ITU and region specific requirements.

For 6G sensing, UE based sensing modes and feasibility/limitations should be carefully studied in RAN4.

Reference

R4-2514592 WF on 6G Sensing, ZTE Corporation