3GPP TSG-RAN WG4 Meeting #117	R4-2521579

Dallas, USA, 17th – 21st November, 2025

Agenda item:		6.13.2.2

Source:	Nokia

Title:	On existing BS RF TX requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#116bis [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the BS RF requirements for SBFD operation. The core part agreements are needed to be implemented to the performance part of the specification. Below is Nokia’s view of the existing BS RF TX requirements for SBFD operation.

# Discussion

## Work-split for CRs drafting

The following work split was agreed during RAN4#116bis regarding the CRs drafting:

| Content for both 38.141-1 and 38.141-2 | Current/ New clauses | Responsible company |
| --- | --- | --- |
| References Defintions, symbols and abbreviations Operating bands and channel arrangement | 2, 3, 5 | CATT |
| General conducted and radiated test conditions and declarations | 4 | Ericsson |
| Conducted and Radiated Transmitter characteristics - General - Base station output power - Output power dynamics - Transmit ON/ OFF power | Old clauses: 6.1 to 6.4 New potential clauses to align how it was done with Core specs: 9.1 to 9.4 | Samsung |
| Conducted and Radiated Transmitter characteristics - Transmitted signal quality - Unwanted emissions - Transmitted intermodulation | Old clauses: 6.5 to 6.7 New potential clauses to align how it was done with Core specs: 9.5 to 9.7 | Nokia |
| Conducted and Radiated Receiver characteristics - General - Reference sensitivity level - Dynamic range - In-band selectivity and blocking | Old clauses: 6.1 to 6.4 New potential clauses to align how it was done with Core specs: 9.1 to 9.4 | Huawei |
| Conducted and Radiated Receiver characteristics - Out of band blocking - Receiver spurious emissions - Receiver intermodulation - In-channel selectivity | Old clauses: 6.5 to 6.8 New potential clauses to align how it was done with Core specs: 9.5 to 9.8 | ZTE |
| Big CR 38.141-1 and 38.141-2 |  | Moderator (Ericsson) |

The list above is missing the transient period between SBFD and non-SBFD operation, which needs to be introduced to the conformance specifications aligned with the core specification. Below one can find what has been currently agreed for the core specification for conducted transient period between SBFD and non-SBFD operation:

### 12.2.8	Transient period between SBFD and non-SBFD operation

#### 12.2.8.1	General

Transmitter transient period between SBFD and non-SBFD operation requirements apply only to SBFD-capable BS.

The transmitter transient period between SBFD and non-SBFD operation is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period between SBFD and non-SBFD operation is illustrated in figure 12.2.8.1-1.

Transmitter output power frequency regions for transition

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transient period between

SBFD and non-SBFD

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 12.2.8.1-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period between SBFD and non-SBFD operation

The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases:

Table 12.2.8.1-1: Transmitter transient period between SBFD and non-SBFD operation

|  | Frequency regions for transition | BS transmitter behavior during transition |
| --- | --- | --- |
| SBFD symbol to DL symbol | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| SBFD symbol to UL symbol | SBFD DL subband | BS transmitter ON-to-OFF |
| DL symbol to SBFD symbol | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| UL symbol to SBFD symbol | SBFD DL subband | BS transmitter OFF-to-ON |

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

#### 12.2.8.2	Minimum requirement for BS type 1-H

For BS type 1-H, the transmitter transient period between SBFD and non-SBFD operation shall be shorter than the values listed in the minimum requirement table 12.2.8.2-1.

Table 12.2.8.2-1: Minimum requirement for the transmitter transient period between SBFD and non-SBFD operation for BS type 1-H

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |

Transient period between SBFD and non-SBFD operation needs to be included in transmitter test specifications.

## Manufacturer declarations

The following was agreed in RAN4#116bis:

Agreement: - Introduce new manufacture declarations related to SBFD capable BSs in TS 38.141-1 and TS 38.141-2 based on the work-split. - Remove multi-band operation from this release and align to Core specs. - Add the declarations in existing table in clause 4.6

The following was also agreed:

Agreement: - For configuration-limited SBFD-capable BS, all RF channels that implement analogue filters shall be tested and shall be declared together with subband configuration declaration. - FFS RF channels to be tested for configuration-flexible SBFD-capable BS.

The second agreement must also be included in the manufacturer declaration. This needs to be introduced to the transmission bandwidth configurations for SBFD DL subband(s) and UL subbands for SBFD operation. Also, another option could be via a new declaration and declaration identifier.

All RF channels that implement analogue filters shall be declared.

Include all RF channels that implement analogue filters in the manufacturer declarations.

# Conclusion

In this contribution, our views on the existing SBFD BS RF Tx requirements are presented. Our observations are listed below.

- All RF channels that implement analogue filters shall be declared.

- Transient period between SBFD and non-SBFD operation needs to be included in transmitter test specifications.

- Include all RF channels that implement analogue filters in the manufacturer declarations.

# References

- R4-2515063 “Way Forward for SBFD BS conformance”, RAN4#116bis, Prague, Czech, 13th – 17th October, 2025.