3GPP TSG-RAN WG4 Meeting #117	R4-2521585

# Dallas, USA, November 17th – 21st, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | 0750 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.104 for SBFD-capable BS RF requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correct typos, wrong title numbering and references to tables. Also alignment between different clauses to maintain unity within specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 12 for SBFD has corrections for typos, wrong title numbering and references to tables. TS is now more aligned between different clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SBFD-capable BS requirements contain errors and are unaligned. Wrong references and title numbering remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.3, 12.2.3.2, 12.2.8, 12.3.2, 12.3.5, 12.3.6, 12.5, 12.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 12.1.3	SBFD-capable BS Types and Configuration Constraints

#### 12.1.3.1 General

SBFD-capable BSs are categorized as either configuration-limited or configuration-flexible according to the classification defined in Table 12.1.3.1-1. The classification is based on BS class and rated output power specified in clause 6.2.1.

Table 12.1.3.1-1: SBFD-capable BS classification

| BS Class | Rated Output Power (Prated,c,sys) | SBFD BS type |
| --- | --- | --- |
| FR1 WA BS | -- | Configuration-limited |
| FR1 MR BS | ≥ 36 dBm | Configuration-limited |
| FR1 MR BS | < 36 dBm | Configuration-flexible |
| FR1 LA BS | -- | Configuration-flexible |
| FR2 WA BS | -- | Configuration-flexible |
| FR2 LA BS | -- | Configuration-flexible |
| NOTE 1: A power level threshold of Prated,c,sys = 36 dBm is used to distinguish between configuration-limited and configuration-flexible SBFD-capable BSs in the FR1 MR BS class. |  |  |

==============Next change==============

#### 12.2.3.2		RE power control dynamic range for SBFD

##### 12.2.3.2.1	General

The RE power control dynamic range for SBFD operation is the difference between the power of an RE and the average RE power for a BS at maximum output power (Pmax,c,AC or Pmax,c,TABC) during SBFD symbol for a specified reference condition.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

##### 12.2.3.2.2	Minimum requirement for BS type 1-H

The same requirement as clause 6.3.2.2 is applicable to RE power control dynamic range requirement for SBFD operation.

#### 12.2.3.3	Total power dynamic range for SBFD

##### 12.2.3.3.1	General

The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

NOTE 1:	The upper limit of the total power dynamic range for SBFD is the OFDM symbol power for a BS when transmitting on all DL RBs in transmission bandwidth configuration for SBFD at maximum output power. The lower limit of the total power dynamic range is the average power for single RB transmission as non-SBFD. The OFDM symbol shall carry PDSCH and not contain RS or SSB.

NOTE 2:	The requirement does not apply to operation with shared spectrum channel access.

##### 12.2.3.3.2	Minimum requirement for BS type 1-H

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log10(NRB,SBFD,DL) if one SBFD DL subband is configured, i.e., DU or UD pattern, and as 10log10(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured, i.e., DUD pattern.

==============Next change==============

### 12.2.8	Transient period between SBFD and non-SBFD operation

#### 12.2.8.1	General

Transmitter transient period between SBFD and non-SBFD operation requirements apply only to SBFD-capable BS.

The transmitter transient period between SBFD and non-SBFD operation is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period between SBFD and non-SBFD operation is illustrated in figure 12.2.8.1-1.

Transmitter output power frequency regions for transition

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transient period between

SBFD and non-SBFD

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 12.2.8.1-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period between SBFD and non-SBFD operation

The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases as defined in table 12.2.8.1-1:

Table 12.2.8.1-1: Transmitter transient period between SBFD and non-SBFD operation

|  | Frequency regions for transition | BS transmitter behavior during transition |
| --- | --- | --- |
| SBFD symbol to DL symbol | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| SBFD symbol to UL symbol | SBFD DL subband | BS transmitter ON-to-OFF |
| DL symbol to SBFD symbol | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| UL symbol to SBFD symbol | SBFD DL subband | BS transmitter OFF-to-ON |

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

#### 12.2.8.2	Minimum requirement for BS type 1-H

For BS type 1-H, the transmitter transient period between SBFD and non-SBFD operation shall be shorter than the values listed in the minimum requirement table 12.2.8.2-1.

Table 12.2.8.2-1: Minimum requirement for the transmitter transient period between SBFD and non-SBFD operation for BS type 1-H

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |

==============Next change==============

### 12.3.2	Reference sensitivity level

#### 12.3.2.1	General

The reference sensitivity power level is the minimum mean power received at the TAB connector for BS type 1-H inside the received SBFD UL subband for SBFD operation at which a throughput requirement shall be met for a specified reference measurement channel.

#### 12.3.2.2	Minimum requirements for BS type 1-H

The same requirements as clause 7.2.2 is applicable to reference sensitivity power level for SBFD operation, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

==============Next change==============

### 12.3.5	Out-of-band blocking for SBFD

#### 12.3.5.1 General

The general requirement of out-of-band blocking for SBFD-capable BS is the same as general requirement specified in clause 7.5.1.

#### 12.3.5.2 Minimum requirement for BS type 1-H

The same requirements as specified in clause 7.5.2 shall be applied. The PREFSENS depends on the SBFD UL subband as specified in clause 12.3.2.

12.3.5.3 Co-location minimum requirement for BS type 1-H

The same requirements as specified in clause 7.5.3 shall be applied. The PREFSENS depends on the SBFD UL subband as specified in clause 12.3.2.

==============Next change==============

### 12.3.6	Receiver spurious emissions for SBFD

#### 12.3.6.1	General

The general description given in clause 7.6.1 for BS type 1-H shall be applied.

#### 12.3.6.2	Minimum requirement for SBFD-capable BS type 1-H

The same requirements as specified in clause 7.6.4 shall be applied.

==============Next change==============

## 12.5 Radiated transmitter characteristics for SBFD-capable BS

### 12.5.1	General

Radiated transmitter characteristics requirements apply on the SBFD-capable BS type 1-H, BS type 1-O or BS type 2-O including all its functional components active and for SBFD operation.

### 12.5.2	Radiated transmit power for SBFD

#### 12.5.2.1	General

The BS radiated transmit power for SBFD operation is specified over SBFD symbol, in which the EIRP for SBFD symbol may be different from the EIRP for transmitter ON period as specified in clause 9.2.1.

#### 12.5.2.2	Minimum requirement for BS type 1-H and BS type 1-O

The same requirement as clause 9.2.2 is applicable to BS radiated transmit power requirement for SBFD operation.

#### 12.5.2.3	Minimum requirement for BS type 2-O

The same requirement as clause 9.2.3 is applicable to BS radiated transmit power requirement for SBFD operation.

### 12.5.3	OTA base station output power for SBFD

#### 12.5.3.1	General

The OTA base station output power for SBFD operation is specified over SBFD symbol, in which the TRP for SBFD symbol may be different from the TRP for transmitter ON period as specified in clause 9.3.1.

#### 12.5.3.2	Minimum requirement for BS type 1-O

The same requirement as clause 9.3.2 is applicable to OTA base station output power requirement for SBFD operation.

#### 12.5.3.3	Minimum requirement for BS type 2-O

The same requirement as clause 9.3.3 is applicable to OTA base station output power requirement for SBFD operation.

### 12.5.4	OTA output power dynamics for SBFD

#### 12.5.4.1	General

The requirements in clause 12.5.4 apply during SBFD symbol. Transmit signal quality (as specified in clause 12.5.6) shall be maintained for the output power dynamics requirements of this clause.

The OTA output power requirements are directional requirements and apply to the beam peak directions over the OTA peak directions set.

#### 12.5.4.2		OTA RE power control dynamic range for SBFD

##### 12.5.4.2.1	General

The RE power control dynamic range for SBFD operation is the difference between the power of an RE and the average RE power for a BS at maximum output power (Pmax,c,AC or Pmax,c,TABC) during SBFD symbol for a specified reference condition.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

##### 12.5.4.2.2	Minimum requirement for BS type 1-O

The OTA RE power control dynamic range for SBFD operation is specified the same as the conducted RE power control dynamic range requirement for BS type 1-H in clause 12.2.3.2.

#### 12.5.4.3	OTA total power dynamic range for SBFD

##### 12.5.4.3.1	General

The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

NOTE 1:	The upper limit of the OTA total power dynamic range for SBFD is the BS maximum carrier EIRP (Pmax,c,EIRP) when transmitting on all DL RBs in transmission bandwidth configuration for SBFD at maximum output power. The lower limit of the OTA total power dynamic range is the average EIRP for single RB transmission in the same direction using the same beam. The OFDM symbol carries PDSCH and not contain RS or SSB.

##### 12.5.4.3.2	Minimum requirement for BS type 1-O and BS type 2-O

The OTA total power dynamic range minimum requirement for each NR carrier shall be larger than or equal to the level calculated as 10log10(NRB,SBFD,DL) if one SBFD DL subband is configured, i.e., DU or UD pattern, and as 10log10(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured, i.e., DUD pattern.

### 12.5.5	OTA transmit ON/OFF power for SBFD

#### 12.5.5.1	OTA transmitter OFF power for SBFD

##### 12.5.5.1.1		General

The general description for OTA transmitter OFF power as clause 9.5 shall be applied.

##### 12.5.5.1.2		Minimum requirement for BS type 1-O and BS type 2-O

For the OTA transmitter OFF power requirements for non-SBFD symbols, the same requirement as specified in clause 9.5 shall be applied. Requirement is not applicable for SBFD symbols.

### 12.5.6	OTA transmitted signal quality for SBFD

#### 12.5.6.1	General

The general description for OTA transmitted signal quality as clause 9.6 shall be applied.

#### 12.5.6.2		Minimum requirement for BS type 1-O and BS type 2-O

For frequency error, modulation quality (EVM) and time alignment error (TAE), the same requirements as specified in clause 9.6 shall be applied.

### 12.5.7	OTA unwanted emissions for SBFD

#### 12.5.7.1	General

The general description for OTA unwanted emissions as clause 9.7 shall be applied.

#### 12.5.7.2	Minimum requirement for BS type 1-O and BS type 2-O

For OTA occupied bandwidth, OTA adjacent channel leakage power ratio (ACLR), OTA operating band unwanted emissions (OBUE), and OTA transmitter spurious emissions the same requirements as specified in clause 9.7 shall be applied.

Requirements related with multi-band and non-contiguous spectrum are not applied to SBFD.

### 12.5.8	OTA Transmitter intermodulation for SBFD

#### 12.5.8.1	General

The general description for OTA transmitter intermodulation as clause 9.8 shall be applied.

#### 12.5.8.2	Minimum requirements for BS type 1-O and BS type 2-O

For the OTA transmitter intermodulation requirement, the same requirement as specified in clause 9.8 shall be applied. Receiver requirement is not specified during the OTA transmitter intermodulation test.

### 12.5.9	OTA spatial emission for SBFD

#### 12.5.9.1	General

OTA spatial emission requirements apply to SBFD-capable BS during SBFD symbol.

#### 12.5.9.2	Protection of FSS UL

For SBFD-capable BS type 1-H and BS type 1-O operating in band n104 during SBFD symbol, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz, shall not exceed the limits specified in table 9.9.2.2-1.

### 12.5.10	OTA transient period between SBFD and non-SBFD operation

#### 12.5.10.1	General

The transmitter transient period between SBFD and non-SBFD operation is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period between SBFD and non-SBFD operation is illustrated in figure 12.5.10.1-1.

Transmitter output power frequency regions for transition

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transient period between

SBFD and non-SBFD

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 12.5.10.1-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period between SBFD and non-SBFD operation

The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases as defined in table 12.5.10.1-1:

Table 12.5.10.1-1: Transmitter transient period between SBFD and non-SBFD operation

|  | Frequency regions for transition | BS transmitter behavior during transition |
| --- | --- | --- |
| SBFD symbol to DL symbol | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| SBFD symbol to UL symbol | SBFD DL subband | BS transmitter ON-to-OFF |
| DL symbol to SBFD symbol | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| UL symbol to SBFD symbol | SBFD DL subband | BS transmitter OFF-to-ON |

This requirement shall be applied at each RIB supporting transmission in the operating band.

#### 12.5.10.2	Minimum requirement for BS type 1-O

For BS type 1-O, the OTA transmitter transient period between SBFD and non-SBFD operation shall be shorter than the values listed in the minimum requirement table 12.5.10.2-1.

Table 12.5.10.2-1: Minimum requirement for the OTA transmitter transient period between SBFD and non-SBFD operation for BS type 1-O

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |

#### 12.5.10.3	Minimum requirement for BS type 2-O

For BS type 2-O, the OTA transmitter transient period between SBFD and non-SBFD operation shall be shorter than the values listed in the minimum requirement table 12.5.10.3-1.

Table 12.5.10.3-1: Minimum requirement for the OTA transmitter transient period between SBFD and non-SBFD operation for BS type 2-O

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 3 |
| ON to OFF | 3 |

### 12.5.11	OTA In-channel adjacent subband leakage for SBFD

#### 12.5.11.1 General

OTA In-channel adjacent subband leakage (ICASL) is the filtered mean power centred on the declared frequency region of UL subband configuration within NR carrier. The measured power is TRP.

The requirement shall be applied per RIB during the transmitter ON period.

#### 12.5.11.2 Minimum requirement for BS type 1-O

The ICASL shall be less than or equal to a basic limit defined in table 12.2.9.2-1 + X (where X = 9 dB).

#### 12.5.11.3 Minimum requirement for BS type 2-O

The ICASL shall be less than or equal to the value specified in table 12.5.11.3-1.

Table 12.5.11.3-1: ICASL absolute basic limit

| BS category / BS class | ICASL absolute basic limit |
| --- | --- |
| Wide Area BS | -13 dBm/MHz |
| Medium Range BS | -20 dBm/MHz |
| Local Area BS | -20 dBm/MHz |

Table 12.5.11.3-2: Filter parameters for the UL subband

| RAT | Filter on the assigned UL subband and corresponding filter bandwidth |
| --- | --- |
| NR | Square (the declared bandwidth of UL subband) |

==============Next change==============

## 12.6 Radiated receiver characteristics for SBFD-capable BS

### 12.6.1	General

Radiated receiver characteristics are specified at RIB for BS type 1-H, BS type 1-O, or BS type 2-O, with full complement of transceivers configured for SBFD operation in normal operating condition.

### 12.6.2	OTA sensitivity for SBFD

#### 12.6.2.1	BS type 1-H and BS type 1-O

##### 12.6.2.1.1	General

The general description of OTA sensitivity requirement as clause 10.2.1.1 shall also be applied for SBFD operation.

##### 12.6.2.1.2	Minimum requirement

For a received signal whose AoA of the incident wave is within the active sensitivity RoAoA of an OSDD, the error rate criterion as described in clause 7.2 shall be met when the level of the arriving signal is equal to the minimum EIS level with 0.7dB degradation allowed in the respective declared set of EIS level and UL subband bandwidth, in SBFD symbols with the transmitted signals in SBFD DL subband(s) present.

### 12.6.3	OTA reference sensitivity level for SBFD

#### 12.6.3.1	General

The general description of OTA reference sensitivity requirement as clause 10.3.1 shall also be applied for SBFD operation.

#### 12.6.3.2	Minimum requirement for BS type 1-O

The OTA reference sensitivity requirement as specified in clause 10.3.2 with 0.7dB degradation allowed is applicable for SBFD operation in SBFD symbols with the transmitted signals in SBFD DL subband(s) present, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

#### 12.6.3.3	Minimum requirement for BS type 2-O

The OTA reference sensitivity requirement as specified in clause 10.3.3 with 0.7dB degradation allowed is applicable for SBFD operation in SBFD symbols with the transmitted signals in SBFD DL subband(s) present, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

### 12.6.4	OTA dynamic range for SBFD

#### 12.6.4.1	General

The general description of OTA dynamic range requirement as clause 10.4.1 shall also be applied for SBFD operation.

#### 12.6.4.2	Minimum requirement for BS type 1-O

The same requirement as clause 10.4.2 is applicable to dynamic range requirement for SBFD operation, in which the BS channel bandwidth specified in tables shall be replaced by SBFD UL subband bandwidth.

### 12.6.5	OTA in-band selectivity and blocking for SBFD

#### 12.6.5.1	OTA Adjacent Channel Selectivity for SBFD

##### 12.6.5.1.1	General

OTA Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive an OTA wanted signal at its assigned channel frequency in the presence of an OTA adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

NOTE: SBFD UL subband is not configured adjacent to the Radio Bandwidth edge where cross link interference from adjacent channel may appear.

##### 12.6.5.1.2	Minimum requirement for BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For SBFD operation, the OTA wanted and the interfering signal are specified in table 12.6.5.1.2-1 for OTA ACS. The reference measurement channel for the OTA wanted signal is further specified in annex A.1. The characteristic of the interfering signal is further specified in annex D.

The OTA ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

Table 12.6.5.1.2-1: OTA ACS requirement for BS type 1-O for SBFD

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB | Wide Area BS: -28 - ΔOTAREFSENS Medium Range BS: -32 - ΔOTAREFSENS Local Area BS: -35 - ΔOTAREFSENS | ±10 | 20 MHz CP-OFDM NR signal 30 kHz SCS, 51 RBs |
| NOTE 1: EISREFSENS is given in clause 12.6.3. NOTE 2: For lowest/highest carrier is configured in DU/UD SBFD operation, the interference only applies when DL subband is adjacent to Base Station RF Bandwidth edge. |  |  |  |  |

##### 12.6.5.1.3	Minimum requirement for BS type 2-O

The general minimum requirements are the same as those specified in clause 10.5.1.3, except for EISREFSENS. The EISREFSENS depends on the SBFD-capable BS class and the SBFD UL subband, and it is specified in clause 12.6.3.

#### 12.6.5.2	OTA in-band blocking for SBFD

##### 12.6.5.2.1	General

The OTA in-band blocking characteristics is a measure of the receiver's ability to receive an OTA wanted signal at its assigned channel in the presence of an unwanted OTA interferer, which is an NR signal for general blocking or an NR signal with one RB for narrowband blocking.

NOTE 1: For FR1 WA BS, the in-band blocking requirements are derived based on the simulation assumption of 4GHz with 288 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs. This NOTE shall not be interpreted as part of RF requirements.

NOTE 2: For FR1 MR BS, the in-band blocking requirements are derived based on the simulation assumption of 4GHz outdoor deployment with 167 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs. This NOTE shall not be interpreted as part of RF requirements or in the context of indoor.

NOTE 3: SBFD UL subband is not configured adjacent to the Radio Bandwidth edge where cross link interference from adjacent channel may appear.

##### 12.6.5.2.2	Minimum requirement for BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with OTA wanted and OTA interfering signal specified in tables 12.6.5.2.2-1 for general OTA blocking requirements. The reference measurement channel for the OTA wanted signal is further specified in annex A.1. The characteristic of the interfering signal is further specified in annex D.

The OTA in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

Table 12.6.5.2.2-1: General OTA blocking requirement for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB | Wide Area BS: -25 - ΔOTAREFSENS Medium Range BS: -32 - ΔOTAREFSENS Local Area BS: -35 - ΔOTAREFSENS | ±30 | 20 MHz CP-OFDM NR signal 30 kHz SCS, 51 RBs |
| NOTE 1: EISREFSENS is given in clause 12.6.3. NOTE 2: For lowest/highest carrier is configured in DU/UD SBFD operation, the interference only applies when DL subband is adjacent to Base Station RF Bandwidth edge. |  |  |  |  |

##### 12.6.5.2.2	Minimum requirement for BS type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For BS type 2-O, the OTA wanted and OTA interfering signals are provided at RIB using the parameters in table 12.6.5.2.2-1 for general OTA blocking requirements. The reference measurement channel for the wanted signal is further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The OTA blocking requirements are applicable outside the Base Station RF Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges.

Table 12.6.5.2.2-1: General OTA blocking requirement for BS type 2-O

| Frequency Range | BS channel bandwidth of the lowest/highest carrier received (MHz) | OTA wanted signal mean power (dBm) | OTA interfering signal mean power (dBm) | OTA interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of OTA interfering signal |
| --- | --- | --- | --- | --- | --- |
| FR2-1 | 50, 100, 200, 400 | EISREFSENS + 6 dB | -44 | ±75 | 50 MHz DFT-s-OFDM NR signal, 60 kHz SCS, 64 RBs |
| NOTE 1: EISREFSENS and EISREFSENS_50M are given in clause 12.6.3. NOTE 2: For lowest/highest carrier is configured in DU/UD SBFD operation, the interference only applies when DL subband is adjacent to Base Station RF Bandwidth edge. |  |  |  |  |  |

### 12.6.6	OTA out-of-band blocking for SBFD

#### 12.6.6.1 General

The general requirement of OTA out-of-band blocking for SBFDcapable BS is the same as general requirement specified in clause 10.6.1.

#### 12.6.6.2	Minimum requirement for BS type 1-O

##### 12.6.6.2.1	General minimum requirement

The general minimum requirements are the same as those specified in clause 10.6.2.1, except for EISminSENS. The EISminSENS depends on the SBFD-capable BS class and the SBFD UL subband, and it is specified in clause 12.6.2.

##### 12.6.6.2.2	Co-location minimum requirement

The co-location minimum requirements are the same as those specified in clause 10.6.2.2, except for EISminSENS. The EISminSENS depends on the SBFD-capable BS class and the SBFD UL subband, and it is specified in clause 12.6.2.

#### 12.6.6.3	Minimum requirement for BS type 2-O

##### 12.6.6.3.1	General minimum requirement

The general minimum requirements are the same as those specified in clause 10.6.3.1, except for EISREFSENS. The EISREFSENS depends on the SBFD-capable BS class and the SBFD UL subband, and it is specified in clause 12.6.3.

### 12.6.7	OTA receiver spurious emissions for SBFD

#### 12.6.7.1	General

The general description given in Clause 10.7.1 shall be applied.

#### 12.6.7.2	Minimum requirement for BS type 1-O

The same requirements as specified in Clause 10.7.2 shall be applied.

#### 12.6.7.3	Minimum requirement for BS type 2-O

The same requirements as specified in Clause 10.7.3 shall be applied.

### 12.6.8	OTA in-channel selectivity for SBFD

The same requirements as specified in clause 10.9 shall be applied to SBFD-capable BS type 1-O and BS type 2-O during SBFD symbol, except for the note in table 10.9.2-1, table 10.9.2-1a, table 10.9.2-2, table 10.9.2-2a, table 10.9.2-3 and table 10.9.2-3a, which shall be adjusted to: Wanted and interfering signal are placed adjacently around FC,SBFD,UL. The aggregated wanted and interferer signal shall be centred in the SBFD UL subband bandwidth of the wanted signal. For DU/UD pattern, place the wanted signal closer to the DL subband.

### 12.6.9	OTA in-channel adjacent subband blocking for SBFD

#### 12.6.9.1	General

OTA in-channel adjacent subband blocking requirement is a measure of the receiver’s ability to receive an OTA wanted signal at its assigned channel frequency in the presence of an OTA interfering signal from other sector(s) and/or site(s) within the same operator’s network, which is an NR signal.

#### 12.6.9.2	Minimum requirement for BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with an OTA wanted signal and OTA interfering signal(s) using the parameters specified in table 12.6.9.2-1. The reference measurement channel for the wanted signal is identified in clause 10.3.2 for each SBFD UL subband bandwidth and are further specified in annex A.1. The characteristic of the interfering signal(s) is further specified in annex D.

The OTA in-channel adjacent subband blocking requirement shall apply to SBFD UL subband bandwidth. The interfering signal(s) shall be positioned at the centre frequency of the SBFD DL subband bandwidth(s).

Table 12.6.9.2-1: In-channel adjacent subband blocking requirement for BS type 1-O

| SBFD UL subband bandwidth (MHz) | Wanted signal mean power (dBm) (NOTE 1) | Interfering signal mean power (dBm) (NOTE 5) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) （NOTE 2, 3, 4） |
| --- | --- | --- | --- | --- | --- |
| 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | EISREFSENS + 6 dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | FC,SBFD,DL | CP-OFDM NR signal 30 kHz SCS, BW = X (in the unit of RB) |
|  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL,2 | CP-OFDM NR signal 30 kHz SCS, BW1 = X1 and BW2 = X2 (in the unit of RB) |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2. NOTE 2: For DU or UD case, interfering signal is placed at FC,SBFD,DL. For DUD case, two interfering signals are placed at FC,SBFD,DL,1 and FC,SBFD,DL,2, respectively. NOTE 3: For DU or UD case, X is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL. For DUD case, X1 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,1 and X2 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,2. NOTE 4: In case the SBFD BS is configured with intra-band carrier aggregation, the set of interfering signals consists of the interfering signal(s) in this table and the signals from other intra-band contiguous carrier(s) operated in non-SBFD operation. The power spectral density of each interfering signal is set to be the same level. NOTE 5: For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dBc coupling loss between interfering BS transmitter and SBFD BS receiver. |  |  |  |  |  |

#### 12.6.9.3	Minimum requirement for BS type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with an OTA wanted signal and OTA interfering signal(s) using the parameters specified in table 12.6.9.3-1. The reference measurement channel for the wanted signal is further specified in annex A.1. The characteristic of the interfering signal(s) is further specified in annex D.

The OTA in-channel adjacent subband blocking requirement shall apply to SBFD UL subband bandwidth. The interfering signal(s) shall be positioned at the centre frequency of the SBFD DL subband bandwidth(s).

Table 12.6.9.3-1: In-channel adjacent subband blocking requirement for BS type 2-O

| Frequency Range | SBFD UL subband bandwidth (MHz) | OTA wanted signal mean power (dBm) (NOTE 1) | OTA interfering signal mean power (dBm) (NOTE 5) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) （NOTE 2, 3, 4） |
| --- | --- | --- | --- | --- | --- | --- |
| FR2-1 | 50, 100, 200 | EISREFSENS + 6 dB | Wide Area BS: -52 | DU or UD | FC,SBFD,DL | CP-OFDM NR signal 60 kHz SCS, BW = X (in the unit of RB) |
|  |  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL,2 | CP-OFDM NR signal 60 kHz SCS, BW1 = X1 and BW2 = X2 (in the unit of RB) |
| NOTE 1: EISREFSENS depends on the SBFD BS UL subband bandwidth as specified in clause 12.6.3. NOTE 2: For DU or UD case, interfering signal is placed at FC,SBFD,DL. For DUD case, two interfering signals are placed at FC,SBFD,DL,1 and FC,SBFD,DL,2, respectively. NOTE 3: For DU or UD case, X is the largest transmission bandwidth configuration as specified in table 5.3.2-2 that is not larger than NRB, SBFD, DL. For DUD case, X1 is the largest transmission bandwidth configuration as specified in table 5.3.2-2 that is not larger than NRB, SBFD, DL,1 and X2 is the largest transmission bandwidth configuration as specified in table 5.3.2-2 that is not larger than NRB, SBFD, DL,2. NOTE 4: In case the SBFD BS is configured with intra-band carrier aggregation, the set of interfering signals consists of the interfering signal(s) in this table and the signals from other intra-band contiguous carrier(s) operated in non-SBFD operation. The power spectral density of each interfering signal is set to be the same level. NOTE 5: For FR2-1 WA BS, in-channel adjacent subband blocking requirement assumes 87 dBc coupling loss between interfering BS transmitter and SBFD BS receiver. |  |  |  |  |  |  |

==============End of change==============