3GPP TSG-RAN WG4 Meeting #117	R4-2521695

Dallas, US, November 17th – 21st 2025

Agenda Item:	6.13.2.3

Source:	Ericsson

Title:	Impact on SBFD BS RF RX requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of impact on conformance testing of BS RF RX requirements for SBFD. Discussion on conformance testing of BS RF TX requirements for SBFD and new requirements for SBFD are presented in separate submissions [1-2].

# 2	Discussion

Conformance testing for both SBFD and non-SBFD slots

It is important to ensure that conformance testing is performed for both SBFD and non-SBFD slot. Even for requirements that remain unchanged, the transmitter and receiver operate under different conditions in SBFD, where simultaneous uplink and downlink transmission leads to different hardware behaviour. Therefore, separate conformance testing is necessary for both cases to capture any deviations in RF performance.

- The transmitter and receiver operate under different conditions in SBFD and non-SBFD slots, for which separate testing is necessary.

Conformance testing shall be performed separately for both SBFD and non-SBFD slots.

RF channels to be tested for configuration-flexible SBFD-capable BS

Agreement: - For configuration-limited SBFD-capable BS, all declared RF channels that implement analogue filters shall be tested. - FFS RF channels to be tested for configuration-flexible SBFD-capable BS.

For conformance testing of configuration-flexible SBFD-capable BS, conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are needed.

For conformance testing of SBFD slots, conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are needed.

# Conclusion

In this contribution, overview of impact on conformance testing of SBFD BS RF RX requirements is presented with the following observations and proposals.

Observation 1	The transmitter and receiver operate under different conditions in SBFD and non-SBFD slots, for which separate testing is necessary.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Conformance testing shall be performed separately for both SBFD and non-SBFD slots.

Proposal 2	For conformance testing of SBFD slots, conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are needed.

# References

R4-2521694, Impact on SBFD BS RF TX requirements, Ericsson

R4-2521693, New SBFD BS RF requirements, Ericsson