3GPP TSG-RAN4 Meeting #117	R4-2521734

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

Source:	Nokia

Title:	On removing support of segmented pre-compensation for IoT NTN TDD

Agenda item:	4.27.4

Document for:	Discussion

Introduction

In the plenary meeting RAN #105, a new WI for Rel-19 was approved to introduce a TDD frame operation for NTN [1]. The work item is designed to provide extended support of 3GPP NB-IoT features leveraging existing satellite systems deployed in Non-Geostationary Satellite Orbit (NGSO). It also aims at enabling low-complexity payloads for satellite operations that are non-reliant on a diplexer.

To achieve the aforementioned goals, the WID was designed to introduce an NTN-compatible TDD frame, that while based on minimum necessary changes on the NB-IoT NTN FDD specification provides a new approach to TDD periodicity.  RAN4 has analysed the potential impacts and specified the requirements for the new frame structure. The core part work has been finalized in RAN4#116, but in RAN4 #117, RAN4 has received a LS from RAN1 that modifies previous agreements related to the segmentation of repetition bursts in NTN [2]. In this document, we discuss the effects of RAN1 LS and how to update RAN4 specification.

Discussion

Contents of RAN1 LS

RAN1 has sent the LS in [2], that provides the following information related to RAN1 agreements on segment pre-compensation (the highlights are our own, whereas the red marks are part of the RAN1 LS):

For precompensation, from RAN1 perspective: - The UE may adjust its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - The UE may adjust its time/frequency pre-compensation at the beginning of an NPUSCH/NPRACH transmission (same behavior as Rel-18) - Segmented precompensation is not supported. - It is not supported to perform precompensation within the set of 8 consecutive uplink subframes other than at the beginning of an NPUSCH/NPRACH transmission - FFS: whether spec impact is in RAN1, RAN4 or both. NOTE: RAN1 may revisit this agreement if RAN4 reply LS shows concerns, including concerns on meeting the requirements without segmented precompensation

From RAN4 perspective, the agreements above exclude the possibility of UE to perform pre-compensation between segments of a repetition period within the set of 8 consecutive uplink subframes. However, RAN1 also states the UE may adjust the pre-compensation at the beginning of each set of 8 consecutive uplink subframes. The key difference with previous releases is that the pre-compensation gap is not needed since the UE is not transmitting immediately before the first subframe of the 8 consecutive uplink subframes.

RAN1 intention is to simplify the PHY procedure for NB-IoT UEs, as within a burst of 8 UL subframes (8 ms) there is no significant change on UE timing (caused by satellite and UE movements). Although there might be different views on this issue, RAN1 seems to consider that the gains of the extra allowed timing adjustment do not compensate the added complexity in deployment of IoT TDD UEs in NTN.

However, as the UL bursts are apart of each other by 90 ms, that would cause the time reference to slide significantly (the distance error would sum up to more than 500 m) between the set of uplink subframes. Therefore, the RAN1 agreement defines the UE may adjust the pre-compensation at the beginning of each set of consecutive uplink subframes, but without the need for the segmentation & pre-compensation gap, since there is no transmission prior to the set.

From RAN1 LS it is possible to confirm there is no need for a pre-compensation gap in IoT NTN TDD, but that UEs may change the pre-compensation prior to the set of 8 consecutive uplink subframes.

Based on this we propose the following:

Confirm to RAN1 that there is no issues with their agreements.

Besides this, the consequences of RAN1 agreements require RAN4 to update its specification. RAN4 Specification in TS 38.133 shall be updated and that means that:

- Include in TS 38.133 that the UE is expected to align its timing before the beginning of each UL burst.

- Remove from clause 7.20B any mention to segmented repetitions.

After that, RAN4 shall send the RAN4 agreed text to RAN1 as information.

RAN4 to update the specification and send the agreed text to RAN1 as information.

Conclusion

In this paper we discussed the TDD punctured pattern for IoT NTN UEs. The following Observations and Proposals were made:

Observation 1: From RAN1 LS it is possible to confirm there is no need for a pre-compensation gap in IoT NTN TDD, but that UEs may change the pre-compensation prior to the set of 8 consecutive uplink subframes.

Proposal 1: Confirm to RAN1 that there is no issues with their agreements.

Proposal 2: RAN4 to update the specification and send the agreed text to RAN1 as information.

- References

- RP-242415, “New WID on introduction of IoT-NTN TDD mode”, September 2024

- R1-2508064,  Reply LS on precompensation for NB-IoT NTN TDD mode,   Qualcomm, Prague, CZ, 13th – 17th October, 2025