3GPP TSG-RAN4 Meeting #117	R4-2521746

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

Source:	Nokia

Title:	Discussing the applicability of the new NGSO channel model for RRM test cases

Agenda item:	6.6.3.1.2

Document for:	Discussion

Introduction

In RAN #101, RAN plenary has approved a simplification of the test framework for NGSO, amidst concerns of the time to complete the work item in RAN5 by the end of the Release 17. As a consequence, the specification work in RAN4 has concluded that the test cases for NGSO scenarios would be tested against a fixed (non-varying) propagation delay and fixed doppler delay.

As the speed of satellite in NGSO scenarios is a very relevant aspect of the NTN scenario that has not been covered by similar testing before and considering that NTN is a growing and relevant market, RAN has approved the WID RP-241985 [1] (NR_IoT_NTN_req_test_enh) to develop more meaningful and relevant test cases for this feature considering the motion trajectory of the satellite.

The following objectives were established for this WI:

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

In the present meeting, RAN4 has to discuss the adaptation of the RRM specifications in TS 38.133 to encompass the new channel model to the test cases.

In this contribution we will discuss the open issues identified in the current WF [2].

Discussion

Applicability of the new channel model

The first point to be discussed in this document is related to the scope of changes of the test cases to adopt the new test cases. In the previous meeting, some companies presented their views relative to this matter and there are currently two main views. Some companies believe that the scope of the new channel model shall be limited to the new test cases. Other companies believe it shall be applied to all test cases.

In our view, implementing the new channel model to all test cases will not add significant workload, as the channel model has already being developed by RAN4 in previous meetings, it is just a matter of enabling the configuration in all test cases. Besides, there are the following advantages of doing so:

- It is economically wise for the UE implementation, as there will not be the need to create a new “test mode” in the UE chipset for enabling the NTN test cases

- RRM specs are already prepared for a non-static channel model.

- Minimum specification effort

- Consistency across all test cases and more meaningful tests that approximate the scenarios closer to the real market applications

Based on all the points above we propose:

Apply the channel model to all test cases in RRM.

However, some companies have raised issues that the original scope did not intend to apply the new channel model for all test cases. In order to compromise we may discuss whether the applicability of the channel model for other test cases (besides transmit timing test cases) shall be dependent on UE declaration of support.

It can be discussed whether  the applicability of the channel model for other test cases (besides transmit timing test cases) shall be dependent on UE declaration of support.

Impact of the new channel model in the RRM test cases

One important outcome of previous meeting’s discussions was that the impact of the channel model should be analyzed for all test cases, as some companies were unsure whether the test case could provide impact on the outcome of the feature being tested. In this contribution we analyze the RRM test cases and whether they are impacted by the new channel model:

Table 1.

| Test “Family” | Tested Feature | Impact of the test case | Other Obs: |
| --- | --- | --- | --- |
| RRC_IDLE State Mobility | Intra-frequency cell reselection (time/location/radio triggers) | No impact* | In the test cases where multi-satellite scenario is configured, the test case becomes much more meaningful with the new channel model Events in the test case have significant margin in the radio conditions compared to thresholds levels for applicability of requirements in the core part |
|  | Inter-frequency cell reselection (time/location/radio triggers) | No impact* |  |
|  | Inter-RAT Cell Reslection | No impact |  |
| RRC_Connected | Handover Test Cases | No impact |  |
|  | RRC Re-establishment | No impact* | Events in the test case have significant margin in the radio conditions compared to thresholds levels for applicability of requirements in the core part |
|  | Random Access | No impact |  |
|  | Satellite Switching | No impact |  |
|  | RRC Connection Release with Redirection | RRC Connection Release with Redirection* | Events in the test case have significant margin in the radio conditions compared to thresholds levels for applicability of requirements in the core part |
| Timing for Satellite Access | Already agreed |  |  |
| Signalling Characteristics | RLM | No impact* | Events in the test case have significant margin in the radio conditions compared to thresholds levels for applicability of requirements in the core part |
|  | Beam Failure Detection | No impact |  |
| Measurement Procedures | Intra-frequency Measurements | No impact* | Events in the test case have significant margin in the radio conditions compared to thresholds levels for applicability of requirements in the core part |
|  | Inter-frequency Measurements |  |  |
|  | L1-RSRP Measurement for beam reporting for satellite access |  |  |
| Measurement performance requirements | SS-RSRP | Potential impact | Accuracy requirement needs to be relaxed |
|  | SS-RSRQ |  |  |
|  | SS-SINR |  |  |
|  | L1-RSRP |  |  |

In several test cases, for example the tests involving measurement procedures, the feature being tested depends on detectability levels specified on the core requirements. Some companies have argued that, because the tests channel model emulates a satellite orbit, there might be some small discrepancies on the values of satellite position compared to a real satellite moving in orbit.  This can cause the decoded SINR to be slightly smaller than intended.  However, in the test cases, the levels are not around these thresholds, the events that are defined in the test case use radio levels that are far above (or below) the thresholds in the core requirements. These events are not impacted by some inaccuracy caused by small errors of estimation in the satellite position.

For the test cases where accuracy on the measurements is expected, the small deviations might cause some errors that could be seen as challenging for UEs being tested. For circumvent such problems, we propose to consider an extra margin of 0.5 dB for this test cases.

Add an extra 0.5 dB margin in the accuracy test cases when the new channel model is introduced

RAN4 to adopt Table 1 as reference for the analysis of the impact of the new channel model in the new test cases.

  - Applicatility of the new TE-emulated channel model for pre-R19 UE

RAN4 has discussed whether the new channel model could be applicable to pre-Rel 19 UEs, and achieved the following WF:

WF: FFS on - Proposal 1: - For the pre-R19 UE, if the new TE-emulated channel model defined in Rel19 available for DUT, the test cases defined before Rel19 can be tested under these new channel model instead of the static channel. - Proposal 1a: - The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration

We see the intent of the proposal as beneficial and also striving to achieve the economic benefits of not needing to implement a test mode in such UEs. Therefore, we support such proposals.

- The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration

Conclusion

In this paper, we provided our views on the performance test cases utilizing the new channel model that includes satellite movement. The following proposals were made:

Proposal 1: Apply the channel model to all test cases in RRM.

Observation 1: It can be discussed whether  the applicability of the channel model for other test cases (besides transmit timing test cases) shall be dependent on UE declaration of support.

Proposal 2: Add an extra 0.5 dB margin in the accuracy test cases when the new channel model is introduced

Proposal 3: RAN4 to adopt Table 1 as reference for the analysis of the impact of the new channel model in the new test cases.

Proposal 4: The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration

- References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, Samsung, RAN #105, Melbourne, Australia, September 9-12, 2024+

- R4-2512120, “WF on RRM requirements for NR_IoT_NTN_req_test_enh”, Moderator (Xiaomi), Bengaluru, India, August 25th – 29th, 2025