3GPP TSG-RAN4 Meeting #117	R4-2521875

# Dallas, USA, 17-21 November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6218 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on CSI-RS based BFD requirements with SBFD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on RAN4 116bis meeting agreement from WF R4-2515170: Issue 2-5: Clarification on the meaning of L1=0 when M =1 <Agreement>: - For the case UE does not support SBFD capability or SBFD is not configured, clarify that L1 is not applicable. - For the case UE supports SBFD capability and SBFD is configured, clarify L1 for the case of M=1 with one of the following alternative. - Apply the way of L1 counting for M = 3 to M = 1. The legacy UE which do not support SBFD, the requirement table shall not be applicable to have the L or L1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the L1 in CSI-RS based BFD requirements with SBFD issue above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RS based BFD requirements with SBFD is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 8.5.3	Requirements for CSI-RS based beam failure detection

#### 8.5.3.1	Introduction

The requirements in this clause apply for each CSI-RS resource in the set q0 of resource configurations for a serving cell, provided that the CSI-RS resource(s) in set for beam failure detection are actually transmitted within the UE active DL BWP during the entire evaluation period specified in clause 8.5.3.2. UE is not expected to perform beam failure detection measurements on the CSI-RS configured for BFD if the CSI-RS is not QCL-ed, with QCL-TypeD when applicable, with the RS in the active TCI state of any CORESET configured in the UE active BWP. The requirements in this clause apply when UE is required to perform beam failure detection on no more than 1 serving cell per band unless otherwise specified. For UE supporting intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided for the configured FR1 intra-band non-contiguous CA, the requirements in this clause apply when UE is required to perform beam failure detection on no more than 2 serving cells per band if these 2 serving cells are in non-contiguous carriers, and no more than 1 serving cell per band otherwise.

For UE supporting intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-v19xy is “type 4”

3>	the requirements in this clause apply when UE is required to perform beam failure detection on no more than 2 serving cells per band if these 2 serving cells are in non-contiguous carriers

2> if nonCollocatedTypeNR-CA-v19xy is “type 1”

3>	the requirements in this clause apply when UE is required to perform beam failure detection on no more than 1 serving cell per band

2> else if nonCollocatedTypeNR-CA-v19xy is not provided

3>	the requirements in this clause apply when UE is required to perform beam failure detection on no more than 1 serving cell per band

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3> the requirements in this clause apply when UE is required to perform beam failure detection on no more than 2 serving cells per band if these 2 serving cells are in non-contiguous carriers

2>else

3> the requirements in this clause apply when UE is required to perform beam failure detection on no more than 1 serving cell per band

Table 8.5.3.1-1: PDCCH transmission parameters for beam failure instance

| Attribute | Value for BLER |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | 0 dB |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | 0 dB |
| Bandwidth (PRBs) | 48 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

#### 8.5.3.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the CSI-RS resource in set q0 estimated over the last TEvaluate_BFD_CSI-RS period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS period.

The value of TEvaluate_BFD_CSI-RS is defined in table 8.5.3.2-1 or table 8.5.3.2-3 (deactivated PSCell) for FR1.

The value of TEvaluate_BFD_CSI-RS is defined in table 8.5.3.2-2 or table 8.5.3.2-4 (deactivated PSCell) for FR2 with N=1. The requirements of TEvaluate_BFD_CSI-RS apply provided that the CSI-RS for BFD is not in a resource set configured with repetition ON. The requirements shall not apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for BFD and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

-	For a UE supporting LB-CA via switching, or for a UE supporting concurrentMeasGapsPreMG-r18 and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting concurrentMeasGapsNCSG-r18 and when concurrent measurement gap(s) with NCSG(s) are configured, or a UE supporting concurrentMeasGap-r17 or [musim-GapPreference-r17] or both concurrentMeasGap-r17 and musim-GapPreference-r17, and when concurrent gaps or periodic MUSIM gaps or both concurrent GAPs and periodic MUSIM gaps are configured,

-	an CSI-RS resource occasion for beam failure detection is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8 and 9.1.10,

-	P value for a BFD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable> 0

-	For a window W of duration max(TL1,  xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE measurement gaps or MUSIM gap(s) or NCSGs and per-FR measurement gaps or NCSGs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any BFD-RS resource occasion:

-	Ntotal is the total number of BFD-RS resource occasions within the window W, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window W,

For UEs supporting LB CA via switching and configured with low band CA via switching:

-	for the BFD monitoring on PCell, Ntotal also includes SSB resource occasions that overlap with the SDL ON duration within the window,

-	for the BFD monitoring on SCell, Ntotal also includes SSB resource occasions that overlap with the PCell ON duration within the window, as defined by the configured switching pattern, and

-	Noutside_MG is the number of BFD-RS resource occasions within the window W, that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion, or

-	that are not overlapped with SCell ON duration corresponding to the LB CA switching pattern, for the BFD on PCell for a UE supporting LowBandSwitching-r19 and configured with low band CA via switching,

-	that are not overlapped with PCell ON duration corresponding to the LB CA switching pattern, for the BFD on SCell for a UE supporting LowBandSwitching-r19 and configured with low band CA via switching, , and

-	Navailable is the number of BFD-RS resource occasions that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion nor any SMTC occasion within the window W, and

-	an CSI-RS resource occasion for beam failure detection is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion, and

TL1 is periodicity of the target BFD-RS.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG.

-	for UE not supporting LB CA via switching, switching pattern periodicity is not applicable.

-	BFD requirement in this clause is not applied when Noutside_MG = 0.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor concurrentMeasGapsPreMG-r18 nor concurrentMeasGapsNCSG-r18 nor supporting musim-GapPreference-r17 or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s) and concurrent measurement gap(s) with NCSG(s), and periodic MUSIM gaps,For FR1,

For FR1,

-	P=11-TCSI-RSxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS.

-	P = 1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For FR2,

-	P = 1, when the BFD-RS resource is not overlapped with GAP and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP, when the BFD-RS resource is partially overlapped with GAP and the BFD-RS resource is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when the BFD-RS resource is not overlapped with GAP and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = Psharing factor, when the BFD-RS resource is not overlapped with GAP and the BFD-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=11-TCSI-RSxRP - TCSI-RSTSMTCperiod, when the BFD-RS resource is partially overlapped with GAP and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5 × TSMTCperiod

-	P=Psharing factor1-TCSI-RSxRP, when the BFD-RS resource is partially overlapped with GAP and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TCSI-RS = 0.5 × TSMTCperiod

-	P=11-TCSI-RSMin(xRP, TSMTCperiod), when the BFD-RS resource is partially overlapped with GAP (TCSI-RS < xRP) and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP.

-	P=Psharing factor1-TCSI-RSxRP, when the BFD-RS resource is partially overlapped with GAP and the BFD-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

where,

-	Psharing factor = 1, if the BFD-RS resource outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	If the higher layer in TS 38.331 [2] signalling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

-	When a GAP is configured only and the GAP is not NCSG,

-	a BFD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a GAP occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG GAP only is configured,

-	a BFD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

-	If the UE is configured with Pre-MG only, a BFD-RS resource or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps or concurrent measurement gap(s) with Pre-MG(s) or concurrent measurement gap(s) with NCSG(s) are configured, a BFD-RS resource or an SMTC occasion is not considered to be overlapped by a GAP occasion if the GAP occasion is dropped according to clause 9.1.8, clause 9.1.12, clause 9.1.13, respectively.

NOTE:	The overlap between CSI-RS for BFD and SMTC means that CSI-RS for BFD is within the SMTC window duration.

For the value of L1,

1>	If the UE does not support supportSBFD or SBFD is not configured by the network

2>	L1is not applicable;

1>	Else if the UE supports supportSBFD and SBFD is configured by the network

2>	If DRX is not configured,

3>	L1 is the number of occasions of the configured BFD-RS resource that are overlapping with the dynamic UL transmission on SBFD symbols during TEvaluate_BFD_CSI-RS;

2>	Else if DRX is configured,

3> L1 is the number of DRX cycles in which at least one occasion of the BFD-RS resource is overlapping with the dynamic UL transmission on SBFD symbols during TEvaluate_BFD_CSI-RS.

If the UE supports supportSBFD and SBFD is configured by the network, the requirements in this clause apply provided that all occasions of the BFD-RS resource are in the same type (i.e., SBFD or non-SBFD) of symbols.

For UE supporting measurement gap occasion cancellation, the UE is not required to perform CSI-RS measurements during the cancelled gap occasions.

Longer evaluation period would be expected if the combination of the BFD-RS resource, SMTC occasion and GAP configurations does not meet previous conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

When the configured aperiodic MUSIM gap is overlapping with CSI-RS resource occasion for beam failure detection, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and if CSI-RS resource occasions for beam failure detection are fully overlapped with MUSIM gap(s), or the union of MUSIM gap(s) and GAPs, no requirement applies for CSI-RS based beam failure detection.

For either an FR1 or FR2 serving cell, longer BFD evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of MBFD used in table 8.5.3.2-1 and table 8.5.3.2-2 are defined as

-	MBFD = 10, if the CSI-RS resource(s) in set  used for BFD is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs.

Note: If UE supports supportSBFD and SBFD is configured by the network, the requirements in this clause apply provided that CSI-RS resource(s) in set  used for BFD is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs in at least one DL subband.

The values of PBFD used in table 8.5.3.2-1 and table 8.5.3.2-2 are defined as

For each CSI-RS resource in the set  configured for PCell or PSCell in EN-DC or NE-DC or SA; or PCell in NR-DC

-	PBFD = 1.

For each CSI-RS resource in the set  configured for PSCell in NR-DC

-	PBFD = 2 if UE is configured for beam failure detection on SCell, 1 otherwise.

For each CSI-RS resource in the set  configured for a SCell

-	PBFD = Z in EN-DC or NE-DC or SA.

-	PBFD = 2* Z in NR-DC.

-	Where Z is the number of band(s) on which UE is performing beam failure detection only for SCell.

Table 8.5.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, Ceil((MBFD+L1) P PBFD) Max(TCSI-RS, Tproc)) |
| DRX cycle ≤ 320 ms | Max(50, Ceil(1.5 × (MBFD+L1) P PBFD) Max(TDRX, TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | Ceil((MBFD+L1) P PBFD) TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. NOTE 2: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for BFD is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. NOTE 3: L1 is only applicable for UE supports supportSBFD and SBFD is configured by the network. |  |

Table 8.5.3.2-2: Evaluation period TEvaluate_BFD_CSI-RS for FR2

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, Ceil((MBFD+L1) P N PBFD) Max(TCSI-RS, Tproc)) |
| DRX cycle ≤ 320 ms | Max(50, Ceil(1.5 × (MBFD+L1) P N PBFD) Max(TDRX, TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | Ceil((MBFD+L1) P N PBFD) TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. NOTE 2: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for BFD is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. NOTE 3: L1 is only applicable for UE supports supportSBFD and SBFD is configured by the network. |  |

Table 8.5.3.2-3: Evaluation period TEvaluate_BFD_CSI-RS for deactivated PSCell in FR1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Ceil(MBFD P PBFD) measCyclePscell |
| DRX cycle ≤ 320 ms | Ceil(1.5 × MBFD P PBFD) Max(TDRX, measCyclePscell) |
| DRX cycle > 320 ms | Ceil(MBFD P PBFD) Max(TDRX, measCyclePscell) |
| NOTE: DRX cycle is the configured DRX cycle of the PSCell. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |

Table 8.5.3.2-4: Evaluation period TEvaluate_BFD_CSI-RS for deactivated PSCell in FR2

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Ceil(MBFD P N PBFD) measCyclePscell |
| DRX cycle ≤ 320 ms | Ceil(1.5 × MBFD P N PBFD) Max(TDRX, measCyclePscell) |
| DRX cycle > 320 ms | Ceil(MBFD P N PBFD) Max(TDRX, measCyclePscell) |
| NOTE: DRX cycle is the configured DRX cycle of the PSCell. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |

#### 8.5.3.3	Measurement restrictions for CSI-RS beam failure detection

The SSB mentioned in this clause can be associated with either the serving cell PCI or a PCI different from serving cell PCI or intra-frequency neighbor cell(s) configured with L1-RSRP measurement or inter-frequency neighbor cell(s) configured with L1-RSRP measurement without gap.

The UE is required to be capable of measuring CSI-RS for BFD without measurement gaps. The UE is required to perform the CSI-RS measurements with measurement restrictions as described in the following scenarios.

For both FR1 and FR2, when the CSI-RS for BFD measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for BFD measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement has same SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement has different SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for BFD measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for BFD measurement without any restriction.

For FR2, when the CSI-RS for BFD measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD or L1-RSRP measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.For both FR1 and FR2, when the CSI-RS for BFD measurement fully or partially overlaps with the OFDM symbol as SSB or CSI-RS from candidate LTM neighbour cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, UE is not required to receive CSI-RS for BFD measurement in the PRBs that overlap with an SSB or CSI-RS from candidate LTM neighbour cell.

For FR1, when the CSI-RS for BFD measurement fully or partially overlaps with the OFDM symbol as SSB from candidate LTM neighbour cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, if CSI-RS and SSB have different SCS and UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD is expected, and no requirements are defined.

For FR2, when the CSI-RS for BFD measurement on one CC fully or partially overlaps with the OFDM symbol as SSB from candidate LTM neighbour cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD is expected, and no requirements are defined.

For FR2, when the CSI-RS for BFD measurement on one CC fully or partially overlaps with the OFDM symbol as CSI-RS from candidate LTM neighbour cell for intra-frequency L1-RSRP measurement, UE is required to measure one of but not both CSI-RS for BFD measurement and CSI-RS for candidate LTM neighbour cell for intra-frequency L1-RSRP measurement. Longer measurement period for CSI-RS based BFD is expected, and no requirements are defined.

For FR2, when the CSI-RS for BFD measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band,

-	In the following cases, UE is required to measure one of but not both CSI-RS for BFD measurement and the other CSI-RS. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

-	The CSI-RS for BFD measurement or the other CSI-RS in a resource set configured with repetition ON, or

-	The other CSI-RS is configured in set q1 and beam failure is detected, or

-	The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE,

-	Otherwise, UE shall be able to measure the CSI-RS for BFD measurement without any restriction.

For FR2-1, when the first CSI-RS for BFD measurement is in the same OFDM symbol as the second CSI-RS for RLM, BFD or L1-RSRP measurement on the same serving cell according to the conditions in clause 3.6.19, the UE supporting schedulingMeasurementRelaxation-r18 is required to measure both the first and the second CSI-RSs without measurement restrictions, provided the following conditions are met:

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON, and

-	One CSI-RS has same QCL source as either

-	the active TCI state of a PDSCH scheduled in the same OFDM symbol or

-	the QCL source based on the default QCL assumption to be applied in the same OFDM symbol according to TS 38.214 [26] clause 5.1.5, and

-	the other CSI-RS has same QCL source as either

-	the active TCI state of a PDSCH scheduled in the same OFDM symbol or

-	the QCL source based on the default QCL assumption to be applied in the same OFDM symbol according to TS 38.214 [26] clause 5.1.5, and

-	Resources of the active TCI states of the two PDSCHs, or QCL sources of the default QCL assumption, or the active TCI state of PDSCH and QCL source of the default QCL assumption have been reported as a resource group in Rel-17 group-based RSRP report.

#### 8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria

This clause contains the minimum requirements for CSI-RS based relaxed beam failure detection.

UE shall be able to evaluate whether the downlink radio link quality on the CSI-RS resource in set q0 estimated over the last TEvaluate_BFD_CSI-RS_Relax period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS_Relax period.

The value of TEvaluate_BFD_CSI-RS_Relax is defined in table 8.5.3.4-1 for FR1.

The value of TEvaluate_BFD_CSI-RS_Relax is defined in table 8.5.3.4-2 for FR2 with N=1.

The values of P, MBFD and PBFD is defined in clause 8.5.3.2.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 8.5.3.4-1: Evaluation period TEvaluate_BFD_CSI-RS_Relax for FR1

| Configuration | TEvaluate_BFD_CSI-RS Relax (ms) |
| --- | --- |
| Max(TDRX, TCSI-RS) ≤ 80 ms | Max(50 K3, Ceil(K1 × 1.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)) |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length and no longer than 80 ms. NOTE 2: K1 is the relaxation factor. K1 = 2 for 40 ms < MAX(TDRX, TCSI-RS) ≤ 80 ms, K1 = 4 for MAX(TDRX, TCSI-RS) ≤ 40 ms NOTE 3: K3 is the relaxation factor for the lower bound. K3 = K1, if 1 < K1 ≤ 2; K3 = 1 otherwise. |  |

Table 8.5.3.4-2: Evaluation period TEvaluate_BFD_CSI-RS_Relax for FR2

| Configuration | TEvaluate_BFD_CSI-RS Relax (ms) |
| --- | --- |
| Max(TDRX, TCSI-RS) ≤ 80 ms | Max(50 K4, Ceil(K2 × 1.5 × MBFD P N PBFD) Max(TDRX, TCSI-RS)) |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length and no longer than 80 ms. NOTE 2: K2 is the relaxation factor. K2 = 2 NOTE 3: K4 is the relaxation factor for the lower bound. K4 = K2, if 1 < K2 ≤ 2; K4 = 1 otherwise. |  |

<End of Change 1>