3GPP TSG RAN WG4 Meeting #116	bis 														R4-2521924

Dallas, USA, 17 Nov. 2025 – 21 Nov. 2025

Agenda Item:	7.10.3

Source: 	Ericsson

Title:	Discussion on Rel-20 MIMO RRM requirements

Document for:	Decision

# Introduction

In RP-252936, WID for MIMO phase 6 is agreed. RAN4 objective is copied here for reference.

RAN4:

Specify necessary core requirements for the enhancements listed above.

In this contribution we analyze the potential RAN4 RRM impacts for NR MIMO Phase 6.

# Discussion

From the WID, we can observe that RAN4 scope depends on the RAN1 enhancements introduced for each objective. In this contribution, we look at each objective and the corresponding RAN4 impact based on the RAN1 agreements in the below sections.

## Objective 1: Enhancing UL capacity and coverage

Objective is copied below for reference.

On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

There are two sub-objectives, outlined below with a high-level overview of their intent:

- Enabling multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop, specifically for RB-level partial frequency sounding. (Enhancement of RB-level partial frequency sounding)

- Enabling cross-slot SRS resource usage between the end of a U slot and the beginning of an S slot, within a single SRS resource set. (Cross-slot SRS)

Enhancement of RB-level partial frequency sounding

For the first sub-objective, in last meeting we agreed that there is NO RRM impact. Agreement is listed below for the reference.

- For multiple frequency-domain starting positions for SRS frequency hopping, there is no RRM impact.

Note: This issue can be revisited if any RRM impact is identified based on further RAN1 progress.

Cross-slot SRS

For the cross slot SRS, following WF is agreed.

  - Option 1 (QC, MTK, LG):

    - RAN4 should wait for further RAN1 progress before RRM requirements for cross-slot SRS are discussed.

  - Option 2 (Huawei):

    - No RRM impact.

  - Option 3 (vivo):

    - RAN4 to define SRS AS requirements for the case for cross-slot SRS resource.

    - RAN4 start from Scenario 1 as agreed in RAN1, and whether to consider other scenarios depends on further conclusion in RAN1.

Based on the RAN1 agreement regarding Scenario 1 in their discussions, we understand that SRS antenna switching (AS) is supported for cross-slot SRS transmission. Our interpretation of the scenarios supported by RAN1 is illustrated in the figure below.

In Figure 1, SRS1 to SRS4 are configured for SRS Antenna Switching (AS). Among these, SRS2 is transmitted in a cross-slot configuration, while SRS3 and SRS4 occupy the first part of the slot. Currently, the existing RAN4 requirements support SRS AS only when the SRS symbols are located in the last 6 symbols of the slot. There is no support defined for SRS AS in the first 8 symbols, which limits flexibility in certain deployment scenarios. If the network intends to support SRS AS using cross-slot transmission, it may result in situations where no specific requirements exist for UE interruption behavior. Therefore, we believe that RAN4 should define requirements for cross-slot SRS AS, particularly for cases where SRS symbols are located in the first half of the slot.

- RAN4 to define requirements for cross-slot SRS AS, particularly for the cases where SRS symbols in an SRS resource are spread across the slots.

## Objective 2: Enhancing DL CSI acquisition (Early CSI acquisition)

Objective is copied below for reference.

On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

To analyze the RRM requirements impact, we first look at few RAN1 agreements.

Agreement: For UE transition from IDLE/INACTIVE to CONNECTED mode, support at least aperiodic SRS-AS transmission triggered via MSG4 of 4-Step RACH. Agreement: For UE transition from IDLE/INACTIVE to CONNECTED mode, support aperiodic CSI reporting triggered via MSG4 of 4-Step RACH based on the followings: - The aperiodic CSI reporting is transmitted on PUSCH. - Support at least aperiodic CSI-RS for CSI associated with the aperiodic CSI reporting - Support PMI-based reporting with wideband PMI based on Rel-15 Type-I SP codebook and wideband CQI - FFS: Which report quantity(s) can be configured - Support PMI-free reporting with wideband CQI FFS: Which report quantity can be configured

Based on recent RAN1 agreements, the UE supports at least aperiodic transmission of both SRS and CSI reporting during the transition from IDLE or INACTIVE to CONNECTED mode. These transmissions are triggered via MSG4 of the 4-Step RACH procedure. Aperiodic SRS helps with downlink CSI acquisition, while aperiodic CSI reporting is sent on PUSCH and depends on associated aperiodic CSI-RS for channel state acquisition. CSI reporting can be done in two ways: PMI-based reporting, which uses wideband PMI from the Rel-15 Type-I Single Panel codebook along with wideband CQI, and PMI-free reporting, which includes only wideband CQI.

In addition, RAN1 has agreed on a four-step timeline for CSI acquisition. Same is copied below for reference.

Agreement: For a UE transition from IDLE to CONNECTED mode, support the following procedure at least for early aperiodic SRS-AS/CSI-RS/CSI triggering (i.e., early triggering of aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting): - Step-1: The UE receives the resource/reporting configuration(s) for early SRS-AS/CSI-RS/CSI triggering provided in the system information before MSG3. - FFS: Which SIB is used to carry the resource/reporting configuration(s) for early SRS-AS/CSI/CSI-RS triggering - Step-2: The UE reports its capability on early SRS/CSI-RS/CSI triggering through MSG3 - Step-3: The UE receives MSG4 that triggers early SRS-AS/CSI-RS/CSI based on the capability reported by the UE. - Step-4: The UE performs aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting. - FFS: Timeline of the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting Note: The term “capability” above does not mean legacy RRC based UE capability. Note: The term “capability” above does not mean legacy RRC based UE capability.

Although PMI and CQI reporting requirements are not typically defined by RAN4 RRM, it is important for RAN1 to consider UE transmission and reception limitations during certain stages of the CSI acquisition process. To support this, RAN4 should share its understanding with RAN1. Specifically, RAN4 assumes that the UE can only acquire CSI after fine timing synchronization is completed. This assumption is key to ensuring that CSI-RS transmission from the network aligns with the UE’s ability to perform CSI measurements during connection setup.

IDLE to connected mode:

Figure 1: RRC IDLE to connected state transition

We begin by presenting the UE transition timeline from IDLE to CONNECTED mode as defined in TS 38.300, illustrated in Figure 1. Based on our understanding, during steps 2 and 2a of this timeline, the UE may be unable to transmit or receive for a certain duration. In the last meeting, we proposed that RAN4 discuss whether the UE can transmit or receive during the RRC setup delay periods. Companies agreed that the proposal is valid and recommended identifying different scenarios where the UE may or may not be able to transmit or receive during these delays.

One such scenario is, when the BWP or any of its parameters are changed during RRC setup, the UE may not be able to transmit or receive during the delay period. Other relevant cases still need to be identified. As a starting point, we can agree that when the initial BWP and the first active BWP are the same, the UE will be able to transmit or receive during RRC setup or resume delays.

This limitation arises from the processing of the RRCSetup message, which involves configuring baseband and RF components for access to the target cell and applying the parameters contained in the setup message.

Unless a new mechanism is introduced in RAN1, this processing time results in a temporary inactivity period during which the UE cannot perform transmission or reception. Therefore, we propose that RAN4 first confirm whether the UE can transmit or receive during the RRC setup delay. If the UE cannot, RAN4 should inform RAN1 that, under current procedures, from the moment the UE receives the RRCSetup message, it is unable to transmit or receive on the cell until the RRCSetup processing delay is complete. This clarification is essential for accurately modelling UE behavior and enabling timely CSI-RS transmission from the network, particularly in scenarios where early CSI acquisition relies on aperiodic CSI-RS.

- RAN4 should first identify the scenarios in which the UE is unable to transmit or receive during the RRC setup delay. RAN4 should clarify to RAN1, via LS that under certain conditions, and in accordance with existing procedures, the UE cannot transmit or receive from the time of RRCSetup reception until the RRCSetup processing delay is complete. As specified in TS 38.331, the RRCSetup processing delay is defined as 10 ms.

Inactive to connected mode:

Figure 2: RRC inactive to Connected transition

We begin by presenting the UE transition timeline from INACTIVE to CONNECTED mode as defined in TS 38.300, illustrated in Figure 2. Based on our understanding, during steps 4 and 5 of this timelines, the UE may be unable to transmit or receive for a certain duration. This limitation arises from the processing of the RRCResume message, which involves configuring baseband and RF components for access to the target cell and applying the parameters contained in the message.

Unless a new mechanism is introduced in RAN1, this processing time results in a temporary inactivity period during which the UE cannot perform transmission or reception. Therefore, we propose that RAN4 first confirm whether the UE can transmit or receive during the RRC resume delay. If the UE cannot, RAN4 should inform RAN1 that, from the moment the UE receives the RRCResume message, it is unable to transmit or receive on the cell until the resume processing is complete. This clarification is essential for accurately modelling UE behavior and enabling timely CSI-RS transmission from the network, particularly in scenarios where early CSI acquisition relies on aperiodic CSI-RS. Furthermore, based on our understanding, the network can begin scheduling data from step 5 onward. Therefore, if CSI acquisition is expected to occur before step 5, adjustments to the current timeline may be needed to support such early CSI reporting.

In the last meeting, we proposed that RAN4 discuss whether the UE can transmit or receive during the RRC setup and RRC resume delay. Companies agreed that the proposal is valid and suggested identifying different scenarios where the UE may or may not be able to transmit or receive during these delays. One such scenario is when the BWP or any of its parameters are changed during RRC setup or RRC resume. In this case, the UE may not be able to transmit or receive during the delay period. Other relevant cases still need to be identified. However, as a starting point, we can agree that when the initial BWP and the first active BWP are the same, the UE will be able to transmit or receive during RRC setup or resume delays.

- RAN4 should first identify the scenarios in which the UE is unable to transmit or receive during the RRC Resume delay. RAN4 should clarify to RAN1, via LS that under certain conditions, and in accordance with existing procedures, the UE cannot transmit or receive from the time of RRCResume reception until the RRCResume processing delay is complete. According to TS 38.331, the RRCResume processing delay is defined as 6 ms or 10 ms, depending on the scenario.

- When the BWP or any of the BWP of the parameter’s changes during the RRC IDLE to RRC Setup or from RRC inactive to RRC connected, UE cannot transmit or receive on the cell for the RRC setup or RRC resume delays.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 to define requirements for cross-slot SRS AS, particularly for the cases where SRS symbols in an SRS resource are spread across the slots.

- RAN4 should first identify the scenarios in which the UE is unable to transmit or receive during the RRC setup delay. RAN4 should clarify to RAN1, via LS that under certain conditions, and in accordance with existing procedures, the UE cannot transmit or receive from the time of RRCSetup reception until the RRCSetup processing delay is complete. As specified in TS 38.331, the RRCSetup processing delay is defined as 10 ms.

- RAN4 should first identify the scenarios in which the UE is unable to transmit or receive during the RRC Resume delay. RAN4 should clarify to RAN1, via LS that under certain conditions, and in accordance with existing procedures, the UE cannot transmit or receive from the time of RRCResume reception until the RRCResume processing delay is complete. According to TS 38.331, the RRCResume processing delay is defined as 6 ms or 10 ms, depending on the scenario.

- When the BWP or any of the BWP of the parameter’s changes during the RRC IDLE to RRC Setup or from RRC inactive to RRC connected, UE cannot transmit or receive on the cell for the RRC setup or RRC resume delays.

# References

[1] 	RP-234007, New WID: NR MIMO Phase 5