3GPP TSG-RAN WG4 Meeting #117	R4-2522052

# DALLAS, TEXAS, 17th – 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0109 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (LTE_NBIOT_eMTC_NTN_req-Core) CR to TS 36.102 for correcting error |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Since PC2 for B252 is not specified yet, B252 NS_06N, NS_07N, and NS_08N are only used to control PC3 and PC5 currenlty. - Correct the typo of NS_06 to NS_06N |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Since PC2 of B252 is not specified yet, to provide correct table in “Additional Spectrum Emission Mask” clauses for specifying B252 NS_06N, NS_07N, and NS_08N associated with PC3 and PC5. - Correct the typo of NS_06 to NS_06N |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error is not corrected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.3.3.1, 6.5A.4.4.7, 6.5B.3.3.1, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 36.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

6.5A.3.3.1	Requirements for network signalling value "NS_02N”, “NS_03N”, “NS_06N”, “NS_07N” and “NS_08N”

When "NS_02N", "NS_03N", “NS_06N”, “NS_07N” or “NS_08N” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.1-1, Table 6.5A.3.3.1-2 or Table 6.5A.3.3.1-3.

Table 6.5A.3.3.1-1: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ±0 – 0.7 | 1 for PC2 -2 for PC3 -5 for PC5 | 4 kHz |
| ±0.7 – 2.8 | -9 for PC2 -12 for PC3 -15 for PC5 | 4 kHz |
| ±>2.8 | -13 for PC2, PC3 and PC5 | 4 kHz |

Table 6.5A.3.3.1-2: Additional requirements for "NS_03N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ±0 – 0.7 | -2 for PC3 -5 for PC5 | 4 kHz |
| ±0.7 – 2.8 | -12 for PC3 -15 for PC5 | 4 kHz |
| ±>2.8 | -13 for PC3 and PC5 | 4 kHz |

Table 6.5A.3.3.1-3: Additional requirements for “NS_06N”, “NS_07N” and “NS_08N”

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ±0 – 0.7 | -2 for PC3 -5 for PC5 | 4 kHz |
| ±0.7 – 2.8 | -12 for PC3 -15 for PC5 | 4 kHz |
| ±>2.8 | -13 for PC3 and PC5 | 4 kHz |

<End of change 1>

<Start of change 2>

6.5A.4.4.7	Minimum requirement (network signalled value "NS_06N")

When "NS_06N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.7-1.

Table 6.5A.4.4.7-1: Additional requirements for "NS_06N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 1.4MHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |

<End of change 2>

<Start of change 3>

6.5B.3.3.1	Requirements for network signalling value "NS_02N”, “NS_06N”, “NS_07N” and “NS_08N”

When "NS_02N", “NS_06N”, “NS_07N” or “NS_08N” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.1-1 or Table 6.5B.3.3.1-2.

Table 6.5B.3.3.1-1: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | 6 for PC1 1 for PC2 -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -4 for PC1 -9 for PC2 -12 for PC3 -15 for PC5 | 4 kHz |
| ± >0.85 | -13 for PC1, PC2, PC3 and PC5 | 4 kHz |

Table 6.5B.3.3.1-2: Additional requirements for “NS_06N”, “NS_07N” and “NS_08N”

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -12 for PC3 -15 for PC5 | 4 kHz |
| ± >0.85 | -13 for PC3 and PC5 | 4 kHz |

NOTE:	ΔfOOB = 0.09 MHz corresponds to an authorized bandwidth, as defined in C63.26-2015 [10], of 0.38 MHz.

<End of change 3>