3GPP TSG-RAN4 Meeting #117	R4-2522358

Dallas, USA, 17 November – 21 November 2025

Title: 	WF on 6G BS RF and co-existence

Agenda item:	8.1

Source:	Feature Lead (Ericsson)

Document for:	Approval

# Agreements and Way Forward

## BS RF requirements

### Issue 1-1-1: 5G RF requirements applicable to 6G

Agreement:

The following existing 5G BS RF requirements are applicable to 6G BS RF at least for the 5G re-farming bands.

- Frequency error

- Occupied bandwidth

- Spurious general requirement

- Receiver spurious requirement

### Issue 1-1-2: 5G RF requirements to be re-evaluated without any new study for 6G

Agreement:

The following 5G BS RF requirements would need to be re-evaluated based on TR 38.817-02, e.g. the existing formula is reused, once the spectrum utilization and channel bandwidths list will be decided for 6G. No further study would be needed for those requirements.

- Reference sensitivity

- Receiver intermodulation

And at least for the 5G re-farming bands:

- Sensitivity (OTA)

- Dynamic Range

- In channel selectivity

### Issues 1-1-3 to 1-1-12: BS RF requirements to be studied

Agreement:

The following BS RF requirements will be studied during the 6G SI:

- 1st priority:

  - EVM, detailed scope to be further discussed:

    - For new 6G modulation

    - Study + measurement methodologies

    - More pragmatic EVM requirement framework (e.g., MIMO layers dependent framework, other considerations are not precluded)

  - In-band and Out of band blocking

    - Reconsidering approach taken for UTRA

    - For bands in upper FR1 and/or for new 6G bands only.

  - Protection of the BS receiver of own or different BS

  - OBUE for wider channel BW

  - Time alignment error

- 2nd priority:

  - BS output power

  - RE power control dynamic range

  - Total power dynamic range

  - Transmit ON/OFF, only for new 6G bands

  - ACLR absolute limit clarification

The 1st priority means the technical discussion should start from next meeting, focus will be on these 1st priority topics.

The 2nd priority means tdoc could be submitted from next meeting but the discussion will only happen if there is time left during the meetings.

Note: SBFD related requirements (e.g., In-channel adjacent subband leakage for SBFD, In-channel adjacent subband blocking for SBFD, etc.) to be considered by leveraging the Rel-19 and Rel-20 outcomes.

## Requirements for cmWave bands

### Issue 1-2-1: Conducted requirements and FR2 like methodology

Agreement:

For bands in the 5-8 GHz:

- Conducted and OTA requirements will be studied.

- FFS if similar approach as FR2 could be used (e.g. single declared sensitivity).

### Issue 1-2-2: RF requirements to be studied for bands above 7 GHz

Way Forward:

The following list of requirements for bands above 7 GHz would be considered:

- If RAN4 agrees to redo coexistence study for ~7GHz:

    - ACLR/ACS

    - BS Rx requirements based on coexistence results, including ICS.

    - Dynamic range

- For further discussion:

    - RE power control dynamic range

    - Transmit On/Off power requirements

## Coexistence studies

### Issue 2-1-1: Coexistence studies

Agreement on the 6G coexistence study

Prioritized scenario: ~7 GHz including n104, Urban Macro, PC2 UE (FFS if PC3 and/or PC1.5 or PC1 will be considered). Companies proposing to redo coexistence studies should:

It is for further discussion on the necessity of the new 6G coexistence study.

The companies who propose not to introduce the new 6G coexistence study are encouraged to provide the justification, including how the conclusions of the existing study can be extended to 6G scenarios and how it will impact to extend the existing 5G requirements to 6G.

Interested companies who propose to introduce the new 6G coexistence study are encouraged to

  - Identify the key assumptions differences (comparing to the assumptions and works done in TR 38.921 for 6.425-7.125GHz and TR 38.922 for 7.125 – 8.4 GHz).

  - Identify the potential impacts on the conclusions of previous coexistence studies if any (e.g. how much ACLR/ACS difference is expected).

  - Identify the next steps (e.g. update existing requirements? New requirements/new type? Regulation impacts?).

  - Identify specific band/ frequence range (e.g., n104 and/or 7.125 – 8.4 GHz frequency range)

Other scenarios (e.g. Urban micro, Indoor hotspot, Dense urban, …) are not precluded but the interested companies are encouraged to justify why the proposed scenario(s) would be more stringent comparing to Urban Macro, impacting the coexistence study results.

Note: Coexistence study for less than 1 GHz is on hold.

## NTN aspects

### Issue 3-1-3: NTN-NTN RF coexistence scenarios

Agreement:

The following coexistence studies are considered for the 6G SI with the motivation and needs of such study to be discussed and clarified:

- NTN-NTN coexistence - S-band

- NTN-TN coexistence: New satellite orbit MEO for S-/Ku-/Ka-band

- NTN and TN coverage overlap – adjacent channel – ~2 GHz

Study performance impacts based on existing requirements.

- NTN-TN coexistence: New satellite orbit vLEO for S-band

- NTN-TN coexistence: New 6G C-band considering LEO600 and GEO

- NTN-TN coexistence: New 6G Q/V-band considering LEO600 and GEO

The priority order for #2 to #6 will be decided in RAN4#118 meeting.

### Issue 3-2-1: SAN RF requirements to be studied

Agreement:

- 1st priority:

  - Time Alignment Error

  - Frequency error based on non-ideal feeder link, …

  - Modulation quality based on real world experience from SAN development, SAN PA models + EVM improvement

  - Total power dynamic range, based on evolved SAN architecture, PA model, waveform

- 2nd priority (will be discussed only if time left in next meetings):

  - RE power control dynamic range, based on waveform and PA model

  - Additional spurious coexistence (NTN/NTN coexistence)

  - Receiver sensitivity level based on new channel models

  - Dynamic range based on new channel models

  - In band blocking for corner SAN – SAN cases

  - Out of band blocking for corner SAN – SAN cases

The 1st priority means the technical discussion should start from next meeting, focus will be on these 1st priority topics.

The 2nd priority means tdoc could be submitted from next meeting but the discussion will only happen if there is time left during the meetings.