3GPP TSG-RAN4 Meeting #117	R4-2522449

# Dallas, TX, USA, November 17 – 21, 2025

Agenda item:			8.1

Source:	Feature lead (vivo)

Title:	WF for [117][110] 6G testability and OTA

Document for:	Approval

# Introduction

This is WF covers the discussions of 6G testability and OTA, the discussion scope includes all the testability issues for conducted requirements and OTA requirements for 6G UE/BS requirements (except for sensing requirements), both AI and non-AI cases.

# Topic #1: Improved testability of Conducted Requirements

### Sub-topic 1-1 Improvement of traditional conducted tests

Issue 1-1-1: Discussions on improvement of MSD via OTA approach

Agreements:

- Requirement definition of MSD will be decided in UE RF agenda

- In this thread, group focus on testability discussions, e.g., limitation of radiated MSD testing, pros and cons of radiated MSD performance, improvement of traditional conducted testing, and potential skipping rule to minimize the test burden.

- Before adopting OTA approach and/or skipping rule, feedback will be provided to UE RF group about the feasibility and test complexity.

- The discussion of MSD OTA test methodology will coordinate with the progress of MSD in 6G UE RF thread

Issue 1-1-2: Discussions on whether radiated and/or conducted spurious emission

Agreements:

- RAN4 can further discuss and align understandings of conducted and radiated spurious emission requirements (3GPP and regulatory requirements) first.

Issue 1-1-3: Potential general guidance on radiated methodology or conducted for FR1

Agreements:

- Whether the FR1 core requirements are specified as conductive requirements or radiated requirements belongs to UE RF discussion. This thread focus on potential testability issues.

- RAN4 further discuss whether some conducted TCs could be transitioned to OTA to improve realism from testing perspective.

  - E.g., RAN4 further study the necessity and limitation on introducing multi-Tx/Rx test cases for OTA based on potential radiated metric

  - Whether introducing new requirements based on UE requirement thread discussion.

# Topic #2: New test methodologies for new 6GR frequencies

### Sub-topic 2-1 Testability for new 6GR frequency

Issue 2-1-1: Upper frequency limitation for supporting antenna connector for conductive testing

Agreements:

- RAN4 further discuss the testability issue of conducted and/or radiated metric for frequencies between FR1 and FR2.

  - Whether UE can support conductive connector could be one aspect for consideration

  - The overall work of topic 2 will be done with close coordinations with 6G spectrum discussion

  - FFS other aspects

Issue 2-1-2: lower frequency limitation for supporting antenna beamforming for OTA testing

Agreements:

- Whether the UE is specified as OTA requirements for >[TBD] frequency belongs to RF requirement discussion.

- RAN4 further study the testability issues for the frequencies that UE may not have connector.

Issue 2-1-3: Study on how to extend current test system to cover new frequencies between FR1 and FR2

Agreements:

- To reuse current system as much as possible, RAN4 should first prioritize the study of test system capability extension to cover 6G New frequencies with FR1 or FR2 setups.

  - FFS the range of new frequencies

- Meanwhile, RAN4 study whether a dedicate test system to cover new 6G frequencies is workable or not.

### Sub-topic 2-2 OTA Testability for traditional FR1 and FR2 range

Issue 2-2-1: OTA test methods for traditional FR1 frequency range and FR2

Agreements:

- RAN4 could assume exiting test methodologies for FR1 and FR2 can be reused as baseline.

Issue 2-2-2: Temperature condition for 6GR overlapping frequencies with NR

Agreements:

- Same temperature condition could be baseline if requirements development/applicability condition is not changed.

Issue 2-2-3: Beam peak search for FR2

- Proposals

  - Proposal 1: For 6G FR2 UE RF OTA testing, consider revisiting the need for beam peak searches to reduce MU and overall test time.

Agreements:

  - Postpone above discussion until FR2 RF requirements have progress on beam peak search.

# Topic #3: OTA test methods for Multi-Tx and CA

### Sub-topic 3-1 OTA for 6GR multi-carrier

Issue 3-1-1: Testability for multi frequency range combinations, e.g., CA or single carrier DL/UL decouple

Agreements:

- Whether FR1 and FR2 can be UL/DL decouple operation depends on RAN1 design, the testability study can be postponed until progress is made.

Issue 3-1-2: Testability for multi-Tx schemes (such as coherent and non-coherent UL-MIMO)

Agreements:

- The definition depends on RAN1 physical layer design for MIMO. The testability for detailed feature can be postponed until more progress in RAN1 for multi-Tx.

  - OTA test methodology for 5G multi-Tx (e.g., single-layer UL-MIMO ) TRP can be considered as starting point for multi-Tx radiated performance metric study.

Issue 3-1-3: Testability for multi-Rx schemes

Agreements:

- Multi-Rx operation for TRS measurements without additional restriction was adopted in 5G OTA testing, the same approach can be considered for 6GR as baseline.

# Topic #4: Testability for different Device types

### Sub-topic 4-1 OTA testability for different device types

Issue 4-1-1: Study the limitation of single OTA system to cover different UE types in 6GR day-1

Agreements:

- RAN4 further discuss whether size of different device (i.e., quiet zone) is the only limitation to develop test system

- If no, then further study other aspects of different devices related to test setup discussions

- At least, the following aspects should be considered:

  - whether the device usage is free space or with phantom, if phantom is need then the turn table and quiet zone size should also be considered

  - Subject to RAN decision on the device types, whether the verification of the feature/device type needs significant update of the system, e.g., additional interference, multiple measurement antennas, higher dynamic range

Issue 4-1-2: Which UE types can be considered as baseline for 6G OTA system design

Agreements:

- Take 5G commercialized device types for which the test methodology has been specified, including handheld UE, as baseline for 6G OTA discussion.

- Other new devices can be further considered depends on RAN plenary decisions.

# Topic #5: AI/ML testability

### Sub-topic 5-1 Enhanced test method for 6GR AI/ML cases

Issue 5-1-3: Clarification of AI/ML testability discussion scope

Agreements:

The scope of 6G testability and OTA for AI includes the following objectives:

- Both OTA and conducted test and methodologies will be considered

  - Strive to use conducted test for AI testing if possible for FR1.

- the test methodology will be developed based on use cases, LCM and performance metric. This includes the proper channel model, test framework and corresponding test setup

- 5G related agreements will be taken as the starting point. Further enhancement of 5G methodologies for 5G use cases can be part of 6G study.

- For 6G new use cases, the testability and methodology study will be kicked off after the corresponding use cases and testability issues are identified in AI thread.

# Topic #6: Harmonized testing for TN and NTN

### Sub-topic 6-1 Harmonized OTA test methodologies for TN and NTN

Issue 6-1-1: Harmonized OTA test methodologies for TN and NTN for TN-FR1 range

Agreements:

- Consider the Anechoic chamber (AC) reference method as the baseline for the harmonized TN/NTN test framework specific to FR1

Issue 6-1-3: Supported polarization for Harmonized TN-NTN OTA test methodologies

Agreements:

- For a harmonized test setup for TN and NTN FR1 devices, LP is taken as the baseline. It can be further studied to support CP.

- The detailed device size will be further discussed. Whether a system can cover all NTN device types can also be studied.

Issue 6-1-4: System complexity study for Harmonized TN-NTN OTA test methodologies

Agreements:

- Due to different UE size and different supported feature for TN and NTN, the OTA system complexity should be further discussed considered all the potential metric to be verified.

# Topic #7: Improved test methods and metric for SISO OTA

### Sub-topic 7-1 Enhanced OTA test method and metric for SISO

Issue 7-1-1: Enhanced OTA test method for Tx switching

Agreements:

- 6GR OTA test method development should consider the UE operation condition more closely reflect real-world usage scenarios.

- RAN4 further study methodology to verify UL performance with Tx antenna switching on.

Issue 7-1-2: Enhanced OTA test method for different UE usage scenarios

Agreements:

- Different usage scenarios and corresponding mechanical mode will be considered when discss test methodology for 6G OTA.

- RAN4 further discuss whether OTA methodology to verify high-power related mechanisms.

# Topic #8: Improved test methods and metric for DL MIMO OTA

### Sub-topic 8-1 Dynamic MIMO OTA for 6GR

Issue 8-1-1: Dynamic or Static for 6G MIMO OTA

Agreements:

- RAN4 further discuss whether dynamic MIMO OTA could be considered as higher priority than static.

  - The performance metric of dynamic MIMO OTA should be further studied for 6G UEs.

Issue 8-1-2: Potential channel models for 6G dynamic MIMO OTA

Agreements:

- It is too early to make down-selection of 2D or 3D before discussing the potential channel model and corresponding system setup of probe configurations. RAN4 can further discuss this issue when the channel model in RAN1 is stable.

Issue 8-1-3: Phantom for dynamic MIMO OTA testing

Agreements:

- Whether phantom can be considered depends on the system design and potential channel models to achieve the required test zone.

- The phantom is not precluded before clear justification shows that the system could not support it or the necessity is not confirmed.

Issue 8-1-5: Priority of Dynamic MIMO OTA for 6G frequencies, i.e., FR1/FR2/new frequency

Agreements:

- 6G can start with FR1 dynamic MIMO OTA. Further consider other frequencies if needed.

# Efficiency improvement (UE/BS)

### Sub-topic 9-1 Improvement of OTA efficiency and requirements

Issue 9-1-1: Improve OTA test efficiency in 6GR

- Proposals

  - Proposal 1: RAN4 could consider the following alternatives for 6G test method consideration.

    - Single point offset approach

    - Coarse measurement grid

    - Spiral Scan procedure for TRP

    - Reverberation chamber

    - Fast Tx/Rx Spherical Coverage Method

Agreements:

  - RAN4 furtther discuss above methods for 6G to enhance efficiency.

### Sub-topic 9-2 Improvement of OTA test method and requirement discussions among WIs or WGs

Issue 9-2-1: Improve of discussion between OTA testability and features/core requirements

Agreements:

  - RAN4 futher discuss whether a framework on improvement of feature discussion and corresponding testability to minimize unnecessary delays and confusion is needed or not.

# Topic #10: General Testability issues (UE/BS RF,RRM and Demod)

### Sub-topic 10-1 General testability issues

Issue 10-1-2: 6G Testability scope discussion

Agreements:

The scope of the 6G study of the testability/OTA include

- Identifying and resloving the testability issues (e.g., test feasibility, limitations of testing, test methodologies development) for potential conducted requirements and OTA requirements of 6G UE/BS (except for sensing requirements).

- The scope includes both AI and non-AI cases.