3GPP TSG-WG4 Meeting #117	R4-2522343

# Dallas, US, 17 – 21 November, 2025

# Title:	TP for PRD 01 on guidelines for near-miss harmonic mixing

# Agenda Item:	5.2.6

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

At RAN4 meeting #114 a new set of guidelines on MSD requirements due to near-miss Rx harmonic mixing were presented in [1], and these guidelines were followed in correction CR [2,3,4] and finalized at this meeting in [5]. This TP for TR proposes to capture these guidelines in the RAN4 PRD. We also propose to clarify how to distinguish a direct-hit from a near-miss case for MSD due to harmonic or harmonic mixing interference.

# References

- R4-2501987 On near-miss for Rx harmonic mixing, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc., Anritsu, Qualcomm France, Oppo.

- R4-2511688 [NR_newRAT-Core] CR to TS38.101-1-gn0: Correction to harmonic mixing MSD – CatF, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, Skyworks Solutions, Inc., Anritsu, Qualcomm France

- R4-2511689 (NR_CADC_R17_2BDL_xBUL-Core) CR to TS38.101-1-hi0: Correction to harmonic mixing MSD – CatF, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, Skyworks Solutions, Inc., Anritsu, Qualcomm France.

- R4-2511690 (NR_CADC_R18_2BDL_xBUL-Core) CR to TS38.101-1-ia0: Correction to harmonic mixing MSD – CatF, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, Skyworks Solutions, Inc., Anritsu, Qualcomm France.

- R4-2520221 (NR_CADC_SUL_R19-Core) CR to TS38.101-1-j31: Correction and missing harmonic mixing MSD - Cat-F, 3GPP TSG-RAN WG4 Meeting #117, Dallas, US, Skyworks Solutions, Inc., Anritsu, Qualcomm France.

# Text Proposal

----- Start of TP -----

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Aggregated channel bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Carrier aggregation band: A set of one or more operating bands across which multiple carriers are aggregated with a specific set of technical requirements.

Carrier aggregation bandwidth class: A class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by a UE.

Carrier aggregation configuration: A combination of CA operating band(s) and CA bandwidth class(es) supported by a UE.

Contiguous carriers: A set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block.

Fallback group: Group of carrier aggregation bandwidth classes for which it is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group.

Harmonic direct-hit: Case where there is at least one individual RE within the uplink transmission bandwidth of the (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of an affected (higher) band.

Harmonic near-miss: Case where the 2nd uplink harmonic of the lower band is at ±10+BWChannelHB/2 MHz offset from the downlink affected (higher) band carrier frequency. Only applicable when direct-hit requirements do not apply and only applicable to UL2/DL1 (UL 2nd harmonic/DL fundamental) interference.

Harmonic mixing direct-hit: Case where the is at least one individual RE within the uplink transmission bandwidth of the band for which the fundamental or the 2nd / 3rd / 4th transmitter harmonic (also denoted UL1, UL2, UL3, UL4) is within the range m.FDL-BWDL/2 to m.FDL+BWDL/2 where “m” is the downlink affected band harmonic order and m=2,3,4,5 (also denoted “DL2, DL3, DL4, DL5”).

Harmonic mixing near-miss: Case where one of the odd order DL band LO harmonics overlaps with the ACLR1 of the UL band fundamental carrier, so called “UL1”. Only applicable when direct-hit requirements do not apply and only applicable to UL1/DL3 (3rd harmonic of DL band LO) and to UL1/DL5 (5th harmonic of DL band LO) interference.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

----- Unchanged sections are skipped -----

#### 5.5.2.1	Two-band Inter-band Downlink CA

#### 5.5.2.1.1	One Uplink Band

#### 5.5.2.1.1.1	One Uplink CC

----- Unchanged sections are skipped -----

#### Guidelines for MSD Test Points due to harmonic and harmonic mixing direct-hit interference:

#### These guidelines apply to both MSD due to UL harmonic interference and harmonic mixing interference except when the victim NR DL band is either band n46, band n96 or band n102.

Guidelines for the DL band configuration:

- For MSD due to harmonic and harmonic mixing direct-hit interference, a maximum of two test points can be specified:

- the first test point captures the smallest DL channel bandwidth (CBW) MSD,

- the second test point is optional and may capture the greatest DL channel bandwidth (CBW) MSD. For this second test point, other DL configurations are not precluded.

- The “smallest” and the “greatest” CBW shall not exceed the CBW specified in the BCS configuration tables of clause 5.5A.

Guidelines for the UL band configuration:

UL RB allocation:

- The parameter “LCRB” is specified as follows for SCS 15kHz / SCS 30kHz respectively,

- For the UL2/DL1x test point, LCRB = 12RB / 6RB,

- For the UL3/DL1x test point, LCRB = 8RB / 4RB,

- For the UL4/DL1x test point, LCRB = 6RB / 4RB,

- For the UL5/DL1x test point, LCRB = 5RB / 2RB,

- For the harmonic mixing UL1/DLx test point, LCRB = 25RB / 12RB,

where x=1 for MSD due to UL harmonics, and where the valid “x” for MSD due to harmonic mixing is defined in Table 1.

UL band CBW:

- The UL band is configured with the smallest UL CBW and the smallest SCS that can accommodate the specified UL LCRB,

- The “smallest” and the “greatest” CBW shall not exceed the CBW specified in the BCS configuration tables of clause 5.5A.

#### Guidelines for MSD Test Points due to harmonic near-miss interference.

- Applicability: Harmonic near-miss MSD cases apply only to UL2/DL1 harmonic interference and only when the band combination does not meet the direct-hit criteria.

- Guideline for near-miss harmonic MSD requirements:

  - Only one MSD requirement is specified for MSD due to harmonic near-miss interference,

  - The MSD is specified with:

    - UL band CBW (lower band): smallest mandatory supported CBW, e.g. 5MHz for FDD bands,

    - UL band LCRB: DFT-s-OFDM full allocation, e.g. 25RB for 5MHz CBW,

    - The Note 6 of Table 7.3A.4-1/ Note 6 of TS 38.101-3 Table 7.3B.2.3.1-1 is modified as follows:

NOTE 6: The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from  in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively.

#### Guidelines for MSD Test Points due to harmonic mixing near-miss interference.

- Applicability:

  - Harmonic mixing near-miss MSD requirements should remain exception requirements and should be specified only on a case-by-case basis to address a proponent’s specific concern,

  - Harmonic mixing near-miss MSD cases apply only to UL1/DL3 and UL1/DL5 interference and only when the band combination does not meet the direct-hit criteria.

- Guideline for near-miss MSD requirements:

  - Only one MSD requirement is specified for MSD due to harmonic mixing near-miss interference,

  - The MSD is specified:

    - UL band CBW (higher band): smallest CBW that ensures Rx harmonic mixing of the UL1 ACLR1 lower channel, denoted here “ACLR1_low” or harmonic mixing of the UL1 ACLR1 higher channel, denoted here “ACLR1_high”.

    - UL band LCRB: DFT-s-OFDM full allocation, e.g. 50RBs for 10MHz CBW

    - DL band CBW (lower band): smallest CBW.

  - The near-miss MSD requirements are verified with the following test conditions:

    - Case of near-miss harmonic mixing with UL1 ACLR1_low:

    - The requirements should be verified for the highest NR ARFCN of the affected DL (lower) band and for the lowest NR ARFCN of the UL (higher) band,

    - Case of near-miss harmonic mixing with UL1 ACLR1_high:

The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band,

----- End of TP -----