3GPP TSG-RAN5 Meeting #109	 R5-255992r1

Dallas, US, 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0668 | rev |  | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test applicability for less than 5M phase 2 test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | # NR_FR1_lessthan_5MHz_BW_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The applicability for less than 5 MHz phase 2 test cases has been missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added applicability statement for test case 6.6.1.14, 6.1.1.15, 6.5.3.17 and 6.6.9.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.0, 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: WIC correction required in the coversheet to resolve 3GU issue. |  |  |  |  |  |  |  |  |

----Start of Change 1----

## 4.0	Test case conditions and selection criteria

For the purposes of the present document, the applicability of conformance test cases conditions given in Table 4.0-1 apply. The tested bands selection criteria given in Table 4.0-2 apply. The tested CA/DC configuration selection criteria given in Table 4.0-3 apply. The subtest selection criteria given in Table 4.0-4 apply. The ICS proformas used in Table 4.0-1, Table 4.0-2, Table 4.0-3 and Table 4.0-4 are defined in TS 38.508-2 [8] unless otherwise stated.

Table 4.0-1: Applicability of conformance test cases conditions

| C001 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 THEN R ELSE N/A |  |
| --- | --- |
| C001a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7/3 THEN R ELSE N/A |  |
| C001b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A |  |
| C001c IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.1-2/2e OR A.4.3.1-2/12) THEN R ELSE N/A |  |
| C001d Void |  |
| C001e IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2A.1-1/1 THEN R ELSE N/A |  |
| C001f IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.5-1/1 AND (A.4.3.11-1/1 OR A.4.3.11-1/3) THEN R ELSE N/A |  |
| C001g IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.1-7/5 OR ((A.4.3.1-7/2 OR A.4.3.1-7/3) AND (A.4.3.2-1/84 OR A.4.3.2-1/84A))) THEN R ELSE N/A |  |
| C001h Void |  |
| C001i IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2A.1-1/2 THEN R ELSE N/A |  |
| C001j IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2A.1-1/3 THEN R ELSE N/A |  |
| C001k IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2A.1-1/4 THEN R ELSE N/A |  |
| … |  |
| C415 IF A.4.1-1/2 AND A.4.3.2-1/226 AND A.4.3.2-1/41 OR A.4.3.2-1/32 AND A.4.3.2-1/228 OR A.4.3.2-1/229 THEN R ELSE N/A |
| C416 IF A.4.1-1/1 AND A.4.3.1-7a/2 OR A.4.3.1-7a/1 AND A.4.3.2-1/161 THEN R ELSE N/A |
| C417 IF A.4.1-1/1 AND ([10]A.4.1-1/1 or [10]A.4.1-1/2) AND A.4.1-2/9 AND A.4.1-3/1 AND (A.4.3.2-1/91 OR A.4.3.2-1/92) AND A.4.4-1/17 THEN R ELSE N/A |
| C418 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-2/9 AND A.4.1-3/1 AND (A.4.3.2-1/91 OR A.4.3.2-1/92) AND A.4.4-1/17 THEN R ELSE N/A |
| Cxyz IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-2/9 AND A.4.1-3/1 AND (A.4.3.2-1/91 OR A.4.3.2-1/92) AND A.4.4-1/17 THEN R ELSE N/A |
| NOTE 1: Cxxxx applicability is defined for enhanced type 1 receiver for NR related tests (A.4.3.9-1/1). NOTE 2: Cxxxy applicability is defined for alternative additional DMRS position for co-existence with LTE CRS related tests (A.4.3.2-1/20). NOTE 3: Cxxxz applicability is defined for modified MPR behaviour related test (A.4.3.2-1/25). NOTE 4: Cxxxw applicability is defined for mpr Power Boost related test (A.4.3.2-1/38). |  |

----End of Change 1----

----Skipping unchanged Clauses----

## 4.2	RRM conformance test cases

----Start of Change 2----

Table 4.2-3: Applicability of RRM NR SA FR1 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch | Subtest Selection Criteria |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6.1 | RRC_IDLE state mobility |  |  |  |  |  |  |
| 6.1.1 | NR cell re-selection |  |  |  |  |  |  |
| 6.1.1.1 | NR SA FR1 cell re-selection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx 8Rx |  |
| 6.1.1.2 | NR SA FR1-FR1 cell re-selection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx 8Rx |  |
| 6.1.1.3 | NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx 8Rx |  |
| 6.1.1.4 | NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx 8Rx |  |
| 6.1.1.5 | NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx 8Rx |  |
| 6.1.1.6 | NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx 8Rx |  |
| … |  |  |  |  |  |  |  |
| 6.5.3 | Scell activation and deactivation delay |  |  |  |  |  |  |
| 6.5.3.1 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx 8Rx |  |
| 6.5.3.2 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx 8Rx |  |
| 6.5.3.3 | NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx 8Rx |  |
| 6.5.3.4 | NR SA FR1 direct SCell activation at SCell addition of known SCell | Rel-16 | C244 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and direct SCell activation |  | 2Rx 4Rx 8Rx |  |
| 6.5.3.5 | NR SA FR1 direct SCell activation at handover with known SCell | Rel-16 | C244 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and direct SCell activation |  | 2Rx 4Rx 8Rx |  |
| 6.5.3.10 | NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | Rel-17 | C270 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and fast SCell activation |  | 2Rx 4Rx 8Rx |  |
| 6.5.3.11 | NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle | Rel-17 | C270 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and fast SCell activation |  | 2Rx 4Rx 8Rx |  |
| 6.5.3.15 | NR SA FR1 TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX | Rel-18 | C244a | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and direct SCell activation based on TRS | NOTE 1 | 2Rx 4Rx 8Rx |  |
| 6.5.3.16 | NR SA FR1 Inter-band SSB-less SCell Activation based on A-TRS | Rel-18 | C270a | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and fast SCell activation based on A-TRS |  | 2Rx 4Rx 8Rx |  |
| 6.5.3.17 | NR SA FR1 SCell activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle with less than 5MHz BW | Rel-19 | C343 | UEs supporting 5GS NR SA FR1 and bandwidth of less than 5 MHz | NOTE 1 | 2Rx 4Rx 8Rx |  |
| 6.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |  |
| 6.5.4.1 | NR SA FR1 UE UL carrier RRC reconfiguration delay | Rel-15 | C002 | UEs supporting 5GS NR SA FR1 and SUL | Test execution not necessary if test 4.5.4.1 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.5.5 | Link recovery procedures |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |
| 6.6 | Measurement procedures |  |  |  |  |  |  |
| 6.6.1 | Intra-frequency measurements |  |  |  |  |  |  |
| 6.6.1.1 | NR SA FR1 event-triggered reporting without gap in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.6.1.1 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.6.1.2 | NR SA FR1 event-triggered reporting without gap in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if test 4.6.1.2 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.6.1.3 | NR SA FR1 event-triggered reporting with gap in non-DRX | Rel-15 | C041 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction | Test execution not necessary if test 4.6.1.3 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.6.1.4 | NR SA FR1 event-triggered reporting with gap in DRX | Rel-15 | C041a | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle | Test execution not necessary if test 4.6.1.4 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.6.1.5 | NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading | Rel-15 | C024 | UEs supporting 5GS NR FDD SA FR1 | Test execution not necessary if test 4.6.1.5 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.6.1.6 | NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading | Rel-15 | C041b | UEs supporting 5GS NR FDD SA FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction | Test execution not necessary if test 4.6.1.6 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.6.1.7 | NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Rel-16 | C052 | UEs supporting 5GS NR SA FR1 and measurement enhancements in HST | Test execution not necessary if test 4.6.1.7 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.6.1.8 | NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17 | Rel-17 | C052b | UEs supporting 5GS NR SA FR1 and CA measurement enhancements in HST | Test execution not necessary if test 4.6.1.8 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.6.1.9 | NR SA FR1 event triggered reporting with MUSIM gap configured | Rel-18 | C367 | UEs supporting 5GS NR SA FR1 and providing MUSIM assistance information with MUSIM gap preference and related MUSIM gap configuration |  | 2Rx 4Rx 8Rx |  |
| 6.6.1.12 | NR SA FR1 event triggered reporting with SSB index detection without measurement gap in non-DRX mode for UE operating with 12 PRB SSB bandwidth | Rel-18 | C343 | UEs supporting 5GS NR SA FR1 and bandwidth of less than 5 MHz |  | 2Rx 4Rx 8Rx |  |
| 6.6.1.13 | NR SA FR1 event triggered reporting tests without gap in DRX under Cell DTX | Rel-18 | C001w | UEs supporting 5GS NR SA FR1 and long DRX cycle and Cell DTX operation |  | 2Rx 4Rx 8Rx |  |
| 6.6.1.14 | NR SA FR1 Deactivated PSCell measurement test with 12 PRB SSB bandwidth | Rel-19 | C343 | UEs supporting 5GS NR SA FR1 and bandwidth of less than 5 MHz | NOTE 1 | 2Rx 4Rx 8Rx |  |
| 6.6.1.15 | NR SA FR1 Event triggered reporting test without gap under non-DRX with SSB index reading and 12 PRB SSB for a deactivated SCell | Rel-19 | C343 | UEs supporting 5GS NR SA FR1 and bandwidth of less than 5 MHz | NOTE 1 | 2Rx 4Rx 8Rx |  |
| … |  |  |  |  |  |  |  |
| 6.6.9 | Idle Mode CA/DC Measurements |  |  |  |  |  |  |
| 6.6.9.1 | NR SA FR1 Idle mode CA/DC measurement for FR1 | Rel-16 | C031a | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and NR SSB measurements in RRC_IDLE/RRC_INACTIVE |  | 2Rx 4Rx 8Rx |  |
| 6.6.9.2 | NR SA FR1 Idle mode fast CA/DC eEMR measurement | Rel-18 | C031e | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and NR SSB measurements in RRC_IDLE/RRC_INACTIVE and measurement validation and report based on EMR measurement during connection setup/resume for fast CA/DC setup |  | 2Rx 4Rx 8Rx |  |
| 6.6.9.3 | NR SA FR1 Idle mode fast CA/DC cell reselection measurement without valid reporting | Rel-18 | C031f | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and measurement validation based on reselection measurements during IDLE/INACTIVE state and reporting for fast CA/DC setup |  | 2Rx 4Rx 8Rx |  |
| 6.6.9.4 | NR SA FR1 Idle mode fast CA/DC cell reselection measurement with valid reporting | Rel-18 | C031f | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and measurement validation based on reselection measurements during IDLE/INACTIVE state and reporting for fast CA/DC setup |  | 2Rx 4Rx 8Rx |  |
| 6.6.9.5 | NR SA FR1 Idle mode CA/DC measurement of SCell with less than 5MHz BW | Rel-19 | Cxyz | UEs supporting 5GS NR SA FR1 and bandwidth of less than 5 MHz and NR SSB measurements in RRC_IDLE/RRC_INACTIVE | NOTE 1 | 2Rx 4Rx 8Rx |  |
| … |  |  |  |  |  |  |  |
| 6.7.17 | LTM L1-RSRP measurement |  |  |  |  |  |  |
| 6.7.17.1.1 | NR SA FR1-FR1 L1-RSRP absolute accuracy requirements for neighbour cell | Rel-18 | C360 | UEs supporting 5GS NR SA FR1, inter-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  | 2Rx 4Rx 8Rx |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Test X refers to the corresponding Sub-Test as defined in TS 38.533 [5]. NOTE 3: Test cases in TS 38.533 [5] clause 6 only apply to FR1 non-RedCap UEs. For FR1 RedCap UEs, Test cases in TS 38.533 [5] clause 16 apply. |  |  |  |  |  |  |  |

----End of Change 2----