3GPP TSG-SA5 Meeting #163	S5-254400

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.572 | CR | 0003 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 28.572 Correct multiple errors in stage 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PlanM |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 28.572 contains small errors and inconsistencies in stage 3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Small errorors are corrected and inconsistencies are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Small errors and inconsistencies jeopardize interoperability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1.1, A.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

Annex A (normative):
Solution sets

## A.1	RESTful HTTP-based solution set

### A.1.1	General Considerations

Provisions for mapping from stage 2 to stage 3

The naming of information elements in stage-3 generally is the same as in stage-2, except for the following cases:

A "changeId" may or may not be provided for each change by MnS consumers according to stage 2 definitions. The following snippet shows an example where the "changeId" is provided.

"configChanges": [ { "changeId": "change-1", "modifyOperator": "create", "target": "example.org/3gpp/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": {} } }, { "changeId": "change-2", "modifyOperator": "create, "target": "example.org/3gpp/SubNetwork=SN1/ManagedElement=ME20", "value": { "id": "ME20", "attributes": {} } } ]

Provisions for OpenAPI defined data node trees

The following provisions apply when the planned configuration is for a current configuration that is defined with OpenAPI. The "configChangesContentType" is "OpenAPI". It is based on 3GPP JSON Patch defined in clause 6.4.3 of TS 32.158 [2].

The "target" parameter value is the concatenation of the canonical form of the target URI defined in clause 4.2.3 of TS 32.158 [2] with "scheme://" omitted, and the "path" parameter of 3GPP JSON Patch defined in clause 6.4.3 of TS 32.158 [2], for example:

"target": "example.org/SubNetwork=SN1/ManagedElement=ME1" "target": "example.org/SubNetwork=SN1/ManagedElement=ME1#/attributes/userLabel" "target": "example.org/SubNetwork=SN1/PerfMetricJob=PMJ1#/attributes/perfMetrics/2"

For Managed Object Plans the "target" parameter identifies always a manged object. Therefore, the fragment component (i.e. the part starting with "#") is always missing.

The "value" parameter value is either a JSON object, a JSON array, a string, a number or one of the following three literal names: false, null, true. It is exactly the JSON value of the node identified by "target" without any additional wrappers or containers, for example:

"value": 5 "value": "Bts10" "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" }

For Managed Object Plans the value of the "value" parameter shall comply to the following JSON schema snippet.

type: object properties: id: type: string objectClass: type: string objectInstance: $ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn' attributes: type: object

The allowed values of the "modifyOperator" are as defined in stage 2.

Provisions for YANG defined data node trees

The following provisions apply when the planned configuration is for a current configuration that is defined with YANG. The "configChangesContentType" is "YANG". It is based on YANG Patch defined in RFC 8072 [3].

The "target" parameter value is constructed based on clause 2.4 of RFC 8072 [3] with the following modifications:

- The authority component (host identifier and optional TCP port) shall be added as the first component of the target. The authority component may be omitted, and the MnS producer shall accept it, if the MnS producer serves only one managed element with one authority.

- The datastore resource "{+restconf}/data" shall be omitted.

- The module prefix may be omitted, and the MnS producer shall accept it if the target without the module prefix is unambiguous.

The first example shows a target including the module prefix, and the second example shows the same target without module prefix. The third example shows the same target again without authority component.

"target": "example.org/3gpp-subnetwork:SubNetwork=SN1/3gpp-managed-element:ManagedElement=ME1" "target": "example.org/SubNetwork=SN1/ManagedElement=ME1" "target": "/SubNetwork=SN1/ManagedElement=ME1"

The "value" parameter value is encoded according to RFC 7951 [a]. The module prefix may also be omitted for the "value" parameter.

"value": { "count": 5 } "value": { "name": "Bts10" "value": { "ManagedElement": [ { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } ] }

The allowed values of the "modifyOperator" are as defined in stage 2.

Provisions for mapping the planned configuration in (3GPP) JSON Patch

In case the activation process requires to map the planned configuration into a 3GPP JSON Patch document the following provisions apply:

The "target" parameter value is assigned to the "path" property without modification.

The "value" parameter value is assigned to the "value" property without modification.

The "modifyOperator" parameter value is mapped to the "op" property according to the following rules:

- The modify operator "create" is strict, the JSON Patch operation "add" is relaxed. Therefore, an "add" operation is only included in the JSON Patch document, if the node to be created does not exist in the current configuration. Otherwise, an error is raised.

- The modify operator "delete" is relaxed, the JSON Patch operation "remove" is strict. Therefore, a "remove" operation is only generated in the JSON Patch document, if the target node exists in the current configuration, otherwise the "deleteMoi" operation in the plan is not included in the 3GPP JSON Patch document.

- The modify operator "merge" is strict, the 3GPP JSON Patch operation "merge" is relaxed. Therefore, the "merge" operation is included in the 3GPP JSON Patch document only, if the target node exists in the current configuration. Otherwise an error is raised.

- The modify operator "merge-create" is relaxed, the 3GPP JSON Patch operation "merge" is relaxed. Therefore, the "merge" operation is always included in the 3GPP JSON Patch document.

Table A.1.1-1 summarizes the property names used in "configChanges" and the corresponding names in 3GPP JSON Patch.

# Table A.1.1-1: Names in "configChanges" and 3GPP JSON Patch

| configChanges property name | 3GPP JSON Patch property name |
| --- | --- |
| target | path |
| modifyOperator | op |
| value | value |

Provisions for mapping the planned configuration into NETCONF

In case the activation process requires to map the planned configuration into NETCONF the following provisions apply:

The "target" parameter value is mapped into a series of nested XML elements with the innermost XML element being the target data node.

The "value" parameter value (in JSON encoding according to RFC 7951 [a]) is mapped to its corresponding XML representation.

The "modifyOperator" parameter value is mapped to the "operation" attribute according to the following rules:

- The modify operator "create" is strict, NETCONF "create" operation is strict. Therefore, the "create" operation is always included in the NETCONF request.

- The modify operator "delete" is relaxed, NETCONF "remove" operation is relaxed. Therefore, the "remove" operation is always included in the NETCONF request.

- The modify operator "merge" is strict, NETCONF "merge" operation is relaxed. Therefore, the "merge" operation is included in the NETCONF request only, if the target data node exists in the current configuration. Otherwise an error is raised.

- The modify operator "merge-create" is relaxed, NETCONF "merge" is relaxed. Therefore, the "merge" operation is always included in the NETCONF request.

### A.1.2	Resource structure

Table A.1.2-1 provides an overview of the resources on the MnS producer and applicable HTTP methods.

# Table A.1.2-1: Resources and methods

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
|  | …/plan-management/{/MnSVersion}/\ plan-descriptors/ | POST | Creates a new planned configuration descriptor |
|  |  | GET | Reads all plannd configuration descriptors |
|  | …/plan-management/{/MnSVersion}/\ plan-descriptors/{id} | GET | Reads one planned configuration descriptor |
|  |  | PUT | Replaces one planned configuration. descriptor. HTTP method is optional. |
|  |  | PATCH | Patches one planned configuration descriptor. HTTP method is optional. |
|  |  | DELETE | Deletes one planned configuration descriptor |
|  | …/plan-management/{/MnSVersion}/\ plan-group-descriptors | POST | Creates a new planned configuration group descriptor |
|  |  | GET | Reads all plannd configuration group descriptors |
|  | …/plan-management/{/MnSVersion}/\ plan-group-descriptors/{id} | GET | Read one planned configuration group descriptor |
|  |  | PUT | Replace one planned configuration group descriptor |
|  |  | DELETE | Delete one planned configuration group descriptor |
|  | …/{plan-management}/{/MnSVersion}/\ fallback-descriptors | GET | Read all fallback descriptors |
|  | …/{plan-management}/{/MnSVersion}/\ fallback-descriptors/{id} | GET | Read one fallback descriptor |
|  |  | DELETE | Delete one fallback descriptor |
|  | …/plan-management/{/MnSVersion}/\ trigger-descriptors | POST | Create a new trigger descriptor |
|  |  | GET | Read all trigger descriptors |
|  | …/plan-management/{/MnSVersion}/\ trigger-descriptors/{id} | GET | Read one trigger descriptors |
|  |  | PUT | Replace one trigger descriptors |
|  |  | DELETE | Delete one trigger descriptors |
|  | …/{plan-management}/{/MnSVersion}/\ plan-validation-jobs | POST | Create a new plan validation job |
|  |  | GET | Read all validation jobs |
|  | …/{plan-management}/{/MnSVersion}/\ plan-validation-jobs/{id} | GET | Read on validation job |
|  |  | DELETE | Delete one validation job |
|  | …/{plan-management}/{/MnSVersion}/\ plan-validation-jobs/{id}/validation-details | GET | Read validation details of one validation job |
|  | …/{plan-management}/{/MnSVersion}/\ plan-validation-jobs/{id}/cancel-request | PUT | Cancel one validation job |
|  | …/{plan-management}/{/MnSVersion}/\ plan-validation-jobs | POST | Create a new plan activation job |
|  |  | GET | Read all activation jobs |
|  | …/{plan-management}/{/MnSVersion}/\ plan-activation-jobs/{id} | GET | Read on activation job |
|  |  | DELETE | Deleete one activation job |
|  | …/{plan-management}/{/MnSVersion}/\ plan-activation-jobs/{id}/activation-details | GET | Read validation details of one activation job |
|  | …/{plan-management}/{/MnSVersion}/\ plan-activation-jobs/{id}/cancel-request | PUT | Cancel one activation job |

Creating, reading, updating and deleting planned configurations

The data node tree on the MnS producer is as follows upon system start up.

{ "plan-descriptors": {}, "plan-group-descriptors": {}, "fallback-config-descriptors": {}, "trigger-condition-descriptors": {}, "plan-validation-jobs": {}, "plan-activation-jobs": {} }

### A.1.3	OpenAPI definitions

OpenAPI definitions for the management of planned configurations are specified in Forge, refer to clause 4.3 of TS 28.623 [16] for the Forge location. An example of Forge location is: "https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel18_SA105/".

Directory: OpenAPI

Files:

TS28572_PlanManagement.yaml

### A.1.4	Examples (informative)

A new item of the collection resource "plan-descriptors" is created by MnS consumers using HTTP POST.

POST 3gpp/plan-management/v1/plan-descriptors HTTP/1.1 Host: exampleA.org Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "configChangesContentType": "OPENAPI_BASED", "activationMode": "ATOMIC", "configChanges": [ { "modifyOperator": "create", "target": "example.org/3gpp/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] }

The MnS producer allocates the identifier "p1" for the new resource and returns the response. The location header contains the URI of the new resource. The response body contains the representation of the new resource which is equal to the representation received in the request with the "lastModifiedAt" and "validationState" properties added.

HTTP/1.1 201 Created Date: Wed, 21 Aug 2024 15:39:57 GMT Location: http://exampleA.org/3gpp/plan-management/plan-descriptors/p1 Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "activationMode": "ATOMIC", "configChangesContentType": "OPENAPI_BASED", "configChanges": [ { "modifyOperator": "create", "target": "example.org/3gpp/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ], "lastModifiedAt": "2025-03-06T16:50:26-08:00", "validationState": "NOT_VALIDATED" }

The resource structure on the MnS producer contains the new resource.

{ "plan-descriptors": { "p1": { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "activationMode": "ATOMIC", "configChangesContentType": "OPENAPI_BASED", "configChanges": [ { "modifyOperator": "create", "path": "example.org/3gpp/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ], "lastModifiedAt": "2025-03-06T16:50:26-08:00", "validationState": "NOT_VALIDATED" } }, "plan-group-descriptors": {}, "fallback-config-descriptors": {}, "trigger-condition-descriptors": {}, "plan-validation-jobs": {}, "plan-activation-jobs": {} }

The next example shows how the value of the "location" attribute in the operation can be changed from "Castle Charlottenburg" to "Summer palace Charlottenburg" using JSON Patch.

PATCH 3gpp/plan-management/v1/plan-descriptors/p1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "replace", "path": "/configChanges/0/value/attributes/location", "value": "Summer palace Charlottenburg" } ]

The second example is for the case where a modified JSON Patch is used.

PATCH 3gpp/plan-management/v1/plan-descriptors/p1 HTTP/1.1 Host: example.org Content-Type: application/json [ { "op": "replace", "path": "/configChanges[changeId='pc1']/value/attributes/location", "value": "Summer palace Charlottenburg" } ]

To read the planned configuration "p1" a MnS consumer might send.

GET 3gpp/plan-management/v1/plan-descriptors/p1 HTTP/1.1 Host: exampleA.org Accept: application/json

The MnS producer may respond as follows.

HTTP/1.1 200 OK Date: Tue, 06 Aug 2025 16:50:26 GMT Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "configChangesContentType": "OPENAPI_BASED", "currentConfigAddress": " example.org/3gpp ", "activationMode": "ATOMIC", "lastModifiedAt": "2025-03-06T16:50:26-08:00", "validationState": "NOT_VALIDATED", "configChanges": [ { "modifyOperator": "CREATE", "path": "example.org/3gpp/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] }

To delete the planned configuration "p1" a MnS consumer might send.

DELETE 3gpp/plan-management/v1/plan-descriptors/p1 HTTP/1.1 Host: exampleA.org

In case of success, the MnS producer returns the following message.

HTTP/1.1 204 No Content Date: Wed, 21 Aug 2024 16:12:45 GMT

To read the planned configuration "p1" a MnS consumer might send.

GET 3gpp/plan-management/v1/plan-descriptors/p1 HTTP/1.1 Host: exampleA.org Accept: application/json

The MnS producer may respond as follows.

HTTP/1.1 200 OK Date: Tue, 06 Aug 2025 16:50:26 GMT Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "planConfigContentType": "OPENAPI_BASED", "currentConfigAddress": " example.org/3gpp ", "activationMode": "ATOMIC", "lastModifiedAt": "2025-03-06T16:50:26-08:00", "validationState": "NOT_VALIDATED", "configChanges": [ { "modifyOperator": "create", "path": "example.org/3gpp/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] }

The following shows a snippet reporting member conflicts.

"memberConflicts": [ { "memberConflict": [ { "planConfigDescrId": "pcd-1", "changeId": "change-4" }, { "planConfigDescrId": "pcd-3", "changeId": "change-6" }, { "planConfigDescrId": "pcd-4", "changeId": "change-7" } ] }, { "memberConflict": [ { "planConfigDescrId": "pcd-5", "changeId": "change-1" }, { "planConfigDescrId": "pcd-6", "changeId": "change-9" } ] } ]

The following shows a snippet reporting activation details.

"activationDetails": { "summary": { "notfinished": 1, "succeeded": 1, "failed": 1, "rollBackSucceeded": 0, "rollBackFailed": 0 }, "results": [ { "planConfigDescrId": "pcd1", "changeIndex": 0, "state": "SUCCEEDED" }, { "planConfigDescrId": "pcd1", "changeIndex": 1, "state": "FAILED", "errors": { "type": "REQUEST_OBJECT_TREE_MISMATCH", "title": "The current state of the data node tree prevents activation.", "reason": "NEW_OBJECTS_ID_EXISTS", "detail": "The id of the new object to be created does exist already." } }, { "planConfigDescrId": "pcd1", "changeIndex": 2, "state": "NOT_STARTED" } ] }

The next example shows how a planned configuration can be created for the case where the current configuration data node tree is specified with YANG.

POST 3gpp/ProvMnS/v1/plan-descriptors HTTP/1.1 Host: example.org Content-Type: application/json Accept : application/json,application/problem+json { "name" : "planxyz", "activationMode" : "BEST_EFFORT", # Execute in a best effort manner trying all operations "configurationContentType" : "application/3gpp-yang-patch+json" "planConfig": { " "edit" : [ { "operation": "create", "editId" : "opId-001", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/:GNBDUFunction=1", "value": { "NRCellDU": [ { "id": "1", "attributes": { "administrativeState" : "LOCKED", ... } } } }, { "operation": "merge", "editId" : "opId-002", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=2", "value": { "attributes": { "administrativeState": "LOCKED", } } }, { "operation": "remove", "editId" : "opId-003", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=3" } ] } } }

To get the planConfig :

GET 3gpp/ProvMnS/v1/plan-descriptors/p1/plan-config HTTP/1.1 Host: example.org Content-Type: application/json

To get a specific operation/edit in a planConfig :

GET 3gpp/ProvMnS/v1/plan-descriptors/p1/plan-config/{editId} HTTP/1.1 Host: example.org Content-Type: application/json

The next example shows how the planConfig edit / operation with editId=opId-001 can be modified to change the value of the "administrativeState" attribute from "LOCKED" to "UNLOCKED".  The 'editId' may be used to uniquely identify the specific configuration operation/edit to be modified in the planConfig.

PATCH 3gpp/ProvMnS/v1/plan-descriptors/p1/planConfig/opId-002 HTTP/1.1 Host: example.org Content-Type: application/json { "value": { "attributes": { "administrativeState": "LOCKED", } } }

To create a new edit entry to the end of the edits:

PATCH 3gpp/ProvMnS/v1/plan-descriptors/p1/planConfig HTTP/1.1 Host: example.org Content-Type: application/json { "operation": "merge", "editId" : "opId-004", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=4", "value": { "attributes": { "administrativeState": "UNLOCKED", } } }

To create a new edit entry before/after an existing edit use "before" or "after" parameter options:

PATCH 3gpp/ProvMnS/v1/plan-descriptors/p1/planConfig?after=opId-001 HTTP/1.1 Host: example.org Content-Type: application/json { "operation": "merge", "editId" : "opId-004", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=4", "value": { "attributes": { "administrativeState": "UNLOCKED", } } }

This section shows examples for plan activation job related resources. This example uses the 'inline' form of the plan descriptor:

# Create plan-activation-job POST {apiRoot}/ProvMnS/1900/plan-activation-jobs HTTP/1.1 Host: example.org Content-Type: application/json Accept : application/json,application/problem+json { "name" : "job-xyz", "description" : "optimize the Dublin area network", "isFallbackEnabled" : true, "planConfigDescrId" : "planxyz" } Response { "id": "myjob-111", "name" : "job-xyz", "description" : "optimize the Dublin area network", "isFallbackEnabled" : true, "planConfigDescrId" : "planxyz", "jobState" : "RUNNING", "startedAt" : "<some-time>", "activationState" : "NOT_STARTED", "activationDetails": { "href" : "{apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/activation-details" }, "cancelRequest" : false, "links" : { "self" : { "href" : "{apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111"}, "planDescriptor" : "{apiRoot}/ProvMnS/1900/…/planxyz"}, "status": { "href" : "{apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/status"}, "fallback": { "href" : "…"} } } # GET plan-activation-job status (job COMPLETE). Report all the relevant status information # related to the plan activation job GET {apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/status Response { "jobState": "COMPLETE", "activationState": "ACTIVATED", "startedAt": "2024-12-02T13:16:54.088Z", "stoppedAt": "2024-12-02T13:16:58.088Z" } # GET plan-activation-job activation-details # - jobState=COMPLETED / activationState=ACTIVATED # - All configuration edits/operations are successfully activated # - no detailed info on failed edits/operations are reported (none failed) GET {apiRoot}/ProvMnS /1900/plan-activation-jobs/myjob-111/activation-details Response { "summary" : { "unprocessed": 0, "succeeded": 3, "failed": 0 } } # GET plan-activation-job activation-details with verbose option (expand=all) # - jobState=COMPLETED / activationState=ACTIVATED # - All configuration edits/operations are successfully activated # - no detailed info on failed edits/operations are reported (none failed) GET {apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/activation-details?expand=all Response { "editStatus" : [ { "planDescriptorId" : "plan-desc-001", "edit" : [ { "editId" : "opId-001", "state" : "SUCCEEDED" }, { "editId" : "opId-002", "state" : "SUCCEEDED" }, { "editId" : "opId-003", "state" : "SUCCEEDED" } ] } ], "summary" : { "unprocessed": 0, "succeeded": 3, "failed": 0 } } # GET plan-activation-job result-details with complete configuration edit/operation failure # - jobState=COMPLETED / activationState=ACTIVATION_FAILED # - All configuration edits/operations failed to activate # - Activation details on failed edits/operations are reported GET {apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/activation-details Response { "editStatus" : [ { "planDescriptorId" : "planxyz", "edit" : [ { "editId" : "opId-001", "state" : "FAILED", "errors" : { "error" : [ { "type": "application", "reason": "data-exists", "path": "/_3gpp-common-subnetwork:SubNetwork=Irl/_3gpp-common-mecontext:MeContext=Dublin-1/_3gpp-common-managed-element:ManagedElement=Dublin-1/_3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1/_3gpp-nr-nrm-nrcelldu:NRCellDU=1", "message": "Data already exists; cannot be created" } ] } }, { "editId" : "opId-002", "state" : "FAILED", "errors" : { "error" : [ { "type": "application", "reason": "data-exists", "path": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=2", "message": "Data already exists; cannot be created" } ] } }, { "editId" : "opId-003", "state" : "FAILED", "errors" : { "error" : [ { "type": "application", "reason": "data-exists", "path": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=3", "message": "Data already exists; cannot be created" } ] } } ] } ], "summary" : { "unprocessed": 0, "succeeded": 0, "failed": 3 } } # GET plan-activation-job result-details with operation/edit failure (with expand=all option) # - jobState=COMPLETED / activationState=PARTIALLY_ACTIVATED # - Some configuration edits/operations failed to activate # - Activation details on failed edits/operations are reported # - Activation details on successful edits/operations are reported (expand=all) GET {apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/activation-details?expand=all Response { "editStatus" : [ { "planDescriptorId" : "planxyz", "edit" : [ { "editId" : "opId-001", "state" : "FAILED", "errors" : { "error" : [ { "type": "application", "reason": "data-exists", "path": "/_3gpp-common-subnetwork:SubNetwork=Irl/_3gpp-common-mecontext:MeContext=Dublin-1/_3gpp-common-managed-element:ManagedElement=Dublin-1/3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1/_3gpp-nr-nrm-nrcelldu:NRCellDU=1", "message": "Data already exists; cannot be created" } ] } }, { "editId" : "opId-002", "state" : "SUCCEEDED" }, { "editId" : "opId-003", "state" : "SUCCEEDED" } ] } ], "summary" : { "unprocessed": 0, "succeeded": 2, "failed": 1 } }

Annex B (informative):
PlantUML for figures

@startuml

left to right direction

object Group_A

object Group_B

object Group_C

Group_A ---o Group_B : is_member_of

Group_B ---o Group_C : is_member_of

Group_C ---o Group_A : is_member_of

@enduml

Figure 6.2.2-1: NOT allowed circular planned configuration group membership relation (with 3 groups)

@startuml

hide empty members

object Group_A

object Group_B

object Group_C

object Plan1

object "Plan1 "

Object Plan2

Object "Plan2 "

Group_A -up-o Group_B : is_member_of

Group_B -down-o Group_C : is_member_of

Plan1 -up-o Group_A : is_member_of

Plan2 -up-o Group_A : is_member_of

"Plan2 " -up-o Group_A : is_member_of

"Plan1 " -up-o Group_C : is_member_of

@enduml

Figure 6.2.2-2: NOT allowed multiple containment of planned configuration

End of modifications